standard input (SI)

State of New Jersey

Department of Transportation

Special Provisions

For

Locally Administered Federal Aid Projects

FY 2026 Edition

September 2026

SPECIAL PROVISIONS

GENERAL

All awards shall be made subject to the approval of the New Jersey Department of Transportation.  No construction shall start before approval of said award by the New Jersey Department of Transportation and execution of the Federal Project Agreement between the NJDOT and the LPA. Prior to the start of construction, the contractor must submit the following Material Questionnaires, DC-2891 and SA-11, listing all sources of materials.  Any materials used on the project from a non-approved New Jersey Department of Transportation source will be considered non-participating.   The LPA must notify the District Office, Division of Local Aid and Economic Development of the construction commencement date at least five (5) calendar days prior to the proposed start of construction and the actual construction start date once it is known.

Award of contract and subletting will not be permitted to, material will not be permitted from, and use of equipment will not be permitted that is owned and/or operated by, firms and individuals included in the report of suspensions, debarments and disqualifications of firms and individuals as maintained by the Department of the Treasury, Division of Purchas & Property, Award of Contract Compliance & Administration, Trenton NJ  08625 (609-292-5400).

https://www.nj.gov/labor/wageandhour/registration-permits/register/debarmentlist.shtml

And the Federal Government’s System for Award Management (SAM), located at  http://www.sam.gov/SAM/ for federally assisted contracts.

Payment for a pay item in the proposal includes all the compensation that will be made for the work of that item as described in the contract documents unless the "measurement and payment" clause provides that certain work essential to that item will be paid for under another pay item.

Whenever any section, subsection, subpart or subheading is amended by such terms as changed to, deleted or added it is construed to mean that it amends that section, subsection, subpart or subheading of the 2019 Standard Specifications unless otherwise noted.

Whenever reference to page number is made, it is construed to refer to the 2019 Standard Specifications unless otherwise noted.

Henceforth in these special provisions whenever reference to the State, Department, Regional Construction Engineer, ME, or RE is made, it is construed to mean the particular municipality or county, or their representative, executing this contract unless otherwise noted.

Whenever reference to Title 27 is made, it is construed to mean Title 40.

PROJECT NAME

PROJECT LOCATION

LPA

| Revisions to 2019 SI: |  |  |
| --- | --- | --- |
| date | BDC | Description / reason |
| SEP 3, 2026 |  | undo FEDERAL AID ATTACHMENT 3 revision below |
| AUG 6, 2026 | - | Revision to Federal Aid Attachment 3 |
| JUL 20, 2026 | BDC26S-06 | REVISION TO SUBPART 655.03.01 (SI2019) |
| june 10, 2026 | BDC26S-07 | REVISIONs TO SUBPART 421.03.03 j |
| MAY 29, 2026 | BDC26S-02 | REVISION TO SUBPART 401.03.07.J AND NJDOT TEST METHOD R-1 |
| may 14, 2026 | bdc26s-05 | revision to subsection 108.09 |
| may 08, 2026 | bdc25s-22 | revision to subpart 701.02.01 and addition of subsection 918.15 |
| apr 24, 2026 | bdc26s-03 | revisions to Subpart 105.07.02 |
| apr 06, 2026 | bdc25s-24 | Revisions to Section 65X (SI2019) |
| Mar 24, 2026 | BDC25S-21 | Revisions to subsections 104.02 and 105.05 and subpart 153.03.01, and Subpart 104.02.01 (SI2019) |
| MAR 9, 2026 | BDC26S-01 | Revisions to Subsection 159.04 |
| MAR 2, 2026 | bdc25s-25 | Revisions to Subsections 901.01 and 901.11 |
| Feb 19, 2026 | bdc25s-20 | revision to Subpart 108.05.02 Part 3 |
| Feb 12, 2026 | bdc25s-18 | revision to Subsection 202.02 and Subpart 202.03.02 |
| jan 8, 2026 | bdc25s-19 | revision to subsection 108.02 |
| oct 22, 2025 | bdc25s-17 | revision to subpart 903.06.01 |
| sep 30, 2025 | bdc25s-16 | revisions to Subsection 406.01, subpart 406.02.01, and subsection 406.04, and the addition of subpart 406.03.02 and subsection 902.16 |
| aug 19, 2025 | bdc25s-13 | revisions to subsection 106.04 and subparts 701.02.01, 702.02.01, and 703.02.01 |
| jul 25, 2025 | bdc25s-14 | revisions to subpart 401.03.07 |
| jul 11, 2025 | bdc25s-11 | revisions to subpart 107.11.02 |
| jun 26, 2025 | bdc25s-12 | removal of federal aid attachment 9 |
| Jun 18, 2025 | BDC25S-09 | Revision to Subsection 106.03 |
| jun 5, 2025 | bdc25s-04 | revision to subpart 108.07.01 |
| MAY 15, 2025 | BDC25s-10 | REVISIONS TO SUBPART 102.13.01 AND SUBSECTIONS 102.15 AND 109.09 |
| may 1, 2025 | bdc25s-08 | revision to subsection 154.04 |
| Apr 28, 2025 | BDC25S-07 | Revision to Subsections 108.01, 701.01, and 701.04, and addition to subpart 701.03.16. |
| apr 24, 2025 | bdc25s-03 | revision to subpart 160.03.01 and 160.03.02 |
| Apr 15, 2025 | BDC25S-06 | Revision to Subpart 155.03.01 (SI2019) and subpart 401.03.05 |
| Apr 3, 2025 | BDC25S-01 | Revision to Subpart 107.11.01 and Subsection 159.04. |
| mar 27, 2025 | bdc25s-02 | revision to subparts 406.03.01, 902.08.02, and 902.08.03 |
| Mar 3, 2025 | BDC24S-26 | REvision to Subpart 902.01.04 |
| Feb 11, 2025 | BDC24S-23 | Revision to Subpart 104.03.08 |
| Feb 10, 2025 | BDC24S-22 | Revision to Subsection 106.07 |
| jAN 31, 2025 | bdc24s-24 | rEVISION TO sUBSECTION 106.03 |
| Jan 28, 2025 | BDC24S-20 | Revision to Subpart 105.02.05, State Funded Project Attachment 5, and Federal Aid Project Attachment 7 |
| Jan 16, 2025 | BDC24S-21 | Revision to Subpart 610.03.02 |
| Jan 10, 2025 | BDC24S-25 | Revision to Federal Aid Project Attachment 1 |
| nov 21, 2024 |  | revision to subsection 101.04 |
| nov 07, 2024 | bdc24s-14 | revision to subpart 160.03.03 |
| oct 28, 2024 Sep 16, 2024 | bdc24s-19 BDC24S-18 | revision to subparts 105.02.05.1.a and 105.02.05.2.a Revision to Subparts 104.03.01 and 104.03.02 |
| sep 3, 2024 | bdc24s-17 | revision to subpart 160.03.01 and subsection 160.04 |
| AUG 22, 2024 | BDC24S-16 | REVISION TO SUBPART 504.03.01. G |
| AUG 20, 2024 | BDC24S-15 | Revision to Subpart 159.03.06 and Subsection 159.04 |
| Jul 30, 2024 | BDC24S-12 | Revision to Section 902 |
| Jul 10, 2024 | can 096 | removal of bdc23s-15, shear test requirement |
| JUL 9, 2024 | BDC24S-11 | REVISION TO SUBPART 104.03.07 |
| jul 1, 2024 JUN 25, 2024 JUN 11, 2024 JUN 06, 2024 MAY 28, 2024 MAY 22, 2024 | bdc24s-10 BDC24S-08 BDC24S-07 BDC24S-04 BDC24S-06 BDC24S-05 | revision to subpart 104.03.09, and subsections 109.03, 109.04, & 109.09 REVISION TO SUBSECTION 101.03 REVISION TO SUBSECTION 108.14 REVISION TO SUBPARTS 102.13.01.C, 105.02.05.1.A AND 105.02.05.2.A REVISION TO SUBPARTS 404.03.01, 902.05.01 AND 902.05.03 AND SUBSECTIONS 1009.01 AND 1009.03 REVISION TO SUBPART 104.03.09.1. E |
| apr 18, 2024 | bdc24s-03 | revision to subparts 104.03.03.1 and 104.03.03.3, and subsection 105.11 |
| apr 4, 2024 | bdc24s-02 | revision to subparts 503.03.03, 503.03.04, and 503.03.05 |
| Mar 27, 2024 | BDC24S-01 | REVISION TO Subpart 602.03.03 |
| mar 19, 2024 | bdc23s-22 | revision to subsection 156.03, subparts 156.03.01 and 156.03.03, and subsection 156.04 |
| Mar 5, 2024 | BDC23S-20 | Revision to Subsection 109.05 |
| feb 19, 2024 | bdc23s-21 | revision to subsection 106.03 |
| DEC 22, 2023 | BDC23S-14 | Revision to subpart 609.03.01 |
| Dec 18, 2023 | BDC23S-16 | Revision to Subpart 105.07.01 |
| Nov 24, 2023 | BDC23S-19 | Revision to Federal Aid Attachment 7 |
| Nov 22, 2023 | bdc23s-15 | revision to subparts 401.03.07 and 401.03.08 and addition of test method njdot b-14 |
| Oct 23, 2023 | BDC23S-13 | Revision to Subpart 102.13.01 and Subsection 103.01 |
| OCT 13, 2023 | BDC23S-18 | Revision to Federail Aid Attachment 8 – FHWA-1273 |
| OCT 6, 2023 | BDC23S-12 | REVISION TO TABLE 902.09.03-1 AND TABLE 902.10.03-1 |
| Sep 8, 2023 | BDC22S-20 | Revision to Subpart 106.07.01 |
| aug 17, 2023 | bdc23s-11 | revision to subpart 104.03.08, section 105, subsections 108.02 and 151.04, and section 157. subpart 157.03.01 moved, revised, and renamed to subsection 105.11 |
| Jul 20, 2023 | BDC23S-03 | Revisision to Subpart 407.03.01 |
| jul 17, 2023 | bdc22s-16 | revision to subsection 612.02, subpart 612.03.01, subsection 612.04, table 911.01.01-2, and subparts 911.01.02 and 911.02.01 |
| Jun 6, 2023 | bdc23s-08 | revision to subsections 607.01 and 607.04, subparts 607.02.01 and 607.02.02, and addition of subpart 607.03.08 and subsection 1008.07 |
| Jun 5, 2023 | BDC23S-10 | Revision of Subsection 102.10 and Subpart 102.13.01 |
| May 19, 2023 | bdc23s-09 | deletion of section 51x |
| May 12, 2023 | BDC23S-07 | Revision to Subsections 102.10 and 102.15 |
| Apr 14, 2023 | BDC23S-04 | Revision to Subpart 104.03.08 |
| MAr 31, 2023 | BDC23S-05 | Revision to Subsection 109.06 |
| MAr 30, 2023 | bdc23s-06 | revision to subparts 155.03.01, 155.03.02, 156.03.01, 156.03.02, & 156.03.03 |
| Mar 24, 2023 | BDC22S-21 | Revision to Section 422 and Subsection 610.04 |
| Mar 14, 2023 | BDC23S-01 | Addition of new subpart 160.03.03 |
| MAR 13, 2023 | BDC23S-02 | Revision to Sections 159, 702, 911, and 1001 |
| Jan 30, 2023 | BDC22S13R | Revision to Subsection 102.10 |
| Jan 17, 2023 | BDC22S-09 | Revision to Section 160 |
| JAn 13, 2023 | BDC22S-08 | Revision to Subsections 915.04 and 915.05 |
| Jan 06, 2023 | BDC21S-05 | Revision to Subpart 507.02.01 and subsection 507.04, & new Subpart 507.03.08 |
| DEC 28, 2022 | BDC22S-19 | Revision to Subsection 109.06 |
| DEC 23, 2022 | BDC22S-17 | Revision to Subsection 106.01 (SI2019) |
| Dec 21, 2022 | BDC22S-13 | Revision to Subsection 102.10 |
| Dec 16, 2022 | BDC22S-14 | Revision to Subsection 106.03 and Subpart 106.07.02 |
| Nov 18, 2022 | BDC22S-18 | Revision to Subsections 102.01, 102.10, 102.15, and 103.04, & Subpart 102.13.01 |
| Oct 14, 2022 | BDC22S-11 | Addition of new subpart 160.03.03 (Superseded) |
| Oct 05, 2022 | BDC22S-12 | Revision to Subpart 911.01.01 |
| Sep 12, 2022 |  | Revision to Subpart 156.03.01 part 7. A.4. |
| aUG 29, 2022 | bdc22s-15 | rEVISION TO fhwa-1273 fEDERAL aID pROJECT aTTACHMENT 8 (si2019) |
| Aug 16, 2022 | BDC21S-12 | Revision to Subsections 601.02 and 601.04, & Subparts 601.03.01, 601.03.03, 601.03.04, 601.03.06, 601.03.07 and 909.02.02. |
| Aug 03, 2022 | BDC22S-10 | Revision to Subpart 108.07.01 |
| Aug 01, 2022 | BDC22S-07 | Revision to Subpart 159.03.02 |
| JuL 13, 2022 | BDC22S-06 | Revision to Subparts 159.03.03, 159.03.05, and 912.04.02 |
| JUN 22, 2022 | BDC22S-03 | Revision to Subpart 401.03.03 and Subsection 401.04 |
| JUN 02, 2022 | BDC21S-10 | Revision to Subpart 611.03.02 |
| May 27, 2022 | BDC22S-05 | Revision to Subparts 201.03.01 and 201.03.02 |
| May 13, 2022 | BDC22S-04 | Revision to Subparts 159.02.01 and 401.02.01, & Section 902 and Subsection 65X.02 (SI2019) |
| May 12, 2022 | BDC21S-20 | Revision to Subpart 701.03.15 and Subpart 701.03.15 (SI2019) |
| APR 22, 2022 | BDC22S-01 | Revision to Subpart 159.03.02 and Subsection 1001.03 |
| Apr 21, 2022 |  | Revision to Subsection 108.01 (2)c and Subpart 151.03.01 |
| MAR 21, 2022 | BDC21S-13 | Revision to Subsection 913.04 |
| Mar 04, 2022 | BDC21S-16 | Revision to State Funded Project Attachment 1, 2 & 3 (SI2019) |
| Jan 21, 2022 | BDC21S-15 | Revision to Subpart 610.03.07 |
| Jan 20, 2022 | BDC21S-19 | Revision to Subpart 401.03.01 and Subpart 401.03.01 (SI2019) |
| DEC 30 2021 | BDC21S-14 | Revision to Subsection 902.05 |
| DEC 10 2021 | BDC21S-17 | Revision to Subpart 159.03.08 |
| Nov 18 2021 | BDC21S-08 | Revision to Subsection 105.05 & Subparts 507.03.01 and 914.04.03 |
| Nov 08 2021 | BDC21S-18 | Revision to Subpart 157.03.02 |
| Aug 23 2021 | BDC21S-04 | Revision to Subsection 106.01 |
| Jun 11 2021 | BDC21S-06 | Revision to Test Method R-1 |
| MAY 25 2021 | BDC21S-07 | Revision to Subparts 505.03.01, 505.03.02, 506.03.01, 507.03.02 and 514.03.01 |
| MAY 14 2021 | BDC21S-09 | Revision to Subsection 102.10 and Subpart 102.13.01 |
| MAR 24 2021 | BDC21S-02 | Revision to Subsection 903.01 |
| MAR 08 2021 | BDC21S-01 | Revision to Subsection 109.01 |
| Jan 15 2021 | BDC20S-15 | Revision to Subpart 159.03.02 |
| OCT 09 2020 | BDC20S-14 | Revision to Subsections 106.01 and 106.03, and change to the heading of Section 106 & Subsection 106.03 |
| OCT 02 2020 | BDC20S-11 | Revision to Subsection 610.04 and addition of Subparts 610.03.09 |
| Sep 21 2020 | BDC20s-13 | Revision to Subsection 102.10 |
| Sep 11 2020 | BDC20S-10 | Revision to Sections 504, 507, 513, 609, and 912, and addition of Subparts 504.03.04 and 513.03.03 |
| Jul 17 2020 | BDC20S-03 | Revision to Subsection 108.08 |
| Jul 07 2020 |  | Revision to Subsection 703.03 |
| Jul 06 2020 | BDC20S-09 | Revision to Subparts 401.03.07, 405.03.02, 421.03.03, 422.03.01, and 1009.01 |
| JUN 12 2020 | BDC20S-08 | Revision to Subparts 401.03.07, 610.03.03, and 610.03.07 |
| jun 05 2020 | bdc20s-05 | Revision to Subsection 105.09 (SI2019) and 1003.01 |
| May 29 2020 | BDC20S-07 | Revision to Subsection 109.05 |
| APR 30 2020 | BDC19S-10 | Revision to Subpart 159.03.02 and Subsection 1001.03 |
| MAR 20 2020 | BDC20S-02 | Revision to Subpart 202.03.01 |
| MAR 13 2020 | BDC20S-01 | Revision to Subsection 159.04 |
| Mar 06 2020 |  | Revision to Subpart 155.03.01 (6) (SI2019) |
| FEB 28 2020 | BDC19S-07 | Revision to Subparts 159.03.02 and 159.03.08 |
| Jan 14 2020 | BDC19S-09 | Revision to Subsection 919.05 |
| Dec 30 2019 |  | Revision to Subpart 107.11.01 (SI2019) |
| Dec 11 2019 | BDC19S-08 | Revision to Subpart 159.03.01 |
| Oct 30 2019 |  | Revision to Subpart 107.12.01 |
| oct 15 2019 | BDC19S-06 | Revision to Subpart 155.03.01 (SI2019) |
| Sep 30 2019 | BDC19S-05 | Revision to Subparts 160.03.01 and 160.03.02 |
| Sep 03 2019 | BDC19S-03 | Revision to Subsections 101.03 and 108.20 |
| Aug 26 2019 | BDC19S-04 | Revision to Subpart 104.02.01 |
| May 28 2019 | BDC19S-01 | Release of the 2019 Standard Specifications for Road and Bridge Construction and Standard Inputs (SI2019) |

THE FOLLOWING SECTIONS HAVE BEEN REVISED FOR LOCAL AID PROJECTS:

SECTIONS

101, 102, 103, 104, 105, 106, 107, 108, 109, 151, 152, 153, 159, 304, 305, 401, 403, 405, 421, 454, 701, 704, 902, 915.

Please note: the following sections of the 2019 Specifications have been added to these special provisions at the request of the bureau of civil rights and must be included in this document

subsection 102.10-submission of bids

subpart 102.13.01(A)–bidder pre-award requirements

subsection 102.15–disqualification of bidders

subpart 104.03.01-authority to make changes

subpart 105.02.05 (1)-civil rights requirements-federal aid projects

subsection 107.02-nondiscrimination

subsection 107.03-affirmative action, disadvantaged business enterprises or emerging small business enterprises, and small business enterprises

subsection 108.01-subcontracting

subsection 108.14-default and termination of contractor’s right to proceed

subsection 109.05 estimates

SPECIAL PROVISIONS

1**************************************************************************************************************************1

include PROJECT DESCRIPTION AS SHOWN ON THE KEY sheet AND FEDERAL PROJECT NUMBER (CONSTRUCTION) WHEN APPLICABLE.

1**************************************************************************************************************************1

AUTHORIZATION OF CONTRACT

1**************************************************************************************************************************1

INclude THE FOLLOWING FOR fhwa funded projects

The Contract is authorized by the provisions of Title 40 of the Revised Statutes of New Jersey and supplements thereto, and Title 23 of the United States Code - Highways.

1**************************************************************************************************************************1

SPECIFICATIONS TO BE USED

The 2019 Standard Specifications for Road and Bridge Construction, of the New Jersey Department of Transportation (Department) as amended herein will govern the construction of this Project and the execution of the Contract.

These Special Provisions consist of the following:

1**************************************************************************************************************************1

complete and include the following

Pages 1 to ___ inclusive.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

INclude THE FOLLOWING FOR fhwa funded projects

General wage determinations issued under Davis-Bacon and related acts, published by US Department of Labor, may be obtained from the Wage Determinations online website at sam.gov.  Select state, county and construction type heading: HIGHWAY where the Project is to be performed then click Search.

Pay the prevailing wage rates determined by the United States Secretary of Labor and the New Jersey Department of Labor and Workforce Development.  If the prevailing wage rate prescribed for any craft by the United States Secretary of Labor is not the same as the prevailing wage rate prescribed for that craft by the New Jersey Department of Labor and Workforce Development, pay the higher rate.

State wage rates may be obtained from the New Jersey Department of Labor & Workforce Development 
(Telephone: 609-292-2259) or by accessing the Department of Labor & Workforce Development’s website at https://www.nj.gov/labor/wagehour/wagerate/prevailing_wage_determinations.html.  The State wage rates in effect at the time of award are part of this Contract, pursuant to Chapter 150, Laws of 1963 (N.J.S.A. 34:11-56.25 et seq.)

If an employee of the Contractor or subcontractor has been paid a rate of wages less than the prevailing wage, the Department may suspend the Work and declare the Contractor in default.

The NJDOT must report all suspected or reported violations to the federal agency providing the funding for the project.

Contractor’s compliance is required with the Copeland “Anti-Kickback” Act, (40 U.S.C. 3145), as supplemented by the Department of Labor regulations (29 CFR Part 3, “Contractors and subcontractors on Public Building or Public Work Financed in Whole or In Part by Loans or Grants from the United States”).  Each contractor or subcontractor is prohibited from inducing by any means, any person employed in the construction, completion, or repair of public work, to give up any part of the compensation to which he or she is otherwise entitled.  The NJDOT must report all suspected or reported violations to the federal agency providing the funding for the project.

The following information is located at the end of these Special Provisions:

2**************************************************************************************2

1.	Disadvantaged Business Enterprise Utilization. (Federal Aid Project Attachment 1)

OR

1.	Emerging Small Business Enterprise Utilization. (Federal Aid Project Attachment 1)

The DBE/ESBE goals request form and classification codes worksheet are available at

Forms and Applications, Local Aid & Economic Development, Doing Business

S101-LA DBE-ESBE Goal

Class Code Worksheet (SI03)

THE LPA/lpa CONSULTANT WILL email the completed form and worksheet along with a copy of the engineer’s estimate to the local aid project manager (LAPM) for submittal to NJDOT construction services.

the Local aid project manager will inform the LPA of the DBE/ESBE goal this goal IS TO be inserted into the appropriate attachment 1 located at the end of these special provisions.

delete the inappropriate fhwa ATTACHMENT.

2**************************************************************************************2

2.	Specific Equal Employment Opportunity Responsibilities on NJDOT Federal Aid Projects. (Federal Aid Project Attachment 2)

Follow instructions and fill out the training request form at

Forms and Applications, Local Aid & Economic Development, Doing Business

SI04-LA trainee goal

to be completed by lpa or lpa consultant

the lpa/lpa consultant will email the completed form along with an engineer’s estimate and construction bar chart to the local aid project manager for submittal to NJDOT Civil Rights.  The local aid project manager will inform the LPA of the trainee goal.  the lpa will insert the goal into federal attachment 2 (training special provisions) located at the end of these special provisions.

3.	Requirements for Affirmative Action to Ensure Equal Employment Opportunity on NJDOT Federal Aid Projects. (Federal Aid Project Attachment 3)

4.	Federal Equal Employment Opportunity Contract Specifications for NJDOT Federal Aid Projects. (Federal Aid Project Attachment 4)

5.	State of New Jersey Mandatory Equal Employment Opportunity Language on NJDOT Federal Aid Projects. (Federal Aid Project Attachment 5)

6.	Investigating, Reporting and Resolving Employment Discrimination and Sexual Harassment Complaints on NJDOT Federal Aid Projects. (Federal Aid Project Attachment 6)

7.	Payroll Requirements for NJDOT Federal Aid Projects. (Federal Aid Project Attachment 7)

8.	FHWA-1273 Required Contract Provisions, Federal Aid Construction Contracts. (Federal Aid Project Attachment 8)

9.	State Mandatory Addendum to FHWA 1273 Required Contract Provision, Federal Aid Construction Contracts as Amended or Supplemented. (Federal Aid Project Attachment 9)

10.	Federal Mandatory Equal Opportunity Language on Federal Aid Projects. (Federal Aid Project Attachment 10)

11.	Byrd Anti-Lobbying Certification. (Federal Aid Project Attachment 11)

list here & include additional project specific ATTACHMENTS required for the project at the end of these Special Provisions, SUCH AS ASBESTOS ABATEMENT.

The following additional project specific Attachments are located at the end of these Special Provisions:

12.

1**************************************************************************************************************************1

Division 100 – General Provisions

Section 101 – General Information

101.01 INTRODUCTION

1**************************************************************************************************************************1

The Federal FY begins October 1 of the previous calendar year and the State FY begins July 1 of the previous calendar each year.

1**************************************************************************************************************************1

101.03 TERMS

1**************************************************************************************************************************1

BDC19S-03 dated SEp 03, 2019

THE FOLLOWING TERM IS ADDED:

Full Traffic Access.  All work is complete to allow safe unencumbered use of the final paved portion of roadway throughout the project including but not limited to striping, RPMs, rumble strips, highway lighting, and traffic signals as determined by the RE.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC24S-08 DATED JUN 25,2024

Completion.  When all of the following have occurred:

PART (1) is changed to:

- The Work has been satisfactorily completed in all respects according to the Contract including landscaping Items listed in 811.04, removal of SESC measures, FINAL CLEANUP, and repair of unacceptable work.

1**************************************************************************************************************************1

101.04 Inquiries Regarding the Project

FOR LOCAL AID PROJECTS THIS SECTION IS DELETED AND REPLACED WITH THE FOLLOWING:

Submit inquiries regarding discrepancies, errors, omissions, or concerns regarding the intent or meaning of the Contract to the LPA as follows:

- Before Award of Contract.

INSERT CONTACT INFORMATION FOR THE LPA

Responsible charge or LPA representative

Telephone, email, mailing address

Include the name of the bidder and contact information.  Specify the nature of the inquiry, including anticipated results.

INSERT THE DEADLINE FOR INQUIRES

The LPA will investigate the information provided in the inquiry and if it is determined that a change or response is necessary, the LPA will issue an addendum.

Requests for postponement of bids will not receive a response.  The LPA will issue an addendum postponing bids if warranted.

- After Award of Contract.

INSERT CONTACT INFORMATION FOR THE LPA

Responsible Charge or LPA Representative:

Mailing address:

Telephone:

Email address:

1

Section 102 – Bidding Requirements and Conditions

102.01 QUALIFICATION to Bid

1**************************************************************************************************************************1

BDC22S-18 dated Nov 18, 2022

the first paragraph is changed to:

The Bidder is an individual, firm, or corporation submitting a bid for the advertised Work.  The Department will not accept bids from Bidders who fail to meet all of the following criteria:

1.	 For Local Aid Projects- If pre-qualification of bidders is required, the Bidder has been pre-qualified as per the requirements of NJSA 40A:11-1 et seq.

2.	Before the receipt of the bid or accompanying the bid, the Bidder has disclosed ownership as required by N.J.S.A. 52:25-24.2.

3.	At the time the bid is delivered, the Bidder has an effective maximum and project ratings of not less than the amount of its bid.

4.	If the Bidder is a corporation not incorporated in the State, the Bidder has been authorized to do business in the State as required by N.J.S.A. 14A:15-2, et seq.

5.	The Bidder has a valid, current registration with the New Jersey Department of Labor, Division of Wage and Hour Compliance as required by “Public Works Contractor Registration Act,” N.J.S.A. 34:11-56.48, et seq.

1**************************************************************************************************************************1

102.04 EXAMINATION of Contract and Project Limits

1**************************************************************************************************************************1

2**************************************************************************************2

complete and include THE FOLLOWING

Address all inquiries concerning the contract and the project limits to the following:

LPA Responsible Charge or Representative:

Email Address:

Mailing Address:

2**************************************************************************************2

2**************************************************************************************2

Complete and include the following information if lead paint

The following is a list of structures and the location(s) of lead paint:

| Structure #/Location | Lead Paint Location(s) |
| --- | --- |

2**************************************************************************************2

1.	Evaluation of Subsurface and Surface Conditions.

2**************************************************************************************2

3************************************************3

complete and include International Roughness Index (IRI) values of the existing roadway

(FOR NHS routes and roads under njdot jurisdiction)

THE FOLLOWING IS ADDED:

| International Roughness Index (IRI) Values of the Existing Roadway |  |  |  |  |
| --- | --- | --- | --- | --- |
| Route | Direction | Mile Post |  | Existing IRI Value |
|  |  | From | To |  |

This information is the latest available IRI data of the right most through lane from the Pavement Management Unit.  The pavement information shown herein was obtained by the Department and is made available to the authorized users only so that they may have access to the same information available to the Department.  It is presented in good faith, but is not intended as a substitute for investigations, interpretation, or judgment of such authorized users.

1**************************************************************************************************************************1

3.	Existing Plans and As-Builts.

2**************************************************************************************2

3************************************************3

for roads under njdot jurisdiction, the LPA/LPA consultant should contact the engineering documents unit to determine if there are plans that can be used in the design of the project.  The lpa/lpa consultant will provide a copy of the final plans to the engineering documents unit for their information.

Include the local aid project manager on all transmittals.

3************************************************3

3************************************************3

Existing Plans including structural plans, contour maps, and as-built plans used are as follows:

a.

3************************************************3

2**************************************************************************************2

102.07 PREPARATION of the bid

For local aid projects THE LAST TWO PARAGRAPHS OF THIS SECTION are omitted

1**************************************************************************************************************************1

102.10 SUBMISSION of Bids

1**************************************************************************************************************************1

2**************************************************************************************2

BDC23S-10 dated Jun 5, 2023

the second paragraph is changed to:

The Bidder shall ensure delivery of its bid with all required components and attachments, including, but not limited to the following:

FOR LOCAL AID PRoJECTS

ADDITIONAL ITEMS REQUIRED BY THE LPA CAN BE ADDED TO THE LIST BELOW.

1.	Schedule of Items.

2.	Proposal Electronic Bidding File with Bidder’s Certification. NOT REQUIRED ON LOCAL AID PROJECTS

3.	Proposal Bond form.

4.	Other related documents as specified in the Contract.

5.	For Federal Aid Projects exceeding a bid amount of $100,000 or more, Bidder shall certify to the Byrd Anti-Lobbying Act requirements under 31 USC 1352.

6.	For Projects subject to the New Jersey Prevailing Wage Act, N.J.S.A. 34:11-56.25 to .98, an acknowledgement that the Bidder accounted for the prevailing wage rate in their bid pricing and that the Bidder agrees to pay the prevailing wage rate if awarded the Contract.

2**************************************************************************************2

A directory of certified Disadvantaged Small Businesses Enterprise firms can be found in the New Jersey Unified Certification Program Vendor Certification database, online at https://njucp.dbesystem.com/.

A directory of certified Emerging Small Business Enterprise firms can be found in the Emerging Small Business Program online directory at http://www.state.nj.us/transportation/business/civilrights/pdf/ESBEDirectory.pdf.

A directory of registered Small Businesses Enterprise firms can be found in the New Jersey Selective Assistance Vendor Information (NJSAVI) database online at https://www20.state.nj.us/TYTR_SAVI/vendorSearch.jsp.

All of the above directories are to be used as a source of information only and does not relieve the Bidder of their responsibility to seek out Enterprises not listed, prior to bidding.

When the Bidder submits bids for 2 or more Contracts that the Department will open on the same day, the Bidder may submit a single Updated Financial Statement instead of a separate statement for each Contract.

The Bidder is solely responsible for any and all errors and for timely submission of the bid, all components thereof, and all attachments thereto, through the electronic bidding system; the Department assumes no responsibility for any claim arising from the failure of any Bidder or of the electronic delivery system to cause any bid, bid component, or attachment to not be delivered to the Department on or before the time set for the opening of bids.

By submitting its bid to the Department, the Bidder warrants that no person or selling agency has been employed or retained by the Bidder to solicit or secure such Contract upon an agreement or understanding for a commission, percentage, brokerage or contingent fee, except bona fide employees or bona fide established commercial or selling agencies maintained by the Bidder for the purpose of securing business, for the breach or violation of which warranty the Department shall have the right to annul such Contract without liability or in its discretion to deduct from the contract price or consideration the full amount of such commission, percentage, brokerage or contingent fee as required by N.J.S.A. 52:34-15.

2**************************************************************************************2

BDC20S-13 dated SEp 21, 2020

THE FOLLOWING IS ADDED AT THE END OF THE SUBSECTION:

By submitting its bid to the Department, the Bidder warrants that no person or selling agency has been employed or retained by the Bidder to solicit or secure such Contract upon an agreement or understanding for a commission, percentage, brokerage or contingent fee, except bona fide employees or bona fide established commercial or selling agencies maintained by the Bidder for the purpose of securing business, for the breach or violation of which warranty the Department shall have the right to annul such Contract without liability or in its discretion to deduct from the contract price or consideration the full amount of such commission, percentage, brokerage or contingent fee as required by 
N.J.S.A. 52:34-15.

2**************************************************************************************2

1**************************************************************************************************************************1

102.13 CONSIDERATION of Bids

102.13.01 Bidder Pre-Award Requirements

A. Federal Aid Projects

1. Contract DBE Goal. On projects having a Contract DBE goal, all Bidders shall ensure that DBEs have an equal opportunity to receive and participate in the performance of contracts and subcontracts in Federal Aid Projects with the Department. All Bidders shall take all necessary and reasonable steps in accordance with 49 CFR, Part 26 to ensure that DBEs are given equal opportunity to compete for and to perform on the Department’s Federal Aid Projects. All Bidders shall not discriminate in the award and performance of any Contract obligation including, but not limited to, their performance of their obligations on USDOT assisted contracts as specified in Section 107.

- All Bidders shall demonstrate commitment of meeting the Contract DBE goal that is specified in the Contract.

(1) Submit to DCR/AA at time of Bid, or within 5 days after bid opening as a matter of responsibility:

(i) a completed and signed Form CR-266 – Schedule of DBE/ESBE/SBE Participation listing each DBE firm being used to meet the Contract goal. Revisions to the CR-266 will not be accepted after its initial submission and before award of the Contract.

(ii) a completed and signed Verification of DBE/ESBE/SBE Firm (Form CR-273) for each firm listed on the CR-266 to demonstrate direct written confirmation from each DBE firm of 12 willingness to participate on the Contract, confirming the kind and amount of work that was provided on the Contractor’s CR-266, and, if applicable,

(iii) a completed and signed DBE/ESBE/SBE Regular Dealer/Supplier Verification (Form CR-272) for all Regular Dealers/Suppliers listed on the CR-266 form, and, if applicable,

(iv) a completed and signed DBE/ESBE/SBE Trucking Verification (Form CR-274) for all DBE trucking firms listed on the CR-266.

Firms listed on the CR-266 will not be counted toward the Contract DBE goal unless completed and signed CR-273 form(s), and applicable CR-272 and CR-274 form(s) are submitted to the DCR/AA within the 5 days after bid opening. The CR-273, CR-272, and CR-274 forms must be completed and signed by each respective DBE firm. The Bidder shall not complete any portion of the CR-273, CR-272, and CR-274 forms.

These forms must be submitted through a designated email - DOT-CR.Verifications@dot.nj.gov.

If the last day for a Bidder’s submission of the CR-266, CR-273, and applicable CR-272 and CR-274 forms falls on a Saturday, Sunday, or Federal holiday, the CR-266, CR-273, and applicable CR-272 and CR-274 forms are due on the next day that is not a Saturday, Sunday, or Federal holiday. In circumstances where the Department is closed for all or part of the day the forms are due, the submission of the forms is due on the next day.

(2) If, at time of Submission, the commitment to meet the Contract DBE goal is not shown on the CR-266, the Bidder must submit at time of Bid, or within 5 days after bid opening, documented evidence of good faith effort(s) to attain the Contract DBE goal, for review and approval by the DCR/AA. Submittal of such information does not imply DCR/AA approval. The Department’s DCR/AA has sole authority to determine whether the Bidder met the Contract DBE goal or made adequate good faith efforts to do so.

(i) Good faith efforts are actions taken to achieve a DBE goal or other requirement of the DBE Program which, by their scope, intensity, and appropriateness to the objective, can reasonably be expected to fulfill the program requirement, including affirmative action measures designed to implement the established objectives of an affirmative action plan that a Bidder may utilize to obtain DBE participation. Efforts to include firms not certified as DBEs in New Jersey are consequently not good faith efforts to meet the DBE Contract goal. A promise to use DBEs after contract award is not considered to be responsive and does not constitute good faith efforts. Good faith effort actions include, but are not limited to:

(a) Conducting market research to identify small business contractors and suppliers and soliciting through all reasonable and available means the interest of all certified DBEs that have the capability to perform the work of the Contract. This may include attendance at pre-bid and business matchmaking meetings and events, advertising and/or written notices, posting of Notices of Sources Sought and/or Requests for Proposals, written notices or emails to all DBEs listed in the New Jersey Unified Certification Program Vendor Certification database of transportation firms that specialize in the areas of work desired (as identified by the North American Industry Classification system (NAICS) code noted in the New Jersey Unified Certification Program Vendor Certification database) and which are located in the area or surrounding areas of the project. The Bidder shall solicit this interest as early in the bidding process as practicable to allow the DBEs to respond to the solicitation and submit a timely offer for the subcontract. The Bidder should determine with certainty if the DBEs are interested by taking appropriate steps to follow up initial solicitations.

(b) Selecting portions of the work to be performed by DBEs in order to increase the likelihood that the DBE goals will be achieved. This includes, where appropriate, breaking out Contract work items into economically feasible units (for example, smaller tasks or quantities) to facilitate DBE participation, even when the Bidder might otherwise prefer to perform these work items with its own forces. This may include, where possible, establishing flexible timeframes for performance and delivery schedules in a manner that encourages and facilitates DBE participation.

(c) Providing interested DBEs with adequate information about the Plans, specifications, and requirements of the Contract in a timely manner to assist them in responding to a solicitation with their offer for the subcontract.

(d1) Negotiating in good faith with interested DBEs. It is the Bidder's responsibility to make a portion of the work available to DBE subcontractors and suppliers and to select those portions of the work or material needs consistent with the available DBE subcontractors and suppliers, so as to facilitate DBE participation. Evidence of such negotiation includes the names, addresses, and telephone numbers of DBEs that were considered; a description of the information provided regarding the plans and specifications for the work selected for subcontracting; and evidence as to why additional Agreements could not be reached for DBEs to perform the work.

(d2) Bidder using good business judgment would consider a number of factors in negotiating with subcontractors, including DBE subcontractors, and would take a firm's price and capabilities as well as Contract goals into consideration. However, the fact that there may be some additional costs involved in finding and using DBEs is not in itself sufficient reason for a Bidder's failure to meet the Contract DBE goal, as long as such costs are reasonable. Also, the ability or desire of a Bidder to perform the work of a Contract with its own organization does not relieve the Bidder of the responsibility to make good faith efforts. Bidders are not, however, required to accept higher quotes from DBEs if the price difference is excessive or unreasonable.

(e1) Not rejecting DBEs as being unqualified without sound reasons based on a thorough investigation of their capabilities. The contractor's standing within its industry, membership in specific groups, organizations, or associations and political or social affiliations (for example union vs. non-union status) are not legitimate causes for the rejection or non-solicitation of bids in the Bidder’s efforts to meet the Contract DBE goal. Another practice considered an insufficient good faith effort is the rejection of the DBE because its quotation for the work was not the lowest received. However, nothing in this paragraph shall be construed to require the Bidder to accept unreasonable quotes in order to satisfy the Contract goals.

(e2) A Bidder’s inability to find a replacement DBE at the original price is not alone sufficient to support a finding that good faith efforts have been made to replace the original DBE. The fact that the Bidder has the ability and/or desire to perform the Contract work with its own forces does not relieve the Bidder of the obligation to make good faith efforts to find a replacement DBE, and it is not a sound basis for rejecting a prospective replacement DBE's reasonable quote.

(f) Making efforts to assist interested DBEs in obtaining bonding, lines of credit, or insurance as required by the recipient or Bidder.

(g) Making efforts to assist interested DBEs in obtaining necessary equipment, supplies, materials, or related assistance or services, but not directly or indirectly providing equipment, supplies or materials to the DBE.

(h) Effectively using the services of available minority/women community organizations; minority/women contractors' groups; local, State, and Federal minority/women business assistance offices; and other organizations as allowed on a case-by-case basis to provide assistance in the recruitment and placement of DBEs.

(3) If the Department determines that the apparent lowest responsive Bidder has failed to commit to meet the Contract DBE Goal and made adequate good faith efforts to do so, the Department must, before awarding the Contract, provide the Bidder an opportunity for Administrative Reconsideration.

The apparent lowest responsive Bidder will have the opportunity to provide written documented evidence or argument concerning the issue of whether it met the Contract DBE goal or made adequate good faith efforts to do so to an official who did not take part in the original determination that the Bidder failed to meet the Contract DBE goal or made adequate good faith effort to do so, pursuant to 49 C.F.R. 26.53(d). The apparent lowest responsive Bidder has the opportunity to meet in person with the Reconsideration Official to discuss the issue of whether it met the Contract DBE goal or made adequate good faith efforts to do so.

Within 1 working day of being notified by the Department that it is not a responsible Bidder because it failed to meet the Contract DBE goal and made adequate good faith efforts to do so, a Bidder may make a request for administrative reconsideration in writing to the New Jersey Department of Transportation, Director, Division of Procurement, PO Box 605, Trenton, New Jersey, 08625-0605. The Bidder must specify one of the following types of administrative reconsideration in its request:

- Written Review by the Department. If the Bidder seeks written review by the Department, it must submit written documented evidence or argument proving the Bidder met the Contract DBE goal at time of Bid or submitted adequate good faith efforts to do so within 5 days after bid opening, to the Department within 2 working days of the Bidder’s request for Administrative Reconsideration.

- (ii) In-Person Meeting. If the Bidder seeks an in-person meeting by the Department it must submit written documented evidence or argument proving the Bidder met the Contract DBE goal at time of Bid or submitted adequate good faith efforts to do so within 5 days after bid opening, to the Department within 2 working days of the Bidder’s request for Administrative Reconsideration. The in-person meeting will be scheduled by the Department as soon as time permits.

If the timeframe for a Bidder’s request for Administrative Reconsideration, or submission of written documented evidence or argument proving the Bidder met the Contract DBE goal or submitted adequate good faith effort to do so falls on a weekend or holiday, the written requests are due to the Department on the next working day. The Department, at its discretion, may not review or consider any documentation or argument in its administrative reconsideration that was not contained in the Bidder’s request for written review or in-person meeting with the Department.

Once the Reconsideration Official has made a determination, the Department will send the Bidder a written decision on reconsideration, explaining the basis for finding that the Bidder did or did not meet the DBE goal or make an adequate good faith effort to do so.

Failure to follow this request procedure may result in the Bidder’s waiver of the right for Administrative Reconsideration under this Section.

The result of the reconsideration process is not administratively appealable to the USDOT.

2.Contract ESBE Goal. Where a Contract ESBE goal is set, the Bidder shall follow all requirements and the same administrative reconsideration procedure of 102.13.

1**************************************************************************************************************************1

2**************************************************************************************2

BDC21S-09 dated MAY 14, 2021

Part C is changed to:

C.	All Projects.  Prior to the time of contract award:

- Submit proof of business registration with the Division of Revenue and Enterprise Services in the New Jersey Department of Treasury as required by N.J.S.A. 52:32-44.  Information on how a business can register and obtain proof of business registration can be accessed on the internet at www.nj.gov/njbgs.

- On the Disclosure of Investment Activities in Iran (Form DC-16) provided by the Department, certify pursuant to N.J.S.A. 52:32-58, that neither the Bidder, nor one of its parents, subsidiaries, and affiliates (as defined in N.J.S.A. 52:32-56(e)(3)), is listed on the Department of the Treasury's List of Persons or Entities Engaging in Prohibited Investment Activities in Iran and that neither is involved in any of the investment activities set forth in N.J.S.A. 52:32-56(f).  If the Bidder is unable to certify, the Bidder shall provide a detailed and precise description of such activities to the Department.

3************************************************3

BDC22S-18 dated Nov 18, 2022

the following is added in part c:

3.	FOR LOCAL AID PROJECTS:  The Bidder must certify, in writing, pursuant to NJSA 52:32-44.1, to the LPA that neither the Bidder nor its affiliates are debarred at the federal level from contracting with a federal government agency.  The LPA shall not make, negotiate, or award to any Bidder that does not provide the above certification.  In addition, all Bidders must register with the federal System for Award Management (SAM) prior to contract award.  ln order to comply with this requirement, Bidders must register in SAM at  sam.gov and the Department will verify the successful Bidder's registration in SAM prior to contract award.

3************************************************3

3************************************************3

BDC23S-10 dated Jun 5, 2023

the following is added in part c:

4.	On the “Certification of Non-Involvement in Prohibitive Activities in Russia or Belarus Form” provided by the Department, certify pursuant to N.J.S.A. 52:32-60.1, that neither the Bidder nor its affiliates are engaged in prohibited activities in Russia or Belarus as defined therein.

3************************************************3

3************************************************3

BDC24S-04 dated Jun 6, 2024

the following is added in part c:

5.	Submit proof of valid Public Works Contractor Registration issued by the New Jersey Department of Labor, Division of Wage and Hour Compliance according to N.J.S.A. 34:11-56.48, et seq.

3************************************************3

3************************************************3

BDC25S-10 dated MAY 15, 2025

the following is added to part c:

6.	N.J.S.A. 34:1A-1.16 authorized the New Jersey Department of Labor and Workforce Development (NJDOL) to create a list on its website, dubbed the Workplace Accountability in Labor List (WALL), of any person found in violation of any State wage, benefit, and tax laws and against whom a final order has been issued by the NJDOL for such violation.  Any person or business named on the WALL is prohibited from contracting with the Department until that person or business has been removed from the WALL.  The WALL is found at https://www.nj.gov/labor/ea/osec/wall.shtml

3************************************************3

2**************************************************************************************2

1**************************************************************************************************************************1

102.15 DISQUALIFICATION of Bidders

The Department will disqualify a Bidder and reject a bid submitted by that Bidder if the Bidder is determined by the Department to lack responsiveness. Failure of a Bidder to follow the requirements of 102.10 demonstrates a lack of responsiveness.

1**************************************************************************************************************************1

BDC23S-07 dated May 12, 2023

The second paragraph is changed to:

The Department will disqualify a Bidder and reject a bid submitted by that Bidder if the Bidder is determined by the Department to lack responsibility.  Factors demonstrating a lack of responsibility include, but are not limited to:

1.	Evidence of collusion among Bidders.

2.	Uncompleted work, which in the opinion of the Department, might hinder or prevent completion of additional work if awarded.

3.	Failure to submit at time of bid or within 5 days of bid opening, a completed and signed CR-266 – Schedule of Disadvantaged Business Enterprise/Emerging Small Business Enterprise/Small Business Enterprise Participation.

4.	Failure to submit within 5 days of bid opening, proof of documented evidence of good faith efforts to meet the Contract goal, if the Bidder fails to meet the Contract DBE, ESBE or SBE goal.

5.	Failure to submit within 5 days of bid opening, a completed and signed Confirmation of DBE/ESBE/SBE Firm (Form CR-273) for each DBE/ESBE/SBE firm listed on the CR-266.  The Bidder shall not complete any portion of the CR-273 form.

6.	Failure to submit within 5 days of bid opening, a completed and signed DBE/ESBE/SBE Trucking Verification (Form CR-274) for each DBE/ESBE/SBE firm listed on the CR-266, if applicable.  The Bidder shall not complete any portion of the CR-274 form.

7.	Failure to submit within 5 days of bid opening, a completed and signed DBE/ESBE/SBE Regular Dealer/Supplier Verification (Form CR-272) for each DBE/ESBE/SBE Regular Dealer/Supplier listed on the CR-266, if applicable.  The Bidder shall not complete any portion of the CR-272 form.

8.	Failure of the Bidder to meet the Contract DBE, ESBE, or SBE goal as determined by the DCR/AA, or make adequate good faith efforts to do so.

9.	Failure of the Bidder to acknowledge that the prevailing wage rate is accounted for in their bid pricing and that the Bidder agrees to pay the prevailing wage rate, if awarded the Contract, for Projects subject to the New Jersey Prevailing Wage Act, N.J.S.A. 34:11-56.25 to .98,

10.	Submission of a materially unbalanced bid.  A materially unbalanced bid is a bid where there is a reasonable doubt that award to the Bidder submitting a mathematically unbalanced bid, which is structured on the basis of nominal prices for some work and inflated prices for other work, will result in the lowest ultimate cost to the Department.

11.	Lack of competency or lack of adequate machinery, plant, or other equipment.

12.	Unsatisfactory performance on previous or current contracts.

13.	Questionable moral integrity as determined by the Attorney General of New Jersey or the Department.

14.	Any other outward actions or lack of action that demonstrates the Bidder is not responsible.

15.	Disqualification, suspension, or debarment of an individual firm, partnership, corporation, joint venture, or any combination as required by N.J.A.C. 16:44-11.1 or Federal Government’s System for Award Management (SAM), located at https://www.sam.gov/SAM/ for federally assisted contracts.

16.	If the bidder is Federally debarred pursuant to N.J.S.A. 52:32-44.1.

2**************************************************************************************2

BDC25S-10 dated may 15, 2025

the following is added TO THE SECOND PARAGRAPH:

17.	The Bidder is listed on the Workplace Accountability in Labor List (WALL), prohibiting the Bidder from public contracting under N.J.S.A. 34:1A-1.16, the law establishing the WALL.

2**************************************************************************************2

1**************************************************************************************************************************1

Section 103 – Award and Execution of Contract

103.01 AWARD of Contract

1**************************************************************************************************************************1

BDC23S-13 dated Oct 23, 2023

FOR LOCAL AID PROJECTS

THE FOLLOWING CAN BE DELETED IF THE LPA DOES NOT POST AWARD INFORMATION ON THE LPA’S WEBSITE.

the following is added:

The Department will post public announcement of award on the Department’s website at

ADD LPA WEBSITE ADDRESS

1**************************************************************************************************************************1

103.04  Execution of the Contract

FOR LOCAL AID PROJECTS DELETE THIS SECTION AND REPLACE WITH THE REQUIREMENTS OF THE LPA

1******************************************************************************************************************

Section 104 – Scope of Work

104.02  Value Engineering

1**************************************************************************************************************************1

BDC25S-21 dated MAR 24, 2026

THE SUBSECTION IS CHANGED TO:

104.02  Value Engineering Change Proposal

104.02.01  Purpose and Scope

The intent of Value Engineering Change Proposal (VECP) is to promote Contractor innovation, for which the Department will share with the Contractor 50 percent of the value of cost savings generated as a result of a Department approved VECP.

The Department will only consider VECPs that produce a cost savings to the Department without, in the sole judgment of the Department, impairing essential functions or characteristics of the Project.  Essential functions that cannot be adversely impacted include, but are not limited to, safety, service life, economy of operation, ease of maintenance, environmental compliance, and desired appearance.  The Department will not consider VECPs that change the Project’s original design criteria, merely eliminate work, are based on an unknown factor, do not show a cost savings; or extend Contract Time.

2**************************************************************************************2

INCLUDE THE FOLLOWING LIST OF VECPs THE DEPARTMENT WILL NOT CONSIDER, otherwise delete

SME CONTACT – PROJECT MANAGER

The Department will not consider the following as VECPs:

2**************************************************************************************2

104.02.02  Conditions

The Department will consider VECPs only after Award of the Contract and only in accordance with the following terms and conditions:

1.	If the Department rejects a VECP during any step of the process, the Contractor is required to complete the Work as specified in the Contract.  The Contractor recognizes that the Department has the right to reject VECPs and cannot base its bid on the anticipated approval of a VECP.

2.	All VECPs, approved or not approved by the Department for use in the Contract, apply only to the ongoing Contract or Contracts referenced in the VECP.  The VECPs shall become the property of the Department and shall contain no restrictions imposed by the Contractor on their use or disclosure.  The Department will have the right to use, duplicate, and disclose in whole or in part any data necessary to use the VECP.  The Department retains the right to use any accepted VECP or part thereof on any other or subsequent project without any obligation to the Contractor.  This provision is not intended to deny rights provided by law with respect to patented materials or processes.

3.	If the Department already has under consideration certain revisions to the Contract that are subsequently incorporated in a VECP, the Department will reject the Contractor’s VECP and has the right to proceed with such revisions without obligation to the Contractor.

4.	The RE will determine whether a VECP qualifies for consideration and evaluation by the Department, including whether the savings generated by the VECP are sufficient to warrant review and processing.  The Department may still reject a final VECP after approval of the initial VECP.

5.	The RE has the right to reject all or any portion of work performed according to an approved VECP if the RE determines that results do not meet the requirements of the VECP or the Contract.  The RE has the right to direct the removal of such rejected work and require the Contractor to proceed according to the original Contract.  If the Contractor is directed to proceed according to the original Contract, the Department will not make payment for the rejected work performed under the VECP, or for its removal.  The Contractor is responsible for the consequential costs arising from the rejected work to complete the work as proposed under the original Contract, including the cost to Others.

6.	The Contractor may only submit VECPs based on Items scheduled to be performed by the Contractor.  The Department will not consider anticipated cost savings based on revisions to utility relocations or other work to be performed by Others.  The Contractor is responsible to obtain the concurrence of Utilities, property owners, governing bodies, and agencies for any work affected by the VECP.  The Department has the right to reject VECPs that may increase the time or cost of work done by Others.

7.	In calculating the value of cost savings, the Department has the right to disregard the Contract bid prices, if such prices do not represent the value of the work to be performed or to be deleted, and has the right to calculate the savings based on reasonable costs for such work.

8.	Ensure that VECPs are designed according to the Department design manuals and other Department standards for the proposed work.

9.	If additional information is needed to evaluate VECPs, provide this information in the Department specified time period, otherwise the Department will reject the VECP.  Additional information may include, where design changes are proposed, results of field investigations and surveys, design computations, and field change sheets.

10.	The Department will not deduct the Department’s costs for review and processing of the VECP from the cost savings.

11.	The Department will not make payment for the Contractor’s costs for development and submission of the VECP.  The Contractor waives its right to claim for costs or delays incurred due to the Department’s review or rejection of a VECP.  These costs include, but are not limited to, development costs, loss of anticipated profits, or increased material or labor costs resulting from delays in the review of such VECP.

12.	The Department will not use the value of cost savings for: additional vehicle operating and delay costs incurred by the traveling public, compared to normal operating conditions, resulting from the establishment of construction, maintenance, or rehabilitation work zones (road user costs), or for costs to oversee the Project (construction engineering costs).

13.	The Contractor may submit VECPs for an approved subcontractor, provided that payment is made by the Department to the Contractor and that the terms of the payment to the subcontractor are satisfactorily negotiated and accepted before the proposal is submitted to the Department.  Submit revised agreements as specified in 108.01.1.  Subcontractors may only submit a VECP through the Contractor.

104.02.03  Submittals

The Contractor may submit VECPs electronically.  With electronic submissions, submit the appropriate attachments and a printed copy.

If the initial VECP is only submitted in printed form, provide at least 3 sets of documents.  If the final VECP is only submitted in printed form, the Department will designate the required number of sets of documents.

Submit initial and final VECPs as follows:

1.	Initial VECP.  Submit to the RE an initial VECP outlining the general technical concepts associated with the VECP and the estimated savings that will result.

The Department will review the initial VECP and, if found to be conceptually acceptable, will grant approval to submit a final VECP.  A finding of conceptual acceptability of the initial proposal in no way obligates the Department to approve the final VECP.

2.	Final VECP.  The Department will consider a final VECP only after the approval of the initial VECP.  With each final VECP, submit, at a minimum, to the RE the following information:

a.	A statement that the final VECP is submitted as a VECP with a reference to the approval of the initial VECP.

b.	A description of the differences between the existing Contract requirements and the proposed change, and the comparative advantages and disadvantages of each.  Document the effects on safety, service life, economy of operations, ease of maintenance, environmental requirements, and desired appearance.

c.	Complete plans, specifications, and calculations showing the proposed changes relative to the original Contract features and requirements.  Ensure that plans and engineering calculations are signed and sealed by a Professional Engineer.

d.	A complete cost analysis indicating the final estimated costs and quantities to be replaced by the VECP, the new costs and quantities generated by the final VECP, and the cost effects of the proposed changes on operational, maintenance, and other considerations.

e.	An evaluation of the effect the final VECP has on Contract Time, including a revised schedule and a time impact evaluation as specified in 108.11.01.C.

f.	A description of any previous use or testing of the final VECP on another Department project or elsewhere and the conditions and results.  If the final VECP was previously submitted on another Department project, indicate the date, the project, and the action taken by the Department.

g.	A letter of concurrence to the VECP from Others that are affected.

h.	The specific date that the Change Order adopting the final VECP must be executed to obtain the maximum cost reduction.  Ensure that this date allows the Department ample time, usually a minimum of 60 days, for review and processing a Change Order.  If the Department finds that there is insufficient time available for review and processing, it has the right to reject the final VECP solely on such basis.  If the Department fails to respond to the final VECP by the date specified, the Contractor shall consider the final VECP rejected and shall make no claim against the Department as a result thereof.

i.	Additional information requested by the Department.

104.02.04  Final Documents

If the final VECP is approved, submit working drawings for approval.  The Department will designate the number of sets to be submitted.

104.02.05  Payment

If the VECP is approved, the Department will issue a Change Order incorporating the VECP and establishing payment for the cost savings.

The Department will make payment to the Contractor for 50 percent of the cost savings from the VECP.  It is understood that the payment includes all costs resulting as a consequence of the change, including but not limited to quantity increases to Items, and unanticipated work or expenses arising out of the change.

1**************************************************************************************************************************1

104.03 CHANGES of the Contract

104.03.01 Authority to Make Changes

1**************************************************************************************************************************1

BDC24S-18 dated Sep 16, 2024

THE FIRST PARAGRAPH IS CHANGED TO:

The Department has the right to make changes to the Work at any time, including altering the Contract, altering the requirements of an Item, increasing, or decreasing the quantities of any Item, or deleting any Item. Such changes neither invalidate the Contract, nor release the Surety. The Contractor agrees to perform the Work as changed.  If the Contractor does not perform, or refuses to perform the Work as changed, the Department may perform the work with its own forces.  If the Department mobilizes its forces to perform the work, the Contractor agrees to pay the Department’s cost of performing the work including the cost of material and labor used and the actual costs for police traffic protection and maintenance and protection of traffic as specified in 107.16.

1***********************************************************************************************1

The Contractor may request changes to the Contract.

The Department will direct changes using a written Field Order or Change Order as follows:

1. Field Orders. The RE has the right to unilaterally direct Contractor performance with a written Field Order. Upon receipt of a Field Order, proceed as directed by the Field Order. A Field Order will state whether the directions provided by the Field Order are eligible for payment or modification of Contract Time. The Department will provide such payment or modification to Contract Time in a subsequent Change Order.

2. Change Orders. The Department will modify the Contract with a Change Order.

The Department has the right to unilaterally make the following changes: make minor increases or decreases in quantity; modify Contract Time as specified in 108.11; direct New Work to be performed on a force account basis; and establish payment modifications for damages, credits, bonuses, and penalties provided for in the Contract.

For changes that require the Contractor’s assent, the RE will negotiate the terms of the change and the Department will offer a Change Order. If the Contractor accepts the offer, the Contractor will sign the Change Order, whereupon the Department may approve and issue the Change Order. By signing a Change Order, the Contractor agrees that the Change Order constitutes the full extent of the agreement between the Department and the Contractor, including the full extent of payment and modified Contract Time for the work and for any effects upon all and any part of the Work, and supersedes any prior written or oral agreement between the 23 parties. Additionally, the Contractor acknowledges the sufficiency and propriety of the consideration for the work detailed in the Change Order and understands that the agreement is binding.

Upon receipt of an approved Change Order, proceed as directed by the Change Order.

DBE or ESBE goals apply to work performed through Field Orders and Change Orders. On Federal Aid Projects, the Contractor is responsible for complying with the DBE program, rules and regulations of 49 CFR Part 26, the requirements as specified in 105.02.05, Federal Aid Project Attachments 1 through 11 for this work.

Contractor resubmission of CR-266, CR-273, CR-272, and CR-274 forms may be required on the work performed through Field Orders and Change Orders.

Do not deviate from the requirements of the Contract unless and until a Field Order is issued by the RE or a Change Order is approved by the Department.

104.03.02 Protests to Change Orders

1**************************************************************************************************************************1

BDC24S-18 dated Sep 16, 2024

The Subpart is changed to:

If the Contractor disagrees with any terms or conditions set forth in a Change Order, submit a written protest to the Department within 30 days after the date of receipt of the Change Order.

A protest is notice that the terms and conditions for proposed work are not in accordance with the Contract, quantity adjustments are incorrect, or that the modification for Contract Time is incorrect.  A protest is not a substitute for notice as specified in 104.03.04.  Providing a protest within 30 days after the date of the receipt of a Change Order may not meet the requirements of 104.03.04 or N.J.S.A 59:13-5 and the Department will not make payment for the costs of a claim if recovery is barred by other provisions in the Contract.

In the protest, list the points of disagreement, and, if possible, the specification references, quantities, and costs involved.  Ensure that the protest is a specific, detailed statement of the points of disagreement.  The Department will reject general protests.  If the Department rejects a protest for being a general protest, provide a specific, detailed statement within 7 days of such rejection.

Regardless of whether the Contractor’s assent is required, if the Contractor refuses to sign the change order or submit an acceptable written protest within 30 days after the date of receipt of the Change Order or within 7 days of the initial rejection of a protest, the Department will make payment and modify Contract Time as set forth in the Change Order.  Such payment is full payment for all work included or required by the Change Order and is conclusive as to any Contract Time modifications provided for therein or in establishing that no Contract Time modification was warranted.

When the Contractor signs a change order and the Department processes the Change Order within 15 days of receiving the Contractor’s signature, the Contractor is barred from protesting the Change Order.

Protest does not relieve the Contractor from the obligation to proceed with work directed by an approved Change Order.

1******************************************************************************************

104.03.03 Types of Changes

1**************************************************************************************************************************1

BDC24S-03 dated Apr 18, 2024

1.	Quantity Increases and Decreases.

The Second Paragraph is Changed to:

For minor changes in quantity, the Department will make payment for the quantity of the Item performed at the bid price for the Item.  Construction layout is included in the price of the original work.

3.	Changes in the Character of Work.

THE THIRD PARAGRAPH IS CHANGED TO:

If a modification cannot be reached by agreement, the Department will make payment, only for the change in work, by force account as specified in 104.03.08.

1**************************************************************************************************************************1

104.03.07 Tracking Costs

1**************************************************************************************************************************1

BDC24S-11 dated JUL 09, 2024

THE FIRST SENTENCE OF THE FIRST PARAGRAPH IS CHANGED TO:

For all work directed to be paid for by force account, track and maintain complete records to provide a clear distinction between the costs for the Force Account work and the costs for other operations.  For costs which the Contractor will pursue reimbursement through a Notice or Claim, track and maintain complete records in Force Account style making a clear distinction between Contract work and the work related to the alleged changed condition.

1**************************************************************************************************************************1

104.03.08 Force Account

1**************************************************************************************************************************1

BDC23S-11 dated August 17, 2023

THE FIRST sentence of the first paragraph IS CHANGED to:

Payment made for the work directed to be performed by force account represents full payment for that work including layout.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC24S-23 dated Feb 11, 2025

Part 6 is changed to:

6.	Extraordinary Expenses.  Extraordinary Expenses will be limited to only permit and disposal fees, including any associated required soil testing.  The Department will only make payment for Extraordinary Expenses if they are approved by the RE before incurring the cost and evidenced by paid invoices.  If the cost is, in the opinion of the RE, excessive, then the Department will limit the payment for such expenses to reasonable costs as available in the prevailing market.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC23S-04 dated Apr 14, 2023

THE FIRST PARAGRAPH OF PART 7 IS CHANGED TO FOLLOWING:

7.	Equipment.  The Department will make payment for Contractor-owned or rented equipment required for the force account work.  The Department will not provide payment for equipment that is determined to be more than necessary or unsuitable by the RE for the force account work or that is inoperable due to breakdown or during periods of repair.  In the event the Contractor proposes to use equipment of a higher cost, including equipment with one or more of the following: specialized functionality, features, trim levels, options, or accessories than that suitable and necessary for the work, the Department will make payment at the rate applicable to the suitable and necessary equipment.  The Department will make payment for Contractor-owned or rented equipment as follows:

PART 7.B IS CHANGED TO THE FOLLOWING:

- Rented Equipment.  In the event that the Contractor does not own a specific type of equipment and must obtain it by rental, submit in writing to the RE for approval the need to rent the equipment and the rental rate for that equipment before using it on the work.  Ensure the rental company does not rent equipment exclusively to a single Contractor.  The Department will make payment for the rental of the equipment as specified in the rental agreements and as evidenced by paid invoices for the time that the equipment is used to accomplish the work, plus the cost of moving the equipment to, on, and away from the work site.

1**************************************************************************************************************************1

104.03.09 Delay Damages

1**************************************************************************************************************************1

BDC24S-05 dated may 22, 2024

1.	Non-Productive Activity.  The Department will make payment for the following non-productive activities:

Part E IS CHANGED TO:

e. 	Equipment.  If as the result of the delay, the equipment cannot be used for any active work and is directed by the RE to remain on the work site during the delay, the Department will make payment as specified in 104.03.08.7.a.5.  If as the result of the delay, the RE determines that the equipment cannot be used for any active work, the RE may request the Contractor to remove the equipment.  The Contractor may remove the equipment from the work site or allow it to remain.  If the equipment is removed from the work site, the Department will make payment for labor and equipment costs to remove the equipment and to return it to the work site at the end of the delay period.  If the equipment remains, the Department will not make payment for the equipment as specified in 104.03.08.7.

If the equipment is required for additional maintenance within the Project Limits, maintenance of traffic control devices, maintenance of SESC measures, and similar activities resulting from the delay and approved by the RE, the Department will make payment as specified in 104.03.08.7.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC24S-10 dated jul 1, 2024

The following is added:

4.	Equipment Escalation.  If, as the result of the delay, equipment use, which had a planned late-finish date occurring before an equipment rate increase date, is required to start after the equipment rate increase date, the Department will make payment for the following:

a.	Contractor owned equipment.  Equipment costs as specified in 104.03.08 at the higher rate as evidenced by a comparison of the Blue Book rate between the planned late finish date for the work and the Blue Book rate when the work was required because of the delay.

b.	Rented equipment.  Equipment costs as specified in 104.03.08 at the higher rate as evidenced by a comparison of rental agreements, including paid invoices, between the planned late finish date for the work and the date the work was required because of the delay.  If rental agreements and paid invoices are not available, provide quotes from the rental company for both dates.

1**************************************************************************************************************************1

Section 105 – Control of Work

1**************************************************************************************************************************1

105.02 RESPONSIBILITIES of the Contractor

1**************************************************************************************************************************1

105.02.05 Civil Rights Requirements

The Contractor is obligated to comply with Title VI of the Civil Rights Act of 1964, 49 CFR Part 21 and 28 CFR Section 50.3, 2 C.F.R. Part 200 and 2 C.F.R. Part 200 Appendix II and any other Rules relative to Nondiscrimination as they may be amended from time to time, which are herein and incorporated by reference and made part of the Contract. The Contractor in the performance of the Contract agrees to comply with nondiscrimination regulations and other requirements as specified in Section 107. Failure of a Contractor to comply with the nondiscrimination provisions of the Contract may result in the actions as set forth as specified in Sections 105, 108, and 109.

The source of funding determines which EEO regulations and goals (Federal and/or State goals) apply to a specific project.

- Federal Aid Projects. On contracts containing Federal funding, Federal EEO regulations and goals apply as specified in Federal Aid Project Attachments 1 through 11. The DCR/AA monitors and reviews these projects on behalf of the Federal Highway Administration (FHWA), under Federal statutes (23 USC 140) and Rules (23 CFR 230, 2 CFR Part 200).

Comply with the DBE/ESBE program, rules and regulations of 49 CFR Part 26 in the administration of the Contract. Failure to do so is a material breach of the Contract and may result in termination of the Contract, or other such actions that the Department or the FHWA deem appropriate which may include, but are not limited to, rejection of bids, denial or limit of credit toward the Contract goal, payment being delayed or withheld as specified in Section 105, assessing sanctions as set forth in 49 CFR Part 26, and default as specified in Section 108. Deliberate attempts by the Contractor or subcontractors to circumvent or commit fraud in the DBE/ESBE program may result in termination of the Contract as specified in Section 108, investigation by the Department’s Inspector General or the U.S. Department of Transportation, or both, and prosecution by the State Attorney General’s Office or the U.S. Department of Justice, or both.

1**************************************************************************************************************1

BDC24S-20 dated jan 28, 2025

The third paragraph is changed to:

Ensure compliance with the labor standards provisions of the Contract.  Submit weekly certified payrolls and Statement of Compliance as required in the Contract to the RE.  Monitor and verify the owner-operator status of all DBE and non-DBE truckers working on Federal Aid Projects used for the Contract.  Submit the DBE Trucking Verification (Form CR-274) to the Department.  Failure of a Contractor to meet the requirements of this paragraph may result in payment being delayed or withheld as specified in Section 105, default as specified in Section 108, disqualifying the Contractor from future bidding as non-responsible, or termination of the Contract as specified in Section 108.

For Local Aid Projects

Reference to Diane B. Allen Equal Pay Act has been removed.

1**************************************************************************************************************1

Obtain executed subcontract agreements as specified in Section 108. Failure of a Contractor to meet this requirement may result in payment being delayed or withheld as specified in Section 105, default as specified in Section 108, disqualifying the Contractor from future bidding as non-responsible, or termination of the Contract as specified in Section 108.

The Contractor is responsible for compliance by any subcontractor, lower tier subcontractor as specified in Section 108. On Federal Aid construction contracts, utilize a DBE that performs a commercially useful function (CUF) and performs the work committed to at the time of Contract award. Monitor and report DBE participation on the Contract, on a monthly basis utilizing the CR-267 - Monthly Report of Utilization of DBE/ESBE or SBE form. Failure of a subcontractor or lower tier subcontractor may result in denial or limit of credit toward the Contract DBE goal, payment being delayed or withheld as specified in Section 105; default as specified in Section 108, or termination of the Contract as specified in Section 108.

The Contractor is required to make good faith effort as defined in 23 CFR Part 230 and 41 CFR Part 60 in meeting the Equal Employment Opportunity, Affirmative Action, on-the-job training and female and minority work hour goals. Ensure compliance by subcontractors and lower tier subcontractors. Failure of the Contractor, subcontractor or lower tier subcontractor to meet these requirements may result in payment being delayed or withheld as specified in Section 105; default as specified in Section 108, or termination of the Contract as specified in Section 108.

Utilize the specific DBEs listed to perform the work, manufacturer the materials or goods, and furnish or supply the equipment, materials, supplies or services for which each is listed on the CR-266 unless prior written consent from the DCR/AA is obtained. Unless prior DCR/AA consent is provided, the Contractor shall not be entitled to any payment for work or material unless it is performed or supplied by the listed DBE.

The Contractor is responsible for Equal Employment Opportunity requirements of the Contract, including Affirmative Action, EEO workforce and On-The-Job Training. Failure by the Contractor to meet the requirements of the Affirmative Action Program for Equal Employment Opportunity may result in payment being delayed or withheld as specified in Section 105 pending corrective and appropriate measures by the Contractor to the satisfaction of the Department.

The Contractor is responsible for compliance with the Trainee program. Failure to meet this requirement may result in payment being delayed or withheld as specified in Section 105, default as specified in Section 108, disqualifying the Contractor from future bidding as non-responsible, or termination of the Contract as specified in Section 108.

The Contractor and subcontractors are required to provide all information and reports as specified in Section 107.

a.	Disadvantaged Business Enterprise/Emerging Small Business Enterprise Goals for this Contract

This Contract includes a goal of awarding a percentage of the Total Contract Price to subcontractors, transaction expeditors, regular dealers, manufacturers and truckers qualifying as certified DBEs/ESBEs as specified in Federal Aid Project Attachment 1 – Disadvantaged Business Enterprise Utilization on Federal Aid Projects or Federal Aid Project Attachment 1 – Emerging Small Business Enterprise Utilization on Federal Aid Projects, of the Special Provisions.

To receive DBE credit toward meeting a contract goal in the context of the contract award process, a DBE firm must be certified before the due date for bids or offers on the Contract, as stated in 49 CFR Part 26.81(c). There may be situations after the award of the Contract, however, in which it is appropriate to count DBE credit for the use of a DBE firm certified after the contract is executed. To be eligible to obtain DBE credit, a DBE firm must be certified before the subcontract on which it is working is executed.

A Contractor is deemed to have satisfied the requirements of the DBE Program if the Contractor meets the Contract DBE goal or the approved DBE commitment utilizing the approved, certified DBEs for the kind and type of work shown on the most recent approved CR-266 – Schedule of Disadvantaged Business Enterprise/Emerging Small Business Enterprise/Small Business Enterprise Participation form or demonstrates an adequate GFE. DCR/AA will determine whether or not the Contractor met the Contract DBE goal or demonstrated adequate GFE. Failure to meet the Contract DBE goal or the approved DBE commitment, without demonstrating an adequate GFE, is considered a material breach of the Contract.

The Contract DBE goal may be changed by the Department based on changes in the Work that increase or decrease work assigned to approved DBEs, or to create potential DBE subcontracting opportunities regarding the Contract. The Department’s DCR/AA will evaluate these changes in the Work in the same manner that the original Contract DBE goal or the approved DBE commitment was established.

**************************************************************************************2

bdc24s-19 dated oct 28, 2024

THE LAST SENTENCE OF THE FOURTH PARAGRAPH IS CHANGED TO:

When the Contract DBE goal or the approved DBE commitment is increased or decreased due to changes in firms, type of work, work items or subcontract value, and for each Field / Change Order that increases or decreases the Contract’s dollar value, submit to DCR/AA to determine if and how these changes will apply to the Contract DBE Plan/Program:

2**************************************************************************************2

- a completed and signed Form CR-266 – Schedule of DBE/ESBE/SBE Participation listing each DBE firm being used to meet the Contract goal.

- a completed and signed Verification of DBE/ESBE/SBE Firm (Form CR-273) for each firm listed on the CR-266 to demonstrate direct written confirmation from each DBE firm of willingness to participate on the Contract, confirming the kind and amount of work that was provided on the Contractor’s CR-266, and, if applicable,

- a completed and signed DBE/ESBE/SBE Regular Dealer/Supplier Verification (Form CR-272) for all Regular Dealers/Suppliers listed on the CR-266 form, and, if applicable,

- a completed and signed DBE/ESBE/SBE Trucking Verification (Form CR-274) for all DBE trucking firms listed on the CR-266.

- a written explanation of each revision, and why

In such circumstances, do not complete any portion of the CR-273, CR-272, or CR-274 forms, and the Contractor shall meet the Modified DBE goal or demonstrate an adequate GFE.

1**************************************************************************************************************************1

BDC24S-04 dated jun 6, 2024

a.	Disadvantaged Business Enterprise/Emerging Small Business Enterprise Goals for this Contract

the last paragraph in part a changed to:

If the Contractor fails to meet the Contract DBE goal, without demonstrating an adequate GFE as determined by DCRAA, the Department will make a payment reduction from the total amount of payments made to the Contractor equal to the value of the DBE goal not attained as follows:

| Value of DBE Goal Payment Reduction = (CG – AG) x CP |  |
| --- | --- |
| Where: |  |
| CG = | Contract DBE Goal percentage, or approved DBE commitment, or if modified by the Department, the Modified DBE Contract Goal percentage. |
| AG = | Attained DBE Goal percentage = (total dollar amount paid to DBE suppliers and DBE subcontractors divided by CP) plus the percent value attributed to the Contractor’s GFE approved by the Department. |
| CP = | Total Adjusted Contract Price less the payment adjustments for PERFORMANCE BOND AND PAYMENT BOND, and changes not associated with actual work including but not limited to incentives/disincentives, bonuses/penalties, acceleration, and portions of settlements not associated with work. |

2*************************************************************************************2

THE FOLLOWING IS ADDED AT THE END OF PART A:

If the Contractor fails to meet the Contract DBE goal and DCR/AA deems the Contractor’s GFEs insufficient, the Department will provide the Contractor with an opportunity for an in-person meeting or written review of information and documentation provided with its Final DBE Report (Form CR 268). The Contractor will be required to make its case based on information it provided with Form CR 268.  No new DBE evidence (e.g. revised CR 268 or introduction of new information/documentation) will be considered. The Contractor may make a request for an in-person meeting or written review in writing to the New Jersey Department of Transportation, Director, Division of Procurement, PO Box 605, Trenton, New Jersey, 08625-0605. The Contractor must specify one of the following in their request:

(i)	Written Review by the Department.  If the Contractor seeks written review by the Department, it must submit written documented evidence or argument proving the Contractor submitted adequate good faith efforts; and include a narrative on what information DCR/AA failed to consider and/or where the regulations/rules were misapplied.

(ii)	In-Person Meeting.  If the Contractor seeks an in-person meeting by the Department, it must submit a written request for a meeting along with written documented evidence or argument proving the Contractor submitted adequate good faith efforts; and include a narrative on what information DCR/AA failed to consider and/or where the regulations/rules were misapplied. The in-person meeting will be scheduled by the Department as soon as time permits.

2************************************************************************************************************************2

b. Trainees

This contract includes a trainee goal which is part of the contractor’s equal employment opportunity affirmative action program, on-the-job training aimed at developing full journey people in the type of craft or job classification involved on the project as specified in section h of Federal Aid Project Attachment 2 – Specific Equal Employment Opportunity Responsibilities on NJDOT Federal Aid projects, of the Special Provisions.

1**********************************************************************************************************************1

105.05  Working Drawings

1**************************************************************************************************************************1

2**************************************************************************************2

BDC25S-21 dated mar 24, 2026

TABLE 105.05-1 is changed to:

| Table 105.05-1 – Working Drawing Submission Category |  |
| --- | --- |
| Certified | Approved |
| Breakaway I-Beam GA Sign Support Posts | Catalog Cuts (related to landscape Items) |
| Bridge Drainage | Change in Structural Steel Details |
| Bridge Railing and Fencing Anchorage System | Change of Prestressed Concrete Strand Patterns |
| Catalog Cuts | Demolition Plans |
| Composite Piles | Erection Plans |
| DMS Sign Support Structure | High Load Multi-Rotational (HLMR) Bearings |
| DMS Standard Ground Mounted | Isolation Bearings |
| Elastomeric Bearings Pads | ITS System Drawings, including Block Diagrams |
| Electrical Items Not Pre-Qualified | Machinery and Electrical Items for Movable Bridges |
| Expansion Joint Assemblies (except Modular Expansion Joint Assemblies) | Mechanically Stabilized Earth (MSE) Walls |
| Precast Prestressed Concrete Beams and Piles Fabrication | Modular Expansion Joint Assemblies |
| Reinforced Elastomeric Bearings | Other work shown on the Plans as conceptual |
| Sign Legends | Precast Concrete Arch Structures |
| Sign Support Structures | Precast Concrete Box Culverts |
| Structural Steel Fabrication | Prefabricated Modular Walls |
|  | Stay-In-Place Forms |
|  | Temporary Sheeting and Cofferdams |
|  | Temporary Shielding |
|  | Temporary Structures |
|  | Value Engineering Change Proposal Plans |

2**************************************************************************************2

2**************************************************************************************2

1.	Certified Working Drawings.

complete and include the following if the DEPARTMENT will REQUIRE less or more than 30 days to review and certify, or reject and return, working drawings. provide a justification to increased time frames and obtain project manager’s approval.

sme contact – Structural Design

the last sentence of the FIRST paragraph is changed to:

The Department will require ___ days for review and certification or rejection and return of certified working drawings.

2**************************************************************************************2

2**************************************************************************************2

2.	Approved Working Drawings.

complete and include the following if the REVIEWER will REQUIRE less or more than 45 days to review and approve, or reject and return, working drawings. provide a justification to increased time frames and OBTAIN APPROVAL FROM THE LPA.

the last sentence of the FIRST paragraph is changed to:

The Reviewer will require ___ days for review and approval or rejection and return of working drawings.

2**************************************************************************************2

1**************************************************************************************************************************1

105.07.01 Working in the Vicinity of Utilities

A.	Initial Notice.

1**************************************************************************************************************************1

list all utility companies located within the project limits.

include ADDRESSes and email ADDRESSes of contact personnel.

1**************************************************************************************************************************1

B.	Locating Existing Facilities.

FOR LOCAL AID PROJECTS-INCLUDE THE FOLLOWING IF IMPACTING A NJDOT FACILITY

1**************************************************************************************************************************1

2.

2**************************************************************************************2

Fiber Optic Markout Form is available at:

http://www.state.nj.us/transportation/eng/elec/ITS/requests.shtm.

2**************************************************************************************2

2**************************************************************************************2

include if a njdot facility is impacted

SELECT the APPROPRIATE TRAFFIC OPERATIONS OFFICE(s)

for Sussex, Passaic, Bergen, essex, Morris, warren, Hunterdon, Somerset, union, Hudson, and middlesex counties

Bureau of Traffic Operations, North Region (TOCN)

670 River Drive

Elmwood Park, NJ 07407-1347

Telephone: 732-697-7360

*****OR*****

For Mercer, Monmouth, Ocean, Burlington, Camden, Gloucester, Atlantic, Salem, Cumberland, and Cape May counties

Bureau of Traffic Operations, South Region (TOCS)

1 Executive Campus-Route 70 West

Cherry Hill, NJ 08002-4106

Telephone: 856-486-6650

2**************************************************************************************2

2**************************************************************************************2

include if a njDot facility is impacted

SELECT THE APPROPRIATE TRAFFIC OPERATIONS OFFICE, IN ADDITION, WHEN FACILITIES OTHER THAN FIBER OPTIC EXIST; SELECT OTHER ITS CONTACT FROM BELOW AS NECESSARY

For Weigh-in-Motion and Traffic Volume systems contact:

Transportation Data and Safety Unit

PO Box 600

Trenton, NJ 08625

609-963-1891

For Roadway Weather Information Systems contact:

Permits, Electrical Maintenance & Claims Unit

PO Box 600

Trenton, NJ 08625

609-963-1829

2**************************************************************************************2

3.

2**************************************************************************************2

include if a njdot facility is impacted

SELECT the APPROPRIATE electrical OFFICE(s)

Bureau of Electrical Maintenance, North Region

200 Stierli Court

Mt. Arlington, NJ  07856-1322

Telephone: 973-601-6650

*****OR*****

NJDOT

Central Region Electrical

1035 Parkway Avenue

4th Floor E&O Bldg.

CN 600

Trenton, NJ 08625

Telephone: 609-963-1491

*****OR*****

Bureau of Electrical Maintenance, South Region

One Executive Campus Route 70 West

Cherry Hill, NJ  08002-4106

Telephone: 856-486-6627

2**************************************************************************************2

1**************************************************************************************************************************1

C.	Protection of Utilities.

2**************************************************************************************2

BDC23S-16 dated DEC 18, 2023

THE FOLLOWING IS ADDED AFTER THE FOURTH PARAGRAPH:

Complete a Request for Right of Entry onto railroad property in compliance with the requirements of the specific railroad.

2**************************************************************************************2

2**************************************************************************************2

.

2**************************************************************************************2

2**************************************************************************************2

complete and include THE FOLLOWING WHEN a railroad is located within the project limits.

| Frequency of Trains |  |  |  |
| --- | --- | --- | --- |
| Location | Speed | Number Per Day | Time |

2**************************************************************************************2

1**************************************************************************************************************************1

105.07.02  Work Performed by Utilities

1**************************************************************************************************************************1

BDC26S-03 dated Apr 24, 2026

The subpart is changed to:

The Contractor is responsible for coordinating work performed by Utilities, so that the Utilities’ work does not conflict with the Contractor’s own work, the operation of existing facilities, and work performed by Others.  Coordination includes but is not limited to scheduling the utility work, monitoring the utility work, and ensuring that traffic control and access to the work location is as specified in the Contract.  The Contractor is responsible for delays and costs resulting from failure to coordinate.  Provide a written request to each Utility in the time specified for the advance notice requirements specified in the Special Provisions.  Include the following:

1.	Name and location of the Project.

2.	Name and contact information of the Contractor and Superintendent.

3.	Portion of the approved preliminary schedule or baseline schedule that affects the Utility.

2**************************************************************************************2

complete and include the following for all utility companies that are affected by the project for the advance notice requirements. ensure number of (day/s) advanced notice are based on Calendar days as related to the schedule

provide these affected utility companies with a compact disc containing electronic files of plans & Special provisions

| Company Name & Address | Contact Person | Number of Day/s Advance Notice |
| --- | --- | --- |

2**************************************************************************************2

Provide a copy of the notice and response to the RE.

Where Utilities jointly use poles or duct banks, coordinate the utility work to be performed sequentially.

Ensure that the work site is in a condition that allows the Utility to perform its work at the scheduled time.  If the Contractor fails to provide the work site at the scheduled time, the Contractor is responsible for the resulting delays and costs to the Project.  If the Contractor causes the Utility to incur additional costs, or delays the Utility without prior written approval of the RE, the Contractor is responsible for these costs and delays.  The Department has the right to recover the cost of damages from the Contractor.

Ensure the progress schedule updates, as specified in 153.03, reflect impacts to project duration due to utility work progress.

Immediately notify the RE of failure by the Utility to respond or complete its work as specified in the Special Provisions.

2**************************************************************************************2

list work per stage to be performed by the utility companies. include a general description of the WORK to be performed by the company and provide company TIME FRAME to Mobilize on-site and to perform the work. ensure work duration (day/s) are based on Calendar days as related to the schedule. include seasonal restrictions for the work. ensure that any staging and multiple company coordination is properly accounted for in all contract documents.

| Stage # ______ |  |  |  |
| --- | --- | --- | --- |
| Utility Company Name | Work Description | Work Duration (Day/s) | Restriction |
|  | Stage Total | _________________ |  |

2**************************************************************************************2

1**************************************************************************************************************************1

105.09 LOAD Restrictions

1**************************************************************************************************************************1

BDC20S-05 dated jun 5, 2020

Complete and include the following MTV Structural Load Restriction table for all paving projects

Add other MTV vehicles which are identified By the industry

THE FOLLOWING IS ADDED:

| Material Transfer Vehicles Structural Load Restriction |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Route | Bridge Structures |  | Structure Name | Material Transfer Vehicles |  |  |  |  |
|  | Number | Mile Post |  | MTV SB-2500C by Roadteck | MTV MC-330 by Blaw-Knox | MTV SB-1500B by Roadteck | MTV SB-1000B by Roadteck | ADD OTHER MTV |
|  |  |  |  | Gross Weight 125,500 lbs | Gross Weight 114,100 lbs | Gross Weight 92,000 lbs | Gross Weight 74,000 lbs |  |
|  |  |  |  | Allowed, Allowed with Speed Limit or Not Allowed |  |  |  |  |

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

THE FOLLOWING SUBSECTION IS ADDED:

105.11 Construction Layout

2**************************************************************************************2

BDC24S-04 dated apr 18, 2024

The Last sentence of the first paragraph is changed to:

Perform a site investigation within the Project Limits to locate existing ROW markers, property markers, survey control markers, and monuments.  Before beginning construction operations, protect ROW markers, property markers, survey control markers, and monuments from disturbance and destruction.  Notify the RE of the location of property markers and monuments that are in danger of being removed or disturbed and tie to fixed points or locate from established contract control.  Do not remove or disturb existing ROW markers, property markers, survey control markers, and monuments until obtaining RE approval.  Record and provide a report of the findings of the site investigation to the RE.

2**************************************************************************************2

Before removing a monument that is not owned by the Department, notify the agency to which the monument belongs of the need to remove the monument.  Provide the RE with copies of correspondence with the agency, as well as the agency’s written requirements or guidelines for setting monuments.

If a monument or marker is disturbed or removed without RE approval, the Contractor is responsible for reimbursing the Department for the cost to replace the monument.

Perform layout for the construction of the Contract using the control points and data shown on the Plans.  Perform layout under the direct supervision of a licensed land surveyor.  Preserve control points throughout the duration of the Project.  Reset control points that are damaged, lost, displaced, or removed.

Before beginning construction operations, verify the vertical and horizontal controls provided in the Plans using, at a minimum, third-order, Class I accuracy procedural standards and equipment.  Notify the RE in writing of discrepancies or errors and obtain resolution before proceeding with the work.  Upon request, provide the RE with survey notes and calculations related to the field control verification.

Before beginning construction operations, establish lines for ROW, easement, and other restrictions, such as boundaries for environmentally sensitive areas to define the limits of construction and temporary operations.  Establish limits of tree protection and other vegetation to be preserved.  Do not encroach on private property, except as allowed by easements.  Document existing site conditions, including vegetation, in areas to be used for excavation, temporary construction, storage, parking, movement of equipment, field office, etc. which will have to be later restored to pre-existing conditions.  Provide documentation, including photographs, to the RE.

Provide the Utilities with the layout needed to install relocated utility facilities and coordinate the Work.  Ensure that relocated facilities do not conflict with proposed construction, including High Voltage Proximity Act conflicts.

Establish the exact location of the Work from the control points.  Reference the Work to baselines that are established from the control points.  Maintain baselines until Completion.

Provide and maintain offset baseline stakes for roadways, ramps, jughandles, and turnarounds outside the limits of grading and construction.  Set offset stakes at a maximum interval spacing of 50 feet.  Where baselines have a radius of less than 475 feet, provide offset stakes at a maximum interval spacing of 25 feet.  Identify and mark each stake to show the offset distance from the baseline and provide grade sheets to the RE showing the cut or fill to the finished profile lines with reference to the offset stakes.  Provide grade sheets for construction of subbase that include calculations to establish the typical cross section from the profile grade stake.  Provide adequate and accurate offset lines during construction that requires occupation of the baseline points by construction operations.  Provide the RE with assistance as requested for verification of lines, grades, boundaries, dimensions, and elevations.

Construct the Work to the dimensions and tolerances noted within the Contract.  Except where otherwise noted, construct subbase, base courses, pavements, and structures to within 1/4 inch of the elevations, stations and offsets noted in the Contract.  If work does not conform to the tolerances allowed, the Department may consider the work to not be in conformance with the work as specified in 105.03.

For each bridge and sign structure within the Project Limits, provide the RE as-built measurements of the minimum vertical underclearance at each lane line, shoulder line, curb line, and edge of pavement line under a structure to the nearest hundredth of a foot. For each bridge structure, provide vertical underclearance measurements at each fascia beam and the portions of the structure that govern the minimum vertical underclearance.  Provide minimum vertical underclearance measurements prior to the completion of each stage of construction.  Notify the RE in writing of any discrepancies, errors, or deviations from plan dimensions and clearances prior to opening any bridge or structure or any portion thereof to traffic.

Upon request, provide the RE with survey notes and calculations related to the alignment and horizontal and vertical control, and field notes to document the ROW, including easements and monument locations.  Maintain survey notes in a bound field notebook in a professional manner.

The Department will not make payment for survey, stakeout, and layout.  The cost is to be distributed across the various items of work and to be included in extra work as may be added to the Contract.

If the Department discovers survey errors, including errors that should have been detected during verification of controls provided in the plans, the Department will deduct the costs of checking and correcting these errors from any money due to the Contractor.

1**************************************************************************************************************************1

Section 106 – Control of Material

1**************************************************************************************************************************1

BDC20S-14 dated Oct 09, 2020

THE SECTION HEADING IS CHANGED TO:

Section 106 – Control of Material and Equipment

1**************************************************************************************************************************1

106.01  Source of Supply and Quality Requirements

1**************************************************************************************************************************1

2**************************************************************************************2

BDC22S-17 dated DEC 23, 2022

Determine if NJSA 52:32-62 ‘Required use of unit concrete products that utilize carbon footprint-reducing technology’ and NJSA 52:27D-141.17 ‘Standards, procedures, implementation’ are relevant to the project, and add the appropriate specifications, requirements, and items as necessary to comply with the Statute.

2**************************************************************************************2

2**************************************************************************************2

BDC21S-04 dated AUG 23, 2021

the first paragraph IS changed to:

Ensure that materials furnished for the Project are new, unless otherwise specified in the Contract.  Comply with 2 CFR 200.323 – Procurement of recovered materials, ensuring that materials furnished for the Project contain, “the highest percentage of recovered materials practicable,” where the purchase price of the covered item listed exceeds $10,000.  Use materials that conform to the requirements of the Contract.  When required by the Contract, use only products and suppliers listed on the QPL.  Use sources of materials that have been approved by the Department on a Materials Questionnaire as specified in 106.04.

2**************************************************************************************2

2**************************************************************************************2

BDC20S-14 dated Oct 09, 2020

THE FOLLOWING IS ADDED TO THE END OF THE SUBSECTION:

For telecommunication and video surveillance services or equipment a certification is required to confirm that the telecommunication and video surveillance services or equipment are not from companies as listed in 2 CFR 200.216. Ensure that the certification includes the statement that all telecommunication and video surveillance services or equipment proposed in this project are not produced by companies as listed in 2 CFR 200.216.

2**************************************************************************************2

1**************************************************************************************************************************1

106.02 DEPARTMENT-Furnished Material

1**************************************************************************************************************************1

include the type of materials and their location

1**************************************************************************************************************************1

106.03 FOREIGN Materials

1**************************************************************************************************************************1

BDC20S-14 dated Oct 09, 2020

THE SUBSECTION HEADING IS CHANGED TO:

106.03 FOREIGN Materials and Equipment

1**************************************************************************************************************************1

2**************************************************************************************2

2*************************************************************************************2

BDC23S-21 dated feb 19, 2024

2.	Federal Aid Projects.  Comply with the Federal statutes and regulations that establish the “Buy America” requirements applicable to the Project.  These Federal requirements, which are contained in 23 CFR 635.409 and 23 CFR 635.410, mandate among other things that manufacturing processes for iron and steel materials and all iron and steel coatings must be performed in the United States.

Comply with the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 and implementing regulations.  Comply with the requirements for IIJA’s four categories as described in 2 CFR 184.4(e): iron or steel products, manufactured products, construction materials, and exempt Section 70917(c) materials.  Per 23 CFR 635.410(c)(2), an article, material, or supply is to be classified as one of the four categories as defined in 2 CFR 184.3 and must not be considered to fall into multiple categories.  Ensure that:

a.	All iron or steel products as defined in 635.410(c)(1) used in the project comply with “Buy America” requirements for steel and iron per CFR 635.410(b).  Ensure all iron or steel products used in the project are produced in the United States.  Iron and steel products means articles, materials, or supplies that consist wholly or predominantly of iron or steel or a combination of both.  Ensure all manufacturing processes, from the initial melting stage through the application of coatings, occurred in the United States.

b.	All manufactured products comply with the requirements in the Special Provisions.

3************************************************3

Select the appropriate paragraph for the requirement for all manufactured products based on the advertisement date for the project.

INCLUDE THE FOLLOWING PARAGRAPH ONLY FOR PROJECTS TO BE ADVERTISED BEFORE OCTOBER 1, 2025.

b.	All manufactured products are exempted from the Buy America requirements pursuant to 48 FR 50399 (1983) which excludes manufactured products from 23 CFR 635.410.  Manufactured products means articles, materials, or supplies that have been processed into a specific form and shape; or combined with other articles, materials, or supplies to create a product with different properties than the individual articles, materials, or supplies.

*****OR*****

INCLUDE THE FOLLOWING PARAGRAPH ONLY FOR PROJECTS TO BE ADVERTISED ON AND BETWEEN OCTOBER 1, 2025 AND SEPTEMBER 30, 2026.

b.	Final assembly for all manufactured products permanently incorporated into the project occurs in the United States.  Manufactured products means articles, materials, or supplies that have been processed into a specific form and shape; or combined with other articles, materials, or supplies to create a product with different properties than the individual articles, materials, or supplies.

*****OR*****

INCLUDE THE FOLLOWING PARAGRAPH ONLY FOR PROJECTS TO BE ADVERTISED ON OR AFTER OCTOBER 1, 2026

b.	All manufactured products permanently incorporated into the project are produced in the United States. “Produced in the United States” means both the final assembly of the product occurs in the United States and the cost of the components of the manufactured product that are mined, produced, or manufactured in the United States is greater than 55 percent of the total cost of all components of the manufactured product.  Manufactured products means articles, materials, or supplies that have been processed into a specific form and shape; or combined with other articles, materials, or supplies to create a product with different properties than the individual articles, materials, or supplies.

3************************************************3

c.	All construction materials are manufactured in and manufacturing processes occurred in the United States.  Construction materials includes an article, material, or supply - other than an item of primarily iron or steel; a manufactured product; or Section 70917(c) material – that is or consists primarily of non-ferrous metals, plastic and polymer-based products (including PVC, composite building materials, and polymers used in fiber optic cables), glass (including optic glass), fiber optic cable (including drop cable), optical fiber, lumber, engineered wood, or drywall.  Items that consist of two or more of the listed materials that have been combined through a manufacturing process, and items that include at least one of the listed materials combined with a material that is not listed through a manufacturing process, should be treated as manufactured products, rather than as construction materials.

d.	Build America, Buy America does not extend to materials listed in Section 70917(c) of the Act.  These exempt items are cement and cementitious materials; aggregates such as stone, sand, or gravel; or aggregate binding agents or additives.

For steel and iron products incorporated into the Project, provide the RE a certification from the manufacturer stating the country where the steel or iron product was melted and manufactured including application of coatings which protect or enhance the value of the material.  Ensure that 4 copies of the manufacturer’s certification are provided with each delivery of steel and iron products.  Retain 1 copy and submit 3 copies to the RE.  Ensure that the certification includes, materials description, quantity of material represented by the certification, country of manufacture, and notarized signature of a person having legal authority to bind the supplier.  If a Certification of Compliance as specified in 106.07 contains a statement regarding the country of manufacture, a separate certification is not necessary.

If the use of foreign materials is allowed, the Department may require the Contractor to deliver the material to a Department-approved site for sampling, inspection, and testing.  Do not use or incorporate the foreign material into the Work before the Department approves the material.

If the Department finds the Contractor failed to comply with the Federal or State provisions regarding foreign materials, the Department will require the unapproved foreign material be removed and replaced with acceptable material at no additional cost to the Department.

Comply with 2 CFR 200.216 Prohibition on Certain Telecommunication and Video Surveillance Services or Equipment.

Do not provide Telecommunications equipment produced by Huawei Technologies Company or ZTE Corporation (or any subsidiary or affiliate of such entities).  Do not provide video surveillance and telecommunications equipment produced by Hytera Communications Corporation, Hanghzou Hikvision Digital Technology Company, or Dahua Technology Company (or any subsidiary or affiliate of such entities).  Do not provide Telecommunications or video surveillance services provided by such entities or using such equipment.

Do not provide Telecommunications or video surveillance equipment or services produced or provided by an entity that the Secretary of Defense, in consultation with the director of the National Intelligence or the Director of the Federal Bureau of Investigation, reasonably believes to be an entity owned or controlled by, or otherwise connected to, the government of a covered foreign country.

Do not provide any equipment assembled by others that has an integral component that was manufactured and supplied by the aforementioned companies.

Build America, Buy America domestic preferences for iron or steel products, manufactured products, and construction materials requirements are waived for projects where: 1. The total value of non-compliant products is no more than the lesser of $1,000,000.00 or 5 percent of total applicable costs for the project. This does not apply to iron or steel subject to the requirements of 23 USC 313; or 2. The total amount of Federal financial assistance applied to the project is below $500,000.00. Refer to the August 16, 2023 Federal Register for USDOT’s issuance of a Waiver of Buy America Requirements for De Minimis Costs and Small Grants at the following link: Federal Register: Waiver of Buy America Requirements for De Minimis Costs and Small Grants.

2**************************************************************************************2

1**************************************************************************************************************************1

106.04  Materials Questionnaire

1**************************************************************************************************************************1

BDC25S-13 dated aug 19, 2025

the first paragraph is changed to:

At least 30 days before shipment of a material, and at least 30 days before fabrication of structural steel, precast concrete, and prestressed concrete Items, notify the RE of the proposed source of materials using the Materials Questionnaire form provided by the Department.  For soil aggregate and dense-graded aggregate (DGA) material, submit a copy of recently performed test results for washed gradation test, proctor test, and other tests, as specified in 901.11, for the source with the Materials Questionnaire.  For ITS systems as specified in Section 704 and machinery and electrical items for moveable bridges, obtain approval of system working drawings including individual components and Electrical material prior to submitting a materials questionnaire.  Provide the approval with the materials questionnaire.

1**************************************************************************************************************************1

106.07.01 Certification of Compliance

1**************************************************************************************************************************1

BDC22S-20 dated Sep 8, 2023

THE FIRST PARAGRAPH IS CHANGED TO:

Submit manufacturer’s Certifications of Compliance stating that the materials and assemblies fully comply with the requirements of the Contract when required by the Contract or requested by the Department, except for materials and assemblies that are temporary and not incorporated into the final construction and are not iron or steel such as sheeting and bridge assemblies.  The State reserves the right to determine if a specific material or assembly meets this provision.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC24S-22 dated Feb 10, 2025

The third paragraph is changed to:

Before incorporating the materials into the Project, obtain 3 copies of the manufacturer's Certifications of Compliance for materials, components, and manufactured items that are accepted by certification.  Retain 1 copy and submit 2 copies to the RE.  With the Certification of Compliance, provide a DC-1811, Certificate of Compliance Cover Sheet, identifying the Item for which it is submitted.  For products that contain steel or iron, attach additional documents as required by the certification procedures as specified in 106.07.02.  The Contractor may submit the Certifications of Compliance electronically to the RE in a scanned document.  Include the DC-1811 and all backup documentation in the scanned document.

1**************************************************************************************************************************1

106.07.02 Certification for Iron and Steel

1**************************************************************************************************************************1

BDC22S-14 dated Dec 16, 2022

The Heading and the entire subpart is changed to:

106.07.02 Certification for Iron and Steel, and Construction Materials

A.	Precast Concrete Steel and Concrete Pipe Certification of Compliance.  For precast concrete and concrete pipe items, a Buy America Compliance Plan is required to confirm that the material meets the Buy America requirements and the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 as specified in 106.03. The ME will periodically audit compliance with the program at the precast plant. If the precast concrete item is not inspected by ME, submit a Certification of Compliance for the precast concrete item as required in 106.07.01. When a Certification of Compliance is submitted, ensure that the Certification of Compliance contains a statement that the reinforcing steel used in the precast concrete item complies with the Buy America requirements and the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 as specified in 106.03.

B.	Certification for Construction Materials.  For construction materials, a Buy America Compliance Plan is required to confirm that the material meets the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 as specified in 106.03. When a Certification of Compliance is submitted, ensure that the Certification of Compliance contains a statement that the construction materials used complies with the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 as specified in 106.03.

C.	Step Certification of Compliance.  For products that contain steel or iron components and are not covered in 106.07.02.A, step Certification of Compliance is required to confirm that the item meets the Buy America requirements and the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52 as specified in 106.03. A step certification is a process under which each handler (e.g., supplier, fabricator, manufacturer, processor, coating facility) of the iron and steel components certifies that the steel and iron components were of domestic origin, and that their step in the process was domestically performed.

Every step in the process from melting to coating must be performed in the United States in order for the steel or iron component to be considered domestic and must be documented by step certification. If a domestic source for a steel or iron component cannot be found, submit a request for waiver to the Department. Do not purchase non-domestic steel or iron components without the express written consent of the Department.

Ensure that 3 copies of the Contractor’s Certification of Compliance (Form DC-17) and the step Certifications of Compliance are provided for items containing steel or iron. Retain 1 copy and submit 2 copies to the RE. The Contractor may submit the DC-17 and the step certifications electronically in a scanned document.

2**************************************************************************************2

BDC24S-22 dated Feb 10, 2025

the fourth paragraph is changed to:

Ensure that step Certifications of Compliance contain the following information for each heat of material listed in the DC-17:

1.	Name of the Company supplying the material.

2.	Name and location of the Company the material was shipped to.

3.	Material description.

4.	Quantity of material represented by the Certification.

5.	Means of identifying the consignment, such as label marking or seal number.

6.	Date and method of shipment.

7.	A statement that the material conforms to the Contract material requirements and to the Buy America requirements in 106.03 and the Infrastructure Investment and Jobs Act (“IIJA”), Pub. L. No. 117-58, which includes the Build America, Buy America Act (“the Act”). Pub. L. No. 117-58, §§ 70901-52.

8.	A statement that all steel or iron components in the material or assembly were “melted and manufactured in the US”, unless there is non-domestic steel or iron in the material or assembly.

9.	If there is non-domestic steel or iron in the assembly, describe in detail the non-domestic steel or iron material and the quantity.  Attach a copy of the Department’s approval for the use of non-domestic steel or iron components.

10.	Signature of a person having legal authority to bind the supplier.

11.	Typed or printed name of the person who signed the certification.

2**************************************************************************************2

The Department will not make payment for work containing steel or iron materials until the RE has received the required DC-17 and step Certifications of Compliance, has inspected and accepted the material or assembly.

1**************************************************************************************************************************1

106.09 SUBSTITUTES for Proprietary Items

1**************************************************************************************************************************1

2**************************************************************************************2

the lpa will determine if a substitute proprietary item or the name of a particular supplier will not be permitted for a work. specify in the appropriate subsection where the intended work is specified, as follows:

2**************************************************************************************2

2**************************************************************************************2

include “No substitution is permitted” statement in the appropriate subsection when a substitution in such work is not permitted

for example: if a proprietary fence material is needed, then include the “No substitution is permitted” statement in subsection 605.02.01.

2**************************************************************************************2

1**************************************************************************************************************************1

Section 107 – Legal Relations

107.02 NONDISCRIMINATION

It is the policy of the Department that anyone performing work under any program, activity, or Contract with the Department, shall not discriminate on the basis of race, creed, color, national origin, age, ancestry, nationality, marital or domestic partnership status, gender, disability, affectional or sexual orientation, gender identity or expression, religion, liability for military service, veteran’s status, income level or ability to read, write or speak English.

Pursuant to N.J.S.A. 10:2-1, the Contractor agrees that in the hiring of persons for the performance of work under this Contract or any subcontract hereunder, or for the procurement, manufacture, assembling, or furnishing of any such materials, equipment, supplies, or services to be acquired under this Contract, no contractor, nor any person acting on their behalf of such contractor or subcontractor, shall by reason of race, creed, color, national origin, ancestry, marital status, gender identity or expression, affectional or sexual orientation or sex, discriminate against any person who is qualified and available to perform the work to which the employment relates;

No Contractor, subcontractor, nor any person on his behalf shall, in any manner, discriminate against or intimidate any employee engaged in the performance of work under this contract or any subcontract hereunder, or engaged in the procurement, manufacture, assembling, or furnishing of any such materials, equipment, supplies, or services to be acquired under such contract, on account of race, creed, color, national origin, ancestry, marital status, gender identity or expression, affectional or sexual orientation or sex.

There may be deducted from the amount payable to the contractor by the contracting public agency, under this contract, a penalty of $50.00 for each person for each day during which such person is discriminated against or intimidated in violation of the provisions of the contract; and this contract may be terminated by the Department, and all money due or to become due hereunder may be forfeited, for any violation of this section of the contract occurring after notice to the Contractor from the Department of any prior violation of this section of the contract.

Standard Title VI Assurance. During the performance of this Contract, the Contractor, for itself, its assignees, and successors in interest (hereinafter referred to as the “Contractor”), in accordance with Title VI/Nondiscrimination Assurance – Appendix A, USDOT Order 1050.2A agrees as follows:

1. Compliance with Regulations: The Contractor will comply with the Acts and Regulations relative to Nondiscrimination in Federally assisted programs of the U.S. Department of Transportation, Federal Highway Administration (FHWA), as they may be amended from time to time, which herein incorporated by reference and made a part of this Contract.

2. Nondiscrimination: The Contractor, with regard to the Work performed by it during the Contract, will not discriminate on the grounds race, creed, color, national origin, age, ancestry, nationality, marital or domestic partnership status, gender, disability, affectional or sexual orientation, gender identity or expression, religion, liability for military service, veteran’s status, income level or ability to read, write or speak English in the selection and retention of subcontractors, including procurement of materials and leases of equipment. The Contractor will not participate directly or indirectly in the discrimination prohibited by the Acts and Regulations, including employment practices when the Contract covers any activity, project, or program set forth in Appendix B of 49 CFR Part 21.

3. Solicitations for Subcontracts, Including Procurement of Materials and Equipment: In all solicitations, either by competitive bidding, negotiation made by the Contractor for Work to be performed under a subcontract, including procurements of materials, or leases of equipment, each potential subcontractor or supplier will be notified by the Contractor of the Contractor’s obligations under this Contract and the Acts and Regulations relative to nondiscrimination on the grounds of race, creed, color, national origin, age, ancestry, nationality, marital or domestic partnership status, gender, disability, affectional or sexual orientation, gender identity or expression, religion, liability for military service, veteran’s status, income level or ability to read, write or speak English.

4. Information and Reports: The Contractor will provide all information and reports required by the Acts, the Regulations, and directives issued pursuant thereto and will permit access to its books, records, accounts, other sources of information, and its facilities as may be determined by the Recipient or the FHWA, to be pertinent to ascertain compliance with such Acts, Regulations, and instructions. Where any information required of a Contractor is in the exclusive possession of another who fails or refuses to furnish the information, the Contractor will so certify to the Recipient or the FHWA, as appropriate, and will set forth what efforts it has made to obtain the information.

5. Sanctions for Non-Compliance: In the event of a Contractor’s noncompliance with the Nondiscrimination provisions of this Contract, the Recipient will impose such Contract sanctions as it or the FHWA may determine to be appropriate, including, but not limited to the following: a. Withholding payments to the Contractor under the Contract until the Contractor complies. b. Cancelling, terminating, or suspending a Contract, in whole or in part.

6. Incorporation of Provisions: The Contractor will include the provisions of paragraphs one through six in every subcontract, including procurements of materials and leases of equipment, unless exempt by the Acts, the Regulations and directives issued pursuant thereto. The Contractor will take action with respect to any subcontract or procurement as the Recipient or the FHWA may direct as a means of enforcing such provisions including sanctions for non-compliance. Provided, that if the Contractor becomes involved in, or is threatened with litigation by a subcontractor, or supplier because of such direction, the Contractor may request the Recipient to enter into any litigation to protect the interests of the Recipient. In addition, the Contractor may request the United States to enter into the litigation to protect the interest of the United States.

During the performance of this Contract, the Contractor, for itself, its assignees, and successors in interest (hereinafter referred to as the “Contractor”) in accordance with the Title VI /Nondiscrimination Assurance – Appendix E, USDOT Order 1050.2A, agrees to comply with the following nondiscrimination statutes and authorities; including but not limited to:

a. Title VI of the Civil Rights Act of 1964 (42 U.S.C. § 2000d et seq., 78 stat. 252); and 49 CFR Part 21.

b. The Uniform Relocation Assistance and Real Property Acquisition Policies Act of 1970, (42 U.S.C. § 4601).

c. Section 162(a) of the Federal Aid Highway Act of 1973, (23 U.S.C. § 324 et seq.)

d. Section 504 of the Rehabilitation Act of 1973, (29 U.S.C. § 794 et seq.), as amended; and 49 C.F.R. Part 27.

e. The Age Discrimination Act of 1975, as amended, (42 U.S.C. § 6101 et seq.)

f. Airport and Airway Improvement Act of 1982, (49 U.S.C. § 471, Section 47123), as amended.

g. The Civil Rights Restoration Act of 1987, (PL 100-209).

h. Title II and III of the Americans with Disabilities Act (42 U.S.C. § 12131 - 12189) as implemented by Department of Transportation regulations at 49 C.F.R. parts 37 and 38.

i. The Federal Aviation Administration’s Nondiscrimination statute (49 U.S.C. § 47123).

j. Executive Order 12898, Federal Actions to address Environmental Justice in Minority Populations and Low-Income Populations.

k. Executive Order 13166, Improving Access to services for Persons with Limited English Proficiency (70 Fed. Reg. at 74087 to 74100).

l. 23 CFR Part 230 (EEO, Affirmative Action & OJT).

m. 49 CFR Part 26.

n. Executive Order 11246, as amended.

o. Section 503 of the Rehabilitation Act of 1973, as amended.

p. Section 4212 of the Vietnam Era Veteran’s Readjustment Assistance Act, as amended.

q. New Jersey Statutes N.J.S.A. 10:5-31 et seq.

r. New Jersey P.L. 1975 Chapter 27.

107.03 AFFIRMATIVE ACTION, DISADVANTAGED BUSINESS ENTERPRISES OR EMERGING SMALL BUSINESS ENTERPRISES, AND SMALL BUSINESS ENTERPISES

It is the public policy of the State and of the United States that no individual, group, firm, corporation or joint venture working on or seeking to work on a Public Works Project should be discriminated against on the basis of race, creed, color, national origin, age, ancestry, nationality, marital or domestic partnership status, gender, disability, liability for military service, affectional or sexual orientation, atypical cellular or blood trait, or genetic information (including the refusal to submit to genetic testing). The Department has developed Affirmative Action, Disadvantaged Business Enterprise, or Emerging Small Business Enterprise Programs to implement this policy, and the regulations and requirements applicable to the Contract are contained in the Special Provisions. The Department will resolve conflicts between these regulations and requirements and the other provisions of the Contract to further the above stated public policy.

Contract Assurance. The Contractor, sub-recipient or subcontractor shall not discriminate on the basis of race, color, national origin, or sex in the performance of this Contract. The Contractor shall carry out applicable requirements of 49 CFR Part 26 in the award and administration of DOT-assisted contracts. Failure by the Contractor to carry out these requirements is a material breach of this Contract, which may result in the termination of this contract or such other remedy as the Department deems appropriate, which may include, but is not limited to one or more of the following:

1. Withholding monthly progress payments.

2. Assessing sanctions.

3. Liquidated damages.

4. Disqualifying the Contractor from future bidding as non-responsive.

107.11 RISKS Assumed by the Contractor

107.11.01 Risks

1**************************************************************************************************************************1

1.	Damage Caused by the Contractor.

For projects within the limits noted below, the designer shall include the following additional clause with the respective limits for that project.  If working within the vicinity of the Turnpike, Garden State parkway, or Atlantic City Expressway, note those also.

Rt 38 MP 0.5 to 9.5

RT 42 mp 6.3 to 13.3

Rt 70 MP 0.0 to 5.4

I-80 MP 57.5 to 68.3

I-295 MP 40.6 to 67.79

I-95 MP 0.58 to 8.77

I-195 MP 0.00 to 6.25

The following is added:

For any damages by the Contractor to the fiber optic network along Route _____, MP _____, also notify the G4S Technology LLC at 877-637-2344 within 2 hours.  Only G4S Technology LLC will be allowed to complete repairs on that respective section of the fiber optic network.  Directly pay G4S Technology LLC within 30 days from the receipt of G4S Technology LLC’s invoice for such repairs and provide the RE with a copy of the transmittal letter.  If the Contractor does not make payment within 30 days, the Department may recover the costs incurred for repairs as specified in 107.16.

1**************************************************************************************************************************1

BDC252S-01 dated apr 03, 2025

2.	Risks of Loss or Damage to the Construction.

The 2nd paragraph under Part 2 is changed to:

The Contractor shall not bear such risk of loss or damage for the repair or replacement of any permanent element of work and any temporary crash cushion items as specified in 159.03.02.5, if the element of the work damaged is completed and conforms to the contract as determined by the Department, and is serving its intended function, is subsequently damaged by a public traffic accident, and the Contractor provides the RE satisfactory evidence that such damage was caused by a public traffic accident.  Satisfactory evidence is limited to:

a.	Accident reports filed with the New Jersey Motor Vehicle Commission.

b.	Documents supporting the damage issued by police agencies or insurance companies.

c.	Statements by reliable, unbiased eye witnesses.

d.	Identification of the vehicle involved in the accident.

1**************************************************************************************************************************1

107.11.02  General Insurance

B.	Types

1**************************************************************************************************************************1

2**************************************************************************************2

1.	Comprehensive General Liability Insurance.

If JCP&L related utility work is to be performed, add the following:

the following is added:

Ensure the policy names JCP&L, its officers, employees, and agents as additional insured.

2**************************************************************************************2

or

2**************************************************************************************2

bdc25s-11 dated july 11, 2025

If PSE&G related GAS utility work is to be performed, add the following:

the following is added:

Ensure the policy names Public Service Electric and Gas Company, its officers, employees, and agents as additional insured.

2**************************************************************************************2

2**************************************************************************************2

2.	Comprehensive Automobile Liability Insurance.

If JCP&L related utility work is to be performed, add the following:

the following is added:

Ensure the policy names JCP&L, its officers, employees, and agents as additional insured.

2**************************************************************************************2

or

2**************************************************************************************2

bdc25s-11 dated july 11, 2025

If PSE&G related GAS utility work is to be performed, add the following:

the following is added:

Ensure the policy names Public Service Electric and Gas Company, its officers, employees, and agents as additional insured.

2**************************************************************************************2

2**************************************************************************************2

5.	Excess Liability Insurance.

If JCP&L related utility work is to be performed, add the following:

the following is added:

Ensure the policy names JCP&L, its officers, employees, and agents as additional insured.

2**************************************************************************************2

or

2**************************************************************************************2

bdc25s-11 dated july 11, 2025

If PSE&G related GAS utility work is to be performed, add the following:

THE FIRST SENTENCE IS CHANGED TO:

Procure Excess Liability or Umbrella Liability insurance with limits in excess of the underlying policies for Comprehensive General Liability and Comprehensive Automobile Liability with minimum limits of liability of $12,000,000.

the following is added:

Ensure the policy names Public Service Electric and Gas Company, its officers, employees, and agents as additional insured.

2**************************************************************************************2

1**************************************************************************************************************************1

107.12.01 Satisfying the Notice Requirements

1***********************************************************************************************1

ADD THE FOLLOWING FOR LOCAL AID PROJECTS

For Local Aid Projects all reference to the contractual notice form is construed to mean the format required by the LPA.

dated Oct 30, 2019

The second sentence of the second paragraph is changed to:

Upon request, provide the RE with 5 copies of all documentation submitted in support of the claim.

107.12.02 Steps

for local aid projects section 107.12.02 steps is deleted and replaced with the requirements of the lpa.

This section is deleted and replaced with the requirements of the LPA as follows:

1**************************************************************************************************************************1

Section 108 – Prosecution and Completion

108.01 SUBCONTRACTING

Do not discriminate on the grounds of race, creed, color, national origin, age, ancestry, nationality, marital/domestic partnership/civil union status, gender, disability, religion, affectional or sexual orientation, gender identity or expression, family status, atypical cellular or blood trait, genetic information, military service, or veterans status, in the selection and retention of subcontractors, including procurement of materials and leases of equipment. In all solicitations, either by competitive bidding, or negotiation made by the Contractor for work to be performed under a subcontract, including procurement of materials, leases of equipment, or professional services, each potential subcontractor or firm will be notified by the Contractor of the Contractor’s obligations under this Contract and the Acts and Regulations relative to Nondiscrimination.

The Department will not permit subcontracting without Department approval. The Contractor is responsible for the work performed by subcontractors. Ensure that no work is performed by a subcontractor before receiving written approval for each subcontractor from the Department. Ensure that DBEs/ESBEs have an equal opportunity to receive and participate in the performance of contracts and subcontracts financed in whole or in part with Federal funds in performing work with the Department. Ensure that SBEs have an equal opportunity to receive and participate in the performance of contracts financed in whole with State funds in performing work with the Department. Utilize the specific DBEs, ESBEs, or SBEs listed to perform work and supply materials for which each is listed unless the written consent of DRR/AA is provided. Ensure that work reserved for a subcontractor designated as a DBE, ESBE, or SBE, is not performed by any other firm, including the Contractor’s own organization. Submit requests for approval to subcontract on Department forms to the Department at least 20 days before the anticipated start of the work with the following:

1. A certified copy of the executed subcontract agreement between the Contractor and the subcontractor.

2. Proof of the subcontractor’s valid business registration with the Department of Treasury, Division of Revenue according to N.J.S.A. 52:32-44.

3. Proof of the subcontractor’s valid Public Works Contractor Registration with the Department of Labor, Division of Wage and Hour Compliance according to N.J.S.A. 34:11-56.18.

Make available on request, a copy of all DBE, ESBE, and SBE subcontracts. Ensure that all subcontracts or agreements with DBEs or ESBEs to supply labor or materials require that the subcontract and all lower tier subcontractors be performed in accordance with 49 CFR 26.53.

On Federal Aid Projects, the Contractor shall not terminate a DBE subcontractor, lower tier DBE subcontractor, DBE transaction expeditor, DBE regular dealer, DBE supplier, DBE manufacturer and DBE trucker or an approved substitute DBE firm without good cause as listed in 49 CFR 26.53(f)(1)(ii)(3), and prior written consent of DCR/AA. Prior to replacement of the DBE or ESBE firm, the Contractor shall in writing, notify the DBE or ESBE firm and the DCR/AA of its intent to request to terminate and/or substitute a DBE or ESBE firm, the reason for the request, and that the DBE/ESBE has 5 days to respond to the Contractor’s notice and advise the DCR/AA and the Contractor of reasons why, if any, it objects to the proposed termination of its subcontract and why the Department should not approve the Contractor’s action. Give the DBE or ESBE 5 days to respond to the Contractor’s notice and advise the DCR/AA and the Contractor of reasons why, if any, it objects to the proposed termination of its subcontract and why the Department should not approve the Contractor’s action. If required in a particular case as a matter of public necessity (e.g., safety), the DCR/AA may provide a response period shorter than five days. At the time the Contractor requests termination or replacement of a DBE or ESBE firm, the Contractor must submit to the DCR/AA, documented evidence of its good faith efforts in accordance with 49 CFR Part 26.53 if they are replacing the terminated DBE or ESBE with a non-DBE or non-ESBE firm. The DCR/AA must approve the termination and substitution of all DBE or ESBE subcontractors, lower tier subcontractors, transaction expeditors, regular dealers, suppliers, manufacturers and truckers. The Contractor needs to show they began good faith efforts to replace or substitute with another DBE or ESBE well in advance of the request to terminate or substitute. The Department’s DCR/AA has sole authority to approve the termination, replacement or substitution of DBE and ESBE subcontractors, lower tier subcontractors, transaction expeditors, regular dealers, suppliers, manufacturers and truckers.

If requesting approval for a third tier subcontract, submit a letter from the subcontractor permitting subcontracting to a third tier, and submit the request for approval to subcontract, completed by the second tier subcontractor. Ensure that no work is performed by a third tier subcontractor before receiving written approval from the Department.

The Department will allow the Contractor to subcontract work as follows:

1. Values and Quantities. The total value of the work subcontracted may not exceed 50 percent of the Total Contract Price, except as follows:

a. The Contractor may deduct the value of work for Items designated as Specialty Items as specified in the Special Provisions from the value of the Total Contract Price.

b. The Contractor may deduct the value of work subcontracted to certified DBE, ESBE, and SBE firms indicated on the original DBE/ESBE/SBE Form A approved by the Department from the value of work subcontracted.

The total value of the work subcontracted may not exceed 70 percent of the Total Contract Price less the value of Specialty Items as noted above.

If a partial quantity of work for a unit price Item is subcontracted, the Department will determine the value of the work subcontracted by multiplying the price of the Item by the quantity of units to be performed by the subcontractor.

If only a portion of work of an Item is subcontracted, the Department will determine the value of work subcontracted based on the value of the work subcontracted as indicated in the subcontract agreement and as shown in a breakdown of cost submitted by the Contractor.

If a portion of a lump sum Item, an Item that includes specialty work, or a sign support structure is subcontracted, the Department will determine the value of work subcontracted based on the value of the work subcontracted as indicated in the subcontract agreement and as shown in a breakdown of cost submitted by the Contractor.

2. Limits and Restrictions.

The Department will permit subcontracting of work with the following restrictions:

a. The Contractor is barred from subcontracting MOBILIZATION.

1**************************************************************************************************************************1

BDC25S-07 dated Apr 28, 2025

Part B is changed to:

b.	The Contractor may only subcontract electrical, clearing site electrical, blasting, asbestos removal, landscaping, and lead paint abatement work to subcontractors having the required certificates and licenses.  Submit copies of required certificates and licenses with the request for approval to subcontract.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

dated Apr 21, 2022

Part C is changed to:

c.	The Contractor is barred from subcontracting to firms and individuals suspended or debarred by the Department or included in the State of New Jersey Consolidated Debarment Report maintained by the Department of the Treasury, Division of Building and Construction, Bureau of Contractor Prequalification.  The Contractor must certify that neither the individual, partnership, corporation, joint venture, or limited liability corporation applying to do subcontract work nor any of its corporate officers, stockholders, partners, or members are collectively or individually suspended, debarred, proposed for debarment, disqualified, declared ineligible, or voluntarily excluded from doing business by this or any other State or sub-division thereof or listed in the Federal Government’s System for Award Management (SAM), located at: https://sam.gov/content/exclusions.

1**************************************************************************************************************************1

d. Subcontractors are barred from making claims against the Department, its agents, officers, or employees.

3. Subcontract Requirements. Ensure that subcontract agreements include the following Contract provisions:

a. Federal Aid Projects. When subcontracting work on a Federal Aid Project, physically incorporate the following in the subcontract agreement, and inform subcontractors of their requirement to physically incorporate the information in lower tier subcontract agreements.

1. Disadvantaged Business Enterprise Utilization (Federal Aid Project Attachment 1), or Emerging Small Business Enterprise Utilization (Federal Aid Project Attachment 1)

2. Specific Equal Employment Opportunity Responsibilities on NJDOT Federal Aid Projects (Federal Aid Project Attachment 2).

3. Requirements for Affirmative Action to Ensure Equal Employment Opportunity on NJDOT Federal Aid Projects (Federal Aid Project Attachment 3).

4. Federal Equal Employment Opportunity Contract Specifications for NJDOT Federal Aid Projects (Federal Aid Project Attachment 4).

5. State of New Jersey Mandatory Equal Employment Opportunity Language on NJDOT Federal Aid Projects (Federal Aid Project Attachment 5).

6. Investigating, Reporting, and Resolving Employment Discrimination and Sexual Harassment Complaints on NJDOT Federal Aid Projects (Federal Aid Project Attachment 6).

7. Payroll Requirements for NJDOT Federal Aid Projects (Federal Aid Project Attachment 7).

8. FHWA-1273 Required Contract Provisions, Federal Aid Construction Contracts as amended or supplemented (Federal Aid Project Attachment 8).

9. State Mandatory Addendum to FHWA-1273 Required Contract Provisions, Federal Aid Construction Contracts as Amended or Supplemented (Federal Aid Project Attachment 9).

10. Federal Mandatory Equal Opportunity Language on Federal Aid Projects (Federal Aid Project Attachment 10).

11. Byrd Anti-Lobbying Certification (Federal Aid Project Attachment 11).

12. The Standard Title VI Assurance found in Subsection 107.02, as amended or supplemented.

13. General Wage Determinations Issued Under the Davis-Bacon and Related Acts.

14. New Jersey Department of Labor Prevailing Wage Rate Determination.

15. New Jersey Department of Transportation Code of Ethics for Vendors.

16. Subsection 107.04 as amended or supplemented.

17. Subsection 106.10 as amended or supplemented.

18. The Contract Assurance found in Subsection 107.03, as amended or supplemented.

1**************************************************************************************************************************1

for local aid projects:  prequalification of bidders shall follow the approval process established in njsa 40A:11-25 and shall receive approval from the director of local government services for any prequalification of bidders

include details, in the appropriate sections, regarding: SPECIALIZED KNOWLEDGE, experience, CRAFTSMANSHIP, and/OR EQUIPMENT required. the subcontractor must be qualified to perform certain SPECIALTY items (such as: segmental bridge, drilled shafts, blasting, ETC.).

1**************************************************************************************************************************1

1.	Values and Quantities.

1**************************************************************************************************************************1

THE FOLLOWING IS ADDED to the first PARAGRAPH:

a.

list “SPECIALTY ITEMS"

items MAY BE CONSIDERED “SPECIALTY ITEMS” DUE TO SPECIALIZED KNOWLEDGE, CRAFTSMANSHIP, OR EQUIPMENT REQUIRED TO perform THE WORK.

2**************************************************************************************2

select one of the following.

There are no Specialty Items in this Project.

*****OR*****

Specialty Items are as listed below:

3************************************************3

Drilling and blasting.

3************************************************3

3************************************************3

Select appropriate item(s).

Above ground highway lighting items.

*****AND/OR*****

Above ground sign lighting items.

*****AND/OR*****

Above and below bridge deck lighting items.

*****AND/OR*****

Electrical wire items.

3************************************************3

3************************************************3

ITS items, except for foundations, standards, and junction boxes.

3************************************************3

3************************************************3

list additional “specialty items”

3************************************************3

2**************************************************************************************2

1**************************************************************************************************************************1

2.	Limits and Restrictions.

1**************************************************************************************************************************1

dated Apr 21, 2022

Part C is changed to:

c.	The Contractor is barred from subcontracting to firms and individuals suspended or debarred by the Department or included in the State of New Jersey Consolidated Debarment Report maintained by the Department of the Treasury, Division of Building and Construction, Bureau of Contractor Prequalification.  The Contractor must certify that neither the individual, partnership, corporation, joint venture, or limited liability corporation applying to do subcontract work nor any of its corporate officers, stockholders, partners, or members are collectively or individually suspended, debarred, proposed for debarment, disqualified, declared ineligible, or voluntarily excluded from doing business by this or any other State or sub-division thereof or listed in the Federal Government’s System for Award Management (SAM), located at: https://sam.gov/content/exclusions.

1**************************************************************************************************************************1

108.02 Commencement of Work

1**************************************************************************************************************************

for local aid projects the following is added:

Work shall not begin before the agreement between the NJDOT and the LPA has been fully executed and the contractor has received written Notice to Proceed.

the first and second paragraphs are deleted and replaced with the requirements of the lpa.

the fifth paragraph is deleted and replaced with the requirments of the lpa.

BDC23S-11 dated aug 17, 2023

THE THIRD PARAGRAPH IS CHANGED TO:

Do not perform construction layout and FIELD OFFICE TYPE ___ SET UP until the Department has approved the insurance certificates and the safety program.  Do not begin other construction operations until after the following actions:

1.	A preconstruction conference with the Department has been held.

2**************************************************************************************2

BDC25S-19 dated jan 8, 2026

Part 2 of the third paragraph is changed to:

2.	Approval of the progress schedule as specified in 153.03.

2**************************************************************************************2

3.	The field office has been established.

4.	The ROW limits, limits of construction, environmentally restricted areas, and trees or other vegetation designated to be preserved have been laid out.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

108.05.02  Safety Program

1**************************************************************************************************************************1

BDC25S-20 dated FEB 19, 2026

PART 3 is changed to:

3.	Elements of the Program.  Include information and procedures for the following elements:

a.	Chain of Command.  Include the responsibilities of the management, supervisor, safety officer, and employees.

b.	Traffic Control Coordinator.  Include the name and contact information.  Ensure that the traffic control coordinator meets the requirements specified in 159.03.01.

c.	Environmental Manager.  Include the name and contact information.  Ensure that the environmental manager meets the requirements specified in 158.03.01.

d.	Local Emergency Telephone Numbers.  Include police, fire, medical, and, NJDEP hotline 
1-877-WARN DEP (1-877-927-6337).

e.	Procedures for Handling Emergencies.  Provide guidelines for handling emergencies, including emergency action plans for accidents involving death or serious injury, property damage, fires, explosions, and severe weather.  Include the emergency contact information of the Contractor’s personnel responsible for handling emergencies.

f.	Training Topics.  Include regulatory and jobsite toolbox meetings.  Include the documentation from the training and an attendance sheet for each.

g.	Contractor’s Safety Rules.  Include housekeeping procedures and personal protective equipment requirements.

h.	Employee Disciplinary Policy.  Include the violation forms.

i.	Safety Checklists.  Include project safety-planning, emergency plans and procedures, documentation, and protective materials and equipment.

j.	Forms.  Include OSHA 300 Log and New Jersey Department of Labor L&I-1.

k.	Security Policy Guidelines.  Provide a copy for the office and for the Project Limits.

l.	Hazard Communication Program.  Provide the following:

1.	The location of and instructions for understanding the MSDS.  Ensure that the location and instruction are available to anyone within the Project Limits.

2.	The person responsible for the hazard communication program and the method of informing personnel of the hazardous communication program.  Include attendance sheets of hazard communication meetings.

3.	When performing work that generates airborne crystalline silica, include engineering and work practice controls to limit exposure levels to at or below the permissible exposure limit according to 29 CFR 1910.1000 Table Z-3.  Ensure that the program includes employee training and respiratory protection measures according to 29 CFR 1910.134 and control of the area when the permissible exposure limit is exceeded.  Provide a trained and competent person, according to 29 CFR 1926.30, within the Project Limits at all times when performing work that produces airborne crystalline silica.

m.	Additional Requirements.  Provide additional procedures for Project specific topics including:

1.	Compressed gas cylinders.

2.	Confined spaces.

3.	Cranes.

4.	Electrical.

5.	Equipment operators.

6.	Fall protection.

7.	Hand and power tools.

8.	Hearing conservation.

9.	Highway safety.

10.	Lead.

11.	Lock out/tag out.

12.	Materials handling, storage, use, and disposal.

13.	Night work.

14.	Personal protective equipment.

15.	Project entry and exit.

16.	Respiratory protection.

17.	Sanitation.

18.	Signs, signals, and barricades.

19.	Subcontractors.

20.	Trenching.

n.	Internal Traffic Control Plan (ITCP).  Develop, submit, implement, and maintain an ITCP according to ANSI/ASSP A10.47 Section 6.5 and 6.6.  At a minimum, ensure the plan includes:

1.	A diagram showing internal vehicle and pedestrian routes, site layout, and hazard zones.

2.	A listing of type and number of all on-site personnel and mobile equipment.  Provide names of personnel upon request if needed after an accident.

3.	A site-specific hazard identification checklist with corresponding mitigation measures aimed at reducing struck-by and related risks.

4.	Safety notes emphasizing backing procedures, spotter usage, visibility aids, and run-over/back-over prevention.

5.	A communication plan to ensure the ITCP is effectively communicated to all relevant workers, operators, subcontractors, and personnel.  Distribute the communication plan at an onsite meeting or as otherwise directed by the RE.

Submit an updated ITCP plan during construction if any substantial changes, as determined by the RE, to site layout, access, or equipment occur.  Ensure the updated plan is approved by the RE before implementation.  Ensure plan modifications are communicated to affected personnel.

Provide a certification that the plan complies with ANSI/ASSP A10.47 Section 6.5, 6.6, and all associated requirements.

Within 15 days of receiving the safety program, the Department will determine if the safety program meets the requirements of the Contract.  If the safety program does not meet the requirements, the Department will return the safety program for revision.  Do not begin work within the Project Limits until the Department has approved the safety program.

The Contractor is responsible for implementing, monitoring, updating, and revising the safety program until Acceptance.  Submit updates and revisions to the safety program to the RE for approval when new information, new practices or procedures, or changing site and environmental conditions necessitate modifications to protect site personnel.

Maintain a copy of the updated safety program, including the appropriate documentation associated with each element, within the Project Limits so that it is available to workers and other authorized persons entering the Project Limits.  Provide copies of updates to the safety program to the RE.

If an incident occurs that requires a New Jersey Department of Labor & Workforce Development Form (L&I-1) to be submitted, send a copy to the Office of Capital Project Safety, New Jersey Department of Transportation, 1035 Parkway Avenue, P.O. Box 600, Trenton, NJ 08625-0600.

Failure to comply with the requirements set forth in the Elements of the Program will result in the RE suspending the Work wholly or in part as specified in 108.13 and suspending Estimates, as specified in 109.05, for failure of the Contractor to correct conditions unsafe for the workers or the general public, for failure to carry out provisions of the Contract, including but not limited to failure to develop, submit, implement, and maintain the ITCP.

1**************************************************************************************************************************1

BDC22S-10 dated aug 03, 2022

108.07.01 Interference

THE SECOND PARAGRAPH IS CHANGED TO:

Schedule and perform the Work so that successive construction operations and lane or roadway openings follow preceding operations as closely as possible.  Limit work zones according to the Special Provisions.  Confine construction operations adjacent to traffic to one side of the roadway at a time unless otherwise specified by the Contract.  Where the Work is performed in stages adjacent to traffic, ensure that the road opened to traffic adequately accommodates traffic.  Do not interfere with existing traffic access, except when required to perform the Work or as approved by the RE.

2**************************************************************************************2

complete and include the following maximum work zone restrictions as applicable. the limits should match assumptions used to calculate traffic control quantities. delete any remaining statements.

A.	Limit work zone to ____ lane miles per day, unless otherwise approved by the RE.

B.	Limit the number of work zones to a maximum of ____ concurrently.

2**************************************************************************************2

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC25S-04 dated jun 5, 2025

THE fifth Paragraph is changed to:

The RE has the right to reject or rescind approval of lane or shoulder closures because of the following:

1.	Weather conditions.

2.	The closure is unnecessary to perform the work.

3.	Emergency conditions either on or off the Project that result in an unacceptable impact to the traveling public.

4.	There is an active snow event in progress.

5.	If the National Weather Service (https://www.weather.gov) locally forecasts, 12 hours prior to the lane closure, a 40 percent or greater chance of snow precipitation, tropical storms, or hurricanes during the requested lane closure hours.

1**************************************************************************************************************************1

include the following if lane rental is necessary otherwise delete.

if required for a local aid project THAT IMPACTS A NJDOT FACILITY contact the local aid project manager for lane rental charges

THE FOLLOWING new subpart IS ADDED:

108.07.03 Lane Rental

Lane and shoulder closures are restricted to the schedule provided in the Traffic Control Details of the plans, and as specified in 108.07.01.  The Contractor may extend the allowable hours for lane and shoulder occupancy as provided by Table 108.07.03-1 with the RE’s written approval.  Submit a written request to the RE to rent lanes and shoulders for an extended period at least 14 days prior to the anticipated use.  In the request, provide the following information:

1.	Route, direction, and milepost limits

2.	Closure Description (Lane type/shoulder)

3.	Date(s)/Days

4.	Start Time(s)

5.	Finish Time(s)

6.	Reason

7.	Calculation of Lane Rental Cost for each closure

The Department will assess the lane rental charge for each hour the Contractor occupies a lane in accordance with Table 108.07.03-1.  If the Contractor does not occupy the lane during the extended hours, the Department will not assess a lane rental charge.  If the Contractor occupies the lane for fewer hours than requested, the Department will only assess a lane rental charge for the time that the Contractor has actually occupied the lane.  If the Contractor occupies a lane for a portion of an hour, the Department will round the occupancy time to the next highest half hour.

The Department will assess a lane rental charge for lane and/or shoulder occupancy of the roadway at the rates provided in Table 108.07.03-1.

2**************************************************************************************2

complete and include the appropriate Lane rental Charge information.

| Table 108.07.03-1 Lane Rental Availability and Charge |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Roadway | Direction | Closure Description | Closure Time |  |  | Rental Time |
|  |  |  | Day(s) | Start Time | Finish Time |  |
|  |  |  |  | Hours | Hours | $ per hour |

2**************************************************************************************2

Do not occupy a lane or shoulder beyond the RE’s approved extension of the allowable hours for lane and shoulder occupancy.  If the Contractor’s lane closure exceeds the allowable time period, the Department will assess Occupancy Charges in accordance with 108.08.

1**************************************************************************************************************************1

108.08 LANE Occupancy Charges

1**************************************************************************************************************************1

BDC20S-03 dated Jul 17, 2020

subsection is renamed and changed to:

108.08 OCCUPANCY Charges

The closure schedule shown in the plans indicates the time periods for allowable closures as specified in the Contract.  Allowable closures are permitted for, but not limited to; roadways, lanes, shoulders and ramps.  If the Contractor’s closures exceed these time periods, the Department will deduct from the monthly estimate an occupancy charge for the use and occupancy beyond the time periods shown in the closure schedule until such time that the closure is reopened to traffic or until such time that the closure is allowed to take place again under the closure schedule.  The Department will recover the cost of occupancy charges as specified in 107.16.

The RE will keep record of each occurrence as well as the cumulative amount of time that a closure exceeds the time periods shown in the closure schedule and provide the record to the Contractor.  The Department will calculate an occupancy charge by multiplying the length of time of each delayed opening, in minutes, by the rate of $10 per minute, unless otherwise specified in the Special Provisions.  The total amount per day for occupancy charges that the Department will collect will not exceed $10,000.00.

The Department will waive an occupancy charge where a closure is not reopened to traffic as specified in the closure schedule directly and solely by reason of extraordinary, exigent circumstances not under the control of or reasonably foreseeable by the Contractor.  Equipment breakdowns, supplier deliveries, and weather-related hindrances are not extraordinary, exigent circumstances.  However, the Department has the right to assess an occupancy charge for any period of time that a closure remains closed beyond the reasonable period of time needed by the Contractor to reopen a closure due to an extraordinary, exigent circumstance.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

include appropriate Occupancy Charge information.

if required for a local aid project THAT IMPACTS A NJDOT FACILITY contact the local aid project manager for occupancy charge information

THE FOLLOWING IS ADDED:

The rate to calculate the Occupancy Charge is as follows:

| Description | Rate |
| --- | --- |

1**************************************************************************************************************************1

108.09  Maintenance Within The Project Limits

1**************************************************************************************************************************1

BDC26S-05 dated may 14, 2026

THE FIRST PARAGRAPH IS CHANGED TO:

After construction operations start, continuously perform maintenance requirements within the Project Limits until Acceptance.  The Project Limits to be maintained include the limits of all temporary and permanent construction, excluding those sections of highways, streets, and roads where only signs are placed.  Maintenance includes but is not limited to the following:

1.	Picking up and disposing of trash and debris, including trash and debris not generated by the Contractor.  However, the Contractor is not responsible for the disposal of dead animals.

2.	Keeping the roadway clear of dust, soil, and cleaning up non-hazardous spills.

3.	Maintaining a safe and unobstructed passageway through the construction area for public traffic, including but not limited to constructing escape ramps at the edges of pavement.

4.	Maintaining surface drainage, including the removal of debris, ice, and snow around inlets, to ensure that it is functioning as intended.

5	Removal of ice and snow from bridge sidewalks open to pedestrian access.

1**************************************************************************************************************************1

108.10 CONTRACT Time

1**************************************************************************************************************************1

2**************************************************************************************2

complete and include the following WHEN dates are specified.

delete “A”, if not applicable, and reAssign APPROPRIATE letter to the remaining statements.

A.	Complete all work required for Interim Completion for ____________ on or before __________.

B.	Complete all work required for Substantial Completion on or before __________.

C.	Achieve Completion on or before __________.

2**************************************************************************************2

*****OR*****

2**************************************************************************************2

complete and include the following WHEN a number of DAYS are specified.

delete “A”, if not applicable, and reAssign APPROPRIATE letter to the remaining statements.

A.	Complete all work required for Interim Completion for ____________ in _______ days.

B.	Complete all work required for Substantial Completion in _______ days.

C.	Achieve Completion in _______ days.

2**************************************************************************************2

*****OR*****

2**************************************************************************************2

complete and include the following WHEN a number of WORKING DAYS are specified.

delete Appropriate part(S), if not applicable, and reAssign APPROPRIATE letter to the remaining statements

A.	Complete all work required for Interim Completion for ____________ in _______ working days.

B.	Complete all work required for Substantial Completion in _______ working days.

C.	Achieve Completion in _______ working days.

2**************************************************************************************2

1**************************************************************************************************************************1

108.11.01 Extensions to Contract Time

1**************************************************************************************************************************1

B.	Types of Delays.

2.	Excusable, Non-Compensable Delays.

b.	Utilities.

include the following, IF the project DURATION is less than 1 year and amount of duration of utility work performed by utility company is minor. Subject to review and approval of the lpa

THE last PARAGRAPH is CHANGED TO:

If approved excusable, non-compensable delays exceed a total of 90 days, the time in excess of 90 days will become excusable and compensable as specified in 108.11.01.B.3.

1**************************************************************************************************************************1

108.12 RIGHT-of-Way Restrictions

1**************************************************************************************************************************1

complete and include THE FOLLOWING WHEN RoW has not been secured for those parcels. provide anticipated dates of availability.

The Department has not obtained the following ROW parcels.  The anticipated availability dates are provided:

| Properties and Vacation/Availability Dates |  |  |  |
| --- | --- | --- | --- |
| Demolition and/or Parcel No. | Approximate Baseline Station | Offset/Direction | Date |

1**************************************************************************************************************************1

108.14 DEFAULT and Termination of Contractor’s Right to Proceed

The Department will provide written notice to the Contractor and the Surety of the cause for default, and demand elimination of such cause for default, if the Contractor does any of the following:

1. Fails to begin construction operations within 40 days of execution of the Contract.

2. Fails to perform the Work with sufficient workers and equipment or with sufficient materials to ensure its completion within the Contract Time specified, or any modification thereof.

3. Fails to complete the Contract within the Contract Time specified, as modified. 78

4. Performs the Work unsuitably or neglects or refuses to remove materials or to again perform such Work as may be rejected as unacceptable and unsuitable.

5. Fails to follow the direction of the RE.

6. Works in a reckless or unsafe manner or fails to correct unsafe conditions.

7. Discontinues the prosecution of the Work.

8. Fails to resume Work which has been discontinued within a reasonable time after notice to do so.

9. Becomes insolvent or is declared bankrupt or commits any act of bankruptcy or insolvency.

10. Allows any final judgment to stand against it unsatisfied for a period of 10 days.

11. Makes an assignment for the benefit of creditors.

12. Fails to acquire or maintain the required insurance.

13. Fails to comply with Contract requirements regarding minimum wage payments, 49 CFR Part 26 et seq., the DBE program requirements, SBE program requirements, and equal employment opportunity requirements.

14. Is a party to fraud.

15. For any other cause whatsoever, fails to carry out the Work in an acceptable manner.

If the Contractor or Surety, within a period of 10 days after such notice, does not proceed as specified in the notice, then the Department has full power and authority, without violating the Contract, to declare the Contractor in default and notify the Contractor to discontinue the Work. The Department will provide, in writing, the declaration of default to the Contractor and Surety. The Department has the right to appropriate any or all materials and equipment within the Project Limits to complete the Contract. The Department has the right to direct the Surety to complete the Contract or may enter into an agreement for the completion of the Contract with the Surety or another contractor, or use such other methods required for the completion of the Contract, including completion of the Work by the Department.

1**************************************************************************************************************************1

BDC24S-07 dated JUN 11, 2024

THE THIRD PARAGRAPH IS CHANGED TO:

If the Department directs the Surety to complete the Contract, the Department will provide to the Surety the Contract as defined in 101.03 and the completion status of the Contract.  If the Surety elects to use a completion-contractor to perform the Work, the Surety must promptly submit to the Department a request for approval of the proposed completion-contractor as a subcontractor as specified in 108.01 within 45 days of receipt of the Contract provided by the Department pursuant to this Section.  The Department, in its sole discretion, has the right to reject a request by the Surety to use the Contractor or another contractor as the completion-contractor, either directly or under the direction of a consultant to the Surety.

THE FOLLOWING IS ADDED AFTER THE THIRD PARAGRAPH:

If the Department approves the Surety’s request to use the Contractor as the completion-contractor, the Surety shall cause the Contractor to begin Work, as defined in 101.03, within 45 days of notice of the Department’s approval or as directed by the Department based on factors including but not limited to weather, seasonal restrictions, permits or at its sole discretion.  The failure of the Surety to comply with the deadlines set forth in this Section shall be deemed a material breach of the Contract.

If the Department does not approve the Surety’s proposed completion-contractor, the Surety must submit to the Department a request for approval of an alternative completion-contractor within 60 days of notice of the Department’s disapproval.  If the Department does not approve the Surety’s alternative completion-contractor, the Department may continue to request approval of another alternative completion-contractor.  Within 60 days of notice of the Department’s approval of the Surety’s proposed completion-contractor, the Surety shall cause the completion-contractor to begin Work, as defined in 101.03, or as directed by the Department based on factors including but not limited to weather, seasonal restrictions, permits or at its sole discretion.  The failure of the Surety to comply with the deadlines set forth in this Section shall be deemed a material breach of the Contract.

1**************************************************************************************************************************1

The Contractor and Surety are not relieved of the assessment of liquidated damages, as specified in 108.20, because of the Contractor’s default.

The Department will recover the costs and charges incurred by the Department, together with the cost of completing the Work from the Contractor or Surety.

The rights and remedies of the Department are in addition to any other rights and remedies provided by law or under the Contract and the bonds.

If, after declaration of default, the Department determines for any reason that the Contractor was not in default or that the delay was excusable, the rights and obligations of the parties are the same as if the Department had issued an order of termination for convenience as specified in 108.15.01.

If, after declaration of default, a court determines for any reason that the Department’s default of the Contract was legally improper, the rights and obligations of the parties are the same as if the Department had issued an order of termination for the convenience as specified in 108.15.01.

1**************************************************************************************************************************1

complete and include the following if start of work is restricted by department action

List (1) of the first PARAGRAPH is changed to:

1.	Fails to begin construction operations within __ days of execution of the Contract.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

108.19 COMPLETION and Acceptance

1**************************************************************************************************************************1

include specification prior to the final Design submission for Incentive Payment.

if no incentive payment for early completion is to be specified include the following:

THE FOLLOWING IS ADDED:

No Incentive Payment for Early Completion is specified for this project.

1**************************************************************************************************************************1

108.20 LIQUIDATED Damages

1**************************************************************************************************************************1

BDC19S-03 dated SEp 03, 2019

obtain road user cost

if required on a local aid project THAT IMPACTS A NJDOT FACILITY contact the local aid project manager to obtain road user cost

select APPROPRIATE condition from the following list

Liquidated damages are as follows:

2**************************************************************************************2

complete and include the following.

delete Appropriate part(S), if not applicable and reAssign APPROPRIATE letter to the remaining statements

A.	For each day that the Contractor fails to complete the work as specified in Subsection 108.10 of these Special Provisions, for Interim Completion, the Department will assess liquidated damages in the amount of $______.

use only the road user cost.

B.	For each day that the Contractor fails to complete the work as specified in Subsection 108.10 of these Special Provisions, for Substantial Completion, the Department will assess liquidated damages in the amount of $______.

FULL TRAFFIC ACCESS NOT ACHIEVED - use the total cost of the road user and construction engineering costs.

For each day that the Contractor fails to complete the work as specified in Subsection 108.10 of these Special Provisions, for Substantial Completion but has completed the work as specified for Full Traffic Access, the Department will assess liquidated damages in the amount of $_____.

FULL TRAFFIC ACCESS ACHIEVED - use ONLY the construction engineering cost.

C.	For each day that the Contractor fails to achieve Completion as specified in Subsection 108.10 of these Special Provisions, the Department will assess liquidated damages in the amount of $_____.

Use only the construction engineering cost divided by 2.

2**************************************************************************************2

1**************************************************************************************************************************1

Section 109 – Measurement and Payment

109.01 MEASUREMENT of Quantities

1**************************************************************************************************************************1

BDC21S-01 dated MAr 08, 2021

THE LAST PARAGRAPH IS CHANGED TO:

for local aid projects this section is changed to the following:

All items are to be measured for payment except those designated as lump sum.

1**************************************************************************************************************************1

109.03 PAYMENT for force account

1**************************************************************************************************************************1

BDC24S-10 dated jul 1, 2024

PART (11) is changed to:

11.	For each subcontractor, provide and certify the subcontractor’s documentation for items 1 through 10 above to be complete, accurate, and true.

1**************************************************************************************************************************1

109.04 PAYMENT for Delay Damages

1**************************************************************************************************************************1

BDC24S-10 dated jul 1, 2024

PART (10) is changed to:

10.	Documentation in the same format as above for each subcontractor with certification by the Contractor that the documentation is complete, accurate, and true.

the following is added to the list:

11.	Certification stating that all costs submitted have been incurred because of the delay, and all vendor invoices have been paid.

1**************************************************************************************************************************1

109.05 ESTIMATES

The Department will make monthly payments to the Contractor for work performed and for materials delivered, as specified in 109.06. The RE will calculate the payment in an Estimate consistent with the provisions of the Contract. If not otherwise described in the Contract, the RE will be the sole judge of the amount of progress payment due for partially completed work. The quantities provided in the Estimate may be approximations and may not be based on as-built quantity measurements. The Department will establish the date of the month that the Estimate is processed.

Pay subcontractors and suppliers for satisfactory performance of their work no later than 10 days from receipt of each payment made by the Department.

Pay subcontractors and suppliers the full amount of retainage no later than 10 days from receipt of payment made by the Department for the subcontractor’s or supplier’s work.

1**************************************************************************************************************************1

2**************************************************************************************2

BDC23S-20 dated March 5, 2024

THE fourth PARAGRAPH is CHANGED TO:

The RE will provide a summary of the Estimate to the Contractor.  Before the issuance of each payment, certify, on forms provided by the Department, whether:

- On Federally Funded Projects

1.	No subcontractor or supplier was used on the project; or

2.	Each subcontractor and supplier used on the project has been paid the amount due, from the previous progress payment; retainage is not being held, and

3.	Each subcontractor and supplier used on the project will be paid the amount due from the current progress payment, for the subcontractor or supplier’s work that was paid by the Department; or

4.	There exists a valid basis under the terms of the subcontractor’s or supplier’s contract to withhold payments from the subcontractor or supplier.  Therefore, the subcontractors and suppliers listed on the DL-72 Contractor Certification of Payment to Subcontractors and Suppliers have not been paid for work performed or materials supplied to the project from the proceeds of the previous progress payment or will not be paid for work performed or materials supplied to the project from the proceeds of the current progress payment, or both.

2**************************************************************************************2

2**************************************************************************************2

If the certification indicates that the Contractor has withheld or will withhold payment from a subcontractor or supplier, provide written notice, according to N.J.S.A. 52:32-40 and N.J.S.A. 52:32-41, of such non-payment to the subcontractor or supplier. Provide a copy of the notice to the Department and to the Surety that holds the performance bond. Include the reason for withholding payment and state the amount of payment withheld in the notice.

The date that the Department receives the certification will initiate the 20 day approval period under N.J.S.A. 2A:30A-1, et seq. The Department will not accept the certification before being requested by the RE.

If the Contractor fails to pay the subcontractor or supplier within 30 days after the subcontractor or supplier satisfactorily completes the specified work, the Department may withhold progress payments from the Contractor, until the Contractor pays the subcontractor or supplier all delinquent amounts due, or the Contract is terminated, or the matter is resolved under N.J.S.A. 52:32-40 and N.J.S.A. 52:32-41.

If the Department receives an allegation from a subcontractor or a supplier that the Contractor has not paid the subcontractor or supplier the amount due from a previous progress payment, including retainage, submit to the RE within 10 days of a request made by the RE, evidence that payment has been made.

If no valid basis exists for withholding payment, N.J.S.A. 52:32-40 and N.J.S.A. 52:32-41 authorize any subcontractor or supplier from whom payment is withheld to receive from the Contractor, in addition to any amount due, interest at a rate 84 equal to the prime rate plus 1 percent if the subcontractor or supplier is not paid within 10 days after receipt by the Contractor of payment by the Department for completed work that is the subject of a subcontract or a material supply agreement. This interest begins to accrue on the tenth day after receipt of payment by the Contractor.

If court action is taken by a subcontractor or supplier to collect payments withheld by a Contractor and it is determined that a valid basis existed for the withholding of those payments, the subcontractor or supplier shall be liable for any court costs incurred by the Contractor in connection with the action.

The Department will not make payment for an Estimate having a value less than $5,000.00, unless it is for the Final Certificate.

The Department will deduct and withhold 2 percent in retainage from the total Estimate amount for State Funded Projects. On State Funded Projects, the Contractor may not withhold subcontractor retainage that exceeds the amount of retainage that the Department withholds from the Contractor.

2**************************************************************************************2

BDC20S-07 dated May 29, 2020

THE TWELFTH PARAGRAPH is CHANGED TO:

Regarding Federally Funded Projects, the Department will deduct and withhold 2 percent in retainage from the total Estimate amount, excluding amounts for subcontracted work, until Substantial Completion.  Pursuant to 49 C.F.R. § 26.29(b)(1), the Contractor may not withhold retainage from a subcontractor on Federally Funded Projects.

2**************************************************************************************2

In the first Estimate following Substantial Completion, the Department will reduce the retainage withheld to one percent of the Total Adjusted Contract Price, excluding subcontracted work on Federal Aid Projects, unless it has been determined by the Department that the withholding of additional retainage is required. If retainage is held in cash withholdings, the reduction is to be accomplished by payment under the next Estimate. If retainage is held in bonds, the Department will authorize a reduction in the escrow account.

The RE has the right to not process an Estimate when, in the judgment of the RE, the Work is not performed or proceeding as specified in the Contract or following the Department giving the Contractor and Surety notice of default as specified in 108.14.

The Department’s processing or payment of an Estimate is not an approval of defective or improper work. The Department, upon determining that any payment under a previous Estimate was improper or unwarranted for any reason, has the right to recover erroneous payment from the Contractor

1**************************************************************************************************************************1

1**************************************************************************************************************************1

109.06 MATERIALS Payments and Storage

1**************************************************************************************************************************1

BDC22S-19 dated DEC 28, 2022

THE subsection is changed to:

The Contractor may request payment for the cost of materials, including the storage cost, not incorporated into the Work.  If approved by the RE, the Department will make payment for the cost of materials, including storage costs if such payment exceeds $25,000.00; however, the amount of payment may not exceed 85 percent of the bid price for the associated Item.  The Department may also direct the Contractor to purchase materials ahead of schedule for this purpose.  The Department will not make payment for such materials until the RE is satisfied that:

1.	The Contractor has properly stored and protected materials within the Project Limits or at locations owned or leased by the Contractor or the Department within the State, except that the Contractor may store structural steel outside the State with the prior approval of the Department.  Provide and comply with manufacturers’, suppliers’, and fabricators’ storing and handling recommendations for each material, as specified in 108.04.

2.	The RE has inspected the materials and they appear to be acceptable based upon available supplier’s certification and materials test reports.

2**************************************************************************************2

BDC23S-05 dated Mar 31, 2023

PART 3 IS CHANGED TO:

3.	The Contractor has provided the RE with the paid invoice or paid bill of sale for the materials, a certification from the supplier that the material was paid for, including the transfer of ownership to the Department.

2**************************************************************************************2

4.	For material stored on property not belonging to the Department, the material is stored in a fenced area with access limited to the Department and the Contractor.  Additionally, the Contractor has posted a sign at the location clearly identifying, and printed in large letters, that the materials are without encumbrances and are to be solely used for the Project.

5.	When materials are stored in a leased area, the lease is made out to the Contractor and provides that it shall be canceled only with the written permission of the Department.  Submit a copy of the lease to the RE.

Payment for materials does not constitute Department approval or Acceptance of the materials or work.  If materials paid for are damaged, stolen, or prove to be unacceptable, the Department has the right to recover the costs from the Contractor.  Stored materials are not to be removed from storage except for incorporation into the project.  The Department will not make payment for plant materials until they are planted or installed.

1**************************************************************************************************************************1

109.09 AUDIT

1**************************************************************************************************************************1

BDC24S-10 dated Jul 1, 2024

The first sentence in the first paragraph is changed to:

All claims filed as specified in 107.12 and force account work are subject to audit at any time following the filing, whether or not part of a suit pending in the courts of this State pursuant to N.J.S.A. 59:13-1, et seq.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC25S-10 dated may 15, 2025

THE LAST PARAGRAPH IS CHANGED TO:

Pursuant to N.J.S.A. 52:15C-14(d) and N.J.A.C. 19:70-1.6(b), relevant records of private vendors or other persons entering into contracts with the Department are subject to audit or review by the New Jersey Office of the State Comptroller.  Therefore, the Contractor shall maintain all documentation related to products, transactions or services under the Contract for a period of 5 years from the date of final payment.  Such records shall be made available to the New Jersey Office of the State Comptroller upon request.

1**************************************************************************************************************************1

Division 150 – Contract Requirements

Section 151 – Performance Bond and Payment Bond

151.03.01 Performance Bond and Payment Bond

1**************************************************************************************************************************1

dated APR 21, 2022

The first sentence is changed to:

Provide a performance and payment bond to the LPA within        insert LPA requirements            days of award.

1**************************************************************************************************************************1

151.04 Measurement and Payment

THIS SECTION IS DELETED AND REPLACED WITH THE REQUIREMENTS OF THE LPA.

1**************************************************************************************************************************1

Section 152 – Insurance

152.03.01 Railroad Protective Liability Insurance

1**************************************************************************************************************************1

2**************************************************************************************2

iNCLUDE THE FOLLOWING WHEN RAILROAD INSURANCE IS REQUIRED.

Procure and maintain insurance coverage for the following railroad(s):

3************************************************3

include THE APPROPRIATE RAILROAD(S).

ADD ANY OTHER RAILROAD THAT MAY BE IMPACTED BY THE PROJECT

National Railroad Passenger Corporation (AMTRAK)

*****and/OR*****

Consolidated Rail Corporation (CONRAIL)

*****and/OR*****

New Jersey Transit Rail Operations

*****and/OR*****

New York Susquehanna & Western Railway Corporation (NYS&W)

3************************************************3

2**************************************************************************************2

2**************************************************************************************2

list the name and address of the railroad company representative

2**************************************************************************************2

2**************************************************************************************2

complete and include THE PERCENTAGE OF PROJECT COST WITHIN RAILROAD ROW

It is estimated that ___ percent of the Project cost is located within or adjacent to the railroad ROW.

2**************************************************************************************2

1**************************************************************************************************************************1

Section 153 – Progress Schedule

153.03  Procedure

153.03.01  CPM Progress Schedule

1**************************************************************************************************************************1

BDC25S-21 Dated maR 24, 2026

the fourth Paragraph is changed to:

If the Contractor proposes alternate staging, submit a plan detailing the proposed alternate.  Submit a schedule showing the original staging and a schedule showing the proposed alternate staging.  The RE may extend its review of the schedule submissions by a reasonable length of time.  The Department will not grant time extensions or make payment for delays to construction resulting from extended review time of the proposed alternate staging by the RE, regardless of whether the Department accepts or rejects it.  The RE may require the Contractor to submit the proposed alternate staging as a VECP as specified in 104.02.

1**************************************************************************************************************************1

Section 154 – Mobilization

154.04  Measurement and Payment

1**************************************************************************************************************************1

BDC25S-08 Dated may 1, 2025

The subsection is changed to:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

MOBILIZATION	LUMP SUM

The Department will make payment for MOBILIZATION on a lump sum basis, regardless of the number of times the Contractor shuts down and returns to the Project.  The Department will make payment as follows:

| Work Completed | Payment |
| --- | --- |
| 5% of the Work1 | Lesser of 25% of bid price or 3% of Total Contract Price |
| 10% of the Work1 | Lesser of 50% of bid price or 7% of Total Contract Price |
| 15% of the Work1 | Lesser of 75% of bid price or 11% of Total Contract Price |
| 20% of the Work1 | Lesser of 100% of bid price or 15% of Total Contract Price |
| 100% of the Work | Amount of bid price not previously paid |
| 1. If the baseline schedule is not approved, the Department will not make payment for mobilization. |  |

The Department will calculate the percentage of Work completed from the total of payments compared to the Total Contract Price.  The total of payments excludes the amount paid for Mobilization and the amount paid for materials furnished but not incorporated into the Work as specified in 109.06.

If mobilization is not included in the Proposal, include the costs in the various Items scheduled in the Proposal.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

Choose “Bar Chart Progress Schedule and Updates” FOR ALL CONTRACTS UNLESS OTHER WISE DIRECTED BY THE LPA OR THE LOCAL AID PROJECT MANAGER.

1**************************************************************************************************************************1

Section 155 – Construction Field Office

155.03.01 Field Office

1**************************************************************************************************************************1

BDC23S-06 Dated Mar 30, 2023

THE FIRST PARAGRAPH IS CHANGED TO:

Provide and maintain a safe and secure field office for the exclusive use of the Department, until no longer required by the Department, at a location within or in the immediate vicinity of the Project Limits and approved by the RE. Do not lock out, or make the field office inaccessible to, the RE or any Department employees, consultants, or representatives at any time. Do not use any building scheduled for demolition under the Contract as a field office. If the field office is a temporary structure within State ROW, secure necessary permits from the New Jersey Department of Community Affairs, and remove the structure when directed by the RE.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

provide THE project specific items.

complete and include the following list.

4.	Communication Equipment.

a.	Telephones.  Provide ___ cordless phones with auto-switching.

c.	Cell Phones.  Provide ____ cellular phones.  Ensure the cellular phone plan provides for unlimited mobile to mobile in-network usage and an anticipated monthly usage of 900 anytime minutes for each phone.  Ensure the phones are on the same plan.  Ensure the cellular phone plan has a home rate with no roaming charges within the state.  Ensure each cellular phone has the following features:

1.	Camera with 1 megapixel picture capability.

2.	Battery life capable of 180 minutes of continuous use and 72 hours of standby use.

3.	Equipped with a hands-free headset.

4.	Base charger and car charger.

The Department will maintain possession of the cell phones at the end of the Project.

d.	Computer System.  Provide a computer system meeting the following requirements:

___ computer configurations each meeting the following:

1.	Processor having a clock speed of ___ GHz or faster, ___ GB RAM, ___ MB Video RAM, ___ Gigabyte hard drive designated as drive C, one DVD (+/-) Writer Drive, and one CD Recordable Drive.  Ensure the system is USB 2.0 compatible and has at least 2 front USB ports Include Keyboard, optical mouse and 2 piece desktop speakers.

2.	Wired Router with appropriate number of ports and cables and a print server.  Ensure there is at least 1 wired Ethernet switch.

3.	High-speed broad band connection and service with a minimum speed of ___ Megabits per second (mbps) with dynamic IP address for the duration of the project.

4.	19 inch or larger Flat Screen LCD monitor with tilt/swivel capabilities.

5.	___ Gigabyte or larger external drive with backup software for MS-Windows, and 15 corresponding formatted data cartridges corresponding to the tape drive size.

6.	___ Flatbed USB version 2.0 or greater Color Scanner with automatic document feed.

7.	Uninterruptible power supply (UPS).

8.	Surge protector for the entire computer configuration to be used in conjunction with the UPS.

9.	Computer workstation, chair, printer stand, and/or table having both appropriate surface and chair height.

10.	One can of compressed air and screen cleaning solution every other month of the duration of the contract.

If more than one computer configuration is specified, provide one network interface card for the base computer configuration and hardwire connections between computer configurations as directed by the RE.

Also provide:

___ USB ___ GB Flash/Jump memory drives

___ CD ___ MB (or larger) recordable CD’s compatible with the CD drive and ___ recordable DVD’s.

___ CD/DVD Holder (each holds 50)

___ color laser printers and supplies as follows:

1.	Minimum of 192 Megabytes of expanded memory, printer cable, and legal-size paper tray.

2.	One set of printer ink cartridges every other month for the duration of the construction project for each printer.

2**************************************************************************************2

BDC19s-06 dated oct 15, 2019

Software as follows:

1.	Microsoft Windows, latest version with future upgrades for the duration of the entire project.

2.	Microsoft Office Professional, latest version.

3.	Norton’s System Works for Windows, latest version, or compatible software package with future upgrades and latest virus patches.

4.	Anti-Virus software, latest version with monthly updates for the duration of the contract.

5.	Adobe Acrobat Professional, latest version, or compatible software for Scanner.

2**************************************************************************************2

6.	Office Equipment.  Provide the following:

b.	___ digital camera(s).  Ensure each digital camera has auto-focus, with rechargeable batteries and charger, __ GB memory card, USB Memory Card Reader compatible with camera and field office computer, 3.0 inch LCD monitor, ___ mega pixel resolution, __ X optical zoom lens, built in flash, image stabilization, computer connections, and a carrying case.

c.	___ video camcorder(s) with ___ Memory Cards 32 GB (or larger) compatible with the camcorder.  Ensure each video camcorder is a memory card camcorder with ___X optical zoom, 2" LCD monitor, USB 2.0 compatible, and includes USB 2.0 connections.

7.	Inspection Equipment.

1.	__ Calculators with trigonometric capability.

2.	__ Date/ Received stamp and ink pad.

3.	__ Electronic Smart level, 4 foot.

4.	__ Electronic Smart level, 2 foot.

5.	__ Carpenter rulers.

6.	__ Steel tape, 100 feet.

7.	__ Cloth tape, 100 feet.

8.	__ Illuminated measuring wheel.

9.	__ Plumb bob and cord.

10.	__ Line level and cord.

11.	__ Surface thermometer.

12.	__ Concrete thermometer.

13.	__ Digital infrared asphalt thermometer.

14.	__ Direct Tension Indicator (DTI) Feeler Gage, 0.005 inch.

15.	__ Sledge hammer, 8 pound.

16.	__ Self leveling laser level with range of 100 feet and an accuracy of 1/4 inch per 100 feet.

17.	__ Hard hats - orange, reflectorized hard hats according to ANSI Z89.1.

18.	__ Safety garments – orange, reflectorized, 360º high visibility safety garments according to ANSI/ISEA Class 3, Level 2 standards.  To be replaced yearly for the duration of the contract.

19.	__ Sets of rain gear with reflective sheeting.

20.	__ Sets of hearing protection with a NRR rating of 22 dB.

21.	__ Sets of eye protection according to ANSI Z87.1.

22.	__ Sets of fall arrest equipment according to ANSII Z359.1 standards consisting of a full body harness, lanyard, and anchor.

23.	__ Light meter - capable of measuring the level of luminance in foot-candles.

24.	__ Lantern flashlight, 6V with monthly battery replacements.

25.	__ Digital Psychrometer.

26.	__ Chain Drag according to ASTM D4580-86.

27.	__ Testing equipment and apparatus conforming to AASHTO T23, T119, and T152.

28.	__ Hard Bound Daily Diaries, 5 1/2" X 8" minimum with one day per page.  To be provided yearly for the duration of the contract.

29.	__ Legal size hanging folders.

30.	__ Legal size manila file folders – three tab.

2**************************************************************************************2

BDC25S-06 Dated Apr 15, 2025

provide THE project specific items.

complete and include the following list.

sme contact – Construction Management

THE FOLLOWING IS ADDED:

31.	Balance and shield according to ASTM D2995.

32.	Pad Assemblies of 12″ X 12″ Geotextile Pads or other absorbent pads according to ASTM D2995, weighing a minimum of 8 oz/yd2, stapled to 12″ X 12″ X 1/4″ plywood.

33.	Insulated gloves for thermal protection when handling hot tack coat.

2**************************************************************************************2

1**************************************************************************************************************************1

155.03.02 Field Office Maintenance

1**************************************************************************************************************************1

BDC23S-06 Dated Mar 30, 2023

THE FIRST PARAGRAPH IS CHANGED TO:

Maintain the field office including furnishings, equipment, lavatories including toiletries, and utilities until no longer required by the Department. Provide services for utilities specified in 155.03.01.2. Provide for utility disconnection when the field office is no longer required by the RE. Assume that the field office will be required for a minimum of 3 months after Completion. Provide weekly janitorial and waste disposal service, and snow removal service as needed. Provide 1 case of letter, legal, and ledger sized paper every 2 months.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

Section 157 – Construction Layout and Monuments

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

THE SECTION IS renamed TO:

Section 157 – Monuments

1**************************************************************************************************************************1

157.01 DESCRIPTION

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

tHE FIRST PARAGRAPH IS CHANGED TO:

This Section describes the requirements for constructing monuments and monument boxes.

1**************************************************************************************************************************1

157.03.01 Construction Layout

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

the entire subpart has been revised, moved, and renamed to subsection 105.11

1**************************************************************************************************************************1

157.03.02 Monument

1**************************************************************************************************************************1

BDC21S-18 dated NOV 08, 2021

2**************************************************************************************2

BDC23s-11 dated aug 17, 2023

The subpart is renumbered:

157.03.01 Monument

2**************************************************************************************2

THE subpart is changed to:

Comply with the Map Filing Law N.J.S.A. 46:26B-1 through 8 and N.J.A.C. 13:40-5.1 through 2.  Set non-Department monuments according to the requirements of the agency.  Set Department monuments at the specified location and elevation and ensure that the monuments are held firmly in place.  Excavate so that concrete for the monument base and sides can be placed against undisturbed in-situ material, ensuring that the base is wider than the shaft.  If rock is encountered, drill into the rock to provide a rock socket to the satisfaction of the RE.  Reuse excess excavated material as specified in 202.03.03.C.1.  Place concrete, as specified in 504.03.02.D, and set the reinforcement steel and the monument marker at the time of the concrete pour.  Ensure that the top surface of the monument is level, and the disk is in the true position.  After the concrete has attained strength, punch the disk.

After the monuments have been set, obtain the current horizontal and vertical control datum values on the monument and submit these values, signed and sealed by the land surveyor, to the RE.

1**************************************************************************************************************************1

157.03.03 Monument Box

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

The Subpart is renumbered:

157.03.02 Monument Box

1**************************************************************************************************************************1

157.04 MEASUREMENT and Payment

1**************************************************************************************************************************1

BDC23S-11 dated aug 17, 2023

the construction layout pay item and payment calculation are deleted:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

MONUMENT	UNIT

MONUMENT BOX	UNIT

The Department will make payment for tree or other vegetation protection or preservation under TREE PROTECTION as specified in 158.03.02.20 for work specified in the various Items of this Subsection.

1**************************************************************************************************************************1

Section 158 – Soil Erosion and Sediment Control and Water Quality Control

158.03.02 SESC Measures

1**************************************************************************************************************************1

19.	Oil-Only Emergency Spill Kit.

Provide other type Emergency Spill kit, if needed

1**************************************************************************************************************************1

Section 159 – Traffic Control

1**************************************************************************************************************************1

provide specifications for temporary its or other requirements, if necessary.

1**************************************************************************************************************************1

159.02.01 Materials

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

THE FIRST ITEM IS CHANGED TO:

Tack Coat 64-22	902.01.01

1**************************************************************************************************************************1

159.02.02 Equipment

1**************************************************************************************************************************1

BDC23S-02 dated MAR 13, 2023

The following equipment is changed to:

Arrow Board	1001.01

1**************************************************************************************************************************1

1**************************************************************************************************************************1

Include the following EQUIPMENT if the REMOTELY OPERATED PVMS OR PORTABLE TRAILER MOUNTED CCTV CAMERA ASSEMBLY are needed

The following is added to the list of equipment references:

Portable Variable Message Sign w/Remote Communication	1001.04

Portable Trailer Mounted CCTV Camera Assembly	1001.05

1**************************************************************************************************************************1

159.03.01 Traffic Control Coordinator

1**************************************************************************************************************************1

BDC19S-08 dated DEC 11, 2019

the first PARAGRAPH before the list IS CHANGED TO:

Before starting Work, submit to the RE the name, training, work experience, and contact information of an employee assigned as the on-site Traffic Control Coordinator (TCC).  The TCC must be certified as having successfully completed the Rutgers CAIT Traffic Control Coordinator Program. A copy of the TCC’s Rutgers CAIT Certification is to be provided to the RE.  The TCC must also successfully complete an approved Traffic Coordinator refresher course every 2 years.  The TCC is a full-time position, and the employee designated as TCC must be available on a 24 hour a day, 7 days a week basis.  The TCC shall have the responsibility for and authority to implement and maintain all traffic operations for the Project on behalf of the Contractor.  Ensure that the TCC is present at the work site at all times while the Work is in progress.  The TCC’s responsibilities and duties shall include the following:

1**************************************************************************************************************************1

159.03.02 Traffic Control Devices

1**************************************************************************************************************************1

2**************************************************************************************2

BDC19S-10 dated ApR 30, 2020

The First Paragrapgh is Changed To:

Ensure that FHWA category 1, 2, 3, and 4 traffic control devices (TCDs) conform to the requirements of the 2016 Edition of the Manual for Assessing Safety Hardware (MASH), except that TCDs manufactured on or before December 31, 2019 must have been purchased by the Contractor on or before December 31, 2019, and conform to the requirements of NCHRP 350, MASH 2009, or MASH 2016.  Provide each device’s applicable MASH 2016, MASH 2009, or NCHRP 350 test results and FHWA Eligibility letter, if issued by the FHWA, to the RE.  Provide the RE with the purchase date certification for devices not meeting the MASH 2016 requirements upon delivery to the site.  Ensure that traffic control devices meet or exceed an acceptable condition as described in the ATSSA guide Quality Standards for Work Zone Traffic Control Devices.  Traffic control devices need not be new but must be in good condition.  Provide traffic control devices according to MUTCD.

2**************************************************************************************2

2.	Construction Barrier Curb.

2**************************************************************************************2

BDC19S-07 dated FEB 28, 2020

THE SECOND PARAGRAPH is CHANGED TO:

At least 30 days before delivering construction barrier curb to the Project Limits, provide the RE notice that the barrier curb is available for inspection.  Ensure the barrier curb is not stacked for this inspection.  The RE will inspect the barrier curb, along with a Contractor representative, to determine what pieces are not approved for delivery to the Project Limits.  Final determination of construction barrier approval will be made at the time of placement at the Project.

2**************************************************************************************2

2**************************************************************************************2

BDC23S-02 dated MAR 13, 2023

Part 3 is changed to:

3.	Arrow Board.  Provide an arrow board as specified in 1001.01.

2**************************************************************************************2

2**************************************************************************************2

BDC20S-15 dated Jan 15, 2021

part 5 is changed to:

5.	Temporary Crash Cushion.  Install inertial barrier systems as specified in 611.03.01.  Install temporary compressive crash cushions as specified in 611.03.02.  Immediately repair or replace crash cushions that become damaged or become inoperable.  Begin repair or replacement of the temporary crash cushion within 1 hour of receiving notice of damage from the Department.  Ensure that workers assigned to such repair or replacement work continuously until the temporary crash cushion is repaired or replaced.  If the Contractor fails to respond to a damage notification and begin work within 1 hour of notification or does not continue to work until the temporary crash cushion is repaired or replaced, the Department, will require closure of the adjacent live lane.  Lane occupancy charges will be imposed as specified in 108.08 for the period of time the adjacent lane is closed.  Should the Department have to respond to a repair with its own forces because of a Contractor’s lack of response to a damage notification, the Contractor agrees to pay the Department a sum of $3,000 for costs of mobilizing its forces and equipment.  In addition, the Contractor must pay the Department the actual cost of material used for the repair and pay the actual costs of police traffic protection.  Maintain an adequate number of replacement parts to repair damaged units at all times.  Keep the areas in front, atop, and around the crash cushions clear of snow accumulation of more than 4 inches in depth.

Upon removal of the crash cushion, cut anchor bolts at least 3 inches below the surface of the surrounding roadway.  Repair HMA pavement as specified in 401.03.03.  Repair concrete pavement as specified in 
Section 452.

2**************************************************************************************2

2**************************************************************************************2

BDC22S-07 dated AUG 01, 2022

Part 6 is changed to:

6.	Traffic Control Truck with Mounted Crash Cushions.  Provide the RE with a copy of the crash cushion manufacturer’s recommendations.  Provide the RE a certified weigh ticket of the Traffic Control Truck with arrow board and mounted Crash Cushion.  Position the traffic control truck to ensure that there is adequate stopping distance after impact and to prevent errant vehicles from traveling around the truck and endangering workers.  When used in a fixed position, place manual transmission vehicles in second gear and place automatic transmission vehicles in park.  Ensure that the parking brake is set and the wheels are set straight.  Do not use traffic control trucks in place of other temporary impact attenuators for more than 24 hours.  Relocate the traffic control truck as specified by the TCP, or as directed by the RE.  Do not use the truck to carry additional equipment, materials, or debris.  When using ballast, ensure that it is secured to the truck.  Submit drawings to the RE detailing the manner of securing the ballast, signed and sealed by a Professional Engineer, certifying that it is capable of withstanding the impact forces for which the impact attenuator is rated.

2**************************************************************************************2

9.	Portable Trailer Mounted CCTV Camera Assembly (PTMCCA).

2**************************************************************************************2

2**************************************************************************************2

if a njdot facility is impacted the lpa/lpa consultant must contact the local aid project manager to coordinate WITH MSE’S MOBILITY MANAGEMENT WORK ZONE UNIT AND TRAFFIC OPERATIONS TO CONFIRM IF REAL TIME WORK ZONE TRAFFIC SYSTEM (RTWZTS) IS REQUIRED. if CONFIRMED AND IF RTWZTS IS REQUESTED BY TRAFFIC OPERATIONS, THEN the local aid project manager will REQUEST THE SPECIFICATIONS FROM MSE to be provided to the lpa for inclusion HEREAFTER MODIFYING THE SPECIFICATIONS TO INCLUDE THE NUMBER OF PVMSRC REQUIRED WITH LOCATIONS AND ANY OTHER ADDITIONAL REQUIREMENTS SPECIFIC TO THE PROJECT. DEVELOP AND INCLUDE A REAL TIME MESSAGE TABLE LISTING THE DESIGNATED ROUTE, LOCATION OF EXISTING/PROPOSED SIGN AND THE TRAVEL TIME MESSAGE TO BE DISPLAYED.

10.	REAL TIME WORK ZONE TRAFFIC SYSTEM (RTWZTS)

2**************************************************************************************2

1**************************************************************************************************************************1

159.03.03 Removable Black Line Masking Tape

1**************************************************************************************************************************1

BDC22S-06 dated JUL 13, 2022

THE entire subsection is changed to:

Apply black line masking tape over existing traffic stripes as specified in 159.03.05.  Ensure that the black line masking tape completely covers existing stripes.  Replace black line masking tape that becomes loose after placement within 2 hours.  When black line masking tape is no longer required or directed by the RE, carefully and completely remove without using heat, solvents, grinding, sanding, or water.

1**************************************************************************************************************************1

159.03.05 Temporary Pavement Marking Tape

1**************************************************************************************************************************1

BDC22S-06 dated JUL 13, 2022

THE entire subsection is changed to:

Install tape according to the manufacturer’s recommendations when the weather is favorable as determined by the RE.  Do not install the tape during wet conditions.  Immediately before marking the pavement surface, clean the surface of dirt, oil, grease, and foreign material, including curing compound on new concrete.  Clean the surface 2 inches beyond the perimeter of the marking to be placed.

Install tape on dry surfaces having a surface temperature between 50 °F and 150 °F, when the ambient temperature is at least 50 °F and rising as determined by the National Weather Service (http://www.nws.noaa.gov/).  When splicing is required, install the tape using butt splices.  Do not overlap the tape.

Tamp the tape for initial adhesion and then apply pressure by driving a truck slowly over the tape several times.  Maintain tape by replacing loose or damaged tape within 2 hours.  Remove tape when no longer required or when directed by the RE.

The surface must be dry.  Do not install tape when precipitation is imminent as determined by the RE.  The RE will coordinate with the Contractor to install the tape when there is no anticipated precipitation.  Install the tape in continuous lengths of 20 feet or less.  Any continuous length of more than 20 feet must be removed and replaced at no cost to the Department.  Ensure that the removable tape is capable of being removed manually, intact or in large pieces, at temperatures above 40°F, without the use of solvents, burning, grinding, or blasting and without damage to the underlying surface.

If conditions do not allow for the proper adhesion of the tape, use Latex Traffic Stripes, Latex Traffic Markings Lines, and Latex Traffic Markings Symbols as specified in 159.03.06.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

159.03.06 Traffic Stripes, Latex ,Traffic Markings Lines, Latex and Traffic Markings Symbols, Latex

1**************************************************************************************************************************1

BDC24S-15 dated Aug 20, 2024

The subpart is changed to:

Apply latex traffic stripes and latex markings when they are required for 14 days or less.  Apply epoxy traffic stripes and thermoplastic markings as specified in 610.03.01 and 610.03.02 when they are required for more than 14 days.  For Contractor convenience, on pavement preservation projects, the Department will allow the Contractor to refresh latex traffic stripes and latex markings for stripes and markings required to be in place for more than 14 days and for each 14 day period thereafter.  Do not refresh these stripes with epoxy or thermoplastic material.  Apply latex traffic stripes and latex markings when the ambient and surface temperatures are at least 45 °F and rising and the surface temperature is no more than 140 °F.  Apply the latex paint in a wet film thickness of 6 ± 1 mil.  Apply glass beads to the wet paint in a uniform pattern and at the rate of 12 pounds per gallon of paint.

1**************************************************************************************************************************

159.03.08 Traffic Direction

1**************************************************************************************************************************1

BDC23S-02 dated MAR 13, 2023

Part A is changed to:

A.	Flagger.  Provide a flagger that has received formal training in flagging operations and the proper use of the STOP/SLOW paddle.  The flagger must be able to demonstrate the abilities indicated in the current MUTCD and, when requested, demonstrate competency to the RE.  Immediately replace flaggers who fail to demonstrate competency with a competent flagger.  Ensure that flaggers wear a 360 degree high-visibility retroreflective orange safety garment meeting ANSI/ISEA Class 3, Level 2 standards.  Ensure that the flagger is equipped with a STOP/SLOW paddle and follows MUTCD flagging procedures.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

FOR LOCAL AID PROJECTS THE FOLLOWING SECTION HAS BEEN DELETED AND REPLACED AS FOLLOWS:

Part B is changed to:

B.	Police. Assignment and use of police to be used in the enforcement of the approved Traffic Control Plan (TCP) will follow applicable state law.  The Contractor will be responsible for obtaining local police for use on the project.   The use of police services by the RE does not relinquish or diminish the Contractor’s responsibilities for work zone safety.

The Contractor will submit a request for police services to the local police department per their requirements. The RE will be notified at the time of the request.

Activities requiring police services include:

1.	Traffic direction through signalized intersections, where the integrity of the existing traffic signal system is impacted or where an override of the signal is required.

Emergency situations may prevent police from arriving at the scheduled date or time.  The RE will not permit construction operations that, by law, require police services if police are unavailable.  The Department will not accept claims for interruptions or delays resulting from any failure of police to arrive as requested.

The Contractor must notify local police of cancellations 24 hours in advance.  At least 24 hours before the scheduled start of work, notify the RE of any work cancellation for which police services were requested.

1**************************************************************************************************************************1

159.03.09 Emergency Towing Service

1**************************************************************************************************************************1

BDC25S-05 dated jun 12, 2023

the subpart is changed to:

At least 30 days before the start of construction operations, submit to the RE for approval an emergency towing service plan for removing abandoned vehicles and vehicles that are disabled but not as the result of an accident.  The police will arrange for towing services to remove vehicles that are disabled due to traffic accidents.  Indicate in the plan the proposed manner of providing towing service, including the type, quantity, and location of towing equipment to be used.

Provide towing service to remove disabled and abandoned vehicles from the construction zone to the nearest location where the vehicle can be legally parked.  Ensure that the towing service responds immediately upon notice of a disabled vehicle.  If the towing service fails to respond within a timeframe as determined by the RE, the Department will use its own forces to remove the disabled vehicle and may recover these costs as specified in 107.16.

Provide light duty emergency towing service for emergency towing of vehicles with a Gross Vehicle Weight Rating (GVWR) up to an including 26,000 pounds, and heavy-duty emergency towing service for emergency towing of vehicles with a GVWR greater than 26,000 pounds that require specialized equipment capable of moving large disabled vehicles.

Provide on-site emergency towing service only when lane closures include a lane that is barrier-delineated on both sides (barrier-delineated lane) and when directed by the RE.  Ensure that the tow vehicle is stationed at the end of the barrier-delineated lane and remains on standby as long as the barrier-delineated lane is in place.

1**************************************************************************************************************************1

159.04  Measurement and Payment

1**************************************************************************************************************************1

BDC26S-01 dated MAR 09, 2026

the subsection is changed to:

2**************************************************************************************2

AFTER CHECKING WITH TRAFFIC OPERATIONS INCLUDE PAYMENT CRITERIA FOR PVMSRC AND PTMCCA. NOTE THAT WHEN REAL TIME WORK ZONE SYSTEM (RTWZS) IS ALSO SPECIFIED IN A PROJECT INCLUDE SEPARATE SPECIFIC PAYMENT CRITERIA BASED ON PERFORMANCE OF RTWZS.

SME CONTACT – TRAFFIC OPERATIONS & Mobility and Systems Engineering (MSE)

2**************************************************************************************2

The Department will measure and make payment for Items as follows:

Item	Pay Unit

BREAKAWAY BARRICADE	UNIT

DRUM	UNIT

TRAFFIC CONE	UNIT

CONSTRUCTION SIGNS	SQUARE FOOT

CONSTRUCTION IDENTIFICATION SIGN, ___' X ___'	UNIT

CONSTRUCTION BARRIER Curb	LINEAR FOOT

ARROW Board, ___' X ___'	UNIT

Portable VARIABLE MESSAGE SIGN	UNIT

TEMPORARY CRASH CUSHION, INERTIAL BARRIER SYSTEM, ___ MODULES	UNIT

TEMPORARY crash cushion, compressive barrier, type___, width___	unit

TEMPORARY crash cushion, Low maintenance compressive barrier,

type___, width___	unit

TRAFFIC CONTROL TRUCK WITH MOUNTED CRASH CUSHION	UNIT

TRAFFIC STRIPES, LATEX ___"	LINEAR FOOT

Traffic MARKINGs LINES, LATEX ___"	LINEAR FOOT

TRAFFIC MARKINGS SYMBOLS, LATEX	SQUARE FOOT

channelizing Guide post	UNIT

PORTABLE VARIABLE MESSAGE SIGN WITH REMOTE COMMUNICATION	UNIT

PORTABLE TRAILER MOUNTED CCTV CAMERA ASSEMBLY	UNIT

REMOVABLE BLACK LINE MASKING TAPE, ___"	LINEAR FOOT

Temporary Pavement Markers	Unit

Temporary PAVEMENT MARKING TAPE, ___"	LINEAR FOOT

HMA Patch	Ton

TRAFFIC DIRECTOR, FLAGGER	HOUR

Emergency Towing Service, Light Duty	UNIT

Emergency Towing service, heavy duty	unit

emergency towing service, on-site	hour

The Department will not make payment for repair or replacement of temporary crash cushion items under TEMPORARY CRASH CUSHION, INERTIAL BARRIER SYSTEM, ___ MODULES; TEMPORARY CRASH CUSHION, COMPRESSIVE BARRIER, TYPE___, WIDTH___, and TEMPORARY CRASH CUSHION, LOW MAINTENANCE COMPRESSIVE BARRIER, TYPE___, WIDTH___.  The Department will make payment for repair or replacement of temporary crash cushion items as specified in 109.03.

For traffic control devices measured by the linear foot or unit basis that are specified in 159.03.02, the Department will make payment for the maximum quantity in service at one time as required by the Contract.  For CONSTRUCTION SIGNS, the Department will make payment for the maximum quantity of specific sign types in service at one time as required by the Contract.  If a particular sign type has more than one unique text, each sign with a unique text will be considered to be a specific sign type.  The Department will make payment for 50 percent of the Contract bid price for traffic control devices specified in 159.03.02 that are measured on a linear foot, square foot or unit basis upon approved placement.  The Department will prorate the balance of payment over the duration of the Contract.

The Department will measure TRAFFIC STRIPES, LATEX and TRAFFIC MARKINGS LINES, LATEX by the linear foot for each specified width of stripe.  The Department will not measure gaps in striping.

The Department will measure TEMPORARY PAVEMENT MARKING TAPE by the linear foot of 4 inch wide strips.

The Department will measure REMOVABLE BLACK LINE MASKING TAPE by the linear foot for each 4 inch width of existing stripe that is to be covered.  The Department will not measure replacement temporary pavement stripes and markers for payment.

If the Contractor cancels work without providing at least 24 hour notice, or if the RE rescinds the lane or shoulder closure approval due to weather conditions, the Department will recover the cost for police services as specified in 107.16 in the amount specified in the Special Provisions.  The Department will not deduct the cost of police services from the Contract if the reason for cancellation is unforeseen weather conditions.  Precipitation that occurs after the National Weather Service (https://www.weather.gov/) locally forecasts, 24 hours prior to the lane closure, a 50 percent or greater chance of precipitation during the scheduled closure, is not an unforeseen weather condition.

2*************************************************************************************************2

provide the current police officer hourly rate

SME CONTACT – bureau of construction management

If the Contractor cancels work without providing at least 24 hour notice or if the RE rescinds the lane or shoulder closure approval due to weather conditions, the Department will recover the cost for police services as specified in 107.16.  The Department will deduct from the Contract an amount equal to 4 hours for each police officer scheduled at the hourly individual officer rate of $_______.  The Department will not deduct the cost of police services if the reason for cancellation is unforeseen weather conditions.  Precipitation that occurs after the National Weather Service (https://www.weather.gov/) locally forecasts, 24 hours prior to the lane closure, a 50 percent or greater chance of precipitation during the scheduled closure, is not an unforeseen weather condition.

2*************************************************************************************************2

If after being notified by the Department that the PORTABLE VARIABLE MESSAGE SIGN WITH REMOTE COMMUNICATION or PORTABLE TRAILER MOUNTED CCTV CAMERA ASSEMBLY has failed to function and the equipment has not been restored to good working order within 48 hours, the Department will make payment reductions as follows:

For each occasion the equipment was not restored within 48 hours the Department will assess a liquidated damage of $250.00 for every 48 hours period the equipment is not functioning.

The Department will not include payment for epoxy traffic stripes and thermoplastic traffic markings and symbols under TRAFFIC STRIPES LATEX, TRAFFIC MARKINGS LINES, LATEX, and TRAFFIC MARKINGS SYMBOLS, LATEX.  The Department will make payment for epoxy traffic stripes and thermoplastic traffic markings under TRAFFIC STRIPES, TRAFFIC MARKINGS LINES, and TRAFFIC MARKINGS SYMBOLS as specified in 610.04.  The Department will not make payment under TRAFFIC STRIPES LATEX, TRAFFIC MARKINGS LINES, LATEX, and TRAFFIC MARKINGS SYMBOLS, LATEX when latex stripes, markings, and symbols are refreshed for Contractor convenience.

The Department will make payment for EMERGENCY TOWING SERVICE, LIGHT DUTY and EMERGENCY TOWING SERVICE, HEAVY DUTY by the unit.  A unit consists of 1 tow of a disabled or abandoned vehicle, not as the result of an accident, from the construction zone to the nearest location where the vehicle can be legally parked.

The Department will make payment for EMERGENCY TOWING SERVICE, ON-SITE, based on the total hours the approved tow vehicle and driver are on-site as directed by the RE, which includes compensation for both the standby time and time for the towing service.

1**************************************************************************************************************************1

Section 160 – Price Adjustments

THE entire Section 160 is changed to:

1**************************************************************************************************************************1

BDC22S-09 dated Jan 17, 2023

160.01 DESCRIPTION

This Section describes the requirements for price adjustments for fuel and asphalt usage.

160.02 MATERIALS

(Intentionally Blank)

160.03 PROCEDURE

160.03.01 Fuel Price Adjustment

2**************************************************************************************2

BDC25S-03 dated apr 24, 2025

the entire subpart is changed to:

The Department will make price adjustments for fuel usage for Items listed in Table 160.03.01-1 and the Special Provisions.  Each month may be divided into two periods.  Period one includes the first day of the month through the fourteenth day of the month.  Period two includes the fifteenth day of the month through the last day of the month.  Work starting within period one and continuing past midnight of the fourteenth day into the fifteenth day of the month will be included in period one for any price adjustments.  Work continuing past midnight of the last day of the month into the first day of the next month will be included in period two.

The Department will calculate fuel price adjustments based on the pay quantities of listed Items using the fuel usage factors listed in Table 160.03.01-1 and the Special Provisions.

Price adjustments may result in an increased payment to the Contractor for increases in the price index and may result in a reduction in payment for decreases in the price index.

If the as-built quantity of an Item eligible for fuel price adjustment differs from the sum of the quantities in the Estimates and the as-built quantity cannot be readily distributed among the time periods that the Item was constructed, then the Department will determine fuel price adjustment by distributing the difference in the same proportion as the Item’s Estimate quantity is to the total of the Item’s time period estimates.

| Table 160.03.01-1 Fuel Price Adjustments |  |
| --- | --- |
| Items | Fuel Usage Factor |
| EXCAVATION, UNCLASSIFIED | 0.50 Gallons per Cubic Yard |
| EXCAVATION, REGULATED MATERIAL | 0.50 Gallons per Cubic Yard |
| EXCAVATION, ACID PRODUCING SOIL | 0.50 Gallons per Cubic Yard |
| REMOVAL OF PAVEMENT | 0.25 Gallons per Square Yard |
| MICRO-MILLING | 0.25 Gallons per Square Yard |
| HMA MILLING, 3" OR LESS | 0.25 Gallons per Square Yard |
| HMA MILLING, MORE THAN 3" TO 6" | 0.25 Gallons per Square Yard |
| CONCRETE MILLING | 0.25 Gallons per Square Yard |
| HMA PROFILE MILLING | 0.25 Gallons per Square Yard |
| BREAKING PAVEMENT | 0.25 Gallons per Square Yard |
| RUBBLIZATION | 0.25 Gallons per Square Yard |
| SUBBASE | 1.00 Gallon per Cubic Yard |
| I-___ SOIL AGGREGATE | 1.00 Gallon per Cubic Yard |
| SOIL AGGREGATE BASE COURSE, ___ " THICK | 1.00 Gallon per Cubic Yard |
| SOIL AGGREGATE BASE COURSE, VARIABLE THICKNESS | 1.00 Gallon per Cubic Yard |
| DENSE-GRADED AGGREGATE BASE COURSE, ___ " THICK | 1.00 Gallon per Cubic Yard |
| DENSE-GRADED AGGREGATE BASE COURSE, VARIABLE THICKNESS | 1.00 Gallon per Cubic Yard |
| CONCRETE BASE COURSE, ___ " THICK | 0.25 Gallons per Square Yard |
| CONCRETE BASE COURSE, REINFORCED ___ " THICK | 0.25 Gallons per Square Yard |
| ASPHALT-STABILIZED DRAINAGE COURSE | 2.50 Gallons per Ton |
| OPEN-GRADED ___ FRICTION COURSE | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ SURFACE COURSE | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ SURFACE COURSE HIGH RAP | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ INTERMEDIATE COURSE | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ INTERMEDIATE COURSE HIGH RAP | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ BASE COURSE | 2.50 Gallons per Ton |
| HOT MIX ASPHALT ___ ___ ___ BASE COURSE HIGH RAP | 2.50 Gallons per Ton |
| MODIFIED OPEN-GRADED ___ FRICTION COURSE ___ | 2.50 Gallons per Ton |
| ULTRA-THIN FRICTION COURSE | 2.50 Gallons per Ton |
| STONE MATRIX ASPHALT ___ SURFACE COURSE | 2.50 Gallons per Ton |
| HIGH PERFORMANCE THIN OVERLAY | 2.50 Gallons per Ton |
| BINDER RICH INTERMEDIATE COURSE | 2.50 Gallons per Ton |
| BRIDGE DECK WATERPROOFING SURFACE COURSE | 2.50 Gallons per Ton |
| NON-VEGETATIVE SURFACE, HOT MIX ASPHALT | 2.50 Gallons per Ton |
| NON-VEGETATIVE SURFACE COURSE, POROUS HOT MIX ASPHALT | 2.50 Gallons per Ton |
| COLOR-COATED NON-VEGETATIVE SURFACE, HOT MIX ASPHALT | 2.50 Gallons per Ton |
| CONCRETE SURFACE COURSE, ___ " THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, 4" THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, 5" THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, 6" THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, 8" THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, REINFORCED, 6" THICK | 0.25 Gallons per Square Yard |
| CONCRETE SIDEWALK, REINFORCED, 8" THICK | 0.25 Gallons per Square Yard |
| DIAMOND GRINDING OF CONCRETE SURFACE COURSE | 0.25 Gallons per Square Yard |
| DIAMOND GRINDING EXISTING CONCRETE PAVEMENT | 0.25 Gallons per Square Yard |
| SLURRY SEAL AGGREGATE, TYPE II | 2.5 Gallons per Ton |
| SLURRY SEAL EMULSION | 0.10 Gallons per Gallon |
| CONCRETE BRIDGE APPROACH | 0.50 Gallons per Cubic Yard |
| CONCRETE CULVERT | 1.00 Gallon per Cubic Yard |
| CONCRETE FOOTING | 1.00 Gallon per Cubic Yard |
| CONCRETE WING WALL | 1.00 Gallon per Cubic Yard |
| CONCRETE PIER COLUMN PROTECTION, HPC | 1.00 Gallon per Cubic Yard |
| CONCRETE PIER COLUMNS AND CAP | 1.00 Gallon per Cubic Yard |
| CONCRETE ABUTMENT WALL | 1.00 Gallon per Cubic Yard |
| CONCRETE PIER SHAFT | 1.00 Gallon per Cubic Yard |
| CONCRETE PEDESTRIAN BRIDGE | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE DECK | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE DECK, HPC | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE SIDEWALK | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE SIDEWALK HPC | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE PARAPET | 1.00 Gallon per Cubic Yard |
| CONCRETE BRIDGE PARAPET HPC | 1.00 Gallon per Cubic Yard |
| 15" BY 32" CONCRETE BARRIER CURB, BRIDGE | 0.12 Gallon per Linear Foot |
| 24" BY 32" CONCRETE BARRIER CURB, BRIDGE | 0.17 Gallon per Linear Foot |
| 21" BY 34" CONCRETE BARRIER CURB, BRIDGE | 0.15 Gallon per Linear Foot |
| 24" BY 42" CONCRETE BARRIER CURB, BRIDGE | 0.21 Gallon per Linear Foo |
| CAST-IN-PLACE CONCRETE PILES, DRIVEN ___ " DIAMETER | 1.00 Gallon per Cubic Yard |
| RETAINING WALL, LOCATION NO.___ ___ | 0.10 Gallon per Square Foot |
| CONCRETE MEDIAN BARRIER, HPC | 0.16 Gallon per Linear Foot |
| 15" BY 41" CONCRETE BARRIER CURB | 0.28 Gallon per Linear Foot |
| 24" BY 32" CONCRETE BARRIER CURB | 0.17 Gallon per Linear Foot |
| 15" BY 54" CONCRETE BARRIER CURB | 0.15 Gallon per Linear Foot |
| 38" BY 79" CONCRETE BARRIER CURB | 0.40 Gallon per Linear Foot |
| 24" BY 39" CONCRETE BARRIER CURB | 0.18 Gallon per Linear Foot |
| 18 5/8" BY 65" CONCRETE BARRIER CURB | 0.20 Gallon per Linear Foot |
| 32" BY 41" CONCRETE BARRIER CURB | 0.24 Gallon per Linear Foot |
| 24" BY 41" CONCRETE BARRIER CURB | 0.19 Gallon per Linear Foot |
| 24" BY 45" CONCRETE BARRIER CURB | 0.19 Gallon per Linear Foot |
| 15" BY 35" CONCRETE BARRIER CURB, DOWELLED | 0.09 Gallon per Linear Foot |
| 15" BY VARIABLE HEIGHT CONCRETE BARRIER CURB | 0.28 Gallon per Linear Foot |
| 24" BY VARIABLE HEIGHT CONCRETE BARRIER CURB | 0.15 Gallon per Linear Foot |
| 15" BY VARIABLE HEIGHT CONCRETE BARRIER CURB, DOWELLED | 0.24 Gallon per Linear Foot |
| 24" BY VARIABLE HEIGHT CONCRETE BARRIER CURB, DOWELLED | 0.15 Gallon per Linear Foot |
| 19" BY 32" CONCRET BARRIER CURB, DOWELLED | 0.10 Gallon per Linear Foot |
| 24" BY 32" CONCRETE BARRIER CURB, DOWELLED | 0.13 Gallon per Linear Foot |
| 24 1/2" BY 53" CONCRETE BARRIER CURB, DOWELLED | 0.18 Gallon per Linear Foot |
| 24 1/2" BY VARIABLE HEIGHT CONCRETE BARRIER CURB, DOWELLED | 0.15 Gallon per Linear Foot |
| 24" BY 35" CONCRETE BARRIER CURB, DOWELLED | 0.13 Gallon per Linear Foot |
| GROUND MOUNTED BARRIER CURB | 0.15 Gallon per Linear Foot |
| 15" BY 51" F SHAPE CONCRETE BARRIER CURB | 0.34 Gallon per Linear Foot |
| 24 1/2" BY 51" F SHAPE CONCRETE BARRIER CURB | 0.23 Gallon per Linear Foot |
| 24 1/2" BY ___" F SHAPE CONCRETE BARRIER CURB, DOWELLED | 0.23 Gallon per Linear Foot |
| 15" BY VARIABLE HEIGHT F SHAPE CONCRETE BARRIER CURB, DOWELLED | 0.34 Gallon per Linear Foot |
| 15" BY ___" F SHAPE CONCRETE BARRIER CURB, DOWELLED | 0.34 Gallon per Linear Foot |
| VARIABLE WIDTH BY VARIABLE HEIGHT F SHAPE CONCRETE BARRIER CURB | 0.34 Gallon per Linear Foot |
| 9" BY 16" CONCRETE VERTICAL CURB | 0.04 Gallon per Linear Foot |
| 9" BY 18" CONCRETE VERTICAL CURB | 0.04 Gallon per Linear Foot |
| 9" BY 20" CONCRETE VERTICAL CURB | 0.04 Gallon per Linear Foot |
| 9" BY 22" CONCRETE VERTICAL CURB | 0.05 Gallon per Linear Foot |
| 9" BY 14" CONCRETE VERTICAL CURB | 0.03 Gallon per Linear Foot |
| 9" BY 4" CONCRETE VERTICAL CURB, DOWELLED | 0.01 Gallon per Linear Foot |
| 9" BY 6" CONCRETE VERTICAL CURB, DOWELLED | 0.01 Gallon per Linear Foot |
| 9" BY 8" CONCRETE VERTICAL CURB, DOWELLED | 0.02 Gallon per Linear Foot |
| 9" BY 10" CONCRETE VERTICAL CURB, DOWELLED | 0.02 Gallon per Linear Foot |
| 12” BY 13” CONCRETE SLOPING CURB | 0.04 Gallon per Linear Foot |
| 12" BY 3" CONCRETE SLOPING CURB, DOWELLED | 0.01 Gallon per Linear Foot |
| ___" BY ___" CONCRETE SLOPING CURB, DOWELLED | 0.01 Gallon per Linear Foot |
| 9" BY VARIABLE HEIGHT CONCRETE VERTICAL CURB | 0.04 Gallon per Linear Foot |
| 9" BY VARIABLE HEIGHT CONCRETE VERTICAL CURB, DOWELLED | 0.02 Gallon per Linear Foot |

3************************************************3

ADD AND COMPLETE TABLE 160.03.01-1a IF THERE ARE Non-Standard ITEMS THAT SHOULD BE ELIGIBLE FOR FUEL PRICE ADJUSTMENT. ITEMS SHOULD BE SIGNIFICANT TO THE PROJECT AND SIMILAR IN SCOPE TO EXISTING TABLE ITEMS.

SME CONTACT - BUREAU OF CONSTRUCTION MANAGEMENT

| Table 160.03.01a-1 Additional Fuel Price Adjustments |  |
| --- | --- |
| Items | Fuel Usage Factor |

3************************************************3

If an item listed in Table 160.03.01-1 or the Special Provisions has a payment unit which differs from that listed in Table 160.03.01-1 or the Special Provisions, the Department will apply an appropriate conversion factor to determine the number of gallons of fuel used.

The Department will calculate fuel price adjustment using the following formula:

| F = (MF − BF) ´ G |  |
| --- | --- |
| Where: |  |
| F = | Fuel Price Adjustment |
| MF = | Fuel Price Index for work performed in the time period immediately before the estimate cutoff date. |
| BF = | Basic Fuel Price Index |
| G = | Gallons of Fuel for Price Adjustment |

The Department will post the Fuel Price Index every month on the Department’s website: https://www.state.nj.us/transportation/business/aashtoware/PriceIndex.shtm.

The Basic Fuel Price Index is the Index which is listed for the month prior to the receipt of bids.  For new work added that is eligible for Fuel Price Adjustment, the Basic Fuel Price Index is the index which is listed for the month the new work was added to the Contract.  If the month prior to the receipt of bids or the month the new work was added has two Indices, the Index in effect for the first day of that month will govern for the Basic Fuel Price Index.  If the Fuel Price Index increases by 50 percent or more over the Basic Fuel Price Index, do not perform any work involving Items listed in Table 160.03.01-1 or the Special Provisions without written approval from the RE.

2**************************************************************************************2

1**************************************************************************************************************************1

160.03.02  Asphalt Price Adjustment

The Department will make price adjustments for asphalt binder usage.  The Department will calculate asphalt price adjustments based on the quantities of Items containing asphalt binder constructed.

Each month may be divided into two periods.  Period one includes the first day of the month through the fourteenth day of the month.  Period two includes the fifteenth day of the month through the last day of the month.  Work starting on the fourteenth day of the month and continuing past midnight into the fifteenth day of the month will be included in period one for any price adjustments.  Work continuing through midnight of the last day of the month into the first day of the next month will be included in period two.

2**************************************************************************************2

BDC25S-03 dated apr 24, 2025

the third paragraph is changed to:

The price used for both the Basic and Monthly Price Indexes will be determined based on the performance grade of asphalt binder in the approved mix design for the asphalt mixture.

2**************************************************************************************2

Price adjustments may result in an increased payment to the Contractor for increases in the price index and may result in a reduction in payment for decreases in the price index.

The Department will calculate the asphalt price adjustment by the following formula:

| A = (MA − BA) ´ T |  |
| --- | --- |
| Where: |  |
| A = | Asphalt Price Adjustment |
| MA = | Asphalt Price Index for work performed in the time period immediately before the estimate cutoff date. |
| BA = | Basic Asphalt Price Index |
| T = | Tons of New Asphalt Binder1 |
| 1. The Department will determine the weight of asphalt binder for price adjustment by multiplying the percentage of new asphalt binder in the approved job mix formula by the weight of the item containing asphalt binder. If a Hot Mix Asphalt Item has a payment unit other than ton, the Department will apply an appropriate conversion factor to determine the number of tons of asphalt binder used. |  |

For Tack Coat, Prime Coat, MICRO SURFACING EMULSION, SLURRY SEAL EMULSION, and FOG SEAL SURFACE TREATMENT, the Department will calculate the weight of asphalt as follows:

| T= G ´ C ´ 0.00428 |  |
| --- | --- |
| C = | Petroleum content of the product |
|  | Use 100% for Tack Coat 64-22 and Tack Coat 64E-22 |
|  | Use 60% for Polymer Modified Tack Coat, and all other emulsified asphalts |
| G = | Gallons furnished |
| The constant 0.00428 is derived from the conversion factor of tons per gallon using 8.345 lbs/gallon for water and a factor of 1.025 for the specific gravity of asphalt binder. |  |
| The Department will not calculate an asphalt price adjustment for FOG SEAL STRIP. |  |

The monthly asphalt price index, as determined by the Department, will be the average of quotations from suppliers serving the area in which the Project is located, and will be determined by the Department.  The Department will post the asphalt price index every month on the Department’s website: https://www.state.nj.us/transportation/business/aashtoware/PriceIndex.shtm.

The Basic Asphalt Price Index will be the Index which is listed for the month prior to the receipt of bids.  If the month prior to the receipt of bids has two Indexes, the Index in effect for the first day of the month will govern for the Basic Asphalt Price Index.

The Monthly Asphalt Price Index will be that for the month that the work is constructed in.  If work is constructed over the course of two or more months for a particular pay estimate, then multiple Monthly Indexes will be used corresponding to the date that the work was performed.

If the Asphalt Price Index increases 50 percent or more over the basic asphalt price index, do not perform work on Items containing asphalt binder without written approval from the RE.

2**************************************************************************************2

BDC23S-01 dated Mar 14, 2023

complete and include THE FOLLOWING when a Contract contains 1,000,000 pounds (or 500,000 pounds for contracts with a 3 year or greater duration from AWARD to completion) oF a combination of the below Items except that BEAM GUIDE RAIL, RUB RAIL, OVERHEAD SIGN STrUCTURE and CANTILeVER SIGN STrUcTURE will always be included without meeting the pound requirement.

The following is added:

160.03.03  Steel Price Adjustment

3************************************************3

bdc24s-14 dated nov 7, 2024

The FIRST PARAGRAPH is CHANGED TO:

The Department will make a steel price adjustment for Items listed in table 160.03.03-1 using the price index indicated for the Item.  The Contractor may opt out of a steel price adjustment for one or more of the categories of structural steel, reinforcement steel, or piles, shown in table 160.03.03-1, by providing the Regional Construction Engineer with a list of those categories to which the Contractor does not want to apply the steel price adjustment.  Provide the list within 5 days of award of the Contract.  Ensure the list identifies all items in each opt out category by description and Item number.  Only entire categories will be considered. Only the items listed will be excluded for a price adjustment.  Failure to submit the list within the specified time will result in losing the opportunity to opt out of the steel price adjustment.  The steel price adjustment will always apply to beam guide rail, rub rail, and sign structuresThis adjustment is based solely on the mill provided steel.

3************************************************3

complete and REMOVE ITEMS THAT DO NOT QUALIFY

| Table 160.03.03-1 Steel Price Adjustment Items |  |
| --- | --- |
| Item | Price Index (BS & MS) |
| STRUCTURAL STEEL | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Hot Rolled Steel Bars, Plates, and Structural Shapes WPU 101704 |
| REINFORCEMENT STEEL | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| REINFORCEMENT STEEL, EPOXY-COATED | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| REINFORCEMENT STEEL, GALVANIZED | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| REINFORCEMENT STEEL, STAINLESS | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| CAST-IN-PLACE CONCRETE PILE FURNISHED | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Hot Rolled Steel Bars, Plates and Structural Shapes WPU 101704 |
| STEEL H-PILE, FURNISHED | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Hot Rolled Steel Bars, Plates and Structural Shapes WPU 101704 |
| BEAM GUIDE RAIL | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| RUB RAIL | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| OVERHEAD SIGN STRUCTURE NO. ___ | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| BUTTERFLY SIGN SUPPORT, DMS STRUCTURE NO. ___ | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| CANTILEVER SIGN STRUCTURE NO. ___ | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |
| CANTILEVER SIGN SUPPORT, DMS STRUCTURE NO. ___ | US Dept. of Labor, Bureau of Labor Statistics - Producer Price Index for Semifinished Steel Mill Products WPU 101702 |

3************************************************3

Price adjustments may result in an increased payment to the Contractor for increases in the price index and may result in a reduction in payment for decreases in the price index.

A steel price adjustment will only be made for price increases when  MS-BSBS >10%MS-BSBS>15%, and a price adjustment will only be made for price decreases when BS-MSBS>15% BS-MSBS >10%.

When the Monthly Steel Price Index is greater than the Benchmark Steel Price Index, the Department will calculate STEEL PRICE ADJUSTMENT using the following formula:

S=[BS-MS+BS×0.10]-BS ×CB×W

When the Monthly Steel Price Index is less than the Benchmark Steel Price Index, the Department will calculate STEEL PRICE ADJUSTMENT using the following formula:

S=[MS-BS+BS×0.10]BS ×CB×W

| Where: |  |
| --- | --- |
| S = | Steel Price Adjustment (Dollars). |
| BS = | Benchmark Steel Price Index – the steel preliminary price index for the month before the project is bid. |
| MS = | Monthly Steel Price Index – the steel price index for the month steel is shipped from the mill. |
| CB = | Cost Basis ($/lb). |
| W = | Weight of Steel (lb). |

The Department will post the BS, MS and CB value every month at

https://www.state.nj.us/transportation/business/aashtoware/SteelPriceIndex.shtm.

With each delivery of steel, submit to the RE documentation from the fabricator or supplier, which details the following information:

1.	Weight of the steel shipped from the mill to the fabricator or supplier

2.	Name of the mill, fabricator or supplier or both.

3.	Identifying transmittal or invoice number for each shipment.

4.	Date of the shipment

5.	Item description and Item number(s) for which the steel is associated.

If the documented steel weight is for more than one Item, provide the RE with the weight attributed to each Item.

3************************************************3

CONTACT THE FIELD MANAGER TO DETERMINE IF THE FOLLOWING IS TO BE ADDED. THIS SHOULD ONLY BE ADDED FOR PROJECTS with more than 5 million lbs OF steel.

On a monthly basis, provide the RE with a certified tabulation listing all the eligible steel shipments for the prior month.  The tabulation must list the above items correlating to the identifying marks noted on the mill delivery reports.

On a monthly basis, complete form DC-160(S) Steel Price Adjustment.

3************************************************3

Provide the documentation to the RE within 60 days of the date of the shipment.  The Department will not make a price adjustment for steel shipped before the bid date.

The Department will make a price adjustment for the Items “STRUCTURAL STEEL” based on the weights listed in Table 160.03.03-2.  If the steel for an Item is shipped on dates having different monthly index prices, the Department will proportionately adjust the weight used in the calculation of the price adjustment by multiplying the weight shipped by the ratio of the weight for the Item listed in Table 160.03.03-2 to the sum total of weight shipped for that Item.  If the weight of steel estimated for a structure in Table 160.03.03-2 differs from the actual weight by more than 10 percent, the Department will make a price adjustment based on the actual weight.

3************************************************3

complete and include THE FOLLOWING

| Table 160.03.03-2 Structural Steel Price Adjustment Items |  |  |
| --- | --- | --- |
| Structure | Item No. | Weight (Lb) |

3************************************************3

The Department will make a price adjustment for the Items OVERHEAD SIGN STRUCTURE and CANTILEVER SIGN STRUCTURE based on the weights listed in Table 160.03.03-3.  If the steel for an Item is shipped on dates having different monthly index prices, the Department will proportionately adjust the weight used in the calculation of the price adjustment by multiplying the weight shipped by the ratio of the weight for the Item listed in Table 160.03.03-3 to the sum total of weight shipped for that Item.  If the weight of steel estimated for a structure in Table 160.03.03-3 differs from the actual weight by more than 10percent, the Department will make a price adjustment based on the actual weight.

3************************************************3

complete and include THE FOLLOWING

| Table 160.03.03-3 Overhead Sign Structure and Cantilever Sign Structure Price Adjustment Items |  |  |
| --- | --- | --- |
| Overhead and Cantilever Sign Structure Number | Item No. | Weight (Lb) |

3************************************************3

The Department will only make a price adjustment for reinforcement steel for steel provided in association with the reinforcement steel items.  The Department will make a price adjustment for reinforcement steel based on the weight of reinforcement steel indicated in the Proposal for an Item as adjusted by Change Orders.  If the reinforcement steel for an Item is shipped on dates having different monthly index prices, the Department will proportionately adjust the weight used in the calculation of the price adjustment by multiplying the weight shipped by the ratio of the weight for the Item to the sum total of weight shipped for that Item.

The Department will make a price adjustment for piles based on the weight of steel furnished for pipe piles and for H-piles based on the RE’s order list.  The weight of steel furnished for piles will not include the weight for ancillary materials such as pile shoes and splice collars.

The Department will make a price adjustment for BEAM GUIDE RAIL (including posts) and RUB-RAIL based on the weight of steel furnished.  The weight does not include coating.  The weight does not include associated hardware and the weight of end treatments.

If the preliminary Monthly Steel Price Index increases 100percent or more over the Benchmark Steel Price Index, do not order more steel without written approval from the RE.  The RE will determine if work will continue based on the Steel Price Index increase.

There will be no increase to STEEL PRICE ADJUSTMENT if the work is behind schedule by fault of the Contractor and the steel was not purchased prior to the delay.

2**************************************************************************************2

160.04  Measurement and Payment

The Subsection is changed to:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

FUEL PRICE ADJUSTMENT	dollar

ASPHALT PRICE ADJUSTMENT	dollar

2**************************************************************************************2

BDC23S-01 dated MAR 14, 2023

The following is added:

Item	Pay Unit

STEEL PRICE ADJUSTMENT	dollar

2**************************************************************************************2

2**************************************************************************************2

BDC24S-17 dated sep 3, 2024

the second paragraph is changed to:

The Items Fuel Price Adjustement and Asphalt Price Adjustment must be included in the Proposal or added to the Contract to qualify for payment.

2**************************************************************************************2

1**************************************************************************************************************************1

Division 200 – Earthwork

Section 201 – Clearing Site

201.03.01  Clearing Site

1**************************************************************************************************************************1

2**************************************************************************************2

2**************************************************************************************2

BDC22S-05 dated MaY 27, 2022

Part A is changed to:

A.	Preparation.  Construct SESC measures, as specified in 158.03.02, before clearing site.

The first paragraph in Part B is changed to:

B.	Clearing and Grubbing.  Before beginning excavation or embankment construction, clear the site within the limits of construction.  Clear the ground surface of vegetation (trees of various caliper, brush, weeds, roots, matted leaves), small structures not shown on the Plans for demolition, debris, and other objectionable material where its existing position conflicts with the limits of construction.  In cut sections, grub out tree stumps within the limits of the total cut area.  In fill sections, the Contractor may leave tree stumps extending less than 1 foot above the original ground surface in those areas where the proposed subgrade, or proposed finished grade in non-pavement sections, is greater than 3 1/2 feet above the original ground surface.  Grub out tree stumps that lie within 5 feet horizontally or vertically from any proposed structure, pipe, or duct.

2**************************************************************************************2

1**************************************************************************************************************************1

201.03.02  Clearing Site, Bridge and Clearing Site, Structure

1**************************************************************************************************************************1

2**************************************************************************************2

BDC22S-05 dated MaY 27, 2022

The first paragraph is changed to:

Submit a demolition plan detailing the work area, methods, and equipment to be used to the RE for approval 30 days before demolition operations.  Clear site within work area as specified in 201.03.01.  Remove the substructures of existing structures to at least 3 feet below the natural stream bottom, and remove those parts outside of the stream to at least 2 feet below natural ground surface.  Where such portions of existing structures lie wholly or in part within the limits of a new structure, remove them to accommodate the construction of the proposed structure.  Only the following equipment is permitted for the work:

2**************************************************************************************2

2**************************************************************************************2

include THE FOLLOWING FOR DECK AND/OR BEAM ENCASEMENT REMOVAL WHEn THE REMAINDER OF EXISTING SUPERSTRUCTURE IS TO REMAIN.

the following is added:

The procedure is described below:

1.	Prestressed Concrete Stringers and Concrete Diaphragms.  Repair damage to prestressed concrete stringers and concrete diaphragms using nonshrink grout conforming to 903.08 before deck placement.

2.	Steel Stringers, Floorbeams, Cross Frames, and Diaphragms.

a.	Repair procedures to tensile components in conformance with ASTM A6/A6M and the following:

1	Repair gouges up to 1/8 inch by grinding flush in the direction of principal stress.

2	Repair gouges deeper than 1/8 inch by first grinding; then, depositing weld metal and grinding flush with the surface of the metal in the direction of principal stress.  Weld using low hydrogen electrodes conforming to current AWS Specifications A5.1 and A5.5.

3	Repair kinks and deformations by flame straightening or a combination of flame straightening and jacking.  Ensure flame straightening is performed by personnel having a minimum of three years of documented experience.  Submit the names of the personnel to the RE for review and approval prior to performing the work.

b.	Repair procedures to compression components for kinks and deformations as outlined in 2.a.3 above.  Where more than 5 percent of the cross-sectional area of the member is damaged, submit a repair procedure to the RE for review and approval.

Clean and paint exposed existing top flanges of beams with prime coat as specified in 554.03.

2**************************************************************************************2

2**************************************************************************************2

INCLUDE THE FOLLOWING IF grounding for ELECTRIFIED RAIL LINES is required to be done by the Railroad

1.	Grounding for Electrified Railroad.  Submit a list of required grounding materials to the RE for approval 21 days before construction operation.  In the list, include the material description, manufacturer, and catalog number.  After obtaining the RE’s approval, submit the list to the railroad for review and approval.  Do not order the materials prior to obtaining the railroad’s approval.  Furnish and deliver the grounding materials to the railroad.  Obtain a receipt for the materials from the railroad and provide a copy to the RE.

3************************************************3

ENTER THE QUANTITIES BELOW AND ANY REVISIONS TO THE MATERIALS LISTED AFTER CONSULTATION WITH THE RAILROAD.

| List of Materials |  |
| --- | --- |
| Description | Quantity Required |
| U-bolt, 7/8 inch diameter by 4 inch, BS fastener | -------- |
| Strap, clevis, 1 1/4 by 2 inches stock, 12 inch connecting length, 1 inch diameter hole, 5/8 inch diameter bolt, ultimate strength 25 psi, Brewer Tilchener Corp.-3074 C | -------- |
| Dead end eye bolt, compression type steel, use DIE 6010SH, compression tool, 60A ALCOA 9190-332 | -------- |
| Jumper cable, compression type aluminum, use DIE 6020AH, compression tool 60A ALCOA 5120-781 | -------- |
| Terminal - Bundy AK2C39B1 to 336400 Cable (1) | -------- |
| Ground terminal - Bundy AK2C39B1 to 336400 Cable (2) | -------- |
| Terminal - solid barrier to 0.17 square inch cable Bundy KC28B1 | -------- |
| Compound, aluminum to copper connection (ALNOX) CANS | -------- |
| Termination, dead end strand clamp, ALCO 336 4 KCM | -------- |
| Clip, bronze, complete type BC, Ohio brass | -------- |
| Thimble-Bronx 336 4 KCM | -------- |
| U-bolt, 1 1/4 inch diameter by 1 1/2 inch loop 336 4 KCM 11, 30/7 STR ACSR, ANACONDA insulated aluminum cable having a diameter of 0.17 square inches, ANACONDA | -------- |

3************************************************3

2**************************************************************************************2

1**************************************************************************************************************************1

201.03.03  Clearing Site, Tank Removal

1**************************************************************************************************************************1

Complete and include following information, when tank(s) removal are required

Remove following:

| Parcel No. | Tank Size | Contents | Tank Registration No. |
| --- | --- | --- | --- |

1**************************************************************************************************************************1

201.03.08  Removal of Asbestos

1**************************************************************************************************************************1

include location and other requirements for the removal of Asbestos

1**************************************************************************************************************************1

201.04  Measurement and Payment

THE FOLLOWING IS ADDED:

1**************************************************************************************************************************1

2**************************************************************************************2

complete and include amount using Engineer’s estimate

The Department will not make payment for the Item CLEARING SITE in excess of $_____________ until Completion.

2**************************************************************************************2

2**************************************************************************************2

INCLUDE PARTIAL PAYMENTS, IF REQUIRED for REMOVAL OF THE BRIDGE OR STRUCTURE SCHEDULED IN STAGES.

THE FOLLOWING EXAMPLE REPRESENTS A BRIDGE SCHEDULED TO BE REMOVED IN 3 STAGES.  THE SEQUENCE OF CONSTRUCTION REQUIRED REMOVAL OF 20 PERCENT OF THE BRIDGE IN STAGE I, 40 PERCENT IN STAGE II, AND 40 PERCENT IN STAGE III.  THE ENGINEER’S ESTIMATE FOR THE ITEM AMOUNT WAS $4,000,000.  THE PAYMENT SCHEDULE FOR “CLEARING SITE, BRIDGE (STRUCTURE NO.)” WILL BE AS FOLLOWS:

- for the item “clearing site bridge (structure no.)”, The department will not make IN EXCESS OF $4,000,000 UNTIL SUBSTANTIAL COMPLETION.

- the department will make payment of 20 PERCENT OF THE PRICE BID (OR $800,000, WHICHEVER IS LESS) UPON ACCEPTANCE OF STAGE I DEMOLITION.

- the department will make payment of 40 PERCENT OF THE PRICE BID (OR $1,600,000, WHICHEVER IS LESS) UPON ACCEPTANCE OF STAGE II DEMOLITION.

- the department will make payment of 40 PERCENT OF THE PRICE BID (OR $1,600,000, WHICHEVER IS LESS) UPON ACCEPTANCE OF STAGE III DEMOLITION.

3************************************************3

complete and include THE VALUE

The Department will not make payment for the Item CLEARING SITE, BRIDGE (___) in excess of $_____________ until Substantial Completion.

The Department will not make payment for the Item CLEARING SITE, STRUCTURE (___) in excess of $_____________ until Substantial Completion.

3************************************************3

2**************************************************************************************2

1**************************************************************************************************************************1

Section 202 – Excavation

202.02 Materials

1**************************************************************************************************************************1

BDC25S-18 DATED FEB 12, 2026

THE FOLLOWING MATERIALS ARE ADDED:

Dense-Graded Aggregate	901.10

HMA	902.02

1**************************************************************************************************************************1

202.03.01 Stripping

1**************************************************************************************************************************1

BDC20S-02 dated MAR 20, 2020

THE SECOND PARAGRAPH is CHANGED TO:

Strip vegetation and underlying soil to a depth of 4 to 6 inches below the existing ground surface.  Confirm the thickness of stripping with the RE based on field conditions.  Temporarily store in stockpiles, as specified in 202.03.03.B, stripped material including excess that is determined suitable for the future use of the Department.  The RE will sample and analyze stripped material in stockpiles to determine suitability for use as topsoil.  Reuse or dispose of unsuitable stripped material as specified in 202.03.03.C.

1**************************************************************************************************************************1

202.03.02 Excavation, Test Pit

1**************************************************************************************************************************1

BDC25S-18 DATED FEB 12, 2026

THE FOLLOWING IS ADDED AFTER THE LAST PARAGRAPH:

For test pits within the existing pavement structure, backfill and restore the pavement structure to match the surrounding pavement.  Provide necessary traffic control during the test pit operation.

1**************************************************************************************************************************1

Division 300 – Subbase and Base Courses

Section 304 – Concrete base Course

304.03.01  Concrete Base Course

1**************************************************************************************************************************1

FOR LOCAL AID PROJECTS, INCLUDE THE FOLLOWING

ADD THE FOLLOWING SENTENCE TO THE END OF THE FIRST PARAGRAPH:

If the concrete thickness lot area is less than 5000 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the test strip requirements.

1**************************************************************************************************************************1

Section 305 – RUBBLIZING CONCRETE PAVEMENT

305.03.01.F  Rubblization

1**************************************************************************************************************************1

FOR LOCAL AID PROJECTS, INCLUDE THE FOLLOWING

ADD THE FOLLOWING THIRD PARAGRAPH TO THE END OF THIS SECTION:

If the rubblized concrete thickness lot area is less than 5000 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the test strip requirements.

1**************************************************************************************************************************1

Division 400 – Pavements

Section 401 – Hot Mix Asphalt (HMA) Courses

401.02.01  Materials

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

THE FIRST ITEM IS CHANGED TO:

Tack Coat 64-22, PG 64S-22	902.01.01

1**************************************************************************************************************************1

401.03.01  Milling

A.	HMA Milling.

1**************************************************************************************************************************1

complete and include TIME INTERVAL FOR RESURFACING THE MILLED PAVEMENT BASED ON THE project STAGING. MAXIMUM TIME INTERVAL MAY VARY BUT SHALL NOT EXCEED 72 HOURS.

| Stage | Max. Time Interval Allowed |
| --- | --- |

1**************************************************************************************************************************1

C.	Micro-Milling and Profile Milling.

1**************************************************************************************************************************1

BDC21S-19 dated Jan 20, 2021

THE following is added at the end:

The Department will provide the time interval for resurfacing the micro-milled areas in the Special Provisions.  Extensions to the time interval may be approved by the RE.  The RE is responsible for performing a daily inspection of the micro-milled surface to ensure continued compliance with ASTM E 965.  The Contractor is responsible for any incidental costs incurred as a result of time interval extension.

2******************************************************************************************************2

complete and include TIME INTERVAL FOR RESURFACING THE MILLED PAVEMENT BASED ON THE project STAGING:

for projects with a posted speed limit of 40 mph or less MAXIMUM TIME INTERVAL MAY VARY BUT SHALL NOT EXCEED 72 HOURS.

For projects with a posted speed limit of 45 miles per hour or greater the time interval is to be 0 HOURS.

3************************************************3

The RE may increase the allowable time interval to a maximum of 72 hours based on field conditions and provided that the condition does not deteriorate the pavement or impact the safety of the traveling public.

| Stage | Max. Time Interval Allowed |
| --- | --- |

3************************************************3

2******************************************************************************************************2

1**************************************************************************************************************************1

401.03.03  HMA Pavement Repair

1**************************************************************************************************************************1

BDC22S-03 dated Jun 22, 2022

The title and entire subsection is changed to:

401.03.03  HMA Repair

A.	HMA Pavement Repair.  Arrange a project site meeting with the RE to establish the limits of HMA pavement repair.  Additional repairs, not delineated on the Plans or by the RE during the project site meeting, may be required if the need is established by the RE.

If potholes are discovered, notify the RE immediately.  The RE may immediately direct repairs of small areas.  The RE may require further evaluation of a large area to determine the need for additional milling and paving.

Perform HMA repairs as a separate operation before milling, paving, and other surface treatments.  The Contractor may request approval of the RE to perform the repair work as one operation with the paving or surface treatment.

HMA repairs may be performed on full depth HMA pavement or on composite pavement (HMA over concrete pavement).  For full depth HMA pavement, sawcut existing HMA pavement to a depth of 8 inches.  For composite pavement, sawcut existing HMA to a depth of 8 inches or up to the top of concrete, whichever is less.  Sawcut lines parallel and perpendicular to the roadway baseline and 3 inches away, at the closest point, from the damaged area to be repaired.

Remove damaged and loose material within the boundary of the sawcuts to form rectangular openings with vertical sides to a depth of 8 inches for HMA pavement, or to the top of concrete for composite pavement.  A milling machine may be used to remove damaged pavement to form the repair areas if approved by the RE.

After the existing damaged HMA and loose material has been removed, the RE will examine underlying material to determine its condition.

If the base of the repair area is unbound material, then shape and compact the unbound material to produce a firm and level base.

If water exists in the area, remove the underlying material to the depth as directed by the RE.  Place geotextile, then place and compact coarse aggregate to required grade to provide for a minimum 8 inch thick HMA pavement repair.  Compact coarse aggregate as specified in 203.03.02.B.3.

If the base of the repair is HMA or concrete pavement, then ensure that the remaining pavement is cleaned and dry prior to applying tack coat.

Apply tack coat at an application rate of 0.15 gallons per square yard to the vertical surfaces and base of the opening.  Spread and grade HMA surface course mix in the opening as specified for the roadway surface or a HMA surface course mix approved by the RE.  Ensure that the temperature of the HMA when placed is at least 250 °F, and compact as specified in 401.03.07.F.  Compact areas not accessible to rollers with a flat face compactor.  Compact until the top of the patch is flush with, or 1/8 inch higher than, the adjacent pavement surface.

Reuse removed material as specified in 202.03.03.C.1.

B.	HMA Longitudinal Joint Repair.  Arrange a project site meeting with the RE to establish the limits of HMA longitudinal repair areas.  Additional repairs, not delineated on the Plans or by the RE during the project site meeting, may be required if the need is established by the RE.

Mill 2 feet wide, unless directed otherwise by the RE, centered over the HMA longitudinal joint, rumble strip, longitudinal distress areas or any combination of the three, as shown on the Plans and as directed by the RE.  Mill to a minimum 2 inches in depth, or as required to remove the damaged pavement.  For distress areas wider than 4 feet, the RE may direct the use of HMA pavement repair as specified in 401.03.03.A.

Clean the milled area as specified in 401.03.01.A.  Obtain RE approval of the repair area before proceeding with the repair.

Apply polymerized joint adhesive to the vertical surfaces of the repair area as specified in 401.03.04.  Apply tack coat as specified in 401.03.05 at an application rate of 0.15 gallons per square yard to the bottom surface of the repair area.  Obtain RE approval of the repair area before proceeding with the repair.  Spread and grade Hot Mix Asphalt 9.5M64 Surface Course in the repair area as specified in 401.03.07.E.  Ensure that the temperature of the HMA when placed and compacted is at least 250 ºF.  Compact as specified in 401.03.07.F, ensuring that the top of the compacted HMA is flush with, or not greater than 1/8 inch higher than, the adjacent pavement surface.

Reuse removed material as specified in 202.03.03.C.1.

401.03.05  Tack Coat

1**************************************************************************************************************************1

BDC25S-06 dated APRIL 15, 2025

THE FOLLOWING IS ADDED AFTER THE FOURTH PARAGRAPH:

The Department will perform verification testing of tack coat application rate according to ASTM D2995, Option A, within the first 300 feet of each paving operation for each lane of paving, except that only one pad assembly will be used for the transverse application rate and the longitudinal application rate will not be verified.  Prior to tack coat application, allow the RE access to set geotextile pads for the initial verification test.  After passing over the pads with the tack distributor, allow the RE time to perform the verification test and calculation.  If the application rate is determined to be outside the rate specified in Table 401.03.05-1, stop the tack coat and paving operations, make the required adjustments to achieve a proper application rate, and allow the RE to perform an additional verification test within the next 300 feet.  Allow the RE to perform additional tests until the correct application rate is achieved.  Retack areas that are insufficiently coated.  The frequency of testing may be increased based on a visual inspection or at the discretion of the RE.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

401.03.07  HMA Courses

A.	Paving Plan.

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

part (4) is changed to:

4.	Lighting plan for night operations as specified in 108.06.

1**************************************************************************************************************************1

C. Test Strip

FOR LOCAL AID PROJECTS INCLUDE THE FOLLOWING:

REPLACE THE FIRST PARAGRAPH OF THIS SECTION WITH THE FOLLOWING:

Test Strip. Construct a test strip for each HMA mix for contracts with more than a total of 5,500 tons of HMA. For HMA HIGH RAP, construct the test strip at least 14 days prior to production. Test strips are not necessary for temporary pavement. Ensure that the tack coat or prime coat has been placed as specified in 401.03.05 and 401.03.06, before placing HMA. Transport and deliver, spread and grade, and compact as specified in 401.03.07.D, 401.03.07.E, and 401.03.07.F, respectively, and according to the approved paving plan. Construct a test strip for the first 700 to 1,200 square yards placed for each job mix formula. If the paving lot area is less than 700 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the coring requirements. While constructing the test strip, record the following information and submit to the RE:

D.	Transportation and Delivery of HMA.

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

THE second SENTENCE OF THE first PARAGRAPH IS CHANGED TO:

Do not allow trucks to leave the plant within 1 hour of sunset unless lighting for night operations is provided as specified in 108.06.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC25S-14 dated Jul 25, 2025

The second paragraph is changed to:

Ensure that weigh tickets are signed and sealed by a certified weighmaster.  Individual weigh tickets do not need to be signed and sealed if using an automated batching plant or automated drum plant.

the fourth paragraph is changed to:

When using an automated batching plant, obtain weigh tickets from the printer used in conjunction with an automated batching and mixing system.  Ensure the printed ticket shows the individual weights of the various components of the HMA in a batch, the total weight of each batch, and the sum of all batch weights in the truckload.  If using an automated batching plant or automated drum plant, at the completion of each day’s work, provide certification from the weighmaster that the total net weight supplied was correct.

1**************************************************************************************************************************1

E.	Spreading and Grading.

1**************************************************************************************************************************1

use 100% of the surface course joint length to estimate the quantity of Polymerized joint adhesive

1**************************************************************************************************************************1

G.	Opening to Traffic.

1**************************************************************************************************************************1

BDC20S-08 dated Jun 12, 2020

THE FOLLOWING IS ADDED after the last paragraph:

Ensure that RPMs are installed and rumble strips are constructed within 14 days of opening each day’s surface paving to traffic.

1**************************************************************************************************************************1

2**************************************************************************************2

FOR local aid projects INclude the following

H.	Air Void Requirements

FOR LOCAL AID PROJECTS, THIS SUBSECTION IS DELETED AND REPLACED BY THE FOLLOWING.

Pavement lots are defined as approximately 15,000 square yards of pavement in Surface area. If pavement lot area is

less than 5000 square yards, the LPA may make a request in writing to the Local Aid District Office to waive the air voids requirements.

The RE will designate an independent testing agency (Laboratory) to perform the quality assurance sampling, testing and analysis. The Laboratory is required to be accredited by the AASHTO Accreditation Program (www.amrl.net). The Laboratory’s accreditation must include AASHTO T 166 and AASHTO T 209.

The Laboratory Technician who performs the quality assurance sampling shall be certified by the Society of Asphalt

Technologists of New Jersey as an Asphalt Plant Technologist, Level 2.

The Laboratory will determine air voids from 5 (Five) 6-inch diameter cores taken from each lot in random locations within the traveled way and at least one core in each travel lane. The HMA Core Sampling Plan form provided on the Local Aid Website must be utilized by the Laboratory to determine the random locations of the cores. The Laboratory may rerun the random location functions on the HMA Core Sampling Plan form to resolve any conflicts generated by the HMA Core Sampling Plan form and physical limitations of the HMA lot, such as utility conflicts, or the specifications defined herein. The coring locations must be designated by a station and offset, and offsets are taken from the left edge of the pavement in the direction of travel within the lane lines. The Laboratory must disclose the contents of the HMA Core Sampling Plan with the Contractor to assist in the schedule of construction.

The Laboratory will determine air voids of cores from the values for the maximum specific gravity of the mix and the bulk specific gravity of the core. The Laboratory will determine the maximum specific gravity of the mix according to NJDOT B-3 and AASHTO T 209, except that minimum sample size may be waived in order to use a 6-inch diameter core sample. The Laboratory will determine the bulk specific gravity of the compacted mixture by testing each core according to AASHTO T 166.

The Laboratory will calculate the percent defective (PD) as the percentage of the lot outside the acceptable range of 2 percent air voids to 8 percent air voids. The acceptable quality limit is 15 percent defective. For lots in which PD > 15, the Department will assess a negative pay adjustment.

The Laboratory will use and submit to the RE the DS8S-PD form provided on the Local Aid Website and verify manually the PD calculation.

The Laboratory will calculate pay adjustments based on the following:

1. 	Sample Mean (X̅) and Standard Deviation (S) of the N Test Results (X1, X2,…, XN).

2. 	Quality Index (Q).

3. 	Percent Defective (PD). Using NJDOT ST for the appropriate sample size, the Laboratory will determine PDL and PDU associated with QL and QU, respectively.  PD = PDL + PDU

4. 	Reduction Per Lot. Calculate the reduction per lot as specified in Table 401.03.07-3:

| Table 401.03.07-3 |  |
| --- | --- |
| Reduction in Payment for Nonconformance to Air Void Requirements |  |
| Percent Defective (PD) Per Lot | Reduction Per Lot (%) |
| 0 < PD ≤ 15 | 0 |
| 15 < PD ≤ 30 | 0.5 |
| 30 < PD ≤ 35 | 2 |
| 35 < PD ≤ 40 | 10 |
| 40 < PD ≤ 45 | 15 |
| 45 < PD ≤ 50 | 20 |
| 50 < PD ≤ 60 | 30 |
| 60 < PD ≤ 75 | 45 |
| PD > 75 | Remove & Replace |

5.    Outlier Detection. If PD < 10, the Laboratory will not screen for outliers. If PD ≥ 10, the Laboratory will screen acceptance cores for outliers using a statistically valid procedure. The following procedure applies only for a sample size of 5 or 10.

  - The Laboratory will arrange the core results in ascending order, in which X1 represents the smallest value and XN represents the largest value.

  - If XN is suspected of being an outlier, the Laboratory will calculate:

  - If X1 is suspected of being an outlier, the Laboratory will calculate:

  - For N = 5 if R > 0.642, the value is judged to be statistically significant and the core is excluded.

For N = 10 if R > 0.412, the value is judged to be statistically significant, and the core is excluded.

If an outlier is detected for N = 5 and no retest is warranted, the Contractor may replace that core by taking an additional core at the same offset and within 5 feet of the original station. If an outlier is detected and a retest is justified, take a replacement core for the outlier at the same time as the 5 additional retest cores are taken. If the outlier replacement core is not taken within 15 days, the Laboratory will use the initial core results to determine reduction per lot.

If an outlier is detected for N = 10, the Contractor may replace that core by taking an additional core at the same offset and within 5 feet of the original station. If the outlier replacement core is not taken within 15 days, the Laboratory will use the initial core results to determine the reduction per lot.

6. 	Retest. If the initial series of 5 cores produces a percent defective value of PD 30 for mainline or ramp lots, or PD 50 for other pavement lots, the Contractor may elect to take an additional set of 5 cores at random locations chosen by the HMA Core Sampling Plan form. Take the additional cores within 15 days of receipt of the initial core results. If the additional cores are not taken within the 15 days, the Laboratory will use the initial core results to determine the PPA. If the additional cores are taken, the Laboratory will recalculate the reduction per lot using the combined results from the 10 cores.

7. 	Removal and Replacement. If the final lot PD ≥ 75 (based on the combined set of 10 cores or 5 cores if the Contractor does not take additional cores), remove and replace the lot and all overlying work. The replacement work is subject to the same requirements as the initial work.

For shoulder lots, the Department will assess the calculated reduction per lot instead of removal and replacement. Fog seal the lot as specified in 422.03.01.

- Thickness Requirements

2**************************************************************************************2

2**************************************************************************************2

FOR LOCAL AID PROJECTS INclude the following

3************************************************3

PERFORM THE FOLLOWING FOR RESURFACING PROJECTS

DELETE THIS SUBSECTION AND REPLACE THIS SUBSECTION’S CONTENTS WITH THE FOLLOWING:

This subsection is deleted. In no instance will a compacted average thickness of less than 1.25 inches be acceptable.

************************************************3

PERFORM THE FOLLOWING FOR NEW CONSTRUCTION, COMPLETE RECONSTRUCTION OR WIDENINGS GREATER THAN EIGHT FEET

DELETE THIS SUBSECTION AND REPLACE THIS SUBSECTION’S CONTENTS WITH THE FOLLOWING:

Thickness requirements will apply when full-depth, uniform-thickness HMA pavement construction is shown.

Pavement lots are defined as approximately 15,000 square yards of pavement area. The RE will not include areas consisting of different HMA mixtures or thicknesses in the same lot. If thickness lot area is less than 5000 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the thickness requirements.

The RE  or LPA will designate an independent testing agency (Laboratory) to perform the quality assurance sampling, testing and analysis. The Laboratory is required to be accredited by the AASHTO Accreditation Program (www.amrl.net). The Laboratory’s accreditation must include AASHTO T 166 and AASHTO T 209.

The Laboratory Technician who performs the quality assurance sampling shall be certified by the Society of Asphalt

Technologists of New Jersey as an Asphalt Plant Technologist, Level 2.

The Laboratory will test for thickness using the full-depth cores taken for surface course air voids, evaluated according to NJDOT B-4. The Laboratory will base acceptance on total thickness and thickness of the surface course.

- Total Thickness. The Laboratory will calculate the percent defective (PD) as the percentage of the lot that is less than the design thickness. The Laboratory will consider 25 percent defective as the acceptable quality limit. For lots where PD < 25, the Department will award a positive pay adjustment. For lots where PD > 25, the Department will assess a negative pay adjustment.

The Department will base total thickness acceptance on the percentage of the lot estimated to fall below the specified thickness as follows:

- Sample Mean (X̅) and Standard Deviation (S) of the N Test Results (X1, X2,..., XN). Calculate as specified in 401.03.07.H.1.

- Quality Index (QI)

Where Tdes = design thickness.

- Percent Defective (PD). Using NJDOT ST for the appropriate sample size, determine the percentage of material (PD) falling below the design thickness associated with QL (lower limit).

- Reduction in Payment. The Department will determine the reduction in payment based on the quantity of the surface course multiplied by the percent reduction in payment from Table 401.03.07-5.

- Retest. If the initial series of 5 cores produces a percent defective value of PD ≥ 30, the Contractor may elect to take an additional set of 5 cores at random locations chosen by the RE. Notify the RE within 15 days of receipt of the initial core results to take the additional cores. If the RE is not notified within the 15 days, the Laboratory will use the initial core results to determine the reduction in payment for nonconformance requirements. If the additional cores are taken, the ME will recalculate the reduction in payment for nonconformance requirements using the combined results from the 10 cores.

- Removal and Replacement. If the lot PD 45, remove and replace, or mill and overlay, the lot. The replacement work is subject to the same requirements as the initial work.

  - Surface Course Thickness. The Laboratory will evaluate the surface course solely to determine whether a remove- and-replace or an overlay condition exists, not for pay adjustment. The Laboratory will calculate the percent defective (PD) as the percentage of the lot that is less than the allowable thickness for the nominal maximum aggregate used in the surface course. The Laboratory will accept pavement lots with PD 30 and will reject pavement lots with PD > 30.

The Laboratory will base surface thickness acceptance on the percentage of the lot estimated to fall below the allowable thickness as follows:

    - Sample Mean ( X ) and Standard Deviation (S) of the N Test Results (X1, X2,..., XN). Calculate using the formula as specified in 401.03.03.I.1.

    - Quality Index (Q).

QL = (X – Tall)/S, where Tall is the minimum allowable thickness from Table 401.03.07-6.

- Percent Defective. Using NJDOT ST - Statistical Tables (NJDOT Standard Specs for Roads and Bridges 2019-NJDOT TEST METHODS) for the appropriate sample size, determine the percentage of material (PD) falling below the allowable thickness associated with QL (lower limit).

- Retest. If the initial series of 5 cores produces a percent defective value of PD > 30, the Contractor may take an additional 5 cores at random locations determined by the Laboratory. Notify the RE within 15 days of receipt of the initial core results to take the additional cores. If the RE is not notified within the 15 days, the Laboratory will use the initial core results to determine the PPA. When the additional cores are taken, the Laboratory will recalculate the reduction in payment for nonconformance requirements using the combined results from the 10 cores to obtain the total PD.

- Removal and Replacement. If the surface course fails to meet the acceptance requirement with a PD ≤ 45, the Department will require removal and replacement of the lot. The replacement work is subject to the same requirements as the initial work.

3************************************************3

INCLUDE THE FOLLOWING FOR LOCAL AID PROJECTS

(FOR ROADS ON THE NHS OR UNDER NJDOT JURISDICATION)

J.	Ride Quality Requirements.

FOR Local Aid projects REPLACE THIS SUBSECTION WITH THE FOLLOWING

The Department will evaluate the ride quality of the final riding surface of all constructed pavement on the project, for routes designated as National Highway System (NHS) and routes under NJDOT jurisdiction, using the International Roughness Index (IRI) according to ASTM E 1926. All NHS roadways are listed on the Department’s website here. The Department may evaluate ride quality of other routes not designated as NHS or under NJDOT jurisdiction. The final riding surface is defined as the last lift of the pavement structure where traffic will be allowed.  The pavement will be evaluated using the current average IRI (C) to select the target IRI (T) from Table 401.03.07-8. The current average IRI (C) is defined as the preconstruction ride quality measured not more than two years from the start of the project pavement construction.

The LPA or RE  will designate an independent testing agency to perform the ride quality testing and analysis. The testing agency is required to comply with testing and certification requirements according to NJDOT R-1. If the current average IRI (C) is not available, then the testing agency will test, analyze and report ride quality before pavement construction to measure current average IRI (C). The testing agency will use and submit to the RE the IRI Testing Summary Report form provided from the  Local Aid District Office and verify manually the pay adjustment calculation.

Current IRI data for paving routes designated NHS or NJDOT jurisdiction can be made available by request by contacting Shahid Haji and Peter Brzostowski at Shahid.Haji@dot.nj.gov and Peter.Brzostowski@dot.nj.gov.

Municipal or county officials may request IRI information through this process. Contractors, consultants, and bidding parties shall not request IRI values through this process for their own benefit.

For projects paving routes designated NHS or NJDOT jurisdiction on mainline travel lanes equal to or greater than 2,500 feet length and any lane within the project of at least 1,000 feet length, the Department will evaluate the ride quality of the final riding surface of the mainline travel lanes using IRI.  The Department will use the measured IRI to calculate the pay adjustment (PA) using pay adjustment equation (PAE) type PA1 as specified in Table 401.03.07-7. PA will be based on lots of 0.01 mile length.  The PA will be zero for acceptable quality and negative for inferior quality work.

For projects paving routes designated NHS or NJDOT jurisdiction on mainline travel lanes of less than 2,500 feet length, the RE will visually inspect the final riding surface. Based on visual inspection, if the RE determines that the work may not conform to the ride quality requirements, then the Department will evaluate the ride quality of the final riding surface using IRI. Visual inspection by the RE is considered sufficient grounds for such evaluation.  The Department will use the measured IRI to calculate the PA using pay equation type PA1 as specified in Table 401.03.07-7.

For paving on ramps and shoulders, the RE will visually inspect the final riding surface. Based on visual inspection, if the RE determines that the work may not conform to the ride quality requirements, then the Department will evaluate the ride quality of the final riding surface using IRI. Visual inspection by the RE is considered sufficient grounds for such evaluation. The Department will use the measured IRI to calculate the pay adjustment using pay equation type PA2 as specified in Table 401.03.07-7.

When paving over bridge structures on NHS or NJDOT jurisdiction roadways, the Department will use the measured IRI to calculate the pay adjustment using pay equation type PA3 as specified in Table 401.03.07-7.

For paving on Local roadways other than NHS and NJDOT jurisdiction on mainline travel lanes equal to or greater than 2,500 feet length and any lane within the project of at least 1,000 feet length, the Department may evaluate the ride quality of the final riding surface of the mainline travel lanes using IRI. Local roadways are defined as municipal and county roads that are not designated as part of the NHS. The Department will use the measured IRI to calculate the pay adjustment (PA) using pay adjustment equation (PAE) type PA4 as specified in Table 401.03.07-7.

- Smoothness Measurement.  The Department will test the longitudinal profile of the final riding surface for ride quality with a Class 1 Inertial Profiling System according to NJDOT R-1.  If project conditions preclude the use of the Class 1 Inertial Profiling System, the Department will use a Class 1 Walking Profiler or lightweight profiler.

- Quality Control Testing.  Perform quality control testing during lift placement to ensure compliance with the ride quality requirements specified in Table 401.03.07-8.

- Preparation for IRI Testing.  Notify the RE when all paving is complete, and the RE or LPA will request IRI testing by independent testing agency. Provide traffic control when the independent testing agency performs IRI testing.  Perform mechanical sweeping of the surface before IRI testing.  To facilitate auto triggering on laser profilers, place a single line of temporary pavement marking tape perpendicular to the roadway baseline at the beginning and end of each lane, shoulder, and ramp to be tested or as per direction of the independent testing agency.  Submit the actual stationing for each temporary pavement marking tape location to the RE.

- Quality Acceptance.  The Department will determine acceptance and provide PA based on the following:

a.	Pay Adjustment.  The acceptable IRI for the roadway pavement will be the target IRI (T) from Table 401.03.07-8 rounded to the nearest whole number for which full payment will be made and will be determined using the latest available current average IRI (C) data.  The number of lots for final pay adjustment will be reduced by the number of lots excluded for each segment shown in Table 401.03.07-7.   Lots excluded from final PA will be those with the highest recorded IRI numbers for respective roadway and bridge deck segments.  A single average IRI value and the corresponding PA for each 0.01 mile lot will be reported.  IRI units are in inches per mile.

| Table 401.03.07-7 Pay Adjustment Equations (PAE) for Ride Quality |  |  |  |
| --- | --- | --- | --- |
| Pay Equation Type | Exclusions | Pay Equations |  |
| PA1 | As shown in the Special Provisions Table 401.03.07-7A | IRI<T | PA1=PAE (see note 2) |
|  |  | T≤IRI≤170 | PA1=PAE |
|  |  | IRI>170 | PA1= -A or Corrective action |
| PA2 | Will include, if tested | IRI ≤ 120 | PA2 =0 |
|  |  | 120 < IRI ≤ 170 | PA2 = (IRI − 120) x (−$5.00) |
|  |  | IRI>170 | Maximum Negative Pay or Corrective action |
| PA3 | Will include, if tested | IRI≤120 | PA3=0 |
|  |  | 120<IRI≤170 | PA3=PAE |
|  |  | IRI>170 | PA3= -A or Corrective action |
| PA4 | Will include, if tested | IRI ≤ T | PA4=0 |
|  |  | T < IRI ≤ T+80 or 170 whichever is higher | PA4 = (IRI − T) x (−$1.25) |
|  |  | IRI>T+80 or 170 whichever is higher | Maximum Negative Pay or Corrective action |
| P = Bid price of last lift of the pavement structure to be evaluated or price listed in table 401.03.07-7B, whichever is higher, per Ton D1 = Design thickness of last lift to be evaluated, Inch M = Bid price of Milling, per Square Yard T = Target IRI |  |  |  |
| - For various design thicknesses of last lift to be evaluated within a segment, calculate the thickness using the following equation: Design thickness of last lift to be evaluated (D)=D1N1+D2N2+…DNNNN1+N2+N3+…NN Where: DN = Design thickness of the last lift to be evaluated of N sections having same mix, Inch NN = Number of lots of N section with design thickness DN of last lift to be evaluated - Positive pay adjustment will be used to offset negative pay adjustment. Total pay adjustment for each lane will not be greater than zero, but may result in a negative pay adjustment. |  |  |  |

3************************************************3

ADD THE FOLLOWING FOR NEW CONSTRUCTION, COMPLETE RECONSTRUCTION OR COMPLETE AND INCLUDE TABLE 401.03.07-7A FOR ANY ROADWAYS THAT ARE TO BE EXCLUDED FROM THE RIDE QUALITY REQUIREMENTS OF SUBSECTION 401.03.07.J:

1**************************************************************************************************************************1

send email to sme to request the exclusions in table 401.03.07-7A for roadways within the project and include the following, INCLUDE THE LOCAL PROJECT MANAGER ON ALL TRANSMITTALS

sme CONTACT –Pavement & drainage Management & Technology unit

THE FOLLOWING IS ADDED:

| Table 401.03.07-7A Exclusions for Resurfacing or Reconstruction |  |  |
| --- | --- | --- |
| Roadway | Lane Number | Exclusions |

Lane designation is by increasing numbers from left to right in the direction of traffic with left lane being Lane 1.

3************************************************3

| Table 401.03.07-7B Minimum Value of P |  |
| --- | --- |
| Surface Course Mix | P |
| Hot Mix Asphalt (Dense Graded) with PG 64-22 binder | $60.00 |
| Hot Mix Asphalt (Dense Graded) with PG 64E-22 binder | $70.00 |
| Stone Matrix Asphalt, High Performance Thin Overlay, Ultra-Thin Friction Course, Open Graded or Gap Graded Mixes not specified in this table | $80.00 |
| Bridge Deck Waterproof Surface Course | $250.00 |

| Table 401.03.07-8 Target IRI for Resurfacing or Reconstruction (T)3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Roadway Type |  | Current average IRI (C) |  | New Construction or Reconstruction |  | Number of Operation for other than New Construction or Reconstruction5 |  |  |  |  |  |  |  |
|  |  |  |  |  |  | One4 |  | Two4 |  |  | Three4 |  | Four or More4 |
|  |  |  |  | Target IRI (T) |  |  |  |  |  |  |  |  |  |
| NHS & NJDOT Freeways or Limited Access Highways |  | ≤ 60 |  | 50 |  |  | 50 |  | 50 |  | 50 |  | 50 |
|  |  | 61 to ≤95 |  |  |  |  | 53 |  | 50 |  | 50 |  | 50 |
|  |  | 96 to ≤170 |  |  |  |  | 55 |  | 53 |  | 50 |  | 50 |
|  |  | 171 to≤200 |  |  |  |  | 0.64C7 |  | 55 |  | 53 |  | 50 |
|  |  | 201 to ≤285 |  |  |  |  |  |  | 58 |  | 55 |  | 50 |
|  |  | >2868 |  |  |  |  |  |  | 60 |  | 58 |  | 53 |
| NHS & NJDOT Roadways other than Freeways or Limited Access Highways with speed limit > 35 MPH |  | ≤ 60 |  | 60 |  |  | 60 |  | 60 |  | 60 |  | 60 |
|  |  | 61 to ≤95 |  |  |  |  | 63 |  | 60 |  | 60 |  | 60 |
|  |  | 96 to ≤170 |  |  |  |  | 66 |  | 63 |  | 60 |  | 60 |
|  |  | 171 to≤200 |  |  |  |  | 0.64C7 |  | 66 |  | 63 |  | 60 |
|  |  | 201 to ≤285 |  |  |  |  |  |  | 69 |  | 66 |  | 60 |
|  |  | >2868 |  |  |  |  |  |  | 72 |  | 69 |  | 63 |
| NHS & NJDOT Roadways other than Freeways or Limited Access Highways with speed limit ≤ 35 MPH |  | ≤ 60 |  | 70 |  |  | 70 |  | 70 |  | 70 |  | 70 |
|  |  | 61 to ≤95 |  |  |  |  | 74 |  | 70 |  | 70 |  | 70 |
|  |  | 96 to ≤170 |  |  |  |  | 77 |  | 74 |  | 70 |  | 70 |
|  |  | 171 to≤200 |  |  |  |  | 0.64C7 |  | 77 |  | 74 |  | 70 |
|  |  | 201 to ≤285 |  |  |  |  |  |  | 81 |  | 77 |  | 70 |
|  |  | >2868 |  |  |  |  |  |  | 84 |  | 81 |  | 74 |
| Local Roadway with Posted Speed ≥45 MPH |  | C |  | 80 |  |  | 0.7C or 80 whichever is higher |  | 0.49C or 80 whichever is higher |  | 0.34C or 80 whichever is higher |  | 0.24C or 80 whichever is higher |
| Local Roadway with Posted Speed <45 MPH |  | C |  | 100 |  |  | 0.84C or 100 whichever is higher |  | 0.59C or 100 whichever is higher |  | 0.41C or 100 whichever is higher |  | 0.29C or 100 whichever is higher |
| 1. The Department will determine target IRI (T) of roadways containing multiple speed limits of greater than 35 MPH and less than or equal to 35 MPH based on the following equation: Where TN is the Target IRI of N section and LN is the length of N section in miles to the nearest 0.01 mile 2. Current average IRI (C) is the average of the latest available preconstruction IRI data. 3. The target IRI (T) is selected or calculated from the table and rounded to the nearest whole number. 4. Multiply T with 1.05 for HMA over Concrete, if total HMA after proposed treatment is less than 8 inch thick. 5. Milling is one operation. Paving each layer of asphalt mix is an individual operation unless plans specify paving a mix in two lifts. In such case, each lift is considered as an operation. 6. Construction or reconstruction of full pavement box on subgrade is new construction or reconstruction. 7. Use Pay Equation as below: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IRI≤T |  | PA=PAE* |  |  |  |  |  |  |  |  |  |  |
|  | IRI>T |  | PA=PAE |  |  |  |  |  |  |  |  |  |  |
| *Positive Pay adjustment will be used to offset negative pay adjustment. Total pay adjustment for each lane will not be greater than zero, but may result in a negative pay adjustment. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8. For paving over rubblized concrete, use C >286 to determine target IRI, then multiply T with 1.05 if total HMA after proposed treatment is less than 8-inch thick. 9 Paving in one lift with no corrective work such as milling, grinding or pre-levelling of at least 25 percent of surface area of existing pavement is one operation. |  |  |  |  |  |  |  |  |  |  |  |  |  |

b.	Corrective Action.  The Department may require corrective action or assess the maximum negative pay adjustment as computed in Table 401.03.07-7, if the average IRI after testing is performed of NHS or NJDOT jurisdiction roadway is greater than 170 inches per mile, or average IRI local roadway is greater than T+80 or 170 whichever is higher. If the Department requires corrective action submit a plan for corrective action.  If the plan for corrective action is approved and the lot is corrected, the Department will retest and evaluate the corrected area as a new lot that must meet the same requirements as the initial work.  If the plan for corrective action is not approved, the Department may require removal and replacement.  The replacement work is subject to the same requirements as the initial work.

for local aid projects add the following

401.03.08 Core Samples

This subsection is deleted and replaced with the following:

The LPA will designate an independent testing agency (Laboratory) to perform the quality assurance sampling, testing and analysis. The Laboratory is required to be accredited by the AASHTO Accreditation Program (www.amrl.net). The Laboratory’s accreditation must include AASHTO T 166 and AASHTO T 209. The Laboratory Technician who performs the quality assurance sampling shall be certified by the Society of Asphalt Technologists of New Jersey as an Asphalt Plant Technologist, Level 2.

Upon completion of an HMA lot, the Laboratory shall drill cores at random locations at least 12 hours after paving. Take cores in the presence of the RE.  The Laboratory will determine air voids from 5 (Five) 6 inch diameter cores taken from each lot in random locations within the traveled way and at least one core in each travel lane. The HMA Coring Layout Sheet provided on the Local Aid Website must be utilized by the Laboratory to determine the random locations of the cores. The Laboratory may rerun the random location functions on the HMA Coring Layout Sheet to resolve any conflicts generated by the HMA Core Sampling Plan form and physical limitations of the HMA lot, such as utility conflicts, or the specifications defined herein. The Laboratory must disclose the contents of the HMA Core Sampling Plan with the Contractor to assist in the schedule of construction.

The Laboratory shall use drilling equipment with a water-cooled, diamond-tipped masonry drill bit that produces 6 inch nominal diameter cores for the full depth of the pavement. The Laboratory shall remove the core from the pavement without damaging it. After the Laboratory removes the core, the Laboratory shall remove all water from the hole. The Laboratory shall apply an even coating of tack coat to sides of the hole. The Laboratory shall place cold patching material or HMA in maximum lifts of 4 inches in the hole and compact each lift. If cold patching material is utilized to fill the coring hole, then it is not necessary to apply tack coat to the sides of the hole. The Laboratory shall ensure that the final surface is 1/4 inch above the surrounding pavement surface.

HMA cores are to be taken from the HMA lot for quality assurance sampling, testing and analysis within seven (7) days of completing the HMA lot. For test strip lots and the first traveled way lot, the Laboratory shall deliver cores from the field to the testing Laboratory within 48 hours of completing the lot. The Laboratory shall deliver all other acceptance cores within 7 days of completing the lot.

After each air void lot is placed, the Laboratory shall drill cores so that the full depth of the course is recovered for air void acceptance testing. If thickness acceptance testing is required as specified in 401.03.07.I, the Laboratory shall drill the surface course air void cores for the full depth of pavement.

The Laboratory shall utilize a tamper proof core sample box for core storage and transportation. The Laboratory shall ensure that the core sample box can be locked and sealed and is tamper proof in such a manner that it cannot be opened without removing the seals. The Laboratory shall ensure that the core sample box provides protection for the cores from being disturbed or damaged during transit. The Laboratory shall mark the assigned core number on the side of the sample. The Laboratory shall place core samples in the core sample box. The Laboratory shall transport the sealed core sample boxes to the testing Laboratory.

The Laboratory will not accept damaged core samples for testing. If the core sample box exhibits indications of tampering, the core samples will be rejected. If any core samples are rejected, drill a replacement core at the same offset and within 5 feet of the original station and deliver to the Laboratory as specified above within 48 hours.

If the project is utilizing quality control cores, the Laboratory shall provide the results of the quality control core testing to the Contractor in a timely manner which will not unnecessarily impede construction.

1**************************************************************************************************************************1

401.04  Measurement and Payment

FOR LOCAL AID PROJECTS THIS SECTION IS CHANGED TO THE fOLLOWING:

401.04  Measurement and Payment

REPLACE THIS SUBSECTION WITH THE FOLLOWING:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

HMA MILLING, 3" OR LESS	SQUARE YARD

HMA MILLING, MORE THAN 3" TO 6"	SQUARE YARD

CONCRETE MILLING	SQUARE YARD

MICRO-MILLING	SQUARE YARD

HMA PROFILE MILLING	SQUARE YARD

HOT MIX ASPHALT PAVEMENT REPAIR	SQUARE YARD

HMA LONGITUDINAL JOINT REPAIR	Square yard

SEALING OF CRACKS IN HOT MIX ASPHALT SURFACE COURSE	LINEAR FOOT

POLYMERIZED JOINT ADHESIVE	LINEAR FOOT

TACK COAT	GALLON

TACK COAT 64-22	GALLON

POLYMER MODIFIED TACK COAT	GALLON

PRIME COAT	GALLON

HOT MIX ASPHALT			SURFACE COURSE	TON

HOT MIX ASPHALT			SURFACE COURSE HIGH RAP	TON

HOT MIX ASPHALT			INTERMEDIATE COURSE	TON

HOT MIX ASPHALT			INTERMEDIATE COURSE HIGH RAP	TON

HOT MIX ASPHALT			BASE COURSE	TON

HOT MIX ASPHALT			BASE COURSE HIGH RAP	TON

The specified depth of the milling is measured from the original surface to the top of the high spots of the textured surface.

The Department will measure HMA LONGITUDINAL JOINT REPAIR before overlay by the square yard of the area.

The RE will measure HOT MIX ASPHALT PAVEMENT REPAIR before overlay by the square yard of area bounded by the sawcuts.

The RE will measure TACK COAT, TACK COAT 64-22, PRIME COAT, and POLYMER MODIFIED TACK COAT by the volume delivered, converted to the number of gallons at 60 °F as calculated by the temperature-volume correction factors specified in 902.01.

The RE will measure HOT MIX ASPHALT ___ ___ ___  SURFACE COURSE, HOT MIX ASPHALT ___ ___

INTERMEDIATE COURSE, and HOT MIX ASPHALT ___ ___ ___ BASE COURSE by the ton as indicated on the certified weigh tickets, excluding unused material. When nominal maximum aggregate size 3/8 inch HMA surface course is directed for use in transition (run out) areas, the Department will include this weight with the weight for HOT MIX ASPHALT ___ ___ ___ SURFACE COURSE.

The Department will not include payment for polymerized joint adhesive in the various paving Items. The Department will make payment for polymerized joint adhesive under POLYMERIZED JOINT ADHESIVE.

The Department will make a payment adjustment for HMA air void quality per lot by the following formula:

| Pay Adjustment Per HMA Lot = - Q x BP x Reduction Per Lot (%) |  |  |
| --- | --- | --- |
| Where: |  |  |
| BP = |  | Bid Price of HMA |
| Q = |  | Quantity of HMA in lot receiving payment adjustment |
| Reduction Per Lot (%) = |  | Air void Reduction (%) per lot as specified in 401.03.07.H. |

The Department will make a payment adjustment for HMA thickness quality per lot by the following formula:

| Pay Adjustment Per HMA Lot = - Q x BP x Percent Reduction (%) |  |  |
| --- | --- | --- |
| Where: |  |  |
| BP = |  | Bid Price of HMA |
| Q = |  | Quantity of HMA in lot receiving payment adjustment |
| Percent Reduction (%) = |  | Thickness Percent Reduction (%) per lot as specified in 401.03.07.I. |

The Department will make a payment adjustment for HMA ride quality, as specified in 401.03.07.J.

1

Section 403 – ULTRA-THIN FRICTION COURSE

403.03.01  Ultra-Thin Friction Course

1**************************************************************************************************************************1

INCLUDE THE FOLLOWING FOR local aid PROJECTS:

REPLACE THE FIRST PARAGRAPH OF SECTION 401.03.01.F WITH THE FOLLOWING:

Test Strip. Construct a test strip for the first 700 to 1,200 square yards placed of ultra-thin friction course. If the ultra-thin friction course paving lot area is less than 700 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the coring requirement. Operate spray paver without mix to determine tack coat application rate for the project. Ensure that the polymer modified tack coat has been placed as specified in 401.03.05. Transport and deliver, spread and grade, and compact as specified in 403.03.01.D, 403.03.01.E, and 403.03.01.F, respectively, and according to the approved paving plan. While constructing the test strip, record the following information and submit to the RE:

1**************************************************************************************************************************

Section 404 – Stone Matrix Asphalt (SMA)

404.03  Construction

1**************************************************************************************************************************1

BDC24S-06 dated May 28, 2024

404.03.01  Stone Matrix Asphalt Surface Course

Part C is changed to:

C.	Test Strip.  Construct a test strip as specified in 401.03.07.C.  If using Fiberless SMA, construct a test strip prior to paving as specified in 401.03.07.C, except for the allowance to continue paving.  Ensure that the test strip is at least 100 tons.  Submit test strip results to the RE.  The RE will analyze the test strip results in conjunction with the ME’s results from the HMA plant to approve the test strip.  Do not proceed with production paving until receiving written permission from the RE.  The Contractor may need to construct multiple test strips in order to produce material that meets both the plant production requirements and the field density requirements as directed by the RE.

1**************************************************************************************************************************1

Section 405 – Concrete Surface Course

405.03.02  Concrete Surface Course

A.	Concreting Plan.

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

part (4) is changed to:

4.	Lighting plan for night operations as specified in 108.06.

1**************************************************************************************************************************1

I. Thickness Requirements.

INCLUDE THE FOLLOWING FOR local aid PROJECTS:

ADD THE FOLLOWING AS THE LAST SENTENCE OF THE FIRST PARAGRAPH OF SECTION 405.03.02.I:

If the total thickness course paving lot area is less than 5000 square yards, the LPA may make a request in writing to the District Local Aid Office to waive the coring requirement.

1**************************************************************************************************************************

J.	Ride Quality Requirements. (For NHS and roads under NJDOT Jurisdiction)

4.	Quality Acceptance.

a.	Pay Adjustment.

1**************************************************************************************************************************1

send email to sme to request the exclusions in table 405.03.02-1A for roadways within the project and include the following, INCLUDE THE LOCAL AID PROJECT MANAGER ON ALL TRANSMITTALS

sme CONTACT – Pavement & drainage Management & Technology unit

THE FOLLOWING IS ADDED:

| Table 405.03.02-1A Exclusions for Concrete Surface Course |  |  |
| --- | --- | --- |
| Roadway | Lane Number | Exclusions |

Lane designation is by increasing numbers from left to right in the direction of traffic with left lane being Lane 1.

1**************************************************************************************************************************1

Section 406 – High Performance Thin Overlay

1**************************************************************************************************************************1

BDC25S-16 dated sep 30, 2025

the section is renamed to:

Section 406 – High Performance Thin Overlay and Ultra-high performance THIN overlay

1**************************************************************************************************************************1

406.01  Description

1**************************************************************************************************************************1

BDC25S-16 dated sep 30, 2025

the first paragraph is changed to:

This Section describes the requirements for constructing high performance thin overlay (HPTO) and Ultra-high performance thin overlay (Ultra-HPTO).

1**************************************************************************************************************************1

406.02  Materials

406.02.01  Materials

1**************************************************************************************************************************1

BDC25S-16 dated sep 30, 2025

The following material is added:

Ultra-HPTO	902.16

1**************************************************************************************************************************1

406.03  Construction

406.03.01  High Performance Thin Overlay

1**************************************************************************************************************************1

BDC25S-02 dated mar 27, 2025

H.	Air Void Requirements on Roadway.

The Fourth paragraph of Part H is changed to:

The ME will calculate the percent defective (PD) as the percentage of the lot outside the acceptable range of 2 percent air voids to 7 percent air voids.  The acceptable quality limit is 10 percent defective.  For lots in which PD < 10, the Department will award a positive pay adjustment.  For lots in which PD > 10, the Department will assess a negative pay adjustment.

Part 2 is changed to:

2.	Quality Index (Q).

QL=(X-2.0)S

QU=(7.0-X)S

1**************************************************************************************************************************

1**************************************************************************************************************************1

BDC25S-16 dated sep 30, 2025

the following subpart is added:

406.03.02  Ultra-High Performance Thin Overlay

A.	Paving Plan.  At least 20 days before the start of placing the Ultra-HPTO, submit a detailed plan of operation to the RE for approval as specified in 401.03.07.A.

B.	Weather Limitations.  If within the 3 hours before paving the National Weather Service locally forecasts a 50 percent chance or greater of precipitation during the scheduled placement, postpone the placement of Ultra-HPTO.  Do not place Ultra-HPTO if it is precipitating and do not allow trucks to leave the plant when precipitation is imminent.  The Contractor may resume paving operations when the chance of precipitation is less than 50 percent and the surface is dry.

Do not pave if the surface temperature of the underlying pavement is below 50 °F.

C.	Test Strip.  At least 14 days prior to production of the Ultra-HPTO, construct a test strip as specified in 401.03.07.C except for the allowance to continue paving.  Submit test strip results to the RE.  The RE will analyze the test strip results in conjunction with the ME’s results from the HMA plant to approve the test strip.  Do not proceed with production paving until receiving written permission from the RE.

D.	Transportation and Delivery of HMA.  Transport and deliver HMA as specified in 401.03.07.D.

E.	Spreading and Grading.  Use of a MTV is required for the construction of Ultra-HPTO.  If Ultra-HPTO is only for bridge deck paving, the use of a MTV is optional.  Ensure that the surface where the Ultra-HPTO is placed is clean of foreign and loose material.  Clean the surface of existing pavement using a self-propelled power broom equipped with a vacuum collection system before placement.  Ensure that the surface is dry before paving begins.  Do not start paving of the Ultra-HPTO until the RE has approved the underlying surface.  In areas where the existing pavement is not being milled, remove traffic stripes and traffic markings as specified in 610.03.08.  Apply tack coat as specified in 401.03.05.  Place Ultra-HPTO at the laydown temperature recommended by the supplier of the asphalt binder or the supplier of the asphalt modifier without exceeding 330 ºF maximum discharge temperature.  Spread and grade Ultra-HPTO as specified in 401.03.07.E.

F.	Compacting.  Compact as specified in 401.03.07.F.  If vibratory compaction causes aggregate breakdown, forces liquid asphalt to the surface or creates a surface with undesirable ride quality, then operate rollers in static mode only. If compacting Ultra-HPTO on a bridge deck, then operate rollers in static mode only.

G.	Opening to Traffic.  Remove loose material from the traveled way before opening to traffic.  Do not allow construction equipment or traffic on the Ultra-HPTO until the mat cools to a temperature of less than 140 °F.

H.	Air Void Requirements on Roadway.  Drill cores as specified in 401.03.08. Mainline lots are defined as the area covered by a day’s paving production of the same job mix formula for the traveled way and auxiliary lanes.  The RE may combine daily production areas less than 500 tons with previous or subsequent production areas.  If a day’s production is greater than 2000 tons, the RE may divide the area of HMA placed into 2 lots with approximately equal areas.

Ramp pavement lots are defined as approximately 10,000 square yards of pavement in ramps.  The RE may combine ramps with less than the minimum area into a single lot.  If 2 or more ramps are included in a single lot, the RE will require additional cores to ensure that at least 1 core is taken from each ramp.

Other pavement lots are defined as approximately 10,000 square yards of pavement in shoulders and other undefined areas.

The ME will calculate the percent defective (PD) as the percentage of the lot outside the acceptable range of 2 percent air voids to 7 percent air voids.  The acceptable quality limit is 10 percent defective.  For lots in which PD < 10, the Department will award a positive pay adjustment.  For lots in which PD > 10, the Department will assess a negative pay adjustment.

The ME will determine air voids from 5 cores taken from each lot in random locations.  The ME will determine air voids of cores from the values for the maximum specific gravity of the mix and the bulk specific gravity of the core.  The ME will determine the maximum specific gravity of the mix according to NJDOT B-3 and AASHTO T 209, except that minimum sample size may be waived in order to use a 6-inch diameter core sample.  The ME will determine the bulk specific gravity of the compacted mixture by testing each core according to AASHTO T 166.

The ME will calculate pay adjustments based on the following:

1.	Sample Mean (X̅) and Standard Deviation (S) of the N Test Results (X1, X2,..., XN).  Calculate using the formula specified in 401.03.07.H.1.

2.	Quality Index (Q).

QL=(X-2.0)S

QU=(7.0-X)S

3.	Percent Defective (PD).  Using NJDOT ST for the appropriate sample size, the Department will determine PDL and PDU associated with QL and QU, respectively.  PD = PDL + PDU

4.	Percent Pay Adjustment (PPA).  Calculate the PPA for traveled way and ramp lots as specified in Table 401.03.07-3.

Calculate the PPA for other pavement lots as specified in Table 401.03.07-4.

5.	Outlier Detection.  If PD < 10, the ME will not screen for outliers. If PD ≥ 10, the ME will screen all acceptance cores for outliers using a statistically valid procedure. The following procedure applies only for a sample size of 5 or 10.

1.	The ME will arrange the core results in ascending order, in which X1 represents the smallest value and XN represents the largest value.

2.	If XN is suspected of being an outlier, the ME will calculate:

R= XN-X(N-1)XN-X1

3.	If X1 is suspected of being an outlier, the ME will calculate:

R= X2-X1XN-X1

4.	For N = 5 if R > 0.642, the value is judged to be statistically significant and the core is excluded. For N = 10 if R > 0.412, the value is judged to be statistically significant and the core is excluded.

If an outlier is detected for N = 5 and no retest is warranted, the Contractor may replace that core by taking an additional core at the same offset and within 5 feet of the original station. If an outlier is detected and a retest is justified, take a replacement core for the outlier at the same time as the 5 additional retest cores are taken. If the outlier replacement core is not taken within 15 days, the ME will use the initial core results to determine PPA.

If an outlier is detected for N = 10, the Contractor may replace that core by taking an additional core at the same offset and within 5 feet of the original station. If the outlier replacement core is not taken within 15 days, the ME will use the initial core results to determine PPA.

6.	Retest.  If the initial series of 5 cores produces a percent defective value of PD  30 for mainline or ramp lots, or PD  50 for other pavement lots, the Contractor may elect to take an additional set of 5 cores at random locations chosen by the ME.  Notify the RE within 15 days of receipt of the initial core results to take the additional cores.  If the RE is not notified with the 15 days, the ME will use the initial core results to determine the PPA.  If the additional cores are taken, the ME will recalculate the PPA using the combined results from the 10 cores.

7.	Removal and Replacement.  If the final lot PD  75 (based on the combined set of 10 cores or 5 cores if the Contractor does not take additional cores), remove and replace the lot and all overlying work.  The replacement work is subject to the same requirements as the initial work.

Do not remove and replace shoulder lots.  Perform fog seal as specified in 422.03.01 and the Department will assess the calculated PPA.

I.	Air Void Requirements on Bridge Deck.  The RE may waive the coring of Ultra-HPTO constructed on a bridge deck or may require that the Contractor to test bridge decks with the thin lift nuclear density gauge.  If required by RE, perform nuclear density gauge testing according to ASTM D 2950 at 5 random locations per bridge deck.  Use the maximum specific gravity determined at the HMA plant according to AASHTO T 209 to determine percent air voids.  If the average air voids for the bridge deck are 8 percent or greater, the RE will require a revised paving plan for any subsequent bridge deck placement of Ultra-HPTO and may require the Ultra-HPTO to be removed and replaced.

J.	Ride Quality Requirements.  The Department will evaluate the ride quality of the final riding surface of all constructed pavement on the project as specified in 401.03.07.J.

1**************************************************************************************************************************

406.04  Measurement and Payment

1**************************************************************************************************************************1

BDC25S-16 dated sep 30, 2025

the subpart is changed to:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

high performance thin overlay	TON

ultra-high performance thin overlay	ton

The Department will measure high performance thin overlay and ULTRA-HIGH PERFORMANCE THIN OVERLAY by the ton as indicated on the certified weigh tickets, excluding unused material.

The Department will not include payment for tack coat, polymer modified tack coat, and core samples in HIGH PERFORMANCE THIN OVERLAY and ULTRA-HIGH PERFORMANCE THIN OVERLAY.  The Department will pay for tack coat, polymer modified tack coat, and core samples under Tack Coat, POLYMER MODIFIED TACK COAT, and Core Samples, Hot Mix Asphalt as specified in 401.04, respectively.

1**************************************************************************************************************************

Section 407 - Binder Rich Intermediate Course

407.03  Construction

1**************************************************************************************************************************1

BDC23S-03 dated Jul 20, 2023

407.03.01  BRIC

C.	Test Strip.

Part C is changed to:

Construct a test strip as specified in 401.03.07.C except for the allowance to continue paving, at least 14 days prior to production of BRIC.  Ensure that the test strip is at least 100 tons.  The RE will reject the test strip if compaction causes asphalt bleeding in the surface.  Submit test strip results to the RE.  The RE will analyze the test strip results in conjunction with the ME’s results from the HMA plant to approve the test strip.  Do not proceed with production paving until receiving written permission from the RE.  The Contractor may need to construct multiple test strips in order to produce material that meets both the plant production requirements and the field density requirements as directed by the RE.

E.	Spreading and Grading.

Part E is changed to:

Do not start paving of the BRIC until the RE has approved the underlying surface.  Apply tack coat as specified in 401.03.05.  Place BRIC at the laydown temperature recommended by the supplier of the asphalt binder or the supplier of the asphalt modifier without exceeding 330 ºF maximum discharge temperature at the HMA plant.  Spread and grade BRIC as specified in 401.03.07.E, and according to the approved paving plan and test strip.

F.	Compacting.

Part F is changed to:

Compact as specified in 401.03.07.F, and according to the approved paving plan and test strip; do not over compact.  Operate rollers in static mode if vibratory compaction causes aggregate breakdown, forces liquid asphalt to the surface or creates a surface with undesirable ride quality.  Apply fine aggregate as specified in 901.07.02 using a Mechanical Fine Aggregate Spreader as specified in 1012.02 at a rate of 0.5 to 1.0 pounds per square yard uniformly over the entire affected surface before opening to traffic if excessive asphalt bleeding occurs in the surface.

G.	Opening to Traffic.

Part G is changed to:

Remove loose material from the traveled way, shoulder, and auxiliary lanes before opening to traffic.  Do not allow traffic or construction equipment on the BRIC until the surface temperature is less than 120 °F.  The RE may reject areas where fine aggregate has been applied that is not sufficiently covered or has excess fine aggregate material and rendered unsatisfactory.  Visual inspection by the RE is considered sufficient grounds for such rejection.  Do not open to traffic unless approved by the RE.  The RE may request a speed limit reduction prior to opening to traffic.  Ensure that traffic is not allowed on the BRIC for more than 3 days.

1**************************************************************************************************************************1

Division 420 – Pavement Preservation Treatments

Section 421 – Micro Surfacing and Slurry Seal

421.03.03  Micro Surfacing Aggregate and Micro Surfacing Emulsion

A.	Micro Surfacing Plan.

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

part (4) is changed to:

4.	Lighting plan for night operations as specified in 108.06 for milling and paving.

1**************************************************************************************************************************1

- Ride Quality Requirements. (For NHS and roads under NJDOT Jurisdiction)

1**************************************************************************************************************************1

BDC26S-07 dated June 10, 2026

part j is changed to:

J.	Ride Quality Requirements.  The Department will evaluate the ride quality of the final riding surface of constructed pavement on the project using the International Roughness Index (IRI) according to ASTM E 1926.  The final riding surface is defined as the last lift of the pavement structure where traffic will be allowed.  The roadway sections listed in Special Provisions Table 421.03.03-2A will be evaluated using the target IRI (T) determined from
 Table 421.03.03-4.

In addition to the sections listed in Special Provisions Table 421.03.03-2A, for projects where the total paving on mainline travel lanes is equal to or greater than 2,500 feet in length, the Department will evaluate the ride quality of the final riding surface of the mainline travel lanes within the project limits using IRI.  The Department will use the measured IRI to calculate the pay adjustment (PA) using pay adjustment equation (PAE) type PA1 as specified in Table 421.03.03-2.  PA will be based on lots of 0.01 mile length.  The PA will be positive for superior quality work or negative for inferior quality work.

In addition to the sections listed in Special Provision Table 421.03.03-2A, for projects where the total paving on mainline travel lanes is less than 2,500 feet in length, the RE will visually inspect the final riding surface of all paved sections not listed in Special Provision Table 421.03.03-2A.  Based on visual inspection, if the RE determines that the work conforms to the ride quality requirements, then those sections will be deemed acceptable.  If the RE determines that the work may not conform to the ride quality requirements, then the Department will evaluate the ride quality of the final riding surface using IRI.  Visual inspection by the RE is considered sufficient grounds for such evaluation.  The Department will use the measured IRI to calculate the PA using pay adjustment equation (PAE) type PA1 as specified Table 421.03.03-2.

For paving on ramps and shoulders, the RE will visually inspect the final riding surface of all paved ramps and shoulders not listed in Special Provisions Table 421.03.03-2A.  Based on visual inspection, if the RE determines that the work conforms to the ride quality requirements, then those sections will be deemed acceptable.  If the RE determines that the work may not conform to the ride quality requirements, then the Department will evaluate the ride quality of the final riding surface using IRI.  Visual inspection by the RE is considered sufficient grounds for such evaluation.  The Department will use the measured IRI to calculate the pay adjustment using pay equation type PA2 as specified in Table 421.03.03-2.

1.	Smoothness Measurement.  The Department will test the longitudinal profile of the final riding surface for ride quality with a Class 1 Inertial Profiling System according to NJDOT R-1.  If project conditions preclude the use of the Class 1 Inertial Profiling System, the Department will use a Class 1 Walking Profiler or Lightweight Profiler.

2.	Quality Control Testing.  Perform quality control testing during lift placement to ensure compliance with the ride quality requirements specified in Table 421.03.03-4.

3.	Preparation for IRI Testing.  Notify the RE when all paving is complete and the RE will request IRI testing by Pavement & Drainage Management & Technology (PDMT) unit.  Provide traffic control when the Department performs IRI testing.  Perform mechanical sweeping of the surface before IRI testing.  To facilitate auto triggering on laser profilers, place a single line of temporary pavement marking tape perpendicular to the roadway baseline at the beginning and end of each lane, shoulder, and ramp to be tested or at the direction of the Department.  Submit the actual stationing for each temporary pavement marking tape location to the RE.

4.	Quality Acceptance.  The Department will determine acceptance and provide PA based on the following:

a.	Pay Adjustment.  The acceptable IRI for the roadway pavement will be the target IRI (T) from Table 421.03.03-4 for which full payment will be made and will be determined using the latest available existing current average IRI (C) data of the right most travel lane specified in 102.04 or from PDMT.  The number of lots for final pay adjustment will be reduced by the number of lots excluded for each segment shown in Special Provisions Table 421.03.03-2A.  Lots excluded from final PA will be those with the highest recorded IRI numbers for respective roadway and bridge deck segments.  A single average IRI value and the corresponding PA for each 0.01 mile lot will be reported.  IRI units are in inches per mile.

| Table 421.03.03-2 Pay Equations for Ride Quality |  |  |  |
| --- | --- | --- | --- |
| Pay Equation Type | Excluded Lots | Pay Equation(s) |  |
| PA11 | As shown in the Special Provisions Table 421.03.03-2A | IRI < (T – 25) | PA1 = $ 0.1 x P |
|  |  | (T – 25) ≤ IRI < (T – 5) | PA1 = $ (T – IRI – 5) x 0.005 x P |
|  |  | (T – 5) ≤ IRI ≤ (T + 5) | PA1 = $ 0 |
|  |  | (T + 5) < IRI ≤ (T + 75) | PA1 = – $ (IRI – T – 5) x 0.014286 x P |
|  |  | IRI > (T + 75) | PA = –$ P |
| PA2 | Will include, if tested | IRI ≤ 120 | PA2 = $ 0 |
|  |  | 120 < IRI ≤ 170 | PA2 = $ (IRI − 120) x (−$ 0.1 x P) |
|  |  | IRI > 170 | Maximum Negative Pay or Corrective Action |
| P=3E+200A310 |  |  |  |
| Round up calculated P to a whole number and use that or 100, whichever is greater. Where: E=Bid price of Micro Surfacing or Slurry Seal Asphalt Emulsion, gallon A=Bid price of Micro Surfacing or Slurry Seal aggregate, ton - Maximum Positive Pay Adjustment: The final positive pay adjustment shall not exceed the cumulative positive pay adjustment for all lots with IRI values less than those specified in Table 421.03.03-3. Positive pay adjustment for lots with IRI values equal to or greater than those specified in Table 421.03.03-3 shall be used to offset negative pay adjustment. |  |  |  |

2******************************************************************************************************2

send email to sme to request the exclusions in table 421.03.03-2A for roadways within the project and include the following

sme CONTACT – Pavement & drainage Management & Technology unit

| Table 421.03.03-2A Exclusions for Micro Surfacing or Slurry Seal |  |  |
| --- | --- | --- |
| Roadway | Lane Number | Exclusions |

Lane designation is by increasing numbers from left to right in the direction of traffic with left lane being Lane 1.

2******************************************************************************************************2

| Table 421.03.03-3 Maximum Positive Pay Adjustment |  |
| --- | --- |
| Roadway Type | IRI for Maximum Positive Pay Adjustment |
| Freeways or Limited Access Highways | < 50 Inch/Mile |
| Other than Freeways or Limited Access Highways | < 60 Inch/Mile |

| Table 421.03.03-4 Target IRI for Micro Surfacing or Slurry Seal (T)1 |  |  |  |
| --- | --- | --- | --- |
| Roadway Type | Excluded Lots | Current average IRI (C) | Target IRI (T) |
| Freeways or Limited Access Highways | As shown in the Special Provisions Table 421.03.03-2A | ≤ 50 | 50 |
|  |  | > 50 | 0.9 x C1 |
| Other than Freeways or Limited Access Highways with speed limit > 35 MPH |  | ≤ 60 | 60 |
|  |  | > 60 | 0.9 x C1 |
| Other than Freeways or Limited Access Highways with speed limit ≤ 35 MPH |  | ≤ 70 | 70 |
|  |  | > 70 | 0.9 x C1 |
| 1. Current average IRI (C) is the average of the latest available preconstruction network level IRI data of right most travel lane from PDMT. |  |  |  |

b.	Corrective Action.  If the average IRI is greater than T+75 inches per mile after testing is performed, the Department may require corrective action or assess the maximum negative pay adjustment as computed in Table 421.03.03-2.  If the Department requires corrective action, submit a plan for corrective action.  If the plan for corrective action is approved and the lot is corrected, the Department will retest and evaluate the corrected area as a new lot that must meet the same requirements as the initial work.  If the plan for corrective action is not approved, the Department may require removal and replacement.  The replacement work is subject to the same requirements as the initial work.

1**************************************************************************************************************************1

send email to sme to request the exclusions in table 421.03.03-2A for roadways within the project and include the following, INCLUDE THE LOCAL AID PROJECT MANAGER ON ALL TRANSMITTALS

sme CONTACT – Pavement & drainage Management & Technology unit

THE FOLLOWING IS ADDED:

| Table 421.03.03-2A Exclusions for Micro Surfacing or Slurry Seal |  |  |
| --- | --- | --- |
| Roadway | Lane Number | Exclusions |

Lane designation is by increasing numbers from left to right in the direction of traffic with left lane being Lane 1.

1**************************************************************************************************************************1

Section 422 – Fog Seal

422.01  Description

1**************************************************************************************************************************1

BDC22S-21 dated Mar 24, 2023

The first paragraph is changed to:

This section describes the requirements for furnishing and applying a fog seal surface treatment with a fine aggregate cover.  This section also describes the requirements for applying a fog seal strip over HMA longitudinal cold joints.

1**************************************************************************************************************************1

422.02.01  Materials

1**************************************************************************************************************************1

BDC22S-21 dated Mar 24, 2023

The first paragraph is changed to:

Provide materials as specified:

Fine Aggregate for Fog Seal	901.07.02

1	Asphalt Emulsion.  For fog seal surface treatment and fog seal of HMA longitudinal cold joint provide emulsified asphalt of grades RS-1 or RS-2 in accordance with AASHTO M 140; or provide cationic emulsified asphalt of grades CRS-1 or CRS-2 in accordance with AASHTO M 208; and ensure all emulsified asphalts are provided as specified in 902.01.02.

2	Polymerized Maltene Emulsion.  As an alternative for asphalt emulsion specified above for fog seal strip of HMA longitudinal cold joints provide a polymerized maltene emulsion.

1**************************************************************************************************************************1

422.03.01  Fog Seal Surface Treatment

A.	Fog Sealing Plan.

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

part (5) is changed to:

5.	Lighting plan for night operations as specified in 108.06 for paving.

1**************************************************************************************************************************1

D.	Surface Preparation.

1**************************************************************************************************************************1

BDC22S-21 dated Mar 24, 2023

The first paragraph in Part D is changed to:

Ensure all repairs are completed prior to beginning fog seal installation.  Clean the surface of existing pavement to remove all dust debris, oil and any other materials that may prevent bonding of the fog seal.  Ensure that the surface is clean and dry.  Remove traffic stripes and traffic markings as specified in 610.03.08.

1**************************************************************************************************************************1

422.03.02  Fog Seal Strip

E.	Fog Sealing Application

1**************************************************************************************************************************1

BDC22S-21 dated Mar 24, 2023

The second paragraph in Part E is changed to:

Apply fog seal in a 2 feet wide strip centered over the HMA longitudinal cold joint, ensuring complete coverage of the HMA longitudinal cold joint.  Ensure that the fog seal material completely covers the pavement surface and is not streaked or ribboned.  Ensure that the distribution is even with no uncoated areas or puddles of excess emulsion.  Correct uncoated or lightly coated areas by applying additional fog seal emulsion.  Blot areas showing an excess of fog seal with sand approved by the RE.  Remove excess sand and emulsion material.  In areas inaccessible to distributor spray bars, use hand spraying equipment.

1**************************************************************************************************************************1

Division 450 – Concrete Pavement Rehabilitation

Section 454 – Diamond Grinding Existing Concrete Pavement

454.03.02  Ride Quality Requirements (For NHS and roads under NJDOT Jurisdiction)

4.	Quality Acceptance.  The Department will determine acceptance and provide PA based on the following:

a.	Pay Adjustment.

1**************************************************************************************************************************1

send email to sme to request the exclusions in table 454.03.02-1A for roadways within the project and include the following,

INCLUDE THE LOCAL AID PROJECT MANAGER ON ALL TRANSMITTALS

sme CONTACT – Pavement & drainage Management & Technology unit

THE FOLLOWING IS ADDED:

| Table 454.03.2-1A Exclusions for Diamond Grinding |  |  |
| --- | --- | --- |
| Roadway | Lane Number | Exclusions |

Lane designation is by increasing numbers from left to right in the direction of traffic with left lane being Lane 1.

1**************************************************************************************************************************1

Division 500 – Bridges and Structures

Section 502 – Load Bearing Piles

502.03.03  Driving Piles

C.	Test Piles.

1**************************************************************************************************************************1

1.	Static Pile Load Test.

2**************************************************************************************2

complete and include static test load to be applied

Apply a total of _____ tons of static test load to the piles.

2**************************************************************************************2

2.	Dynamic Pile Load Tests.

2**************************************************************************************2

Specify if restrike is required

2**************************************************************************************2

1**************************************************************************************************************************1

Section 503 – Drilled Shaft Foundations

503.03.03  Load Test

1**************************************************************************************************************************1

BDC24S-02 dated apr 04, 2024

B.	Load Testing and Reporting.

The last paragraph is changed to:

Within 14 days, the Department will notify the Contractor if revisions to the foundation lengths and installation procedures will be made based on the results of the load tests.  Do not begin construction of production drilled shafts without the Department’s approval.

1**************************************************************************************************************************1

503.03.04  Crosshole Sonic Logging

1**************************************************************************************************************************1

BDC24S-02 dated apr 04, 2024

The First paragraph in part c is changed to:

C.	Evaluation of the CSL Test Results.  The Department will evaluate the CSL test results and determine whether or not the drilled shaft construction is acceptable and respond within 14 days.  If the RE determines that the drilled shaft is acceptable, dewater and grout the CSL tubes.  Use the grout with the same strength or higher than the strength of the concrete used in the original drilled shaft.

Part D is changed to:

D. Further Evaluation.  Perform Tomography, Shaft Coring, or other investigated methods as directed by the RE for further evaluation.  Submit a report signed and sealed by a Professional Engineer registered in the State of New Jersey providing the results of further evaluation and recommendations to accept or repair the shaft within 14 days.  The report must contain recommendations for modification of construction procedures to prevent defects for subsequent shaft installation.  The Department will respond within 14 days.

1**************************************************************************************************************************1

503.03.05  Tomography

1**************************************************************************************************************************1

BDC24S-02 dated apr 04, 2024

the subpart is changed to:

Use the same equipment and access tubes as the CSL method.  Submit the Tomography analysis results to RE for review.  Provide the two dimensional (2-D) horizontal and vertical slices and three dimensional (3-D) tomographs for the entire shaft between the respective tube pairs.  Present these images in color and coded to identify the variations in sonic velocity.  Include the complete discussion of the Tomography tests results in the report.  The Department will respond within 14 days.

1**************************************************************************************************************************1

Section 504 – Structural Concrete

504.01  Description

the following is added:

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

This Section also describes the requirements for the application of color stain and primer to concrete structures and staining other areas as described herein and as shown on the plans.

1**************************************************************************************************************************1

504.02  Materials

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

THE FOLLOWING MATERIAL IS ADDED TO THE LIST

Concrete Stain	912.01.04

1**************************************************************************************************************************1

504.03  Construction

1**************************************************************************************************************************1

BDC24S-16 dated AUG 22, 2024

504.03.01  Reinforcement Steel

PART G IS CHANGED TO:

G.	Field Repairing of Coatings.  The RE will allow field repair when there is no greater than 2 percent of the surface area in each linear foot of each bar damaged.  The RE will reject material with more than 2 percent of the surface area in each linear foot of each bar and any material with a damaged area more than 4 square inches in size.  Repairable damage is any bare or loose spots or breaks in the coating that affect an area smaller than 4 square inches.  Ensure that grease, dirt, mortar, concrete, mill scale, injurious rust, or any other foreign substance is removed before repairing.

1.	Epoxy-Coating.  Repair damage to epoxy-coating according to ASTM D3963.

2. 	Galvanized-Coating.  Repair damage to the galvanized coating according to ASTM A780.

1**************************************************************************************************************************

504.03.02  Concrete

1**************************************************************************************************************************1

G.	Removal of Forms and Falsework.

Complete and include the following if concrete strength required for the removal of forms and falsework

Do not remove forms and falsework until the concrete obtains a compressive strength of ____ pounds per square inch.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

THE FOLLOWING SUBpart IS ADDED:

504.03.04  Concrete Staining

Prior to staining, examine the surfaces to be stained.  Bring areas requiring patching or repair to the attention of the RE.

A.	Submittals.  Submit product information and manufacturer color chip sample for approval by the Office of Landscape Architecture.

B.	Cleaning.  After the concrete areas and any patching has cured for at least 28 days, power wash all surfaces for initial preparation at a minimum pressure of 2,500 psi using a 25 degree tip and a standoff distance of 2 feet.  Verify that all foreign materials, such as dirt, dust and form oil, have been removed and surfaces are clean prior to application of primer and stain.  If contaminants are still present after initial preparation, vary power washing parameters or use other suitable methods to clean surfaces.

C.	Test Staining.  Complete a test staining program for porosity, adhesion, and color acceptance before staining operations on approved textural mock-ups or in place according to contract plans.  Test area(s) to include both smooth and textured concrete sections.  Prepare test area for initial surface preparation as described in 504.03.04.B for inspection by the RE.  If the staining mock-ups are approved, they may be included as part of the final construction.

After concrete has cured, test for porosity prior to testing for adhesion or color acceptance, on both the smooth concrete section and the textured concrete section by spraying water onto the surface to be stained.  Notify the RE if the water does not absorb rapidly as per manufacturers recommendations.

If the concrete passed the porosity test, apply production stain to concrete as specified in 504.03.04.D.  After the manufacturer’s recommended drying time of the stain, test for satisfactory adhesion of stain as described in 
Table 912.01.04-1.

The final stain color scheme will be approved by the Office of Landscape Architecture in the field after reviewing and approving the test stain program.  Do not order final quantities of stain and primer until approved.  When approved the sample area will serve as a standard of acceptance for all further work.

D.	Stain and Primer.  Apply one coat of primer and one coat of stain to smooth concrete surfaces.  Apply one coat of stain, only to textured concrete surfaces.

E.	Stain Colors.  See the Special Provisions for stain colors.

2**************************************************************************************2

INCLUDE STAIN COLORS AS NEEDED FOR BASE, HIGHLIGHT, AND SHADOW COATS

2**************************************************************************************2

1**************************************************************************************************************************1

504.04  Measurement and Payment

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

The following Item is added:

Item	Pay Unit

CONCRETE Staining	Square Yard

1**************************************************************************************************************************1

Section 505 – Precast and Prestressed Structural Concrete

505.03.01  Pretensioned Prestressed Concrete Beam and Prestressed Concrete Beam

1**************************************************************************************************************************1

BDC21S-07 dated May 25, 2021

The first paragrapH in Part A IS CHANGED TO:

A.	Working Drawings.  Submit working drawings for certification, as specified in 105.05, that include the class of concrete, the pattern and schedule for releasing strands before detensioning, detensioning concrete strength, and tensioning and detensioning patterns.

Part C IS CHANGED TO:

C.	Erection Plan.  Submit working drawings for approval, as specified in 105.05,regarding the plan of operations to the RE at least 30 days before the pre-erection meeting.  Include, at a minimum, the following in the plan:

1.	Number and type of manpower and equipment.

2.	Shipping procedures.

3.	Lifting procedures.

4.	Erecting sequence.

5.	Temporary bracing.

6.	Manufacturer’s recommendations.

7.	Procedures for employee safety.

8.	Traffic control and protection.

9.	Method of post-tensioning and determining friction loss.

10.	Anchorage details and design calculations, signed and sealed by a Professional Engineer.

1**************************************************************************************************************************1

505.03.02  Precast Concrete Culvert

1**************************************************************************************************************************1

BDC21S-07 dated May 25, 2021

Part A IS CHANGED TO:

A.	Working Drawings.  Submit working drawings for approval, as specified in 105.05,that show plan, elevation, and sections as well as details for all appurtenances such as headwalls, cutoff walls, wingwalls, and aprons.  In addition, include details of the neoprene gasket between the precast concrete culvert units as well as all threaded inserts, bar extensions, waterproofing, and end anchorage details for the post-tensioning reinforcement.  Provide erection details including handling points, neoprene gasket details, the method for pulling the culvert boxes together, section lengths, and the method of installing the units.

1**************************************************************************************************************************1

Section 506 – Structural Steel

506.03.01  Structural Steel

1**************************************************************************************************************************1

BDC21S-07 dated May 25, 2021

Part B IS CHANGED TO:

B.	Erection Plan.  At least 30 days before the pre-erection meeting, submit working drawings for approval, as specified in 105.05,regarding the plan of operations to the RE.  Include, at a minimum, the following in the plan:

1.	Number and type of manpower and equipment.

2.	Shipping procedures.

3.	Lifting procedures.

4.	Beam erecting sequence, including method of setting bearings and diaphragms.

5.	Temporary bracing.

6.	Manufacturer’s recommendations.

7.	Procedures for employee safety.

8.	Traffic control and protection.

1**************************************************************************************************************************1

Section 507 – Concrete Bridge Deck, Bridge Parapet, and approaches

507.02.01  Materials

1**************************************************************************************************************************1

BDC21S-05 dated JAN 06, 2023

THE FOLLOWING IS ADDED:

Provide and use a penetrating type of protective sealer as directed by the RE.

Ensure the protective sealer does not contain an aqueous solvent or carrier.

1**************************************************************************************************************************1

507.03.01  Joint Assemblies

1**************************************************************************************************************************1

BDC21S-08 dated NOV 18, 2021

A.	Working Drawings.

Part A is changed to:

Submit working drawings for approval, as specified in 105.05, for modular expansion joint assemblies.  Submit working drawings for certification, as specified in 105.05, for strip seal expansion joint assemblies, preformed elastomeric joint assemblies, and other types of expansion joint assemblies.  Finger joint assemblies are not allowed.  As a minimum, indicate the following information on the working drawings:

1.	Plan, elevation, and section of the joint system for all movement ratings and roadway width limitations, as well as relative dimensions and tolerances.

2.	All ASTM, AASHTO, or other material designations.

3.	Method of installation including sequence of installation, relative temperature settings, setting anchorage, and installation at curb lines.

4.	Corrosion protection system or material.

5.	Details of temporary supports for shipping and handling.  Include lifting mechanisms and locations.

6.	Design calculations for all structural elements.  As warranted, include fatigue design calculations and strength design calculations.  Indicate locations of all welded splices.

7.	Design the joint system as one continuous unit without field splices.  If due to shipping or construction requirements, splices are necessary indicate that the locations of the splices are to be in areas outside the main traffic lanes.  Provide field splicing procedures.

8.	Ensure that the removal and reinstallation of the strip seal can be accomplished from above the joint without full closure of the roadway.

9.	Ensure that the expansion joint assembly seals do not protrude above the top of the joint.  Use preformed neoprene strip seals that are mechanically held in place.

In addition, for modular expansion joint assemblies, incorporate the following requirements:

1.	Provide the modular expansion joint assembly to accommodate all expected longitudinal movements as well as vertical and horizontal rotations.  Incorporate strip seal glands with a maximum movement range of 3.15 inches per seal.  Support centerbeam(s) with an independent support bar that is welded to the centerbeam.  Suspend the support bars over the joint opening by sliding elastomeric bearings.  Incorporate an equidistant control system that develops its maximum compressive force when the joint is at its maximum opening.  Do not use bolted connections between the centerbeams and support bars unless approved by the Designer.  If bolted connections are approved, lock them into position after tightening.  Provide tightening procedure for Designer’s approval to ensure all bolts cannot loosen during the service life.

2.	Provide continuous centerbeams without any field splice unless approved by the Designer.  If a field splice is inevitable due to a manufacturing constraint, provide a fatigue resistant field splice (welded or bolted, and located away from potential wheel paths) and installation procedure.  Submit welding procedure specification (WPS), welder qualification, weld procedure, and QC inspection plans for field welds for Designer’s approval.  Installation is to be supervised by the Manufacturer and Contractor.  If bolted connections are used, provide a bolt tightening procedure and ensure bolt loosening does not happen during service life.  Provide continuous strip seals even if centerbeams are spliced.

Verify the blockout location, dimensions, and details shown on contract plans, and ensure that installation of a modular expansion joint assembly shall fit in the blockouts on site.

3.	Fatigue test the modular expansion joint assembly, including the transverse centerbeams, support bars, and other structural elements and connections, according to Section 19/A19 of AASHTO LRFD Construction Specifications.  Submit prequalification fatigue test report.  Ensure welded (or bolted, if approved) details used in project products match those that have been fatigued tested.  When a fatigue test report is not available, welded, or bolted details may be fatigue designed in accordance with Section 14 of AASHTO LRFD Bridge Design Specifications as approved by the Designer.

4.	Perform prequalification tests, Open Movement and Vibration (OMV) testing and Seal Push-Out (SPO) testing as specified in 914.04.03 for review and approval by the Designer.

5.	Provide the elastomeric springs and bearings to be removable and replaceable in the event maintenance is necessary.

Verify the blockout location, dimensions, and details shown on contract plans, and ensure that installation of a modular expansion joint assembly shall fit in the blockouts on site.

B.	Installation.

Part 3 is changed to:

3.	Modular Expansion Joint Assemblies.  Modular expansion joint assemblies include preformed neoprene strip seals that are held in place by steel edgebeams and centerbeams, support bars to support the centerbeam and sliding elastomeric bearings.  These assemblies contain no bolted connections unless approved by the Designer.

1**************************************************************************************************************************1

507.03.02  Concrete Bridge Deck

1**************************************************************************************************************************1

BDC21S-07 dated May 25, 2021

A.	Forms.  Construct forms as follows:

the first paragraph in Part 1 IS CHANGED TO:

1.	Stay-In-Place (SIP) Forms.  Submit working drawings for approval, as specified in 105.05, that include the following:

a.	Deck reinforcement location.

b.	Grade of steel.

c.	Galvanizing specification.

d.	Physical and section properties for all permanent steel bridge deck form sheets.

e.	Locations where the forms are supported by steel flanges subject to tensile stresses.

1**************************************************************************************************************************1

507.03.05  Concrete Parapet and Concrete Barrier Curb

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

THE FOLLOWING part 3 is added:

3.	Concrete Staining.  Perform concrete staining as specified in 504.03.04.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC21S-05 dated Jan 06, 2023

THE FOLLOWING SUBpart IS ADDED:

507.03.08  Bridge Deck and Bridge Approach Sealing

The RE will direct if and where to apply BRIDGE DECK AND BRIDGE APPROACH SEALING.

A.	Instructions.  At least 5 days prior to starting the sealing operation provide the RE with the manufacturer’s instructions for application and use.  Use only 1 brand and specific type of sealer on each deck or approach slab.

B.	Surface Preparation.  Ensure the concrete is air dry for 24 hours after saw cut grooving is completed.  If the concrete is subjected to rain or moisture from other project operations, extend the drying time by 24 hours from the time that the concrete has stopped being wetted.  Complete all work such as surface texturing, saw cut grooving, barriers, parapets, sidewalks and safety walks prior to the cleaning the concrete surface.  After the drying period has ended, clean the concrete surface by vacuum methods, to remove loose particles.

After cleaning, ensure no laitance, standing water, oil, dirt or other foreign particles are present.  Do not start sealer application until the RE approves the surface preparation.

C.	Weather Limitations.   Do not apply sealer materials during wet weather conditions or when adverse weather conditions are anticipated within 12 hours of completion of sealer application.  Ensure ambient and surface temperatures, during application, and until the sealed concrete is dry to the touch, is maintained at a minimum of 40ºF.  Spray application is not permitted during windy conditions, if in the opinion of the RE, unsatisfactory results will be obtained.

D.	Sealer Application.  Do not alter or thin the sealer.  Ensure sealer equipment is free of foreign materials.  Apply the sealer by means of brushing, spraying or rolling, as recommended by the manufacturer.

Apply a minimum of 2 coats of the sealer to achieve uniform coverage.  Apply the sealer to an application rate as recommended by the manufacturer and to the satisfaction of the RE.  Apply second and each additional coat perpendicular to the previous coat.  Do not allow the sealer to run or puddle.  Ensure each coat dries for a minimum of 2 hours before the next coat is applied.  Ensure the final coat dries according to the manufacturer's instructions, prior to allowing traffic on the surface.

1**************************************************************************************************************************1

507.04  Measurement and Payment

1**************************************************************************************************************************1

BDC21S-05 dated Jan 06, 2023

THE FOLLOWING pay item IS ADDED:

Item	Pay Unit

BRIDGE DECK AND BRIDGE APPROACH SEALING	SQUARE FOOT

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

The following is added:

The Department will not include Concrete Staining under Concrete Parapet and Concrete Barrier Curb.  The Department will make payment for Concrete Staining under Concrete Staining.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC21S-05 dated Jan 06, 2023

THE FOLLOWING is ADDED:

The Department will not make payment for BRIDGE DECK AND BRIDGE APPROACH SEALING if it is determined by the RE that cracking occurred due to fault of the contractor.

1**************************************************************************************************************************1

Section 510 – Timber Structures

510.03.02  Sheeting and Wales

1**************************************************************************************************************************1

include the corrosion requirements for metal components, if NECESSARY

1**************************************************************************************************************************1

Section 512 – Sign Support Structures

1**************************************************************************************************************************1

512.04  Measurement and Payment

2**************************************************************************************2

INCLUDE THE FOLLOWING WHEN SUPPORT STRUCTURES FOR DYNAMIC MESSAGE SIGN (DMS) (other than ground mounted dms) ARE PROPOSED.

The following itemS ARE added:

Item	Pay Unit

Cantilever Sign Support, DMS Structure No. ____	Unit

Butterfly Sign Support, DMS Structure No. ____	Unit

2**************************************************************************************2

2**************************************************************************************2

INCLUDE THE FOLLOWING WHEN DRILLED SHAFT FOUNDATION IS proposED.

THE FOLLOWING IS ADDED:

The Department will make payment for drilled shaft foundations for sign supports under DRILLED SHAFT FOR SIGN STRUCTURE FOUNDATION as specified in 51X.04.

2**************************************************************************************2

1**************************************************************************************************************************1

1**************************************************************************************************************************1

include the following upon ASSIGNING an APPROPRIATE SECTION/SUBSECTION/subpart NUMBERs if DRILLED SHAFT foundations FOR SIGN SUPPORT STRUCTURES is necessary

The following section is added:

Section 513 – Retaining Walls

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

The following Subpart is added:

513.03.03  Concrete Staining

Perform concrete staining as specified in 504.03.04.

1**************************************************************************************************************************1

513.04  Measurement and Payment

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

The following is added:

The Department will make payment for Concrete Staining as specified in 504.04.

1**************************************************************************************************************************1

Section 514 – Temporary Structures

514.03.01  Temporary Structures

1**************************************************************************************************************************1

BDC21S-07 dated May 25, 2021

Part A IS CHANGED TO:

A.	Working Drawings.  At least 30 days before beginning the work, submit working drawings for approval, as specified in 105.05.

Part B IS CHANGED TO:

B.	Erection Plan.  At least 30 days before beginning work, submit to the RE a plan for approval, as specified in 105.05, that includes the proposed equipment, detailed erection instructions and drawings of all structures, and the proposed scheme for traffic control during the erection and use.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

Division 600 – Miscellaneous Construction

Section 601 – Pipe

601.02  Materials

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

The Following Material is added:

Polypropylene (PP) Pipe	909.02.02

1**************************************************************************************************************************1

601.03.01  Pipe

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

B.	Excavating.

The fifth paragraph in part b is changed to:

If the material at the bottom of the trench is rock or other hard material, remove at least 6 inches of the material for RCP or at least 12 inches of the material below the bottom of the pipe for corrugated metal, steel, aluminum alloy pipe, HDPE or polypropylene (PP) pipe.  Backfill the undercut with Class C bedding.

D.	Installing Pipe

The first paragraph in part D is changed to:

Before the installation of HDPE and polypropylene (PP) pipe and as directed by the RE, provide a technical representative from the pipe manufacturer on the work site for the first day of pipe installation to ensure proper installation procedures.

The last paragraph in part D is changed to:

When using heavy construction equipment (100 kips axle load) over, or within 10 feet of HDPE and polypropylene (PP) pipe or corrugated aluminum alloy pipe, place the manufacturer recommended temporary compacted cover over the top of the pipe. Ensure that the temporary cover is free from stones larger than 1 inch.

E.	Joining Pipe.

The last paragraph in part E is changed to:

Do not use split couplings to join field-cut HDPE and polypropylene (PP) pipe unless approved by the RE.  Ensure that joints are bell and spigot type, or bell and spigot type with a gasket, according to ASTM F 477, to provide a silt-tight seal.  Construct pipe connections according to the manufacturer’s recommendations for assembly of joint components, lubrications, and making of joints.  Ensure that the pipe fittings are free of inclusions and visible defects.  Cut the ends of the pipe squarely so as not to adversely affect joining.

F	Backfilling.

The first paragraph in part F is changed to:

When using corrugated aluminum pipe, backfill from the bottom of the trench to 2 feet above the top of the pipe with Class C bedding. When using HDPE and polypropylene (PP) pipe, backfill from the bottom of the trench to 1 foot above the top of the pipe with Class C bedding. When using pipe other than corrugated aluminum, HDPE or polypropylene (PP) pipe, backfill from the bottom of the trench to 2 feet above the top of the pipe with suitable excavated material free from stones and rock larger than 2 inches in any dimension. For distances 2 feet above the top of the pipe, backfill using suitable excavated material.

601.03.03  End Section

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

The second paragraph is changed to:

Use end sections of the same material as the adjoining pipe or pipe arch, except use concrete end sections for HDPE and polypropylene (PP) pipe.

1**************************************************************************************************************************1

601.03.04  Underdrain

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

A.	Excavating.

The fourth paragraph in part A is changed to:

If the material at the bottom of the trench is rock or other hard material, remove at least 6 inches of the material for RCP or at least 12 inches of the material below the bottom of the pipe for corrugated metal, steel, aluminum alloy pipe, HDPE or polypropylene (PP). Backfill the undercut with Class C bedding.

1**************************************************************************************************************************1

601.03.06  Video Inspection of Pipe

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

the second paragraph is changed to:

Perform the video inspection in dry pipe conditions to ensure viewing of the entire pipe circumference.  Ensure that a visual numerical registration of the distance the video camera is traveling from the starting point to the ending point within the pipe drainage structure is recorded on the digital footage  at all times.  Also, ensure that the name of the pipe run and the name of the structure is recorded on the digital footage at all times.  Stop the video camera at all joints, lateral connections, breaks, and irregularities to ensure full view at these locations.  Ensure that the video provides clear, sharply focused pictures.  The Department will not accept blurred or out of focus footage.  Submit color digital footage of the inspection to the RE to become the property of the Department.  Number the files sequentially and provide a corresponding index for all videos, listing the location, date, size and type of pipe, cross or longitudinal drains, berm, slope, and similar identifying information.

1**************************************************************************************************************************1

601.03.07  Deflection Inspection of HDPE Pipe

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

The Subpart heading is changed to:

601.03.07  Deflection Inspection of HDPE Pipe and Polypropylene (PP) Pipe

The first sentence of the first paragraph is changed to:

Test approximately 25 percent of the length of HDPE and polypropylene (PP) pipe for deflection no sooner than 30 days after installation.

1**************************************************************************************************************************1

601.04  Measurement and Payment

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

The following items are added:

Item	Pay Unit

POLYPROPYLENE PIPE	LINEAR FOOT

DEFLECTION TESTING OF POLYPROPYLENE PIPE	LINEAR FOOT

Section 602 – Drainage Structures

602.03.03  Set Casting, Reset Casting, and Reconstructed Inlet and Manhole

1**************************************************************************************************************************1

BDC24S-01 dated mar 27, 2024

THE FIRST PARAGRAPH IS CHANGED TO:

When modifying less than 1 foot of an inlet or manhole, set a new casting, or reset the existing casting. When modifying 1 foot or more of an inlet or manhole, reconstruct the inlet or manhole.

1**************************************************************************************************************************1

Section 607 – Curb

607.01  Description

1**************************************************************************************************************************1

BDC23S-08 dated Jun 6, 2023

THE PARAGRAPH IS CHANGED TO:

This Section describes the requirements for constructing concrete curb, granite curb, and HMA curb, for resetting granite curb, and for cutting concrete vertical curb.

1**************************************************************************************************************************1

607.02.01  Materials

1**************************************************************************************************************************1

BDC23S-08 dated Jun 6, 2023

THE FOLLOWING IS ADDED:

Epoxy Waterproofing	912.02.02

1**************************************************************************************************************************1

607.02.02  Equipment

1**************************************************************************************************************************1

BDC23S-08 dated Jun 6, 2023

THE FOLLOWING IS ADDED:

Concrete Vertical Curb Saw	1008.07

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC23S-08 dated Jun 6, 2023

THE FOLLOWING SUBpart IS ADDED:

607.03.08  Sawcut Vertical Curb

The RE will determine if the existing concrete vertical curb is suitable for sawcutting.  Construct cast in place curb as specified in 607.03.02 if the existing curb exhibits visible cracking or deterioration or both.

Provide a concrete vertical curb saw as specified in 1008.07.  Construct erosion control measures as specified in 158.03.02.  Set the concrete vertical curb saw height to the desired vertical curb face height.  Sawcut the vertical curb to within ±1/2 inch of the desired curb face dimension.  Sawcut grooves as specified in 507.03.02.L to transition height differentials.

Finish the sawcut concrete vertical curb as specified in 607.03.01.E.  Seal sawcut concrete with epoxy waterproofing as specified in 504.03.03.  Prepare sawcut surface of concrete vertical curb according to manufacturer’s directions before applying epoxy waterproofing.

Dispose of cut material as specified in 201.03.01.H.

1**************************************************************************************************************************1

607.04  Measurement and Payment

1**************************************************************************************************************************1

BDC23S-08 dated Jun 6, 2023

THE FOLLOWING PAY ITEM IS ADDED:

SAWCUT VERTICAL CURB	LINEAR FOOT

1**************************************************************************************************************************1

Section 609 – Beam Guide Rail

609.03.01 Beam Guide Rail

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

THE FOLLOWING IS ADDED:

Remove trees and shrubs as specified in 801.03 and 802.03 from the entire guide rail element extending 4 feet behind the guide rail post.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC23S-14 dated DEC 22, 2023

THE FOLLOWING IS ADDED:

After completing guide rail installation, provide to the RE, coordinates in the New Jersey State Plane Coordinate System, based on the latest realization of the North American Datum of 1981, currently NAD83 (2011) with units of measure in US Survey feet (EPSG Code-3424).  Provide coordinates every 50 feet along the face of the as-built guide rail including at both ends to the accuracy of 3 feet (submeter) or better.  For guide rail runs of 100 feet or less provide coordinates at the beginning, midpoint or point of flares, and end of the run.  For guide rail runs of 1000 feet or more provide coordinates every 100 feet.  Ensure coordinates are collected at every location of change in horizontal and vertical geometry.

Provide the following digital photos tied to the coordinate location where each photo was taken:

1.	At least one digital photo of guide rail viewing the rail splice and at least one post.

2.	Three photos at each end of the guide rail.  Ensure photos of guide rail ends are taken at 3 view angles to present perspective view (front, back and side) of the end terminal, crash cushion, or connection with parapet, barrier, or abutment.

Submit coordinate data in excel format compatible with the Department’s version, and digital photos, electronically to the RE on forms provided by the Department after the guide rail installations have been constructed and accepted.

1**************************************************************************************************************************1

609.03.05 Reset Beam Guide Rail

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

THE FOLLOWING IS ADDED at the end of the subsection:

Remove trees and shrubs as specified in 801.03 and 802.03 from the entire guide rail element extending 4 feet behind the guide rail post.

1**************************************************************************************************************************1

Section 610 – Traffic Stripes, Traffic Markings, and Rumble Strips

610.03.02  Traffic Markings Lines, Traffic Markings Symbols, and Traffic Markings Route Symbols

1***********************************************************************************************1

BDC24S-21 DATED JAN 16, 2025

A.	Marking Plan.  At least 20 days before beginning the work, submit to the RE for approval a marking plan that includes:

1.	Schedule of operations for applying traffic markings.

2.	Number and type of equipment.

3.	Manufacturer’s recommendations for use of the materials, including mixing ratios and application temperatures.

4.	Details on the means and methods for surface preparation.

5.	Details on the means and methods for premarking.

B.	Surface Preparation.  Immediately before marking the pavement surface, clean the surface of dirt, oil, grease, and foreign material, including curing compound on new concrete.  Clean the surface 2 inches beyond the perimeter of the marking to be placed.

C.	Applying Traffic Markings.  Place preformed thermoplastic or hot extruded thermoplastic traffic markings on thoroughly dry surfaces and during dry weather conditions.  Apply using equipment and procedures that produce markings of the specified color, width, and thickness with well-defined edges, uniform retroreflectivity, and proper bonding to the pavement.  Apply the thermoplastic material as follows:

1.	Preformed Thermoplastic.  Melt the preformed thermoplastic tape to bond the traffic markings permanently in position according to the manufacturer’s recommendations.

Meet the minimum initial retroreflectance value, as specified in 610.03.01.D for thermoplastic tape, by applying additional glass beads to the hot-wet material in a uniform pattern as necessary.

2.	Extruded Thermoplastic.  Uniformly heat the thermoplastic material.  When the ambient and surface temperatures are at least 50 °F, apply the melted material at a temperature of between 400 and 425 °F.  Extrude the thermoplastic traffic markings on the HMA or concrete pavement ensuring a thickness of 90  1 mils.

Immediately after, or in conjunction with the thermoplastic extrusion, uniformly apply glass beads to the wet material at a minimum rate of 10 pounds per 100 square feet of markings.  Apply glass beads by mechanical means only.

D.	Applying Transverse Rumble Markings.  Install transverse rumble markings as follows:

1.	Ensure the HMA or concrete pavement is clean and dry before applying thermoplastic materials as specified in 610.03.02.B.

2.	Heat thermoplastic material as specified in 610.03.02.C.2.

3.	Apply one layer of thermoplastic material on the HMA or concrete pavement surface at a thickness of 125 ± 1 mils.  Then while the material is hot, apply a second layer of thermoplastic material at 125 ± 1 mils.

4.	Uniformly apply glass beads as specified in 610.03.02.C.2.

5.	The Contractor will ensure that there is no separation between the first and second layer of thermoplastic material before the RE’s final acceptance of transverse rumble markings.

6.	The Contractor will ensure that the appropriate thickness of 125 ± 1 mils per layer of thermoplastic material is installed, any markings found by the RE to be thinner than 125 ± 1 mils will be removed and reinstalled by the Contractor at no additional cost to the Department.

7.	The Department will make payment for transverse rumble markings by square feet under TRAFFIC MARKINGS SYMBOLS as specified in 610.04. 8.	Thermoplastic material for transverse rumble markings shall be white in color.

E.	Performance.  Ensure that the traffic markings show no fading, lifting, cracking, chipping for any reason including but not limited to traffic wear, maintenance activities including snow plowing, until Acceptance.  Ensure that 60 days after application, traffic markings have a minimum retroreflectance value of:

375 millicandelas per square meter per lux for white traffic markings

250 millicandelas per square meter per lux for yellow traffic markings

F.	Defective work.  Replace thermoplastic traffic markings that are determined by the RE before Acceptance to be defective or that are damaged during construction.  Remove defective markings as specified in 610.03.08.

Replace the entire area of thermoplastic traffic markings determined to be less than the required thickness, to have incorrect color or width, to have failed to bond to the pavement, or to have chipped or cracked.  The minimum replacement area is an individual word or symbol, or for longitudinal lines the entire length from where the deficiency first occurs to where it no longer exists.

The RE will determine initial retroreflectance as follows:

Provide the RE with a Reflectometer that meets a 30 meter geometry as specified in ASTM E 1710, capable of measuring wet and dry conditions as specified in ASTM E 2176 and ASTM E 2177, and that has been certified by the manufacturer as being calibrated within the last 2 years.  The RE will test the retroreflectance of traffic markings with the provided reflectometer according to the manufacturer’s recommendations.  Replace traffic markings that do not meet the retroreflectance values indicated in 610.03.02.E.

G.	Opening to Traffic.  Complete each application of thermoplastic traffic markings and allow to thoroughly dry before opening to traffic.  The RE will determine when the traveled way can be opened to traffic.

1**************************************************************************************************************************1

610.03.03  RPMs (Raised Pavement Markers)

1**************************************************************************************************************************1

BDC20S-08 dated Jun 12, 2020

THE FOLLOWING IS ADDED after the last paragraph:

Ensure that RPMs are installed within 14 days of opening each day’s surface paving to traffic.

1**************************************************************************************************************************1

610.03.07  Rumble Strip

1**************************************************************************************************************************1

2**************************************************************************************2

BDC21S-15 dated Jan 21, 2022

The fourth paragraph is changed to:

Ensure that after cutting centerline rumble strips, the centerline rumble strips are cleaned, all debris is collected and the surface is prepared as specified in 610.03.01.B before placing traffic stripes.

2**************************************************************************************2

2**************************************************************************************2

BDC20S-08 dated Jun 12, 2020

THE FOLLOWING IS ADDED after the last paragraph:

Ensure that rumble strips are constructed within 14 days of opening each day’s surface paving to traffic.

2**************************************************************************************2

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC20S-11 dated oct 02, 2020

THE FOLLOWING subpart IS ADDED:

610.03.09  Removal of Rumble Strip

Prior to shifting lanes, remove rumble strips as indicated in the plans and as directed by the RE.

Centered over the rumble strip, mill the width of the rumble strip plus 4 inches beyond the edge of the rumble strip on both sides.  Mill to a minimum depth of 2 inches.

Clean the milled area as specified in 401.03.01.A.  Obtain RE approval of the removal before proceeding with paving in the milled rumble strip.

Apply polymerized joint adhesive to the vertical surfaces of the milled rumble strip area as specified in 401.03.04.  Apply tack coat as specified in 401.03.05 at an application rate of 0.15 gallons per square yard to bottom surface of the milled rumble strip area.  Spread and grade HMA surface course in the milled rumble strip area as specified in 401.03.07.E.  Ensure that the temperature of the HMA when placed and compacted is at least 250 ºF.  Compact as specified in 401.03.07.F, ensuring that the top of the compacted HMA is flush with, or not greater than 1/8 inch higher than, the adjacent pavement surface.

Reuse removed material as specified in 202.03.03.C.

1**************************************************************************************************************************1

610.04  Measurement and Payment

1**************************************************************************************************************************1

BDC22S-21 dated Mar 24, 2023

The subpart is changed to:

The Department will measure and make payment for Items as follows:

Item	Pay Unit

TRAFFIC STRIPES, ___"	LINEAR FOOT

Traffic Markings Lines, ___"	LINEAR FOOT

TRAFFIC MARKINGS Symbols	SQUARE FOOT

Traffic markings route symbols	SQUARE FOOT

RPM, MONO-DIRECTIONAL, WHITE LENS	UNIT

RPM, MONO-DIRECTIONAL, AMBER LENS	UNIT

RPM, BI-DIRECTIONAL, AMBER LENS	UNIT

REMOVAL OF RPM	UNIT

REMOVAL AND REPLACEMENT OF RPM LENS	UNIT

FLEXIBLE DELINEATOR, GROUND MOUNTED	UNIT

RUMBLE STRIP	LINEAR FOOT

REMOVAL OF TRAFFIC STRIPES	LINEAR FOOT

REMOVAL OF TRAFFIC MARKINGS	SQUARE FOOT

REMOVAL OF RUMBLE STRIP	LINEAR FOOT

The Department will measure TRAFFIC STRIPES and TRAFFIC MARKINGS LINES by the linear foot for each specified width of stripe.  The Department will not measure gaps in striping.

The Department will measure rumble strip by the linear foot measured in the longitudinal direction of the rumble strip without deducting the interval spacing between rumble strips and the gaps for RPM placement and make payment under the Item RUMBLE STRIP.

The Department will not include payment for traffic stripes in RUMBLE STRIP.  The Department will make payment for traffic stripes placed in conjunction with constructing a centerline rumble strip under TRAFFIC STRIPES as specified in 610.04.

The Department will not measure the gaps such as WIM locations in the Rumble Strip.

1**************************************************************************************************************************1

Section 611 – Crash Cushions

1**************************************************************************************************************************1

BDC21S-10 dated Jun 02, 2022

611.03.02  Crash Cushion, Compressive Barrier

The subpart is changed to:

At least 10 days before beginning the work, submit the manufacturer’s installation guide to the RE.  Install compressive crash cushions including transitions according to the manufacturer’s recommendations and as shown on the Plans.  Install permanent compressive crash cushions on a concrete pad foundation according to the manufacturer’s recommendation.  Submit working drawings for the concrete pad foundation for each location.

1**************************************************************************************************************************1

Section 612 – Signs

612.02  Materials

1**************************************************************************************************************************1

BDC22S-16 dated July 17, 2023

The FOLLOWING MATERIAL IS CHANGED TO:

Steel “U” and Square Tube Post Sign Supports	911.02.01

1**************************************************************************************************************************1

612.03  Construction

612.03.01  Regulatory and Warning Signs, and Guide Sign, Type GA Steel “U” Post Supports

1**************************************************************************************************************************1

BDC22S-16 dated July 17, 2023

the subpart is renamed to:

612.03.01  Regulatory and Warning Signs, and Guide Sign, Type GA Steel “U” and Square Tube Post Supports

the second paragraph is changed to:

Mount sign panels on breakaway steel “U” or square tube post sign supports.  For signs located behind guide rails or other roadside barriers, mount sign panels on steel “U” or square tube post sign supports without the breakaway assembly.

1**************************************************************************************************************************1

612.04  Measurement and Payment

1**************************************************************************************************************************1

BDC22S-16 dated July 17, 2023

the following item is changed to:

Item	Pay Unit

GUIDE SIGN, TYPE GA, STEEL “U” OR SQUARE TUBE POST SUPPORTS	SQUARE foot

1**************************************************************************************************************************1

Division 650 – Utilities

Section 651 – Water

1**************************************************************************************************************************1

651.02  Materials

include additional materials requirements as needed

1**************************************************************************************************************************1

651.03.02  Ductile Iron Water Pipe, Bridge

1**************************************************************************************************************************1

include specifications for water mains on bridges

1**************************************************************************************************************************1

Section 652 – Sanitary Sewers

1**************************************************************************************************************************1

652.02  Materials

include additional materials requirements as needed

1**************************************************************************************************************************1

652.03.01  Sewer Pipe

G.	Sewer Pipe Testing.

1.	Gravity Main Sewer Testing.

1**************************************************************************************************************************1

include leakage testing list

1**************************************************************************************************************************1

652.03.02  Ductile Iron Sewer Pipe, Bridge

1**************************************************************************************************************************1

INCLUDE Specifications for sanitary sewer pipes on bridges

1**************************************************************************************************************************1

Section 653 – Gas

653.03.01  Gas Main

1**************************************************************************************************************************1

C.	Handling and Storing.

1**************************************************************************************************************************1

INCLUDE utility location for pickup and return

1**************************************************************************************************************************1

J.	Air-Pressure Test.

1**************************************************************************************************************************1

INCLUDE pressures and durations for test

1**************************************************************************************************************************1

Section 65X – JCP&L Facility

1**************************************************************************************************************************1

BDC25S-24 dated april 06, 2026

THE SECTION IS CHANGED TO:

2*****************************************************************************************************2

include the following if JCP&L related utility work is to be performed

ensure that the work package has been received from jcp&l; provide an electronic file (PDF format) of the construction order detail & construction order compatible unit summary only to the project manager for it to be posted on bid express as mentioned in QIA no: 046 dated Aug 01, 2007

2*****************************************************************************************************2

Section 655 – JCP&L Facility

655.01  Description

This Section describes the requirements for installing, relocating, and removing Jersey Central Power and Light (JCP&L) electric utility facilities including utility poles, risers, sheathing, conduits, manholes, transformer vaults, handholes, and appurtenances and also includes the requirements for transferring electric services.

655.02  Materials

Except for the materials noted below, JCP&L will supply all materials necessary for the work at no cost to the Contractor.  Provide JCP&L written notice 30 days in advance of when materials will be required.  Ensure the electric subcontractor takes delivery of the materials from JCP&L’s storage facility within 2weeks of the notice from JCP&L indicating that the material is available.  Materials may be located at more than one JCP&L storage facility.  If the electric subcontractor fails to take delivery, the material may not be available, and the electric subcontractor may be required to provide an additional request for materials.  The Contractor is responsible for compensating the Department for any additional handling costs incurred by JCP&L resulting from the failure to take delivery within the time required.

The electric subcontractor is responsible for loading the material, delivering it to the job site, and all subsequent handling and delivery within the jobsite.  Store and protect all materials received from JCP&L. Return and deliver all excess materials furnished by JCP&L to JCP&L’s storage facility.  Obtain a receipt for all material received from JCP&L, maintain a documented inventory of materials used and obtain a receipt for all material returned to JCP&L.

Provide materials as specified:

Tack Coat 64-22, PG 64S-22	902.01.01

Hot Mix Asphalt (HMA)	902.02

Concrete	903.03

Controlled Low Strength Material (CLSM)	903.09

Curing Materials	903.10

Joint Sealer, Hot-Poured	914.02

Polymerized Joint Adhesive	914.03

655.03  Construction

655.03.01  Electric

2**************************************************************************************2

BDC26S-06 dated JULY 20, 2026

THE SUBPART IS CHANGED TO:

A.	Prequalification.  Only a prequalified electric subcontractor, approved by JCP&L, may construct and relocate JCP&L electric facilities.

A list of the prequalified electric subcontractors is listed below:

3************************************************3

INCLUDE THE LIST OF PREQUALIFIED ELECTRIC SUBCONTRACTORS

CHECK THE UTILITY MANAGEMENT WEBPAGE UNDER JCP&L APPROVED SUBCONTRACTORS LIST

3************************************************3

The Contractor is responsible for soliciting from a subcontractor that will be approved by JCP&L when preparing its Bid.  Work restricted to the electric subcontractor does not preclude the Contractor from performing the work of layout, traffic control, sawcutting, pavement removal, temporary or final pavement restoration, and landscape restoration associated with the work of installing or relocating JCP&L electrical facilities.

B.	Scheduling of Work and Interruption to Utilities.  Provide the RE and the designated JCP&L representative with a detailed schedule of when the electric utility work will be performed.  Indicate in the schedule for each activity the following information: the work locations; the number of crews; and whether the work will be performed during a day shift or night shift, or on weekends.  Coordinate all electric utility work with the JCP&L representative, and notify the RE and the JCP&L representative at least 2 weeks prior to starting electric utility work.  Do not interrupt existing electric service until approved by the JCP&L representative.

Weather conditions may prevent connections to existing systems between June 1 and September 30.  Do not perform work which will require electric transmission service interruptions from June 1 through September 30 without the approval of JCP&L.  JCP&L may extend this period based on weather conditions and system demand.  Notify JCP&L at least 1 month in advance of commencing conductor work.

If service transfers are required, coordinate service transfers with the JCP&L representative.  Notify the property owner and all tenants affected by service interruptions or transfers prior to making the service transfer.  Minimize disruption to normal operations of existing facilities and minimize any interruption of electric service to JCP&L customers.  Protect existing facilities during construction and installation of the service transfer.

C.	Quality Control and Quality Assurance.  Provide access to the work for the JCP&L representative at all times.  Perform all electric utility work in a manner acceptable to the JCP&L representative.  Perform all electric utility work in accordance with JCP&L standards and details.

D.	Safety.  Perform work in accordance with applicable OSHA regulations, N.J.S.A. 34:6-47.1 et seq. “High Voltage Proximity Act”, and JCP&L safety standards.

E.	Abandonment and Removal.  Prior to beginning work, review the condition of all existing electric utility facilities noted to be removed with the JCP&L representative.  If the JCP&L representative designates the material to be salvaged, remove the material and deliver it to a JCP&L storage facility.  Remove and dispose of all other electrical utility material designated for removal.

F.	Excavation.  When excavation is required in areas having existing pavement and sidewalk, sawcut to the full depth of the existing pavement and sidewalk.  Excavate trenches for conduit, manholes and vaults and appurtenances.  Provide vertical sides for excavations within the traveled way, shoulder, sidewalk areas, and where existing facilities require protection.  Remove unstable material at the bottom of the excavation and backfill with granular material.  Do not excavate trenches more than 300 feet in advance of installing conduit unless approved by the RE.  Provide and maintain trench crossings where necessary to maintain access.  Do not leave trenches open overnight unless protected by temporary fencing or steel plates.  Remove and dispose of excess or unsuitable material as specified in 202.03.03.C.2.

G.	Backfill.  Backfill with suitable material in lifts not exceeding 6 inches thick, loose measurement.  If the backfill is predominantly granular material, compact the backfill material with a vibratory plate compactor.  For material that is not predominately granular, compact the backfill material with a vibratory rammer compactor.  If it is not possible to compact the backfill material, the Contractor may backfill with CLSM with the approval of the JCP&L representative.  If using CLSM, install as specified in 601.03.01.F.

H.	Restoration.  Restore areas disturbed in the performance of electrical utility relocations to its original condition.  In areas that are disturbed for which the plans provide final grading, pavement or landscaping, provide temporary restoration to the satisfaction of the RE.  If open-cut trenching across a road is required, restore the pavement with in-kind construction.

I.	Field Testing.  Perform a high-potential test (also known as a dielectric voltage withstand test) on all cables and splices prior to energizing.  Testing must be performed by a person who is qualified to operate the test equipment, and is familiar with the cable system.  Ensure that the cables are disconnected from non-cable systems equipment, and that adequate physical clearances are maintained between all cable ends, energized cables, and electrical grounds and all other equipment during the test.  Prior to performing the test, verify that all taps or laterals in the circuit are cleared.  In the event hot poured compound filled splices and terminations are involved, do not perform testing until they have cooled to ambient temperature.  Set the relays in the high voltage direct current test equipment to operate between 5 and 25 milliamperes leakage.  The shape of the leakage curve under constant voltage is more important than the absolute leakage current of a “go or no go” withstand test result.  The field test voltage is related to the final factory applied dc potentials using a factor of 80 percent.

Ensure the high potential test is performed in the presence of the JCP&L representative.  Apply a direct current field test voltage according to the following table:

| Field Test Values |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rated Voltage | dc Hi-Pot Test |  | dc Hi-Pot Test |  |
| Phase to | (15 Minutes) |  |  |  |
| Phase | Wall - mils | Kv | Wall - mils | kV |
| 5000 | 90 | 25 | 115 | 35 |
| 8000 | 115 | 35 | 140 | 45 |
| 15000 | 175 | 55 | 220 | 65 |
| 25000 | 260 | 80 | 320 | 95 |
| 28000 | 280 | 85 | 345 | 100 |
| 35000 | 345 | 100 | 420 | 125 |
| 46000 | 445 | 130 | 580 | 170 |
| 69000 | 650 | 195 | 650 | 195 |
| Note: If the leakage current quickly stabilizes, the duration may be reduced to 10 minutes. |  |  |  |  |

After the voltage has been applied and the test level reached, record the leakage current at 1 minute intervals.  If the leakage current decreases or stays steady after it has leveled off, the cable is considered satisfactory.  If the leakage current starts to increase, excluding momentary spurts due to supply-circuit disturbances, extend the test to see if the rising trend continues.  At the conclusion of the test, discharge the circuit through the test set and voltmeter circuit.  After the potential drops below 95 percent of the test value, ground the cable, and discharge the circuit.  Leave the grounds on all conductors for a minimum of 4 times as long as the test voltage was applied.

Remove and replace cables that fail to meet the requirements of the direct current field test.  The Contractor is responsible for reimbursing the Department for any additional material costs incurred by the Department resulting from the failure to meet the requirements of the direct current field test.

J.	Energizing Lines.  Energize lines with the guidance of the JCP&L representative.  Prior to energizing lines, submit a request to JCP&L.  Switching orders may only originate from JCP&L employees.  Submit a request for permission to energize transmission lines 10 days in advance of when the work will be performed.  Request permission to energize distribution lines in a manner that will permit the JCP&L representative to submit a request to JCP&L’s Dispatch Office by 12:00 p.m. the previous working day.

K.	As-builts.  Upon completion of the work, submit to JCP&L as-built drawings in accordance with JCP&L standards.  Prints of construction drawings, marked to show the final location, are acceptable.  Provide a copy of the as-built drawings to the RE.

2**************************************************************************************2

655.04  Measurement and Payment

The Department will measure and make payment for Items as follows:

Item	Pay Unit

ELECTRICAL UTILITY RELOCATION, JCP&L	LUMP SUM

1*********************************************************************************************************************1

Division 700 – Electrical

Section 701 – General Items

1**************************************************************************************************************************1

BDC25S-07 dated Apr 28, 2025

The subsection is changed to:

This Section describes the requirements for constructing or installing various electrical Items.  This Section also includes the requirements of clearing site electrical.

1**************************************************************************************************************************1

701.02.01  Materials

1**************************************************************************************************************************1

BDC25S-22 dated May 8, 2026

the following material IS CHANGED TO:

Junction Box Frames and Covers 	 918.15

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC25S-13 dated aug 19, 2025

the third paragraph is changed to:

The Department will allow the use of pre-qualified materials provided the materials meet the requirements of the Contract.  Submit materials for approval on a Materials Questionnaire as specified in 106.04.  Include Certified or Approved Working Drawings, as appropriate, with the Materials Questionnaire for materials not pre-qualified.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

701.03.01  Existing Systems

2**************************************************************************************2

if there are any its materials to be salvaged, provide the location to deliver and unload salvaged its materials

if a njdot facility is impacted the following is included:

Deliver and unload salvaged ITS materials to:

Mobility Management North (MMN)- ITS Maintenance

670 River Drive

Elmwood Park, NJ 07407-1347

Telephone: 732-697-7360

Mobility Management South (MMS) – ITS Maintenance

One Executive Suite Route 70 West

Cherry Hill, NJ  08002-4106

Telephone: 856-486-6615

2**************************************************************************************2

2**************************************************************************************2

inCLUDE THE FOLLOWING WHEN EXISTING CONDUIT SYSTEMS ARE PROPOSED TO BE USED for PULLING electrical CONDUCTORS OR FIBER OPTIC CABLES.

THE FOLLOWING IS ADDED:

If new cable or wire is designated to be installed into existing conduit systems, clean and swab the conduit system prior to installing the cable or wire.  After cleaning, test each conduit by pulling through a metal ball with a diameter at least 85 percent of the nominal inside diameter of the conduit to ensure the conduit is free of any obstruction or foreign material.  If the ball fails to pass through the conduit, repair or replace the defective conduit as directed by the RE.  Restore disturbed areas to original condition.

2**************************************************************************************2

1**************************************************************************************************************************1

701.03.15  Cable and Wire

C.	Connection and Coordination with Utility Services.

1**************************************************************************************************************************1

BDC21s-20 dated MAY 12, 2022

THE LAST PARAGRAPH in Part C IS CHANGED TO:

Provide temporary services if required for testing and operation of the electrical systems until Substantial Completion or as directed by the RE.  Coordinate temporary services with the Utility Company.

2**************************************************************************************2

for its facilities design, confirm on the requirements for interim communication and power connections, and connections to njta network. Also, for establishment of ip addresses, interim and permanent.

if required FOR PROJECTS THAT IMPACT NJDOT FACILITIES CONTACT the local aid project manager

2**************************************************************************************2

2**************************************************************************************2

Include the following for Transportation Mobility and Mobility Engineering Projects which require any new utility services.

THE LAST PARAGRAPH IS CHANGED TO:

Obtain and provide for utility services required for testing and operation of ITS systems until interim acceptance of each system or device.  Utility Services may be governed by differing Authorities Having Jurisdiction (AHJ).  Along with Utility Requirements, comply with all AHJ requirements.  Upon successful completion of level C testing and acceptance of any device, provide the RE with a letter requesting transfer of utility services providing the latest copy of the utility bill from each utility company.  Such transfers are to be effective beginning the next monthly billing cycle after completion of successful ITS system testing as specified in Section 704 and interim acceptance of the device or as directed by the RE.

Once new utility services have been energized or activated and the utility company has de-energized and unhooked the old service connection; remove existing pole risers and service heads, cut back 1 foot below grade, and plug the conduits.

| Service Requests |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Device Site No. | Primary Route | Final MP | Direction | Location | Township | County | Utility Territory | Job No. | Utility Contact Person | Utility Pole No. |

2**************************************************************************************2

1**************************************************************************************************************************1

BDC25S-07 dated Apr 28, 2025

The following Subpart is added:

701.03.16  Clearing Site Electrical

Ensure the requirements of 108.01 are adhered to.  Comply with the requirements specified in 701.03.01.  Deenergize electrical systems scheduled to be removed prior to beginning clearing operations.

1**************************************************************************************************************************1

701.04  MEASUREMENT AND PAYMENT

1**************************************************************************************************************************1

BDC25S-07 dated Apr 28, 2025

The following pay item is added:

Item	Pay Unit

CLEARING SITE ELECTRICAL	LUMP SUM

1**************************************************************************************************************************1

1**************************************************************************************************************************1

Section 702 – Traffic Signals

702.02.01  Materials

1**************************************************************************************************************************1

BDC25S-13 dated aug 19, 2025

the third paragraph is changed to:

The Department will allow the use of pre-qualified materials provided the materials meet the requirements of the Contract.  Submit materials for approval on a Materials Questionnaire as specified in 106.04.  Include Certified or Approved Working Drawings, as appropriate, with the Materials Questionnaire for materials not pre-qualified.

1**************************************************************************************************************************1

702.03.07  Push Button

1**************************************************************************************************************************1

BDC23S-02 dated Mar 13, 2023

The first paragraph is changed to:

Install a push button that operates on logic ground including its housing, and instruction signage.  Install cable from the push button to the foundation and make connections.  Ensure the traffic signal pedestrian and bicycle actuation sign complies with the MUTCD.

1**************************************************************************************************************************1

Section 703 – Highway Lighting

703.02.01  Materials

1**************************************************************************************************************************1

bdc25s-13 dated aug 19, 2025

the third paragraph is changed to:

The Department will allow the use of pre-qualified materials provided the materials meet the requirements of the Contract.  Submit materials for approval on a Materials Questionnaire as specified in 106.04.  Include Certified or Approved Working Drawings, as appropriate, with the Materials Questionnaire for materials not pre-qualified.

1**************************************************************************************************************************1

703.03  Construction

1**************************************************************************************************************************1

dated July 7, 2020

The second sentence in the fourth paragraph is changed to:

Begin repair of the highway lighting system within 2 hours of receiving notice of damage or malfunction from the Department, State police, local police or local authorities.

1**************************************************************************************************************************1

703.03.07  Temporary Highway Lighting System

2**************************************************************************************2

Determine whether or not the Contractor must design the Temporary lighting system at a given location and include the following

The Contractor must design the Temporary lighting system at ___________________________.

2**************************************************************************************2

2**************************************************************************************2

provide the locations to deliver and unload salvaged materials

Deliver and unload salvaged materials to:

2**************************************************************************************2

1**************************************************************************************************************************1

Section 704 – Intelligent Transportation Systems (ITS)

1**************************************************************************************************************************1

if impacting a njdot facility Prior to final design submission contact the local aid project manager to confirm with the respective traffic operations center if any of the proposed its systems have to be completed early in the project THAT ARE REQUIRED TO BE available to use for traffic mitigation during construction. include respective interim completion dates in subsection 108.10 and confirm with Mobility and Systems Engineering (MSE) on the requirements for interim communication and power connections.

if project includes connection to njta network, including direct fiber or wireless, contact the local aid project manager to confirm with Mobility and Systems Engineering (MSE) on coordination requirements with njta.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

704.02.01  Materials

MOST COMMON ITS MATERIAL SPECifICATIONS ARE AVAILABLE ON THE DEPARTMENT’S WEBSITE AND A QPL of prequalified materials is posted, DOWNLOAD THE REQUIRED SPECIFICATIONS FROM THE DEPARTMENT’S WEBSITE AND LIST THEM HERE IN THE TABLE REFERRING TO THE APPROPRIATE SUBSECTION OF SECTION 918 AND INCLUDE THE SPECIFICATIONS IN THAT SECTION.

1**************************************************************************************************************************1

704.03.01  General System (GS)

1**************************************************************************************************************************1

B.	Installation.

2**************************************************************************************2

if a njdot facility is impacted contact the local aid project manager to confirm with

TRAFFIC OPERATIONS THE EXISTING SYSTEM SHUTDOWN TIME. REVISE AS DIRECTED and insert here.

2**************************************************************************************2

2**************************************************************************************2

Provide the cost of damages for exceeding the allowable time frames

2**************************************************************************************2

2**************************************************************************************2

1.	Junction Box ITS.

a.	Installation.

IF REMOVAL OR RELOCATION OF ITS JUNCTION BOX IS NOT FEASIBLE IN A PROJECT THAT REQUIRES TRAFFIC TO BE SHIFTED IN THE AREA OVER THIS JUNCTION BOX DURING ANY STAGE OF CONSTRUCTION THEN IT MUST BE PROTECTED DURING CONSTRUCTION AND MUST INCLUDE DETAILS FOR PROTECTING THE ITS JUNCTION BOX.

2**************************************************************************************2

2**************************************************************************************2

6.	Control Center System.

include the work to be performed at eACH control center (toc, HUB, OR ANY BUILDING/CABINET WITH A NETWORK NODE) clearly and remove the portion of the work that is not applicable to any particular project as the bid price for this item will be based on the work involved at the designated control center. THE DESIGNERS MUST INCLUDE SYSTEM BLOCK DIAGRAMS AND FIBER ASSIGNMENT DIAGRAMS IN THE SET OF CONSTRUCTION PLANS. THE CONTRACTOR WILL USE THESE AS WORKING DRAWINGS BY ADDING A LIST OF EQUIPMENT AND NETWORK IP ADDRESSES.

2**************************************************************************************2

2**************************************************************************************2

7.	Meter Cabinet ITS.

This includes a fnmc reserved for electrical service conductors that may require separate junction boxes at short intervals. coordinate with MSE before PROCEEDING WITH THE DESIGN plans to determine what is exactly required. If a fiber glass sleeve is required, specify it and include with bridge structure attachments and expansion fittings as per manufacturer requirements. When Fiberglass conduit is not proposed across an existing structure, built-in sleeves within the parapet can be utilized if empty and available.

ITS CONDUIT TYPE A INCLUDES 3 fnmc. IF THE PROJECT SPECIFIC CONSTRUCTION REQUIREMENTS NEED DIFFERENT SIZE CONDUITS, modify THE STANDARD detail TO incorporate the proposed changes AND includE IT IN THE SET OF PLANS reflecting the correct size and type of conduits. ALSO, REVISE THE SPECIFICATIONS BELOW TO LIST ANY CHANGES NEEDED TO MATCH THE TYPE OF CONDUITS OTHER THAN FNMC.

2**************************************************************************************2

1**************************************************************************************************************************1

704.03.02  Camera Surveillance System (CSS)

1**************************************************************************************************************************1

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

1**************************************************************************************************************************1

704.03.03  Fiber Optic Cable

1**************************************************************************************************************************1

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

1**************************************************************************************************************************1

704.03.04  Controlled Traffic Signal System (CTSS)

1**************************************************************************************************************************1

2**************************************************************************************2

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

2**************************************************************************************2

2**************************************************************************************2

G.	Warranty.

cONTACT the local aid project manager for information from the mse’s aam unit for any specific warranty requirements and modify the above TO meet their requirements and insert here

2**************************************************************************************2

2**************************************************************************************2

H.	Networking Requirements.

contact the local aid project manager  to provide additional networking requirements from njoit

2**************************************************************************************2

2**************************************************************************************2

I.	IT Requirements.

provide additional IT requirements

2**************************************************************************************2

1**************************************************************************************************************************1

704.03.05  Travel Time Systems (TTS)

1**************************************************************************************************************************1

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

1**************************************************************************************************************************1

704.03.06  Road Weather Information System (RWIS)

1**************************************************************************************************************************1

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

1**************************************************************************************************************************1

704.03.07  Dynamic Message System (DMS)

1**************************************************************************************************************************1

2**************************************************************************************2

A.	Components.

complete and include the following WHEN dms signs are included in the project.

also, include THE APPROPRIATE notes and INFORMATION FOR EACH DMS PROPOSED IN THE CONTRACT.

The following are the Model numbers for the various DMS to be provided and installed in this project:

| Location | Communication Type | DMS Type | Manufacturer/Model No. |
| --- | --- | --- | --- |

Ensure that the designated Model numbers for the various DMS signs are provided as specified in the Contract documents.

Ensure that Controller, DMS is purchased with pre-installed controller, pre-wired with the equipment listed below along with specialized communications cables (minimum 120 feet Fiber Optic Cable with Connectors for each sign).

As part of the specified model numbers, ensure the DMS manufacturer supplies the cabinet and controller for each DMS sign with pre-installed uninterruptable power supply (UPS), a media converter, and a TCP/IP wireless modem conforming to the wireless provider requirements.  Provide other equipment not listed here but required for the remote operation of the DMS.

2**************************************************************************************2

2**************************************************************************************2

B.	Installation.

3************************************************3

Construct the DMS sign mounting structure and foundation as specified in Division 500.

DMS STRUCTURE REQUIRES THE CONTRACTOR TO follow DIVISION 500 specifications FOR STRUCTURAL DETAILS AND other REQUIREMENTS. INCLUDING FOUNDATION.

3************************************************3

3************************************************3

4*************************4

1.	DMS Sign.

THIS ITEM IS FOR DMS SIGNS OF VARIOUS types and SIZES WHEN PROPOSED TO BE PROVIDED AND INSTALLED BY THE CONTRACTOR. INCLUDE THE MAKE AND MODEL NUMBER OF DMS SIGN in section 918 AND in the table in subsection 704.03.07.a. also, include it IN THE ITS PLANS. CONTACT MSE FOR FURTHER DETAILS. WHEN THIS ITEM IS USED IN A PROJECT, “CONTROLLER DMS” AND “FOUNDATION ITS TYPE D” OR “FOUNDATION ITS TYPE D-MC” AS APPLICABLE WILL BE REQUIRED FOR CONTROLLER/METER CABINET MOUNTING.

4*************************4

4*************************4

2.	Controller, DMS.

thIS ITEM is required when installing new dms signs and must be purchased from the dms sign manufacturer. PROVIDE THE MODEL NUMBER OF the dms SIGNS to the manufacturer for which controller dms are required. ADDITIONAL PAY ITEMS LIKE “FOUNDATION ITS TYPE D” OR “FOUNDATION ITS TYPE D-MC” FOR MOUNTING CONTROLLER/METER CABINETS AS APPLICABLE WILL BE REQUIRED.

4*************************4

3************************************************3

2**************************************************************************************2

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

2**************************************************************************************2

1**************************************************************************************************************************1

704.03.08  Weigh-in-Motion System (WIMS)

1**************************************************************************************************************************1

2**************************************************************************************2

B.	Installation.

cONTACT the local aid project manager  to confirm with TRAFFIC OPERATIONS  THE EXISTING SYSTEM SHUTDOWN TIME. REVISE AS DIRECTED and insert here

Provide the cost of damages for exceeding the allowable time frames

2**************************************************************************************2

2**************************************************************************************2

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

2**************************************************************************************2

1**************************************************************************************************************************1

704.03.09  Traffic Volume System (TVS)

1**************************************************************************************************************************1

B.	Installation.

2**************************************************************************************2

complete and include the following.

INsert THE existing system shutdown time frames, including days of the week, specific dates, and/or hours of the day(s).

The Department will allow existing TVS system shutdowns from _____.

Provide the cost of damages for exceeding the allowable time frames

2**************************************************************************************2

2**************************************************************************************2

F.	Equipment Training.

provide recommendation prior to final design submission for any special training, and/or if more than 10 personnel require the training.

2**************************************************************************************2

1**************************************************************************************************************************1

1**************************************************************************************************************************1

704.03.10  Variable Speed Limit System (VSLS)

B.	Installation.

2.	Controller, VSLS.

thIS ITEM is required when installing new VSLs signs and must be purchased from the VSLS sign manufacturer. PROVIDE THE MODEL NUMBER OF the VSLs SIGNS to the manufacturer for which controller VSLs ARE required. ADDITIONAL PAY ITEMS FOR MOUNTING CONTROLLER/METER CABINETS AS APPLICABLE WILL BE REQUIRED.

1**************************************************************************************************************************1

704.04  Measurement and Payment

1**************************************************************************************************************************1

contact the local aid project manager to obtain the latest revised standard detail SHEETS that are available from NJDOT Mobility and Systems Engineering (MSE) for inclusion in the contract plans until they are issued via bdc.

1**************************************************************************************************************************1

Division 800 – Landscaping

Section 811 – Planting

811.03.02  Plant Establishment and Maintenance Period

1**************************************************************************************************************************1

2**************************************************************************************2

complete and include THE FOLLOWING

The Department will reinspect the plants annually for _____ years.

2**************************************************************************************2

2.	Maintenance Bond.

2**************************************************************************************2

complete and include the bond amount (a percentage of planting value)

Provide a bond to the Department in the amount of $_____________.

2**************************************************************************************2

1**************************************************************************************************************************1

Division 900 – Materials

Section 901 – Aggregates

901.01  Source

1**************************************************************************************************************************1

BDC25S-25 dated MAR 2, 2026

THE FOLLOWING IS ADDED AFTER THE SECOND PARAGRAPH:

Do not source aggregates from other Department projects unless approved by the ME.  Ensure that aggregates sourced from other Department projects or non-QPL sites are not classified as regulated or hazardous according to NJDEP Solid Waste Regulations, N.J.A.C. 7:26-1 et seq. or N.J.A.C 7:26 E-1-8.  Provide test results to the Department upon request.

1**************************************************************************************************************************1

901.11  Soil Aggregate

1**************************************************************************************************************************1

BDC25S-25 dated MAR 2, 2026

THE SECOND PARAGRAPH is changed to:

The Contractor may use soil aggregate produced from RPCSA produced by a NJDEP approved “Class B” recycling center operating pursuant to N.J.A.C. 7:26A. Ensure soil aggregates are not comprised of material designated as regulated or hazardous as specified in 901.01.

1**************************************************************************************************************************1

Section 902 – Asphalt

902.01.01 Asphalt Binder

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

THE FIRST TWO PARAGRAPHS ARE CHANGED TO:

Use an asphalt binder that is storage-stable and conforms to AASHTO M 332.  Include compliance with the elastic response requirement in Figure 1 of AASHTO R 92, if applicable.

1**************************************************************************************************************************1

902.01.04  Warm Mix Asphalt (WMA) Additives and Processes

1**************************************************************************************************************************1

BDC24S-26 dated Mar 3, 2025

THE FIRST PARAGRAPH IS CHANGED TO:

Use a WMA additive or process that is listed on the QPL.

1**************************************************************************************************************************1

902.02.01  Mix Designations

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

PART (4) IS CHANGED TO:

4.	“E”  The fourth field in the Item description designates the high temperature designation of the performance-graded binder.  Options are “64” for PG 64S-22 and “E” for PG 64E-22.

1**************************************************************************************************************************1

INCLUDE THE FOLLOWING FOR LOCAL AID PROJECTS

902.02.03 Mix Design

THE FOLLOWING IS ADDED TO THE FIRST PARAGRAPH:

Unless otherwise approved by the engineer, only one source of supply for hot mix asphalt surface course may be used on the project.

2**************************************************************************************2

902.02.04 Sampling and Testing

A. General Acceptance Requirements.

2**************************************************************************************2

BDC22S-04 DATED MAY 13, 2022

THE SECOND PARAGRAPH OF SECTION (A) IS CHANGED TO:

For PG 64S-22, ensure that the temperature of the mixture at discharge from the plant or surge and storage bins is at least 290 °F when the ambient temperature is less than 50 °F or is at least 275 °F when the ambient temperature is greater than or equal to 50 °F. For PG 64E-22, ensure that the temperature of the mixture at discharge from the plant or surge and storage bins is at least 10 °F above the manufacturer’s recommended laydown temperature. For mixes produced using a WMA additive or process, ensure that the temperature of the mixture at discharge from the plant or surge and storage bins is at least 10 °F above the WMA manufacturer’s recommended laydown temperature.

2**************************************************************************************2

D. Acceptance Testing and Requirements.

1***********************************************************************************************1

BDC24S-12 DATED JUL 30, 2024

PART D IS CHANGED TO:

The ME will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant. The ME will compact HMA to the number of design gyrations (Ndes) specified in Table 902.02.03-2, using equipment according to AASHTO T 312. The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166. The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the HMA.

The ME will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

Ensure that the HMA mixture conforms to the requirements specified in Table 902.02.04-1, and to the gradation requirements in Table 902.02.03-1. If the test results are outside of the gradation or volumetric requirements specified in Table 902.02.03-1 or Table 902.02.04-1 for an acceptance sample, immediately run a quality control sample. If the quality control sample is also outside of the requirements specified in Table 902.02.03-1 or Table 902.02.04-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance. Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the requirements. If the mix is within the requirements based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meet gradation and volumetric requirements specified in Table 902.02.03-1 and Table 902.02.04-1. If 2 consecutive acceptance or quality control samples are outside the gradation or volumetric requirements specified in Table 902.02.03-1 or Table 902.02.04-1, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production. Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets gradation and volumetric requirements specified in Table 902.02.03-1 and Table 902.02.04-1 and ME approval. The ME will reject mixture produced at initial restarting that does not meet gradation and volumetric requirements specified in Table 902.02.03-1 and Table 902.02.04-1.

The ME will test a minimum of 1 sample per 3,500 tons for moisture, basing moisture determinations on the weight loss of an approximately 1,600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

| Table 902.02.04-1 Hot Mix Asphalt Requirements for Control |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compaction Levels | Required Density (% of Theoretical Max. Specific Gravity) | Voids in Mineral Aggregate (VMA), % (minimum) |  |  |  |  |  | Dust-to-Binder Ratio |
|  |  | Nominal Max. Aggregate Size, mm |  |  |  |  |  |  |
|  | @Ndes1 | 37.5 | 25.0 | 19.0 | 12.5 | 9.5 | 4.75 |  |
| L, M | 95.0 – 97.0 | 11.0 | 12.0 | 13.0 | 14.0 | 15.0 | 16.0 | 0.6 – 1.3 |
| 1. As determined from the values for the maximum specific gravity of the mix and the bulk specific gravity of the compacted mixture. Maximum specific gravity of the mix is determined according to AASHTO T 209. Bulk specific gravity of the compacted mixture is determined according to AASHTO T 166. |  |  |  |  |  |  |  |  |

1**************************************************************************************************************************1

2**************************************************************************************2

DETERMINATION OF CONFORMANCE TO THE VOLUMETRIC PROPERTIES BY SAMPLING AND TESTING AT THE HMA PLANT BY AN INDEPENDENT TESTING AGENCY AND/OR LABORATORY IS PREFERRED; HOWEVER, THE FOLLOWING CHANGES TO SUBSECTION 902.02.04 MAY BE USED AS AN ALTERNATE TO THE SAMPLING AND TESTING PROVISIONS LISTED IN SUBSECTION 902.02.04 TO DETERMINE CONFORMANCE TO THE SPECIFICATION REQUIREMENTS.

INCLUDE THE FOLLOWING FOR LOCAL AID PROJECTS

902.02.04  Sampling and Testing

THE FOLLOWING SUBSECTION IS ADDED:

E. Acceptance of HMA. The Department may accept the HMA as specified in 902.02.04.A through 902.02.04.E by employing staff or an independent testing agency at the HMA plant during production. The inspector who performs the quality assurance sampling shall be certified by the Society of Asphalt Technologists of New Jersey as an Asphalt Plant Technologist, Level 2. Form “DS-8 HMA Testing Summary Report – State Aid” provided on the Local Aid Website must be utilized by the Laboratory to report their findings to the RE.

Alternatively, the Department may accept the HMA by Certification of Compliance according to 106.07.

902.03 OPEN-Graded Friction Course (OGFC) and Modified Open-Graded Friction Course

902.03.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

The entire subpart is changed to:

Ensure that the mix meets the requirements as specified in 902.02.04.A, otherwise the RE or ME will reject the material.  Maintain the temperature of the mix between 300 °F and 330 °F.  Perform and meet requirements for quality control testing as specified in 902.02.04.C.

Ensure that a technical representative from the lab which designed the mix is present during the first night of production to make adjustments as needed for mix compliance.  During production, the ME will take one random acceptance sample from each 700 tons of production to verify composition.  Conduct draindown tests as directed by the ME.

If the composition testing results are outside of the production control tolerances specified in Table 902.04.02-1 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances in Table 902.04.02-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the production control tolerances.  If the mix is within tolerance based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the production control tolerances specified in Table 902.04.02-1.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.04.02-1, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

The ME will perform sampling according to NJDOT B-2 or ASTM D 3665, and will perform testing for composition according to AASHTO T 308.  Perform testing for draindown according to AASHTO T 305 for every 3,500 tons or as directed by the ME.  The ME may require testing and calculations of film thickness according to NJDOT B-13.  The ME may require adjustment or redesign of the UTFC for failure of draindown or film thickness based on the requirements in Table 902.04.02-2.  During production at the plant, the ME will take a sample of the asphalt binder once every 3,500 tons or as directed by the ME.

| Table 902.03.03-1 JMF Master Ranges and Mixture Requirements Open-graded Friction Course |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mixture Designations (% Passing1) |  |  |  |  |  |  |  |
| Sieve Sizes | OGFC – 9.5 mm |  | MOGFC – 12.5 mm |  | MOGFC – 9.5 mm |  |
| 3/4" | – | 100 |  | – |  |
| 1/2" | 100 | 85 – 100 |  | 100 |  |
| 3/8" | 80 – 100 | 35 – 60 |  | 85 – 100 |  |
| No. 4 | 30 – 50 | 10 – 25 |  | 20 – 40 |  |
| No. 8 | 5 – 15 | 5 – 10 |  | 5 – 10 |  |
| No. 200 | 2.0 – 5.0 | 2.0 – 5.0 |  | 2.0 – 4.0 |  |
| Minimum asphalt binder, %1 | 5.5 | 5.7 |  | 6.0 |  |
| Minimum % Air Voids, design | 15% | 20% |  | 18% |  |
| Minimum lift thickness, design | 3/4" | 1 1/4" |  | 3/4" |  |
| 1. Aggregate percent passing to be determined based on dry aggregate weight. Asphalt binder content to be determined based on total weight of mix. |  |  |  |  |  |  |  |

| Table 902.03.03-2 Production Control Tolerances for OGFC and MOGFC Mixtures |  |
| --- | --- |
| Sieve Sizes | Production Control Tolerances from JMF1 |
| 1/2" | 3.0 |
| 3/8" | 4.0 |
| No. 4 | 3.0 |
| No. 8 | 1.0 |
| No. 200 | 1.0 |
| Asphalt Binder Content, % (AASHTO T 308)2 | 0.40 |
| Asphalt Binder Content, % (NJDOT B-5)2 | 0.15 |
| Minimum % Air Voids | 1.0% less than design requirement |
| 1. Production tolerances may not fall outside of the wide band gradation limits in Table 902.03.03-1. 2. The asphalt binder content may not be lower than the minimum after the production tolerance is applied. |  |

1**************************************************************************************************************************1

902.04  Ultra-Thin HMA

902.04.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

The entire subpart is changed to:

Ensure that the mix meets the requirements as specified in 902.02.04.A, otherwise the RE or ME will reject the material.  Maintain the temperature of the mix between 300 °F and 330 °F.  Perform and meet requirements for quality control testing as specified in 902.02.04.C.

Ensure that a technical representative from the lab which designed the mix is present during the first night of production to make adjustments as needed for mix compliance.  During production, the ME will take one random acceptance sample from each 700 tons of production to verify composition.  Conduct draindown tests as directed by the ME.

If the composition testing results are outside of the production control tolerances specified in Table 902.04.02-1 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances in Table 902.04.02-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the production control tolerances.  If the mix is within tolerance based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the production control tolerances specified in Table 902.04.02-1.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.04.02-1, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

The ME will perform sampling according to NJDOT B-2 or ASTM D 3665, and will perform testing for composition according to AASHTO T 308.  Perform testing for draindown according to AASHTO T 305 for every 3,500 tons or as directed by the ME.  The ME may require testing and calculations of film thickness according to NJDOT B-13.  The ME may require adjustment or redesign of the UTFC for failure of draindown or film thickness based on the requirements in Table 902.04.02-2.  During production at the plant, the ME will take a sample of the asphalt binder once every 3,500 tons or as directed by the ME.

1**************************************************************************************************************************1

902.05  Stone Matrix Asphalt (SMA)

1**************************************************************************************************************************1

BDC21s-14 dated DEC 30, 2021

902.05.01  Composition of Mixture

2**************************************************************************************2

BDC24S-06 dated May 28, 2024

The SECOND paragraph is changed to:

The composition of the SMA mixture is coarse aggregate, fine aggregate, mineral filler, mineral fibers or cellulose fibers, and polymer modified asphalt binder and may include a WMA additive.  If the supplier utilizes a Fiberless SMA, the mineral or cellulose fibers may be removed and must utilize a WMA additive to properly reduce production temperature.

2**************************************************************************************2

THE FIFTH PARAGRAPH IS CHANGED TO:

For fine aggregate, use 100 percent manufactured stone sand conforming to 901.05.02.  Ensure that the combined fine aggregate of 100 percent manufactured stone sand in the mixture conforms to the requirements in Table 902.02.02-2.

2**************************************************************************************2

BDC24S-06 dated May 28, 2024

The NINTH paragraph is changed to:

If the asphalt supplier is to utilize a Fiberless SMA, the supplier must provide proper mixture design documentation, including a graph indicating the appropriate production temperature range where both draindown and air voids meet the material specifications.  This graph should evaluate draindown and air voids using at least four temperature data points.  If used, ensure that WMA additives or processes conform to 902.01.04.  If a WMA additive is pre-blended in the asphalt binder, ensure that the asphalt binder meets the requirements of the specified grade after the addition of the WMA additive.  If a WMA additive is added at the HMA plant, ensure that the addition of the additive will not negatively impact the grade of asphalt binder.  Follow the manufacturer’s recommendations for percentage of WMA additive needed.  Do not use a controlled asphalt foaming system WMA for Fiberless SMA.

2**************************************************************************************2

902.05.02  Mix Design

TABLE 902.05.02-1 AND TABLE 902.05.02-2 ARE CHANGED TO:

| Table 902.05.02-1 SMA Specification Band (% Passing) Nominal-Maximum Aggregate Size |  |  |  |  |
| --- | --- | --- | --- | --- |
| Production Control Tolerances from JMF1 | Sieve Size | 19 mm % Passing | 12.5 mm % Passing | 9.5 mm % Passing |
| 0% | 1" | 100 | 100 | 100 |
| ±3% | 3/4" | 90 – 100 | 100 | 100 |
| ±5% | 1/2" | 50 – 88 | 90 – 100 | 100 |
| ±5% | 3/8" | 25 – 60 | 50 – 80 | 70 – 95 |
| ±6% | No. 4 | 20 – 28 | 20 – 35 | 30 – 50 |
| ±3% | No. 8 | 16 – 24 | 16 – 24 | 20 – 30 |
| ±4% | No. 16 | – | – | 0 – 21 |
| ±3% | No. 30 | – | – | 0 – 18 |
| ±3% | No. 50 | – | – | 0 – 15 |
| ±2% | No. 200 | 7.0 – 11.0 | 7.0 – 11.0 | 7.0 – 12.0 |
| – | Coarse Aggregate Fraction | Portion Retained on No. 4 Sieve | Portion retained on No. 4 Sieve | Portion retained on No. 8 Sieve |
| 1. Production tolerances are for the approved JMF and may not fall outside of the wide band gradation limits. |  |  |  |  |

| Table 902.05.02-2 SMA Mixtures Volumetrics for Design and Plant Production |  |  |
| --- | --- | --- |
| Property | Production Control Tolerances | Requirement |
| Air Voids | ±1% | 3.5% |
| Voids in Mineral Aggregate (VMA) | – | 17.0% minimum |
| VCAmix | – | Less than VCADRC |
| Draindown @ production temperature1 | – | 0.30% maximum |
| Asphalt Binder Content (AASHTO T 308)2 | ±0.40% | 6% minimum3 |
| Tensile Strength Ratio (AASHTO T 283) | – | 80% minimum |
| 1. For design, conduct draindown test at anticipated mixing temperature and 15°F higher. Ensure draindown test meets requirement at both temperatures. For production, conduct draindown test at 15°F higher than anticipated mixing temperature. 2. Asphalt binder content may not be lower than the minimum after the production tolerance is applied. 3. Aggregate blends with a Gsb value of 2.90 and higher may produce the mix with a minimum asphalt content of 5.8% with ME approval. |  |  |

902.05.03  Sampling and Testing

THE ENTIRE SUBSECTION IS CHANGED TO:

A.	General Acceptance Requirements.  Ensure that the mix meets the requirements as specified in 902.02.04.A.

2**************************************************************************************2

BDC24S-06 dated May 28, 2024

Part b is changed to:

B.	Sampling.  The ME will take a random sample from each 700 tons of production for volumetric acceptance testing and to verify composition.  The ME will perform sampling according to AASHTO T 168, NJDOT B-2, or ASTM D 3665.  During production at the plant, a sample of asphalt binder will be taken once every 3,500 tons or as directed by the ME.  If a WMA additive is added at the HMA plant, asphalt binder sample will be taken after in-line blending.

2**************************************************************************************2

C.	Quality Control Testing.  Perform quality control testing as specified in 902.02.04.C.

For each acceptance test, perform testing for draindown according to AASHTO T 305 at 15°F above the mixing temperature.

Perform bulk specific gravity of coarse aggregates (GCA) and dry-rodded voids in coarse aggregate VCADRC testing according to AASHTO R 46 by sampling and drying aggregates according to AASHTO R 90 and AASHTO T 255, respectively.  The ME shall ensure that the GCA and VCADRC of the aggregate blend has been determined within 7 days of the start of production.  Stop production and conduct VCADRC test immediately when there are major changes to aggregate blend percentages.  When performing the VCADRC test during production, use the GCA value that was determined prior to production.  The ME may request additional VCADRC tests to be conducted with or without requiring a new GCA test to be performed at any time.

D.	Acceptance Testing and Requirements.  The ME will determine volumetric properties at 75 gyrations for acceptance from samples taken, compacted, and tested at the HMA plant according to AASHTO T 312.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the QC maximum specific gravity test result in calculating the volumetric properties of the SMA.  The ME will determine VCAmix according to AASHTO R 46.

If the acceptance sample is outside of the control tolerances for the No. 4 sieve in Table 902.05.02-1 or VCAmix is greater than VCADRC, immediately take a quality control sample for testing.  If the quality control sample is outside of the control tolerances for the No. 4 sieve in Table 902.05.02-1 or VCAmix is greater than VCADRC, immediately stop production and shipping.

If the test results other than the No. 4 sieve or VCADRC are outside of the production control tolerances specified in Table 902.05.02-1 or Table 902.05.02-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances in Table 902.05.02-1 or Table 902.05.02-2, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within tolerances.  If the mix is within tolerance based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition, air voids, draindown, VCAmix, and VMA meet the production control tolerances specified in Table 902.05.02-1 and Table 902.05.02-2.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.05.02-1 or Table 902.05.02-2, immediately stop production and shipping.

1**************************************************************************************************************************1

2**************************************************************************************2

BDC24S-12 dated Jul 30, 2024

The fourth paragraph in Part D is changed to:

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

2**************************************************************************************2

1**************************************************************************************************************************

1**************************************************************************************************************************1

902.06  Asphalt-Stabilized Drainage Course (ASDC)

902.06.01  Composition

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

THE SECOND PARAGRAPH IS CHANGED TO:

The mixture shall consist of asphalt binder and aggregate and may contain a WMA additive.  Use asphalt binder that is PG 64S-22 as specified in 902.01.01.  Use aggregate that conforms to 901.05.01 or 901.05.02 and the gradation requirements specified in Table 902.06.01-1.

1**************************************************************************************************************************1

902.06.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

The entire subpart is changed to:

Perform quality control testing as specified in 902.02.04.C.  Ensure that the mix meets the requirements as specified in 902.02.04.A, except that the temperature of the mix at discharge is required to be between 230 °F and 275 °F, otherwise the RE or ME will reject the material.  For mixes produced using a WMA additive or process, ensure that the temperature of the mixture at discharge from the plant or surge and storage bins is at least 10 °F above the WMA manufacturer’s recommended laydown temperature.

During production, the ME will take 1 random acceptance sample from each 700 tons of production to verify composition.  Conduct draindown tests as directed by the ME.

If the composition testing results are outside of the production control tolerances specified in Table 902.06.01-1 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances specified in Table 902.06.01-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within tolerances.  If the mix is within tolerance based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the production control tolerances specified in Table 902.06.01-1.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.06.01-1, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

The ME will perform sampling according to NJDOT B-2 or ASTM D 3665 and will perform testing for composition according to AASHTO T 308.  If directed by the ME, perform testing for draindown according to AASHTO T 305.  During production at the plant, a sample of asphalt binder will be taken once every 3,500 tons or as directed by the ME.

1**************************************************************************************************************************1

902.07.02  Asphalt-Rubber Binder

1**************************************************************************************************************************1

BDC22S-04 dated MAY 13, 2022

PART (2) SUBSECTION (a) IS CHANGED TO:

2.	Asphalt Binder.

a.	Use asphalt binder that conforms to AASHTO M 332, Table 1; PG 64S-22, PG 58-28 or an approved blend of both grades.  The asphalt binder producer is required to provide the asphalt binder quality control plan annually to the ME for approval.  Ensure that the quality control plan conforms to AASHTO R 26.  Submit to the ME a certification of compliance, as specified in 106.07, for the asphalt binder.  The ME will perform quality assurance sampling and testing of each asphalt binder lot as defined in the approved quality control plan.

1**************************************************************************************************************************1

902.07.04  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

C.	Acceptance Testing.

part c is changed to:

During production, the ME will take one random acceptance sample from each 700 tons of production to verify composition.  The ME will perform sampling according to NJDOT B-2 or ASTM D 3665 and will perform testing for composition according to AASHTO T 308.  Perform testing for air voids according to T 209 and either B-6 or T 331.  Perform testing for draindown according to NJDOT B-8.  During production at the plant, a sample of asphalt binder will be taken once every 3,500 tons or as directed by the ME.

Conduct air voids and draindown tests as directed by the ME.

If the composition testing results are outside of the production control tolerances specified in Table 902.07.04-1 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances in Table 902.07.04-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the production control tolerances.  If the mix is within tolerance based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the production control tolerances specified in Table 902.07.04-1.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.07.04-1, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

| Table 902.07.04-1 Production Control Tolerances for AR-OGFC Mixtures |  |
| --- | --- |
| Sieve Sizes | Production Control Tolerances from JMF1 |
| 1/2" | 6.0 |
| 3/8" | 2.0 |
| No. 4 | 4.0 |
| No. 8 | 1.0 |
| No. 200 | 1.0 |
| Asphalt-rubber binder, % (AASHTO T 308)2 | 0.40 |
| Minimum % Air Voids | 1.0% less than design requirement |
| 1. Production tolerances may fall outside of the wide band gradation limits in Table 902.07.03-1. 2. Asphalt-rubber binder content may not be lower than the minimum in Table 902.07.03-1 after the production tolerance is applied. |  |

1**************************************************************************************************************************1

902.08  High Performance Thin Overlay (HPTO)

902.08.02  Mix Design

1**************************************************************************************************************************1

BDC25S-02 dated Mar 27, 2025

the subpart is changed to:

At least 45 days before initial production, submit a job mix formula for the HPTO on forms supplied by the Department.  Include a statement naming the source of each component and a report showing the results meet the criteria specified in Table 902.08.02-1 and Table 902.08.02-2.

For the job mix formula for the HPTO mixture, establish the percentage of dry weight of aggregate passing each required sieve size and an optimum percentage of asphalt binder based upon the weight of the total mix.  Determine the optimum percentage of asphalt binder according to AASHTO R 35 and M 323 with an Ndes of 50 gyrations.  Before maximum specific gravity testing or compaction of specimens, condition the mix for 2 hours according to the requirements for conditioning for volumetric mix design in AASHTO R 30, Section 7.1.  If the absorption of the combined aggregate is more than 1.5 percent according to AASHTO T 84 and T 85, condition the mix for 4 hours according to AASHTO R 30, Section 7.2 prior to compaction of specimens (AASHTO T 312) and determination of maximum specific gravity (AASHTO T 209).  Ensure that the job mix formula is within the master range specified in Table 902.08.02-1.

| Table 902.08.02-1 JMF Requirements for HPTO |  |  |
| --- | --- | --- |
| Sieve Sizes | Percent Passing1 | Production Control Tolerances2 |
| 3/8" | 100 | ±0.0% |
| No. 4 | 65 – 85 | ±4.0% |
| No. 8 | 30 - 45 | ±4.0% |
| No. 16 | 20 – 35 | ±3.0% |
| No. 30 | 15 – 30 | ±3.0% |
| No. 50 | 10 – 20 | ±2.0% |
| No. 100 | 5 – 15 | ±2.0% |
| No. 200 | 5.0 – 8.0 | ±1.0% |
| Asphalt Binder Content (Ignition Oven) |  | ±0.40% |
| 1. Aggregate percent passing to be determined based on dry aggregate weight. 2. Production tolerances are applied to the approved JMF for gradation and asphalt binder content. Gradation results may not fall outside of the wide band gradation limits when tolerances are applied. |  |  |

Design the HPTO to meet the requirements in Table 902.08.02-2.

| Table 902.08.02-2 Volumetric Requirements for Design and Control of HPTO |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Required Density (% of Max Sp. Gr.) |  | Voids in Mineral Aggregate | Dust-to-Binder Ratio | Draindown1 AASHTO T 305 |
|  | @ Ndes (50 gyrations) | @ Nmax (100 gyrations) | (VMA) |  |  |
| Design Requirements | 96.0 | ≤ 99.0 | ≥ 17.0 % | 0.6 – 1.2 | ≤ 0.1 % |
| Control Requirements | 95.0 – 97.0 | ≤ 99.0 | ≥ 17.0 % | 0.6 – 1.3 | ≤ 0.1 % |
| 1. Draindown testing is at the discretion of the ME. |  |  |  |  |  |

Ensure that the job mix formula provides a mixture that meets a minimum tensile strength ratio (TSR) of 85 percent when prepared according to AASHTO T 312 and tested according to AASHTO T 283 with the following exceptions:

1.	Before compaction, condition the mixture for 2 hours according to AASHTO R 30 Section 7.1.

2.	Compact specimens with 40 gyrations.

3.	Extrude specimens as soon as possible without damaging.

4.	Use AASHTO T 269 to determine void content.

5.	Record the void content of the specimens.

6.	If less than 55 percent saturation is achieved, the procedure does not need to be repeated, unless the difference in tensile strength between duplicate specimens is greater than 25 pounds per square inch.

7.	If visual stripping is detected, modify or readjust the mix.

For each mix design, submit 3 gyratory specimens and 1 loose sample corresponding to the composition of the job mix formula, including the design asphalt content.  The ME will use these samples for verification of the properties of the job mix formula.  Compact the specimens to the design number of gyrations (Ndes).  To be acceptable all 3 gyratory specimens must comply with the gradation requirements in Table 902.08.02-1 and with the design requirements in 
Table 902.08.02-2.  The ME reserves the right to be present at the time of molding the gyratory specimens.

In addition, submit 11 gyratory specimens and 2 boxes of loose mix to the ME.  The ME will use these additional gyratory samples for performance testing of the HPTO mix.  The ME reserves the right to be present at the time of molding the gyratory specimens.  Ensure that the additional gyratory specimens are compacted according to AASHTO T 312.  Compact 6 of the specimens to 77 millimeters in height and an air void content of 5.0 ± 0.5 percent.  The ME will test the six 77 millimeter specimens using an Asphalt Pavement Analyzer (APA) according to AASHTO T 340 at 64 °C, 100 pound per square inch hose pressure, and 100 pound wheel load.  Compact the other 5 specimens to 115 millimeter in height.  These 5 specimens will be cut, from the middle of each 115 millimeter in height specimen, to 38 millimeter in height test specimens.  The air void content of the 5 cut specimen will be determined to ensure compliance with the target air void content of 5.0 ± 0.5 percent.  The ME will use the five 38 millimeter in height specimens to test using an Overlay Tester (NJDOT B-10) at 25 °C and a joint opening of 0.025 inch.  The ME will eliminate the high and low Overlay test results then average and report the middle 3 test results.  The ME will ensure that all submitted specimens are within the target air void content as tested at the Materials’ Central Lab.  The ME will not accept specimens lower than the target air void content, but may accept and test specimens higher than the target air void content.

The ME will approve the JMF if the average rut depth for the 6 specimens in the Asphalt Pavement Analyzer testing is not more than 3.0 millimeters at 8,000 loading cycles and the average number of cycles to failure in the Overlay Tester is not less than 1,200.  If the JMF does not meet the APA and Overlay Tester criteria, redesign the HPTO mix and submit for retesting.  The JMF for the HPTO mixture is in effect until modification is approved by the ME.

When unsatisfactory results for any specified characteristic of the work make it necessary, the Contractor may establish a new JMF for approval.  In such instances, if corrective action is not taken, the ME may require an appropriate adjustment to the JMF.

Should a change in sources or changes in the properties of materials occur, the ME will require that a new JMF be established and approved before production can continue.

The ME may verify a mix on an annual basis rather than on a project-to-project basis if the properties and proportions of the materials do not change.  If written verification is submitted by the HMA supplier that the same source and character of materials are to be used, the ME may waive the requirement for the design and verification of previously approved mixes.

1**************************************************************************************************************************1

902.08.03  Sampling and Testing

1**************************************************************************************************************************1

BDC25S-02 dated mar 27, 2025

part b is changed to:

B.	Sampling.  The ME will take a sample of HPTO for volumetric acceptance testing from each 700 tons of a mix.  The ME will perform sampling according to AASHTO T 168, NJDOT B-2, or ASTM D 3665.  During production at the plant, a sample of asphalt binder will be taken once every 1,400 tons or as directed by the ME.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

D.	Acceptance Testing and Requirements.

Part D is changed to:

The ME will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant.  The ME will compact HPTO to 50 gyrations, using equipment according to AASHTO T 312.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the HPTO.

The ME will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

2**************************************************************************************2

BDC25S-02 dated mar 27, 2025

the third paragraph of part d is changed to:

Ensure that the HMA mixture conforms to the requirements specified in Table 902.08.02-2, and to the gradation and asphalt content requirements in Table 902.08.02-1.  If the test results are outside of the requirements specified in Table 902.08.02-1 or Table 902.08.02-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control requirements specified in Table 902.08.02-1 or Table 902.08.02-2, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the requirements.  If the mix is within the requirements based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the requirements specified in Table 902.08.02-1 and Table 902.08.02-2.  If 2 consecutive acceptance or quality control samples are outside the requirements specified in Table 902.8.02-1 or Table 902.08.02-2, immediately stop production and shipping.

2**************************************************************************************2

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets the requirements specified in Table 902.08.02-1 and Table 902.08.02-2 and ME approval.  The ME will reject mixture produced at initial restarting that does not meet the requirements specified in Table 902.08.02-1 and Table 902.08.02-2.

The ME will test a minimum of 1 sample per 3,500 tons for moisture, basing moisture determinations on the weight loss of an approximately 1,600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

1**************************************************************************************************************************1

BDC25S-02 dated mar 27, 2025

E.	Performance Testing.

the fourth paragraph of part e is changed to:

If a sample does not meet the criteria for performance testing as specified in Table 902.08.03-1, the Department will assess a pay adjustment as specified.  The Department will calculate the pay adjustment by multiplying the percent pay adjustment (PPA) by the quantity in the lot and the bid price for the HPTO item.  If APA rutting is greater than 8.0 millimeters or Overlay cycles is less than 400 or both, the Department will assess the maximum pay adjustment of PPA = -100 percent or may require removal and replacement.  PPA for both APA and Overlay are cumulative and may not exceed -100 percent in total.  If samples received are lower than the target air void range, 5.0 ± 0.5 percent, the Department will consider the samples untestable and assess a PPA of -100 percent or may require removal and replacement of the lot.  If the Department requires removal and replacement, then the replacement work is subject to the same requirements as the initial work.

tABLE 902.08.03-1 is changed to:

| Table 902.08.03-1 Performance Testing Pay Adjustments for HPTO |  |  |  |
| --- | --- | --- | --- |
| Test | Requirement | Test Result | PPA |
| APA @ 8,000 loading cycles, mm (AASHTO T 340) | 3.0 maximum | t ≤ 3.0 3.0 < t ≤ 8.0 t > 8.0 | 0 -10(t-3) -100 or Remove & Replace |
| Overlay Tester, cycles (NJDOT B-10) | 1,200 minimum | t ≥ 1,200 1,200 > t ≥ 400 t < 400 | 0 -(1,200-t)/16 -100 or Remove & Replace |

1**************************************************************************************************************************1

1**************************************************************************************************************************1

902.09  Micro Surfacing

1**************************************************************************************************************************1

BDC23S-12 dated oCT 6, 2023

902.09.03  Sampling and Testing

TABLE 902.09.03-1 is changed to:

| Table 902.09.03-1 Gradation Requirements for Aggregate and Mineral Filler |  |  |  |
| --- | --- | --- | --- |
| Sieve Size | Type II Percent Passing | Type III Percent Passing | Stockpile Tolerances from JMF1 |
| 3/8″ | 100 | 100 | – |
| No. 4 | 90 – 100 | 70 – 90 | ±4% |
| No. 8 | 65 – 90 | 45 – 70 | ±5% |
| No. 16 | 45 – 70 | 28 – 50 | ±4% |
| No. 30 | 30 – 50 | 19 – 34 | ±3% |
| No. 50 | 18 – 30 | 12 – 25 | ±3% |
| No. 100 | 10 – 21 | 7 – 18 | ±2% |
| No. 200 | 5 – 15 | 5 – 15 | ±2% |
| 1. Stockpile tolerances are for the approved JMF and may not fall outside of the wide band gradation limits. |  |  |  |

1**************************************************************************************************************************1

902.10  Slurry Seal

1**************************************************************************************************************************1

BDC23S-12 dated oCT 6, 2023

902.10.03  Sampling and Testing

TABLE 902.10.03-1 is changed to:

| Table 902.10.03-1 Gradation Requirements for Aggregate and Mineral Filler |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sieve Size | Type I Percent Passing | Type II Percent Passing | Type III Percent Passing | Stockpile Tolerances from JMF1 |
| 3/8″ | 100 | 100 | 100 | – |
| No. 4 | 100 | 90 – 100 | 70 – 90 | ±5% |
| No. 8 | 90 – 100 | 65 – 90 | 45 – 70 | ±5% |
| No. 16 | 65 – 90 | 45 – 70 | 28 – 50 | ±5% |
| No. 30 | 40 – 65 | 30 – 50 | 19 – 34 | ±5% |
| No. 50 | 25 – 42 | 18 – 30 | 12 – 25 | ±3% |
| No. 100 | 15 – 30 | 10 – 21 | 7 – 18 | ±3% |
| No. 200 | 10 – 20 | 5 – 15 | 5 – 15 | ±2% |
| 1. Stockpile tolerances are for the approved JMF and may not fall outside of the wide band gradation limits. |  |  |  |  |

1**************************************************************************************************************************1

902.11 Binder Rich Intermediate Course (BRIC)

902.11.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

D.	Acceptance Testing and Requirements.

Part D is changed to:

The ME will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant.  The ME will compact HMA to the number of design gyrations (Ndes) of 50 gyrations, using equipment according to AASHTO T 312.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the HMA.

The ME will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

Ensure that the HMA mixture conforms to the requirements specified in Table 902.11.03-2 and to the gradation requirements in Table 902.11.03-1.  If the test results are outside of the requirements specified in Table 902.11.03-1 or Table 902.11.03-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the requirements in Table 902.11.03-1 or Table 902.11.03-2, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the requirements.  If the mix is within the requirements based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the requirements specified in Table 902.11.03-1 and Table 902.11.03-2.  If 2 consecutive acceptance or quality control samples are outside the requirements specified in Table 902.11.03-1 or Table 902.11.03-2, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets the requirements specified in Table 902.11.03-1 and Table 902.11.03-2 and ME approval.  The ME will reject mixture produced at initial restarting that does not meet the requirements specified in Table 902.11.03-1 and Table 902.11.03-2.

The ME will test a minimum of 1 sample per lot for moisture, basing moisture determinations on the weight loss of an approximately 1,600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

1**************************************************************************************************************************1

902.12  Asphalt Rubber Gap Graded Course

902.12.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

The entire subpart is changed to:

Perform quality control testing as specified in 902.02.04.C.  Ensure that the mix meets the requirements as specified in 902.02.04.A, otherwise the RE or ME will reject the material.  Ensure that the temperature of the mixture at discharge from the plant or surge and storage bins meets the WMA additive manufacturer’s recommendations.  Do not allow the mixture temperature to exceed 300 °F at discharge from the plant.

During production at the plant, the ME will take a sample from each 700 tons of production to verify composition and air voids.  Conduct draindown, VCAmix, VCAdry, and VMA testing every 3,500 tons or as directed by the ME.  Perform tests according to AASHTO R 46.

If the testing results are outside of the production control tolerances specified in Table 902.12.02-1 and 
Table 902.12.02-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control tolerances in Table 902.12.02-1, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within tolerances.  If the mix is within tolerances based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the production control tolerances specified in Table 902.12.02-1 and Table 902.12.02-2.  If 2 consecutive acceptance or quality control samples are outside the tolerances specified in Table 902.12.02-1 and Table 902.12.02-2, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets JMF tolerances and ME approval.  The ME will reject mixture produced at initial restarting that does not meet tolerances.

The ME will perform sampling according to NJDOT B-2 or ASTM D 3665, and will perform testing for composition according to AASHTO T 308 or NJDOT B-5.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166. The ME will use the most current QC maximum specific gravity test result, obtained according to AASHTO T 209, in calculating the volumetric properties of the ARGG.  Perform testing for draindown according to AASHTO T 305.  During production at the plant, the ME will take a sample of the asphalt binder once every 3,500 tons or as directed by the ME.

1**************************************************************************************************************************1

902.13. Hot Mix Asphalt High RAP

FOR LOCAL AID PROJECTS THIS SECTION IS DELETED AND REPLACED WITH THE FOLLOWING

This section is deleted and replaced with the following:

902.13  Hot Mix Asphalt High RAP

902.13.01  Mix Designations

The requirements for specific HMA mixtures with required minimum amounts of RAP are identified by the abbreviated fields in the Item description as defined as follows:

HOT MIX ASPHALT 12.5ME SURFACE COURSE HIGH RAP

1.	“HOT MIX ASPHALT”  “Hot Mix Asphalt” is located in the first field in the Item description for the purpose of identifying the mixture requirements.

2.	“12.5”  The second field in the Item description designates the nominal maximum size aggregate (in millimeters) for the job mix formula (sizes are 4.75, 9.5, 12.5, 19, 25, and 37.5 mm).

3.	“M”  The third field in the Item description designates the design compaction level for the job mix formula based on traffic forecasts as listed in Table 902.02.03-2 (levels are L=low and M=medium).

4.	“E”  The fourth field in the Item description designates the high temperature designation of the performance-graded binder.  Options are “64” for PG 64S-22 and “E” for PG 64E-22.

5.	“SURFACE COURSE”  The last field in the Item description designates the intended use and location within the pavement structure (options are surface, intermediate, or base course).

6.	“HIGH RAP”  This additional field designates that there will be a minimum percentage of RAP required for the mixture in 902.13.02.

902.13.02  Composition of Mixture

Provide materials as specified:

Aggregates for Hot Mix Asphalt	901.05

Use a virgin asphalt binder that will result in a mix that meets the performance requirements specified in 
Table 902.13.03-2.  Ensure that the virgin asphalt binder meets the requirements of 902.01.01 except the performance grade.  Use a performance grade of asphalt binder as determined by the mix design and mix performance testing.  Submit a certificate of analysis (COA) showing the PG continuous grading (AASHTO R 29) for the asphalt binder used in the mix design.

For quality assurance testing of the asphalt binder, the ME may sample the asphalt binder during production of the mix and compare the results with the COA submitted during test strip approval.  To analyze the binder the ME will test the binder at the nearest standard PG temperature then compare the results with the COA.  If the high (G*/ sin δ) and low (stiffness and m value) temperature passing test results are within 5 percent of the results from the passing temperature on the COA, then the ME will consider the asphalt binder comparable to the binder used during the test strip.

Mix HMA HIGH RAP in a plant that is listed on the QPL for HMA Plants and conforms to the requirements for HMA Plants as specified in 1009.01.

Composition of the mixture for HMA HIGH RAP surface course is coarse aggregate, fine aggregate, asphalt binder, and greater than 15 percent Reclaimed Asphalt Pavement (RAP), and may also include mineral filler, asphalt rejuvenator, and Warm Mix Asphalt (WMA) additives or processes as specified in 902.01.04.  When WMA is used it must meet the requirements as specified in 902.10.  Ensure that the finished mix does not contain more than a total of 1 percent by weight contamination from Crushed Recycled Container Glass (CRCG).

The composition of the mixture for HMA HIGH RAP base or intermediate course is coarse aggregate, fine aggregate, asphalt binder, and greater than 25 percent Reclaimed Asphalt Pavement (RAP),  and may also include mineral filler, up to 10 percent of additional recycled materials, asphalt rejuvenator, and Warm Mix Asphalt (WMA) additives or processes as specified in 902.01.04.  When WMA is used it must meet the requirements as specified in 902.10.  The recycled materials may consist of a combination of RAP, CRCG, Ground Bituminous Shingle Material (GBSM), and RPCSA, with the following individual limits:

| Table 902.13.02-1 Use of Recycled Materials in Base or Intermediate Course |  |  |
| --- | --- | --- |
| Recycled Material | Minimum Percentage | Maximum Percentage |
| RAP | Greater than 25 | – |
| CRCG | – | 10 |
| GBSM | – | 5 |
| RPCSA | – | 20 |

Combine the aggregates to ensure that the resulting mixture meets the grading requirements specified in 
Table 902.02.03-1.  In determining the percentage of aggregates of the various sizes necessary to meet gradation requirements, exclude the asphalt binder.

Ensure that the combined coarse aggregate, when tested according to ASTM D 4791, has less than 10 percent flat and elongated pieces retained on the No. 4 sieve and larger.  Measure aggregate using the ratio of 5:1, comparing the length (longest dimension) to the thickness (smallest dimension) of the aggregate particles.

Ensure that the combined fine aggregate in the mixture conforms to the requirements specified in Table 902.02.02-2.  Ensure that the material passing the No. 40 sieve is non-plastic when tested according to AASHTO T 90.

902.13.03  Mix Design

At least 45 days before initial production, submit a job mix formula for the HMA HIGH RAP on forms supplied by the Department, to include a statement naming the source of each component and a report showing that the results meet the criteria specified in Table 902.02.03-1 and Table 902.13.03-1.

Include in the mix design the following based on the weight of the total mixture:

1.	Percentage of RAP or GBSM.

2.	Percentage of asphalt binder in the RAP or GBSM.

3.	Percentage of new asphalt binder.

4.	Total percentage of asphalt binder.

5.	Percentage of each type of virgin aggregate.

| Table 902.13.03-1 HMA HIGH RAP Requirements for Design |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compaction Levels | Required Density (% of Theoretical Max. Specific Gravity) |  | Voids in Mineral Aggregate (VMA)2, % (minimum) |  |  |  |  | Voids Filled With Asphalt (VFA) % | Dust-to-Binder Ratio |
|  |  |  | Nominal Max. Aggregate Size, mm |  |  |  |  |  |  |
|  | @Ndes1 | @Nmax | 25.0 | 19.0 | 12.5 | 9.5 | 4.75 |  |  |
| L | 96.0 | ≤ 98.0 | 13.0 | 14.0 | 15.0 | 16.0 | 17.0 | 70 – 85 | 0.6 – 1.2 |
| M | 96.0 | ≤ 98.0 | 13.0 | 14.0 | 15.0 | 16.0 | 17.0 | 65 – 85 | 0.6 – 1.2 |
| 1. As determined from the values for the maximum specific gravity of the mix and the bulk specific gravity of the compacted mixture. Maximum specific gravity of the mix is determined according to AASHTO T 209. Bulk specific gravity of the compacted mixture is determined according to AASHTO T 166. For verification, specimens must be between 95.0 and 97.0 percent of maximum specific gravity at Ndes. 2. For calculation of VMA, use bulk specific gravity of the combined aggregate include aggregate extracted from the RAP. |  |  |  |  |  |  |  |  |  |

The job mix formula for the HMA HIGH RAP mixture establishes the percentage of dry weight of aggregate, including the aggregate from the RAP, passing each required sieve size and an optimum percentage of asphalt binder based upon the weight of the total mix.  Determine the optimum percentage of asphalt binder according to AASHTO R 35 and M 323 with an Ndes as required in Table 902.02.03-2.  Before maximum specific gravity testing or compaction of specimens, condition the mix for 2 hours according to the requirements for conditioning for volumetric mix design in AASHTO R 30, Section 7.1.  If the absorption of the combined aggregate is more than 1.5 percent according to AASHTO T 84 and T 85, ensure that the mix is short term conditioned for 4 hours according to AASHTO R 30, Section 7.2 prior to compaction of specimens (AASHTO T 312) and determination of maximum specific gravity (AASHTO T 209).  Ensure that the job mix formula is within the master range specified in Table 902.02.03-1.

Ensure that the job mix formula provides a mixture that meets a minimum tensile strength ratio (TSR) of 80 percent when prepared according to AASHTO T 312 and tested according to AASHTO T 283.  Submit the TSR results with the mix design.

Determine the correction factor of the mix including the RAP by using extracted aggregate from the RAP in the proposed proportions when testing is done to determine the correction factor as specified in AASHTO T 308.  Use extracted aggregate from the RAP in determining the bulk specific gravity of the aggregate blend for the mix design.

For each mix design, submit with the mix design forms 3 gyratory specimens and 1 loose sample corresponding to the composition of the JMF.  Ensure that the samples include the percentage of RAP that is being proposed for the mix.  The ME will use these to verify the properties of the JMF.  Compact the specimens to the design number of gyrations (Ndes).  For the mix design to be acceptable, all gyratory specimens must comply with the requirements specified in 
Table 902.02.03-1 and Table 902.13.03-1.  The ME reserves the right to be present at the time the gyratory specimens are molded.

In addition, submit 11 gyratory specimens and two 5 gallon buckets of loose mix to the ME.  The ME will use these additional gyratory samples for performance testing of the HMA HIGH RAP mix.  The ME reserves the right to be present at the time of molding the gyratory specimens.  Ensure that the additional gyratory specimens are compacted according to AASHTO T 312.  Compact 6 of the specimens to 77 millimeter height, and have an air void content of 6.5 ± 0.5 percent.  The ME will test 6 specimens using an Asphalt Pavement Analyzer (APA) according to AASHTO T 340 at 64 °C, 100 pound per square inch hose pressure, and 100 pound wheel load.  Compact the other 5 specimens to 115 millimeter height.  These 5 specimens will be cut, from the middle of each 115 millimeter height specimen, to 38 millimeter height test specimens.  The air void content of the 5 cut specimen will be determined to ensure compliance with the target air void content of 6.5 ± 0.5 percent.  The ME will use the five 38 millimeter height specimens to test using an Overlay Tester (NJDOT B-10) at 25 °C and a joint opening of 0.025 inch.  The ME will eliminate the high and low Overlay test results then average and report the middle 3 test results.  The ME will ensure that all submitted specimens are within the target air void content as tested at the Laboratory.

The ME will approve the JMF if the results meet the criteria in Table 902.13.03-2.

| Table 902.13.03-2 Performance Testing Requirements for HMA HIGH RAP Design |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test | Requirement |  |  |  |
|  | Surface Course |  | Intermediate and Base Course |  |
|  | PG 64S-22 | PG 64E-22 | PG 64S-22 | PG 64E-22 |
| APA @ 8,000 loading cycles (AASHTO T 340) | ≤ 7 mm | ≤ 4 mm | ≤ 7 mm | ≤ 4 mm |
| Overlay Tester (NJDOT B-10) | ≥ 200 cycles | ≥ 275 cycles | ≥ 100 cycles | ≥ 150 cycles |

If the JMF does not meet the APA and Overlay Tester criteria, redesign the HMA HIGH RAP mix and submit for retesting.  The JMF for the HMA HIGH RAP mixture is in effect until modification is approved by the ME.

When unsatisfactory results for any specified characteristic of the work make it necessary, the Contractor may establish a new JMF for approval.  In such instances, if corrective action is not taken, the ME may require an appropriate adjustment to the JMF.

Should a change in sources be made or any changes in the properties of materials occur, the ME will require that a new JMF be established and approved before production can continue.

902.13.04  Sampling and Testing

A.	General Acceptance Requirements.  The RE or ME may reject and require disposal of any batch or shipment that is rendered unfit for its intended use due to contamination, segregation, improper temperature, lumps of cold material, or incomplete coating of the aggregate.  For other than improper temperature, visual inspection of the material by the RE or ME is considered sufficient grounds for such rejection.

Ensure that the temperature of the mix at discharge from the plant or storage silo meets the recommendation of the supplier of the asphalt binder, supplier of the asphalt modifier, and WMA manufacturer.  For HMA, do not allow the mixture temperature to exceed 330 °F at discharge from the plant.  For WMA, do not allow the mixture temperature to exceed 300 °F at discharge from the plant.

Combine and mix the aggregates and asphalt binder to ensure that at least 95 percent of the coarse aggregate particles are entirely coated with asphalt binder as determined according to AASHTO T 195.  If the ME determines that there is an ongoing problem with coating, the ME may obtain random samples from 5 trucks and will determine the adequacy of the mixing on the average of particle counts made on these 5 test portions.  If the requirement for 95 percent coating is not met on each sample, modify plant operations, as necessary, to obtain the required degree of coating.

B.	Sampling.  The Laboratory, as defined in subsection 912.13.04.D, will take 5 stratified random samples of HMA HIGH RAP for volumetric acceptance testing from each lot of approximately 3,500 tons of a mix.  When a lot of HMA HIGH RAP is less than 3,500 tons, the Laboratory will take samples at random for each mix at the rate of one sample for each 700 tons.  The Laboratory will perform sampling according to AASHTO T 168, NJDOT B-2, or ASTM D 3665.  During production at the plant, a sample of asphalt binder will be taken once every 3,500 tons or as directed by the Laboratory.

Use a portion of the samples taken for volumetric acceptance testing for composition testing.

C.	Quality Control Testing.  The HMA HIGH RAP producer shall provide a quality control (QC) technician who is certified by the Society of Asphalt Technologists of New Jersey as an Asphalt Technologist, Level 2.  The QC technician may substitute equivalent technician certification by the Mid-Atlantic Region Technician Certification Program (MARTCP).  Ensure that the QC technician is present during periods of mix production for the sole purpose of quality control testing.  The RE will not perform the quality control testing or other routine test functions in the absence of, or instead of, the QC technician.

The QC technician shall perform sampling and testing according to the approved quality control plan, to keep the mix within the limits specified for the mix being produced.  The QC technician may use acceptance test results or perform additional testing as necessary to control the mix.

To determine the composition, perform ignition oven testing according to AASHTO T 308.

For each acceptance test, perform maximum specific gravity testing according to AASHTO T 209.  Sample and test coarse aggregate, fine aggregate, mineral filler, and RAP according to the approved quality control plan for the plant.

Ensure that the supplier has in operation an ongoing daily quality control program to evaluate the RAP.  As a minimum, this program shall consist of the following:

1.	An evaluation performed to ensure that the material conforms to 901.05.04 and compares favorably with the design submittal.

2.	An evaluation of the RAP material performed using a solvent or an ignition oven to qualitatively evaluate the aggregate components to determine conformance to 901.05.

3.	Quality control reports as directed by the ME.

D.	Acceptance Testing and Requirements.  The RE or LPA will designate an independent testing agency (Laboratory) to perform the quality assurance sampling, testing and analysis. The Laboratory Technician (QC technician) who performs the quality assurance sampling shall be certified by the Society of Asphalt Technologists of New Jersey as an Asphalt Plant Technologist, Level 2. The Laboratory will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant.  The Laboratory Technician will compact HMA HIGH RAP to the number of design gyrations (Ndes) specified in Table 902.02.03-2, using equipment according to AASHTO T 312.  The Laboratory will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the HMA HIGH RAP.

The Laboratory will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

Ensure that the HMA HIGH RAP mixture conforms to the requirements specified in Table 902.13.04-1, and to the gradation requirements in Table 902.02.03-1.  If 2 samples in a lot fail to conform to the gradation or volumetric requirements, immediately initiate corrective action.

The Laboratory will test a minimum of 1 sample per lot for moisture, basing moisture determinations on the weight loss of an approximately 1,600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

| Table 902.13.04-1 HMA HIGH RAP Requirements for Control |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Compaction Levels | Required Density (% of Theoretical Max. Specific Gravity) | Voids in Mineral Aggregate (VMA), % (minimum) |  |  |  |  | Dust-to-Binder Ratio |
|  |  | Nominal Max. Aggregate Size, mm |  |  |  |  |  |
|  | @Ndes1 | 25.0 | 19.0 | 12.5 | 9.5 | 4.75 |  |
| L, M | 95.0 – 98.5 | 13.0 | 14.0 | 15.0 | 16.0 | 17.0 | 0.6 – 1.3 |
| 1. As determined from the values for the maximum specific gravity of the mix and the bulk specific gravity of the compacted mixture. Maximum specific gravity of the mix is determined according to AASHTO T 209. Bulk specific gravity of the compacted mixture is determined according to AASHTO T 166. |  |  |  |  |  |  |  |

E.	Performance Testing for HMA HIGH RAP.

Table 902.13.04-2 and Table 902.13.04-3

Provide 6 gyratory specimens that are compacted according to AASHTO T 312 and 2 boxes of loose mix.  Compact all 6 specimens to 62 ± 1 millimeters in height and an air void content of 6.5 ± 0.5 percent.  The Laboratory will ensure that all specimens are within the target air void content.  The Laboratory will test 3 specimens for High Temperature Indirect Tensile Strength (HT-IDT) in accordance with ASTM D6931 and conditioned at 44 °C.  The Laboratory will test the remaining 3 specimens for IDEAL-CT Index in accordance with ASTM D8225 and conditioned at 25 °C.

Ensure that the first sample is taken during the construction of the test strip as specified in 406.03.01.C.  Thereafter, sample every lot.  If the test strip is done within the Project Limits and the performance testing results are acceptable to the RE, the results will be included into the first lot.  A lot is defined as 1,400 tons.

If a sample does not meet the criteria for performance testing as specified in 902.08.03-1, the RE will assess a pay adjustment as specified in Table 902.13.04-2 and Table 902.13.04-3 .  The RE will calculate the pay adjustment by multiplying the percent pay adjustment (PPA) by the quantity in the lot and the bid price for the HMA HIGH RAP item.  If High Temperature Indirect Tensile Strength is less than 20 and/or the IDEAL-CT Index is less than 200, the RE will assess the maximum pay adjustment of PPA = -100 percent or may require removal and replacement.  PPA for both High Temperature Indirect Tensile Strength and IDEAL-CT are cumulative and may not exceed -100 percent in total.  If samples received are lower than the target air void range of 6.5 ± 0.5 percent, the RE will consider the samples untestable and assess a PPA of -100 percent or may require removal and replacement of the lot.  If the RE requires removal and replacement, then the replacement work is subject to the same requirements as the initial work.

| Table 902.13.04-2 Surface Course Performance Testing Pay Adjustments for HMA HIGH RAP |  |  |  |
| --- | --- | --- | --- |
|  | Surface Course |  | PPA |
|  | PG 64S-22 | PG 64E-22 |  |
| High Temperature IDT (psi) (ASTM D6931) | t ≥ 25 | t ≥ 34 | 0 |
|  | 21 < t ≤ 25 | 25 < t ≤ 34 | PG 64S-22: -(t-25)/0.08 PG 64E-22: -(t-34)/0.18 |
|  | t < 21 | t < 25 | -100 or Remove & Replace |
| IDEAL-CT Index (ASTM D8225) | t ≥ 130 | t ≥ 150 | 0 |
|  | 130 > t ≥ 120 | 150 > t ≥ 130 | Surface PG 64S-22: -(t-130)/0.2 Surface PG 64E-22: -(t-150)/0.4 |
|  | t < 120 | t < 130 | -100 or Remove & Replace |

| Table 902.13.04-3 Intermediate and Base Course Performance Testing Pay Adjustments for HMA HIGH RAP |  |  |  |
| --- | --- | --- | --- |
|  | Intermediate and Base Course |  | PPA |
|  | PG 64S-22 | PG 64E-22 |  |
| High Temperature IDT (psi) (ASTM D6931) | t ≥ 25 | t ≥ 34 | 0 |
|  | 21 < t ≤ 25 | 25 < t ≤ 34 | PG 64S-22: -(t-25)/0.08 PG 64E-22: -(t-34)/0.18 |
|  | t < 21 | t < 25 | -100 or Remove & Replace |
| IDEAL-CT Index (ASTM D8225) | t ≥ 100 | t ≥ 120 | 0 |
|  | 100 > t ≥ 91 | 120 > t ≥ 104 | Intermediate PG 64S-22: -(t-100)/0.18 Intermediate PG 64E-22: -(t-120)/0.32 |
|  | t < 91 | t < 104 | -100 or Remove & Replace |

1**************************************************************************************************************************1

902.14  Bridge Deck Waterproof Surface Course (BDWSC)

902.14.03  Sampling and Testing

1**************************************************************************************************************************1

BDC24S-12 dated Jul 30, 2024

D.	Acceptance Testing and Requirements.

Part D is changed to:

The ME will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant.  The ME will compact HMA to the 50 design gyrations (Ndes), using equipment according to AASHTO T 312.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the BDWSC.

The ME will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

Ensure that the HMA mixture conforms to the requirements specified in Table 902.14.02-1 and Table 902.14.02-2.  If the test results are outside of the requirements specified in Table 902.14.02-1 or Table 902.14.02-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the requirements in Table 902.14.02-1 or Table 902.14.02-2, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the requirements.  If the mix is within the requirements based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the requirements specified in Table 902.14.02-1 and Table 902.14.02-2.  If 2 consecutive acceptance or quality control samples are outside the requirements specified in Table 902.14.02-1 or 
Table 902.14.02-2, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets the requirements specified in Table 902.14.02-1 and Table 902.14.02-2 and ME approval.  The ME will reject mixture produced at initial restarting that does not meet the requirements specified in Table 902.14.02-1 and Table 902.14.02-2.

The ME will test a minimum of 1 sample per lot for moisture, basing moisture determinations on the weight loss of an approximately 1,600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC25s-16 dated sep 30, 2025

the following subsection is added:

902.16  Ultra-High Performance Thin Overlay (ULTRA-HPTO)

902.16.01  Composition of Mixture

Mix Ultra-HPTO in a plant that is listed on the QPL and conforms to the requirements for HMA Plants as specified in 1009.01.  The composition of the mixture for Ultra-HPTO is coarse aggregate, fine aggregate, and asphalt binder, and may also include mineral filler and a WMA additive.  Do not use Reclaimed Asphalt Pavement (RAP), Ground Bituminous Shingle Material, Remediated Petroleum Contaminated Soil Aggregate, or Crushed Recycled Container Glass (CRCG).  Use asphalt binder and aggregates that meet the following requirements:

1.	Use polymer modified asphalt binder that is specially formulated for meeting the mix performance criteria as specified in 902.16.02 and conforms to AASHTO M 332 including compliance with the elastic response requirement in Figure 1 of AASHTO R 92.  Consult with the asphalt binder supplier to obtain the appropriate material for the specific mix design.  Submit a certificate of analysis (COA) showing the PG continuous grading (AASHTO R 29) for the asphalt binder used in the mix design.

For quality assurance testing of the asphalt binder, the ME may sample the asphalt binder during production of the mix and compare the results with the COA submitted at the time of test strip.  To analyze the binder the ME will test the binder at the nearest standard PG temperature then compare the results with the COA.  If the high (G*/ sin δ) and low (stiffness and m value) temperature passing test results are within 5 percent of the results from the passing temperature on the COA, then the ME will consider the asphalt binder comparable to the binder used during test strip.

2.	WMA additives may be used in the mix and must conform to 902.01.04.  If a WMA additive is pre-blended in the asphalt binder or added at the HMA plant, ensure that the mix meets the mix performance criteria in this specification and will not be negatively impacted by the WMA additive.  Follow the manufacturer’s recommendations for percentage of WMA additive needed.  Controlled asphalt foaming system WMA is prohibited for use in this mixture.

3.	Use coarse aggregate that is gneiss, granite, quartzite, or trap rock and conforms to 901.05.01.

4.	For fine aggregate, use 100 percent stone sand conforming to 901.05.02 and having an uncompacted void content of at least 45 percent when tested according to AASHTO T 304, Method A.  In addition, the minimum sand equivalent is 45 percent when tested according to AASHTO T 176.

5.	If necessary, use mineral filler as specified in 901.05.03.

902.16.02  Mix Design

At least 45 days before initial production, submit a job mix formula for the Ultra-HPTO on forms supplied by the Department.  Include a statement naming the source of each component and a report showing the results meet the criteria specified in Tables 902.16.02-1 and 902.16.02-2.

For the job mix formula for the Ultra-HPTO mixture, establish the percentage of dry weight of aggregate passing each required sieve size and an optimum percentage of asphalt binder based upon the weight of the total mix.  Determine the optimum percentage of asphalt binder according to AASHTO R 35 and M 323 with an Ndes of 50 gyrations.  Before maximum specific gravity testing or compaction of specimens, condition the mix for 2 hours according to the requirements for conditioning for volumetric mix design in AASHTO R 30, Section 7.1.  If the absorption of the combined aggregate is more than 1.5 percent according to AASHTO T 84 and T 85, condition the mix for 4 hours according to AASHTO R 30, Section 7.2 prior to compaction of specimens (AASHTO T 312) and determination of maximum specific gravity (AASHTO T 209).  Ensure that the job mix formula is within the master range specified in, Table 902.16.02-1.

| Table 902.16.02-1 JMF Requirements for Ultra-HPTO |  |  |
| --- | --- | --- |
| Sieve Sizes | Percent Passing1 | Production Control Tolerances2 |
| 3/8" | 100 | ±0.0% |
| No. 4 | 65-85 | ±4.0% |
| No. 8 | 30-45 | ±4.0% |
| No. 16 | 20-35 | ±3.0% |
| No. 30 | 15-30 | ±3.0% |
| No. 50 | 10-20 | ±3.0% |
| No. 100 | 5-15 | ±3.0% |
| No. 200 | 5.0-8.0 | ±2.0% |
| Asphalt Binder Content (Ignition Oven) |  | ±0.40% |
| 1. Aggregate percent passing to be determined based on dry aggregate weight. 2. Production tolerances are applied to the approved JMF for gradation and asphalt binder content. Gradation results may not fall outside of the wide band gradation limits when tolerances are applied |  |  |

Design the Ultra-HPTO to meet the requirements in Table 902.16.02-2.

| Table 902.16.02-2 Volumetric Requirements for Design and Control of Ultra-HPTO |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Required Density (% of Max Sp. Gr.) |  | Voids in Mineral Aggregate | Dust to Binder Ratio | Draindown1 AASHTO T 305 |
|  | @ Ndes (50 gyrations) | @ Nmax (100 gyrations) | (VMA) |  |  |
| Design Requirements | 96.0 | ≤ 99.0 | ≥ 17.0 % | 0.6 - 1.2 | ≤ 0.1 % |
| Control Requirements | 95.0 - 97.0 | ≤ 99.0 | ≥ 17.0 % | 0.6 - 1.3 | ≤ 0.1 % |
| 1. Draindown testing is at the discretion of the ME. |  |  |  |  |  |

Ensure that the job mix formula provides a mixture that meets a minimum tensile strength ratio (TSR) of 85 percent when prepared according to AASHTO T 312 and tested according to AASHTO T 283 with the following exceptions:

1.	Before compaction, condition the mixture for 2 hours according to AASHTO R 30 Section 7.1.

2.	Compact specimens with 40 gyrations.

3.	Extrude specimens as soon as possible without damaging.

4.	Use AASHTO T 269 to determine void content.

5.	Record the void content of the specimens.

6.	If less than 55 percent saturation is achieved, the procedure does not need to be repeated, unless the difference in tensile strength between duplicate specimens is greater than 25 pounds per square inch.

7.	If visual stripping is detected, modify or readjust the mix.

For each mix design, submit 3 gyratory specimens and 1 loose sample corresponding to the composition of the job mix formula, including the design asphalt content.  The ME will use these samples for verification of the properties of the job mix formula.  Compact the specimens to the design number of gyrations (Ndes).  To be acceptable all three gyratory specimens must comply with the gradation requirements in Table 902.16.02-1 and with the design requirements in Table 902.16.02-2.  The ME reserves the right to be present at the time of molding the gyratory specimens.

In addition, submit 11 gyratory specimens and two boxes of loose mix to the ME.  The ME will use these additional gyratory samples for performance testing of the Ultra-HPTO mix.  The ME reserves the right to be present at the time of molding the gyratory specimens.  Ensure that the additional gyratory specimens are compacted according to AASHTO T 312.  Compact 6 of the specimens to 77 millimeters in height and an air void content of 5.0 ± 0.5 percent.  The ME will test the six 77 millimeter specimens using an Asphalt Pavement Analyzer (APA) according to AASHTO T 340 at 64 °C, 100 pound per square inch hose pressure, and 100 pound wheel load.  Compact the other 5 specimens to 115 millimeter in height.  These 5 specimens will be cut, from the middle of each 115 millimeter in height specimen, to 38 millimeter in height test specimens.  The air void content of the 5 cut specimen will be determined to ensure compliance with the target air void content of 5.0 ± 0.5 percent.  The ME will use the five 38 millimeter in height specimens to test using an Overlay Tester (NJDOT B-10) at 25 °C and a joint opening of 0.025 inch.  The ME will eliminate the highest and lowest Overlay test results then average and report the middle 3 test results.  The ME will ensure that all submitted specimens are within the target air void content as tested at the Material’s Central Lab. The ME will not accept specimens lower than the target air void content, but may accept and test specimens higher than the target air void content.

The ME will approve the JMF if the average rut depth for the 6 specimens in the Asphalt Pavement Analyzer testing is not more than 3 millimeters at 8,000 loading cycles and the average number of cycles to failure in the Overlay Tester is not less than 2500.  If the JMF does not meet the APA and Overlay Tester criteria, redesign the Ultra-HPTO mix and submit for retesting.  The JMF for the Ultra-HPTO mixture is in effect until modification is approved by the ME.

When unsatisfactory results for any specified characteristic of the work make it necessary, the Contractor may establish a new JMF for approval.  In such instances, if corrective action is not taken, the ME may require an appropriate adjustment to the JMF.

Should a change in sources or changes in the properties of materials occur, the ME will require that a new JMF be established and approved before production can resume.

The ME may verify a mix on an annual basis rather than on a project-to-project basis if the properties and proportions of the materials do not change.  If written verification is submitted by the HMA supplier that the same source and character of materials are to be used, the ME may waive the requirement for the design and verification of previously approved mixes.

902.16.03  Sampling and Testing

A.	General Acceptance Requirements.  The RE or ME may reject and require disposal of any batch or shipment that is rendered unfit for its intended use due to contamination, segregation, improper temperature, lumps of cold material, or incomplete coating of the aggregate.  For other than improper temperature, visual inspection of the material by the RE or ME is considered sufficient grounds for such rejection.

Ensure that the temperature of the Ultra-HPTO at discharge from the plant or surge and storage bins is maintained between 300 and 330 °F.  For mixes produced using a WMA additive or process, ensure that the temperature of the mixture at discharge from the plant or surge and storage bins is at least 10 °F above the WMA manufacturer’s recommended laydown temperature.

Combine and mix the aggregates and asphalt binder to ensure that at least 95 percent of the coarse aggregate particles are entirely coated with asphalt binder as determined according to AASHTO T 195.  If the ME determines that there is an on-going problem with coating, the ME may obtain random samples from 5 trucks and will determine the adequacy of the mixing on the average of particle counts made on these 5 test portions.  If the requirement for 95 percent coating is not met on each sample, modify plant operations, as necessary, to obtain the required degree of coating.

B.	Sampling.  The ME will take a sample of Ultra-HPTO for volumetric acceptance testing from each 700 tons of a mix.  The ME will perform sampling according to AASHTO T 168, NJDOT B-2, or ASTM D 3665.  During production at the plant, a sample of asphalt binder will be taken once every 1,400 tons or as directed by the ME.

C.	Quality Control Testing.  The HMA producer is required to provide a quality control (QC) technician who is certified by the Society of Asphalt Technologists of New Jersey as an Asphalt Technologist, Level 2.  The QC technician may substitute equivalent technician certification by the Mid-Atlantic Region Technician Certification Program (MARTCP).  Ensure that the QC technician is present during periods of mix production for the sole purpose of quality control testing and to assist the ME.  The ME will not perform the quality control testing or other routine test functions in the absence of, or instead of, the QC technician.

The QC technician is required to perform sampling and testing according to the approved quality control plan, to keep the mix within the limits specified for the Ultra-HPTO mix being produced.  The QC technician may use acceptance test results or perform additional testing as necessary to control the mix.

To determine the composition, perform ignition oven testing according to AASHTO T 308.  For each acceptance test, perform maximum specific gravity testing according to AASHTO T 209 on a test portion of the sample taken by the ME.  Sample and test coarse aggregate, fine aggregate, mineral filler, and asphalt binder according to the approved quality control plan for the plant.

D.	Acceptance Testing and Requirements.  The ME will determine volumetric properties at Ndes for acceptance from samples taken, compacted, and tested at the HMA plant.  The ME will compact Ultra-HPTO to 50 gyrations, using equipment according to AASHTO T 312.  The ME will determine bulk specific gravity of the compacted sample according to AASHTO T 166.  The ME will use the most current QC maximum specific gravity test result in calculating the volumetric properties of the Ultra-HPTO.

The ME will determine the dust-to-binder ratio from the composition results as tested by the QC technician.

Ensure that the HMA mixture conforms to the requirements specified in Table 902.16.02-2, and to the gradation and asphalt content requirements in Table 902.16.02-1.  If the test results are outside of the requirements specified in Table 902.16.02-1 or Table 902.16.02-2 for an acceptance sample, immediately run a quality control sample.  If the quality control sample is also outside of the control requirements specified in Table 902.16.02-1 or 
Table 902.16.02-2, determine if a plant adjustment is needed and take corrective action to bring the mix into compliance.  Take an additional quality control sample immediately after completing the corrective action to ensure that the mix is within the requirements.  If the mix is within the requirements based on the quality control sample results, then the ME will immediately take an acceptance sample to test and verify that the composition meets the requirements specified in Table 902.16.02-1 and Table 902.16.02-2.  If 2 consecutive acceptance or quality control samples are outside the requirements specified in Table 902.16.02-1 or Table 902.16.02-2, immediately stop production and shipping.

After a production stop, obtain ME approval of a plant correction plan before resuming production.  Upon restarting production, do not transport mixture to the Project Limits before the results of a quality control sample from the mixture indicate that the mixture meets the requirements specified in Table 902.16.02-1 and Table 902.16.02-2 and ME approval.  The ME will reject mixture produced at initial restarting that does not meet the requirements specified in Table 902.16.02-1 and Table 902.16.02-2.

The ME will test a minimum of 1 sample per 3500 tons for moisture, basing moisture determinations on the weight loss of an approximately 1600 gram sample of mixture heated for 1 hour in an oven at 280 ± 5 °F.  Ensure that the moisture content of the mixture at discharge from the plant does not exceed 1.0 percent.

E.	Performance Testing.  Provide 11 gyratory specimens that are compacted according to AASHTO T 312 and 2 boxes of loose mix.  Compact 6 of the specimens to 77 millimeter in height and an air void content of 5.0 ± 0.5 percent.  Compact the other 5 specimens to 115 millimeter in height.  These 5 specimens will be cut, from the middle of each 115 millimeter in height specimen, to 38 millimeter in height test specimens.  The air void content of the 5 cut test specimens will be determined to ensure compliance with the target air void content of 5.0 ± 0.5 percent.

The ME will use the boxes of loose mix to determine the maximum specific gravity of the mix according to AASHTO T 209.  The ME will use the gyratory samples for performance testing of the Ultra-HPTO mix.  The ME will test six 77 millimeter in height specimens using an Asphalt Pavement Analyzer (APA) according to AASHTO T 340 at 64 °C, 100 pound per square inch hose pressure, and 100 pound wheel load.  The ME will use the five 38 millimeter in height specimens to test using an Overlay Tester (NJDOT B-10) at 25 °C and a joint opening of 0.025 inch.  The ME will eliminate the highest and lowest Overlay test result then average and report the middle 3 test results.  The ME will ensure that all submitted specimens are within the target air void content as tested at the Material’s Central Lab.  The ME will not accept specimens lower than the target air void content, but may accept and test specimens higher than the target air void content.

Ensure that the first sample is taken during the construction of the test strip as specified in 406.03.01.C.  Thereafter, sample every lot or as directed by the ME.  If the test strip is done within the project limits and the performance testing results are acceptable to the ME, the results will be included into the first lot.  A lot is defined as material placed on the traveled way within the project limits.

If a sample does not meet the criteria for performance testing as specified in Table 902.16.03-1, the Department will assess a pay adjustment as specified.  The Department will calculate the pay adjustment by multiplying the percent pay adjustment (PPA) by the quantity in the lot and the bid price for the Ultra-HPTO item.  If APA rutting is greater than 8.0 millimeters or Overlay cycles is less than 1000 or both, the Department will assess the maximum pay adjustment of PPA = -100 percent or may require removal and replacement.  PPA for both APA and Overlay are cumulative and may not exceed -100 percent in total.  If samples received are lower than the target air void range, 5.0 ± 0.5 percent, the Department will consider the samples un-testable and assess a PPA of -100 percent or may require removal and replacement of the lot.  If the Department requires removal and replacement, then the replacement work is subject to the same requirements as the initial work.

| Table 902.16.03-1 Performance Testing Pay Adjustments for Ultra-HPTO |  |  |  |
| --- | --- | --- | --- |
| Test | Requirement | Test Result | PPA |
| APA @ 8,000 loading cycles, mm (AASHTO T 340) | 3.0 maximum | t ≤ 3.0 3.0 < t ≤ 8.0 t > 8.0 | 0 -10(t-3) -100 or Remove & Replace |
| Overlay Tester, cycles (NJDOT B-10) | 2500 minimum | t ≥ 2500 2500 > t ≥ 1000 t < 1000 | 0 -(2500-t)/30 -100 or Remove & Replace |

1**************************************************************************************************************************1

Section 903 – Concrete

903.01  Cement

1**************************************************************************************************************************1

BDC21s-02 dated Mar 24, 2021

the entire Subsection text is changed to:

Use cement, listed on the QPL, that is either portland cement or blended hydraulic cement and conforms to the following:

Portland Cement, Type I, II, and Type III	ASTM C 150

Blended Hydraulic Cement, Type IS, IP, and IL	ASTM C 595

Only use Type III portland cement for Class V concrete, prestressed Items, and precast Items.

Use portland cement pre-blended with a maximum of 25 percent fly ash, by weight, or a maximum of 5 percent silica fume by weight, or with a maximum of 50 percent slag by weight for blended hydraulic cement Type IS or IP.  Use portland cement pre-blended with a minimum of 5 percent limestone content and a maximum of 15 percent limestone content by weight for blended hydraulic cement Type IL.  Ensure that a scaling test according to ASTM C 672 is completed on the mix design if more than 30 percent slag is used and that the concrete has a visual rating less than 3 after 50 cycles.

Do not add additional mineral admixtures to blended hydraulic cements Type IS or IP at the concrete plant unless approved by the ME.  The use of additional mineral admixtures in blended hydraulic cement Type IL at the concrete plant is permitted if the mineral admixture is listed on the QPL

Do not mix different brands of cement, the same brand of cement from different mills, or different types of cement.

Provide suitable means for storing and protecting the cement against dampness.  The ME will reject cement that has become partially set or that contains lumps of caked cement.  Ensure that the temperature of the cement at the time of delivery to the mixer does not exceed 160 °F.

1**************************************************************************************************************************1

903.03.05  Control and Acceptance Testing Requirements

1**************************************************************************************************************************1

E.	Acceptance Testing for Strength for Pay-Adjustment Items.

INCLUDE ONLY THE following ITEMS, as APPLICABLE, for pay adjustment

(do not include any variance such as: concrete bridge deck, hpc)

Concrete Items which are subject to pay adjustment and the base prices are as follows:

| ITEMS | DESCRIPTION | UNIT | BASE PRICE |
| --- | --- | --- | --- |
| 507021P | CONCRETE BRIDGE DECK | CY | $500.00 |
| 507036P | CONCRETE BRIDGE PARAPET | LF | $305.00 |
| 505039P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SII-36), 36" X 15" | LF | $125.00 |
| 505042P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SIII-36), 36" X 18" | LF | $130.00 |
| 505015P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BI-36), 36" X 27" | LF | $170.00 |
| 505045P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SIV-36), 36" X 21" | LF | $160.00 |
| 505018P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BII-36), 36" X 33" | LF | $170.00 |
| 505021P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BIII-36), 36" X 39" | LF | $175.00 |
| 505024P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BIV-36), 36" X 42" | LF | $185.00 |
| 505003P | PRETENSIONED PRESTRESSED CONCRETE BEAM, 45" | LF | $155.00 |
| 505006P | PRETENSIONED PRESTRESSED CONCRETE BEAM, 54" | LF | $155.00 |
| 505048P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SII-48), 48" X 15" | LF | $160.00 |
| 505051P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SIII-48), 48" X 18" | LF | $135.00 |
| 505009P | PRETENSIONED PRESTRESSED CONCRETE BEAM, 63" | LF | $185.00 |
| 505027P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BI-48), 48" X 27" | LF | $215.00 |
| 505054P | PRESTRESSED CONCRETE SLAB BEAM, (TYPE SIV-48), 48" X 21" | LF | $215.00 |
| 505030P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BII-48), 48" X 33" | LF | $185.00 |
| 505033P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BIII-48), 48" X 39" | LF | $220.00 |
| 505036P | PRESTRESSED CONCRETE BOX BEAM, (TYPE BIV-48), 48" X 42" | LF | $230.00 |
| 505012P | PRETENSIONED PRESTRESSED CONCRETE BEAM, 72" | LF | $200.00 |
| 502045M | CAST-IN-PLACE CONCRETE PILE, DRIVEN, 12" DIAMETER | LF | $50.00 |
| 502090M | PRECAST CONCRETE PILE, DRIVEN, 12" X 12" | LF | $90.00 |
| 502132M | PRESTRESSED CONCRETE PILE, DRIVEN, 12" X 12" | LF | $50.00 |
| 502135M | PRESTRESSED CONCRETE PILE, DRIVEN, 14" X 14" | LF | $50.00 |
| 502138M | PRESTRESSED CONCRETE PILE, DRIVEN, 16" X 16" | LF | $50.00 |
| 502141M | PRESTRESSED CONCRETE PILE, DRIVEN, 18" X 18" | LF | $50.00 |
| 502144M | PRESTRESSED CONCRETE PILE, DRIVEN, 20" X 20" | LF | $75.00 |
| 502147M | PRESTRESSED CONCRETE PILE, DRIVEN, 22" X 22" | LF | $75.00 |
| 502150M | PRESTRESSED CONCRETE PILE, DRIVEN, 24" X 24" | LF | $75.00 |
| 502151M | PRESTRESSED CONCRETE PILE, DRIVEN, 30" X 30" | LF | $75.00 |
| 502156M | PRESTRESSED CONCRETE PILE, DRIVEN, 54" DIAMETER | LF | $200.00 |

1**************************************************************************************************************************1

903.06.01  SCC for Drilled Shafts

1**************************************************************************************************************************1

BDC25S-17 dated oct 22, 2025

THE SUBPART IS CHANGED TO:

903.06.01  SCC For Cast-In-Place Concrete

A.	Composition.  Produce SCC conforming to the composition requirements specified in 903.03.01, except use a Type F admixture and a viscosity modifying admixture (VMA).  Use Type F and VMA admixtures, as specified in 903.02.02 and 903.02.04, at a dosage to produce a flowable concrete that does not require vibration for consolidation.  Proportion the aggregates so that the fine aggregate is less than 50 percent by weight of the total aggregate.

B.	Mix Design and Verification.  Design the mix as specified in 903.03.02 or 903.05.02 to conform to the strength requirements, water-cement ratio, and cement content for the specified concrete of the item being cast. Ensure the SCC conforms to the requirements specified in Table 903.06.01-1.

| Table 903.06.01-1 Requirements for SCC for Cast-in-Place Concrete |  |  |
| --- | --- | --- |
| Property | Test Method | Requirement |
| Air Content for Drilled Shafts |  |  |
| Coarse Aggregate No. 57 | AASHTO T 152 | 6.5 ± 1.5 percent |
| Coarse Aggregate No. 67 |  | 6.5 ± 1.5 percent |
| Coarse Aggregate No. 8 |  | 7.5 ± 1.5 percent |
| Air Content for All Other Items |  |  |
| Coarse Aggregate No. 57 |  | 6.0 ± 1.5 percent |
| Coarse Aggregate No. 67 | AASHTO T 152 | 6.0 ± 1.5 percent |
| Coarse Aggregate No. 8 |  | 6.5 ± 1.5 percent |
| Slump Flow for Drilled Shaft | NJDOT C-4 | 18 to 24 inches |
| Slump Flow for All Other Items | NJDOT C-4 | 22 to 30 inches |
| Visual Stability Index |  |  |
| Plastic Concrete | NJDOT C-4 | 1 maximum |
| Hardened Concrete | NJDOT C-5 | 1 maximum |

Perform mix design verification as specified in 903.03.02 or 903.05.02 verification batch, ensure that the air content is in the top half of the allowable range and the slump flow is between 21 to 24 inches for drilled shaft items and between 22 and 28 inches for all other items.  Perform air content, slump flow, and visual stability index (plastic concrete) testing on the verification batch.  Make concrete cylinders for compression testing as specified in 903.03.02 and make 2 additional 4 × 8 inch cylinders for evaluation of the visual stability index of the hardened concrete.  Saw the additional cylinders length-wise according to NJDOT C-5.  The ME will perform the compressive strength testing and the visual evaluation to assign a visual stability index in order to approve the mix.

C.	Verification of Pumpability.  Verify pumpability at least 10 days before pouring the SCC.  Demonstrate the pumpability of the SCC to the ME by pumping a trial batch through the pump proposed for placing the SCC.  Use the proposed methods for mixing the concrete including any anticipated time delays.  The ME will test the SCC before and after pumping to verify that the SCC meets the requirements of Table 903.06.01-1 after pumping.

D.	Mixing.  Mix SCC as specified in 903.03.03.

E.	Control and Acceptance Testing.  Perform quality control testing as specified in 903.03.05.

The ME will perform acceptance testing as specified in 903.03.05 for the specified class of concrete for the item, except that the provisions for slump testing are replaced with requirements for slump flow testing and visual `stability index on the plastic concrete.  The ME will perform the slump flow testing and the visual stability index according to NJDOT C-4, at the sampling rate specified for slump testing for the specified class of concrete.  The ME will perform visual stability index on the hardened concrete according to NJDOT C-5 at a rate of at least 1 per day.  If the visual stability index on the hardened concrete does not conform to the criteria specified in Table 903.06.01-1, the ME will require redesign of the mix.

In the performance of quality control or acceptance testing, without remixing the sample, fill cylinder molds, slump flow cones, and air buckets in one lift.  Do not vibrate, rod, or tap to consolidate the SCC.

1**************************************************************************************************************************1

Section 909 – Drainage

909.02.02  HDPE Pipe

1**************************************************************************************************************************1

BDC21S-12 dated Aug 16, 2022

The Subpart heading is changed to:

909.02.02  HDPE Pipe and Polypropylene (PP) Pipe

The entire subpart is changed to:

Use corrugated HDPE drainage pipe that conforms to AASHTO M 294 and is Type S (smooth interior with annular corrugations) with gasketed silt-tight joints.

Use corrugated polypropylene (PP) drainage pipe that conforms to AASHTO M 330 and is Type S (smooth interior with annular corrugations) with gasketed silt-tight joints.

Use HDPE and polypropylene (PP) pipe from a manufacturer who is an AASHTO NTPEP (National Transportation Product Evaluation Program) certified manufacturer.  For a list of NTPEP certified manufacturers, see the following webpage: https://data.ntpep.org/.

Submit a certification of compliance, as specified in 106.07, for HDPE and polypropylene (PP) pipe.

1**************************************************************************************************************************1

Section 910 – Masonry Units

910.04  Stone Curb

1**************************************************************************************************************************1

provide lithology, color, and texture of stone curb

1**************************************************************************************************************************1

910.05  Stone Facing for Pier Shafts

1**************************************************************************************************************************1

provide lithology, color, and texture of Stone Facing for Pier Shafts

1**************************************************************************************************************************1

910.06  Stone Paving Block

1**************************************************************************************************************************1

provide color, texture, and uniformity of Stone Paving Block

1**************************************************************************************************************************1

Section 911 – Signs, Sign Supports, and Delineators

911.01.01  Materials

B.	Retroreflective Sheeting.

1**************************************************************************************************************************1

BDC22S-12 dated Oct 05, 2022

Table 911.01.01-2 is changed to:

2**************************************************************************************2

BDC22S-16 dated July 17, 2023

the second sign type in table 911.01.01-2 is changed to:

| Table 911.01.01-2 Allowable Sign Sheeting Types |  |  |
| --- | --- | --- |
| Sign Type | Test Method | Type |
| Regulatory and Warning Signs | ASTM D 4956 | Type III, Type VIII, Type IX or Type XI1 |
| Guide Signs Mounted on Steel “U” or Square Tube Posts | ASTM D 4956 | Type III, Type VIII, Type IX or Type XI |
| Guide Signs Mounted on Overhead Sign Structures, Breakaway Sign Supports, or Non-breakaway Sign Supports | ASTM D 4956 | Type VIII, Type IX or Type XI2 |
| 1. Ground-mounted signs with white background shall be ASTM Type IX. 2. Do not use ASTM Type XI sheeting with any existing sign lighting improvement or signs that are lit. |  |  |

2**************************************************************************************2

1**************************************************************************************************************************1

911.01.02  Fabrication

1**************************************************************************************************************************1

BDC23S-02 dated Mar 13, 2023

A.	Sign Panels.

The second paragraph in Part A is changed to:

Fabricate flat sheet signs from a single piece of sheet aluminum without joints, using the thicknesses specified in Table 911.01.02-1.  Drill or punch 3/8 inch diameter holes in the sign blank for attachment to sign supports.  Locate holes according to the 2004 Edition of Standard Highway Signs and Markings Book.  If the panel is larger than 5 feet in any dimension, reinforce the panel with z-bars.

2**************************************************************************************2

BDC22S-16 dated July 17, 2023

the third paragraph of part a is changed to:

For multiple panel signs, use 1 foot wide extruded sections bolted together.  Join sign panel sections together at the flanges with 3/8 inch bolts.  Attach the sign panels to vertical supports, ensuring that the span between vertical supports is a maximum of 18 feet.  Do not use extruded sign panels with steel “U” or square tube post sign supports.  Use the same material and color for trim molding that is used for the sign face.

2**************************************************************************************2

C.	Legends and Borders.

The first paragraph in Part C is changed to:

The legend for each sign consists of letters, numerals, shields, and other symbols.  Use Series E Modified 2000 lettering that conforms to the 2004 Edition of Standard Highway Signs and Markings Book.  Ensure that the lettering is aligned, spaced, and sized according to 2004 Edition of Standard Highway Signs and Markings Book, its 2012 Supplement, and the working drawings.  Apply the legend and borders using the following methods:

1**************************************************************************************************************************1

911.01.04  Acceptance Inspection

1**************************************************************************************************************************1

BDC23S-02 dated Mar 13, 2023

The first paragraph is changed to:

Notify the RE at least 3 days before shipping to the Project so that arrangements for inspection can be made.  The RE will reject signs not fabricated according to the 2004 Edition of Standard Highway Signs and Markings Book, its 2012 Supplement, and the Plans.  The RE will ensure that finished signs are clear and legible without smudging, blisters, delamination, loose edges, or other blemishes.  The RE will also ensure that the colors have a consistent chromaticity across all signs of the same color.

1**************************************************************************************************************************1

911.02 Sign Supports

911.02.01 Steel “U” Post Sign Supports

1**************************************************************************************************************************1

BDC22S-16 dated July 17, 2023

the subpart is renamed to:

911.02.01 Steel “U”and Square Tube Post Sign Supports

The following is added at the end:

Use steel “U” post sign supports conforming to ASTM A 499, Grade 50 or 60, with the length of post and minimum pounds per foot as shown on the Plans.  Provide 18-8 stainless steel 5/16 × 18 UNC hexagonal headed bolts and nuts conforming to ASTM A 320, Grade B8, Class 1, for securing the signs to the steel “U” post.  Provide sign mounting bolts that are sized to extend beyond the end of each nut by no more than 3/4 inches when fully tightened.

Submit a certification of compliance, as specified in 106.07, for “U” posts.

Use square steel tube post sign supports conforming to ASTM A 1011, Grade 50, with the length of post and minimum pounds per foot as shown on the Plans.  Provide 18-8 stainless steel 5/16 × 18 UNC hexagonal headed bolts and nuts conforming to ASTM A 320, Grade B8, Class 1, for securing the signs to the square steel tube post.  Provide sign mounting bolts that are sized to extend beyond the end of each nut by no more than 3/4 inches when fully tightened.

Submit a certification of compliance, as specified in 106.07, for square steel tube posts.

1**************************************************************************************************************************1

Section 912 – Paints, Coatings, Traffic Stripes, and Traffic Markings

912.01.04 Concrete Stain

1**************************************************************************************************************************1

BDC20S-10 dated Sep 11, 2020

The subsection is changed to:

Provide a penetrating stain that is a single component, water-based acrylic coating, alkali resistant, and water repellant.  Primer is required for application on smooth concrete.  Provide a primer that is a penetrating, water based, water repellent concrete sealer.  Ensure that the stain conforms to the requirements in Table 912.01.04-1.

| Table 912.01.04-1 Requirements for Concrete Stain |  |  |
| --- | --- | --- |
| Property | Value | ASTM Test Method |
| Dry – to – Touch Time | Max. 1 hour | D 1640 |
| Dry – to – Recoat Time | Max. 4 hour | D 1640 |
| Weight per Gallon | 11.0 + 0.5 lbs. | D 1475 |
| Weight Solids | 52 + 2% | D 2369 |
| Adhesion, Tape Test | Min. 4A | D 3359 |
| Gloss | Flat | D 523 |
| Weathering | <3.0 dE @ 3,000 hrs. | G 154 / D 4587 |
| VOC | <100 g/L | D 2369 |

1**************************************************************************************************************************1

912.04.02  Removable Pavement Marking Tape

- Temporary Pavement Tape.

1**************************************************************************************************************************1

BDC22S-06 dated JUL 13, 2022

THE fourth paragraph in part a is changed to:

Provide a certification of compliance, as specified in 106.07, for temporary pavement tape.

1**************************************************************************************************************************1

- Removable Black Line Masking Tape.

1**************************************************************************************************************************1

BDC22S-06 dated JUL 13, 2022

THE third paragraph in part b is changed to:

Provide a certification of compliance, as specified in 106.07, for removable black masking tape.

1**************************************************************************************************************************1

Section 913 – Guide Rail, Fence, Railing, and Box Beam

1**************************************************************************************************************************1

BDC21S-13 dated mAR 21, 2022

913.04  Box Beam for Construction Barrier Curb

The first paragraph is changed to:

Ensure that the box beam is made of cold-formed welded and seamless structural tubing.  Ensure that the box beam conforms to ASTM A 500, Cold-Formed Welded and Seamless Carbon Steel Structural Tubing in Rounds and Shapes, Grade C.

1**************************************************************************************************************************1

Section 914 – Joint Materials

914.04.03  Modular

1**************************************************************************************************************************1

BDC21S-08 dated NOV 18, 2021

The SUBPART is renamed TO:

914.04.03  Modular Expansion Joint Assemblies

B.	Materials.

Part 1 is changed to:

1.	Use structural steel conforming to the requirements of AASHTO M 270M / M 270 (ASTM A 709 / A 709M), Grade 50, or Grade 50W with Zone 2 CVN requirements for centerbeams, supporting beams, and yokes.  Use structural steel AASHTO M 270 M / M (ASTM A 709 / A 709M) Grade 36, Grade 50, or Grade 50W for other steel components.  Do not use aluminum components.

Part 3 is changed to:

3.	Use PTFE that is 100 percent virgin material, woven PTFE fabric, or dimpled PTFE conforming to the material requirements in Section 14 of the AASHTO LRFD Bridge Design Specifications, and Section 19 of AASHTO LRFD Bridge Construction Specifications.

The 2nd PARAGRAPH is changed to:

Perform prequalification tests, Open Movement and Vibration (OMV) testing and Seal Push-Out (SPO) testing, according to AASHTO LRFD Construction Specifications Section 19/A19 and provide testing report.  Certify that the tested modular expansion joint assembly has passed the prequalification tests required in AASHTO LRFD Construction Specifications.

The following is added AT THE end:

Provide certification that materials of bearings, springs, and equidistant devices used in the project product are identical to those used in the prequalification tests in material composition and formulation, and conform to the same manufacturing process, fabrication procedure, and configuration as those used in the prequalification tests.  In addition, certify that these materials and fabrication procedures are as per working drawing plans and specifications provided by the manufacturer at the time of approval.

1**************************************************************************************************************************1

Section 915 – Timber and Timber Treatment

915.04  Dimension Lumber, Timber, and Decking for Structures

1**************************************************************************************************************************1

BDC22s-08 dated JAN 13, 2023

the First paragraph is changed to:

Use dimension lumber, timber, glued-laminated timber, and decking for structures that conform to AASHTO M 168 with the following modifications:

1.	Manufacture dimension lumber and timber from Southern pine or Douglas fir of structural grade that conforms to the grading rules of the Southern Pine Inspection Bureau or the Western Lumber Inspection Bureau.  Ensure that the grading is performed by an agency approved by the Board of Review of the American Lumber Standards Committee.  Use Southern pine that is designated and graded as No. 2 if 2 to 4 inches nominal thickness and as No. 1 if 5 inches nominal thickness or thicker.  Use Douglas fir that is designated and graded as Dense No. 1.

2.	For decking subjected to vehicular traffic, use timber that is graded as No. 1 Dense Southern Pine or 
Douglas-fir lumber and timbers .  Dress timber for bridge decking square edged S4S.

3.	For decking used exclusively by pedestrians, use No. 1 grade Southern Pine or Douglas-fir lumber and timbers that are dressed square edged S4S.

4.	Treat timber as specified in 915.05, except do not preserve timber railing systems and decking used by pedestrians with creosote.  In addition, do not preserve timber decking to be covered with an HMA overlay with creosote.

1**************************************************************************************************************************1

915.05  Timber Treatment

1**************************************************************************************************************************1

BDC22s-08 dated JAN 13, 2023

The entire subsection is changed to:

Treat wood species according to AASHTO M 133 and AWPA Standards U1-20 and T1-20, including those as summarized in Table 915.05-1, Table 915.05-2, and Table 915.05-3.

| Table 915.05-1 Treatment for Sawn Timber Posts |  |  |  |
| --- | --- | --- | --- |
| Type of Wood | Location/Environment | AASHTO Treatments | AWPA Standard Reference for Minimum Retention Level |
| Southern Pine | Soil or Fresh Water | CCA or Pentachlorophenol | UC4A |
| Douglas Fir | Soil or Fresh Water | ACZA | UC4A |

| Table 915.05-2 Treatment for Round Timber Piles |  |  |  |
| --- | --- | --- | --- |
| Type of Wood | Location/Environment | AASHTO Treatments | AWPA Standard Reference for Minimum Retention Level |
| Southern Pine | Soil or Fresh Water | CCA | UC4C |
| Southern Pine | Marine | CCA | UC5B |
| Douglas Fir | Soil or Fresh Water | ACZA | UC4C |
| Douglas Fir | Marine | ACZA | UC5B |

| Table 915.05-3 Treatment for Timber Sheet Piling and Timber for Structures |  |  |  |
| --- | --- | --- | --- |
| Type of Wood | Location/Environment | AASHTO Treatments | AWPA Standard Reference for Minimum Retention Level |
| Southern Pine | Soil or Fresh Water | CCA or Pentachlorophenol | UC4B |
| Southern Pine | Marine | CCA | UC5B |
| Douglas Fir | Soil or Fresh Water | ACZA | UC4B |
| Douglas Fir | Marine | ACZA | UC5B |

Notify the RE at least 14 days before treating timber.  If directed by the RE, perform an assay to determine the retention of preservative according to AASHTO M 133 and referenced AWPA standards.  Submit certification of compliance as specified in 106.07.  Attach the assay report to the certification.

The use of uncoated pressure-treated timber using the above timber treatments, as specified in Table 915.05-1, Table 915.05-2, and Table 915.05-3, may not be permitted in areas containing shellfish or submerged aquatic vegetation, or in other environmental sensitive areas.  Alternative materials, such as plastic, natural cedar or other untreated wood, polymer coated pressure-treated wood, concrete or other inert products, may be required by regulatory agencies.  Prior to using treated timber products, contact the RE .

All lumber and timber, including those specified in Table 915.05-1, Table 915.05-2, and Table 915.05-3, must be pressure treated according to current AWPA or AASHTO M 133 standards.  Preservatives are subject to EPA Guidelines 2004 with restricted use of CCA preservatives.

1**************************************************************************************************************************1

Section 917 – Landscaping Materials

1**************************************************************************************************************************1

917.07  Sod

Specify if required other SOD

1**************************************************************************************************************************1

917.08  Plant Materials

H.	Inspection.

1**************************************************************************************************************************1

complete and include THE FOLLOWING if the time frame for the delivery notification to the RE is other than 72 hours

Notify the RE at least ____ (hours or days) in advance of delivery to the Project Limits for installation.

1**************************************************************************************************************************1

Section 918 – Electrical Materials

1**************************************************************************************************************************1

BDC25s-22 dated may 08, 2026

THE FOLLOWING SUBSECTION IS ADDED:

918.15  Junction Box Frames and Covers

Provide junction box frames and covers that conform to the following:

1.	Gray Iron Castings.  Use gray iron castings conforming to AASHTO M 306, except that the manufacturer may use gray cast iron conforming to AASHTO M 105, Class 30B or Class 35B.  Ensure that castings are true to pattern in form and dimensions and are free from pouring faults, sponginess, cracks, blowholes, and other defects in composition affecting their strength or integrity.  To present a smooth, clean, and uniform surface, sandblast or clean the castings to remove sand and scale.

2.	Carbon Steel Frames and Anchors.  Use carbon steel frames and anchors for junction boxes.  When specified, galvanize the frames and anchors according to AASHTO M 111.  Perform welding of fabricated steel shapes and structures according to AWS D1.1.  Do not punch, drill, ream, weld, or cut frames and anchors in the field.  Repair any damage to the galvanized coating.

Submit certifications of compliance as specified in 106.07.

1**************************************************************************************************************************1

Section 919 – Miscellaneous

919.05  Geomembrane Liner

1**************************************************************************************************************************1

BDC19s-09 dated JAN 14, 2020

Table 919.05-1 is changed to:

| Table 919.05-1 Requirements for HDPE Resin |  |  |
| --- | --- | --- |
| Property | Test Method | Requirements |
| Specific Gravity (Resin & Carbon Black) | ASTM D 792 | > 0.940 |
| Melt Index | ASTM D 1238 | < 0.4 g/10 min |
| Carbon Black Content | ASTM D 1603 | 2 – 3 % |

1**************************************************************************************************************************1

Division 1000 – Equipment

Section 1001 – Traffic Control Equipment

1001.01  Flashing Arrow Board

SUBSECTION HEADING AND The entire TEXT ARE changed to:

1**************************************************************************************************************************1

BDC23S-02 dated Mar 13, 2023

1001.01  Arrow Board

Provide either Type A or Type C or both types of arrow boards.  Ensure the arrow board elements conform to the MUTCD and the following requirements:

1.	Non-reflective, black boards equipped with battery-operated amber lights.

2.	A minimum peak luminous intensity of 8,800 candelas and equipped with photocells that will automatically reduce the luminous intensity to 1,500 candelas when the ambient light level drops to 5 foot-candles.

3.	A light on the rear face of the board to indicate that the lights are operating.

4.	Solid state controls with polarity and surge protection.

5.	Panel operation controls mounted in a lockable enclosure.

Ensure the arrow boards are equipped with a diesel charged battery system.  Do not use gasoline powered systems.  With RE approval, the Contractor may use the arrow boards equipped with solar charged battery systems in non-moving operations.  The Department may require a solar charged battery system in noise sensitive areas.

Securely mount arrow boards on a manufacturer-approved 2 wheeled towing trailer.

1**************************************************************************************************************************1

1001.03  Traffic Control Truck with Mounted Crash Cushions

1**************************************************************************************************************************1

BDC23S-02 dated Mar 13, 2023

The entire subsection is changed to:

Provide a truck affixed with a bed-mounted type C arrow board, as specified in 1001.01, and a rear mounted crash cushion.  Ensure the weight of the truck with the type C arrow board and the rear mounted crash cushion is minimum total weight of 10 tons.  The Contractor may use ballast to meet the weight requirement.  When using ballast, ensure that it is securely fastened to the truck.  Provide crash cushions that conforms to the following requirements:

1.	Meets crash-worthiness requirements as specified in 159.03.02.

2.	Designed to be attached to the rear of a truck.

3.	Equipped with a 90 degree hydraulic tilt system to raise and lower the crash cushion.  The tilt system shall have a locking mechanism to secure the crash cushion when in the raised position.

4.	If equipped with energy absorbing modules, ensure that they are painted yellow.

5.	Displays alternating 6 inch wide black and yellow bands, composed of Type III-retroreflective sheeting, as specified in ASTM D 4956, in an inverted “V” chevron pattern on the surface of the rear module that faces traffic.  When in the raised position, ensure that the surface of the rear facing module also displays the chevron pattern.

6.	Equipped with standard trailer lighting systems, including brake lights, taillights, and turn signals that are visible in the raised and lowered positions.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

include the following subsections if ANY OF THESE equipment are REQUESTED BY Traffic Operations

The following subsection is added:

1001.04  Portable Variable Message Sign with Remote Communication

IF REQUESTED FOR A NJDOT FACILITY

Provide a NTCIP compliant portable variable message sign as described under 1001.02 with the exceptions noted below and each equipped with broadband cellular modem.

Ensure that the sign panel is color full matrix model that displays a combination of letters and graphic images.

Ensure that the sign panel is capable of displaying 3 lines of text with variable size characters.

Ensure 9 characters are displayed per line for posting travel times.  For this 9 character requirement, smaller size characters may be allowed that meets MUTCD guidelines.

Ensure that the panel is also capable of displaying 8 characters per line with a minimum character height of 18 inches.

Ensure that the PVMSRC can be integrated with the Department’s central DMS control software for remote operation.

1001.05  Portable Trailer Mounted CCTV Camera Assembly

IF REQUESTED FOR A NJDOT FACILITY ADD THE FOLLOWING:

2**************************************************************************************2

Provide a Portable Trailer Mounted CCTV Camera Assembly (PTMCCA) with the following:

A.	Trailer Platform

1.	Maximum size, including tongue, 14 feet long by 7 feet wide by 8 feet high.

2.	NJDOT approved lighting package to include electrical brake and marker lights with wire connections.

3.	Primed and painted with powder coated orange color.

4.	Fitted with manual telescoping outriggers with adjustable jacks sized to counter full mast extension.

5.	Four 3,500 pounds, drop leg, top wind screw jacks.

6.	All equipment secured to prevent theft or separation from platform.

7.	24/7 operation in all weather conditions.

8.	One locking NEMA-4 equipment box for operational controls.

9.	Removable wheels (with wheel locks) when trailer is in deployed position.

10.	Operation manual with a copy placed in the storage bin.

B.	Mast

1.	150 pounds payload capacity.

2.	29 feet to 32 feet of extension with capability to mount antenna at 20 feet, 25 feet or at the top, 10 feet maximum nested length of mast - 3 to 9 sections.

3.	Un–guyed.

4.	Driven by galvanized steel cable.

5.	Spiral conduit for cables.

6.	Compactly retractable when nested into storage container at the bottom, and foldable for easy transport.

7.	Operated by a power winch with a safety brake.

8.	Capable of being raised or lowered during sustained wind speeds of 30 miles per hour.

C.	Power Source

Equip the PTMCCA with either a diesel charged or a solar charged battery system.  Ensure that the PTMCCA is also capable of operating on 120 volt AC electrical service.  The Department may require a solar charged battery system in noise sensitive areas.  Provide the power with a battery backup system capable of providing continuous operation when the primary power source fails.  Ensure that the power source meets the following requirements:

1.	Diesel.  Ensure that the fuel tank is capable of operating the sign for a period of 72 hours without refueling.  Equip with an exhaust muffler and a United States Department of Forestry approved spark arrester.  Ensure that the engine is shock mounted to reduce vibration and locked in a ventilated enclosure.

2.	Solar.  Provide solar panels capable of recharging the batteries at a rate of 4 hours of sun for 24 hours of camera usage.  Ensure that the battery capacity is capable of operating the sign for a period of 18 days without sunlight.

D.	Electronics

1.	Cellular (CDMA), microwave, or 802.11 bandwidth option.

2.	Work lights in all cabinets.

3.	Remote trailer diagnostics (battery level, charging output, etc.)

E.	Camera and Software

Ensure that the camera has the following characteristics:

1.	Dome Camera in a heavy duty plastic dome or with a weather resistant case.

2.	Impact resistant viewing window.

3.	Minimum resolution of NTSC 704 (H) x 480 (V).

4.	Backlight compensation.

5.	Image stabilization.

6.	Light Sensitivity 0.02 lux NIR Mode.

7.	Auto Focus with Manual Focus capability.

8.	Auto White Balance with Manual White Balance capability.

9.	Motorized Zoom up to 16x optical, 10x digital.

10.	Motorized Pan-Tilt, pan 360°, tilt 180°.

11.	Thermostatically controlled heater and defroster -50° to 140°F operating range.

12.	Windshield wiper.

13.	24/7 operation in all weather conditions.

14.	Time and date stamp.

Ensure the software provides the following functionality:

1.	Remote control of pan, tilt, and zoom.

2.	Display of streaming video in MPEG format, motion-JPEG, and single snapshot JPEG images, remotely interchangeable by using central software.

3.	Preset controls of pan/tilt/zoom combinations.  Ensure all presets are accessible from a drop-down menu with descriptive name of preset.  Set first 8 presets with quick-launch icons with graphical representation of the preset views.

4.	Display of all the project’s webcams in a single view screen.

5.	Display of local time and weather conditions including temperature and humidity.

6.	Saving images and sending email images.

3************************************************3

7.	Viewing archived images via a graphical calendar control and storing archived images at least every 5 minutes.

3************************************************3

8.	Three levels of password protection: administrator, user, and guest individual user accounts.

9.	Monitoring and controlling the cameras using web access.

2**************************************************************************************2

1**************************************************************************************************************************1

Section 1003 – HMA Site Equipment

1003.01  Materials Transfer Vehicle (MTV)

1**************************************************************************************************************************1

BDC 20S-05 dated jun 5, 2020

THE FOLLOWING IS ADDED AFTER THE LAST PARAGRAPH:

Ensure the MTVs Gross Weight and maximum speed limit do not exceed the load restrictions as shown in 105.09 Special Provisions.

1**************************************************************************************************************************1

Section 1008 – Miscellaneous Equipment

1**************************************************************************************************************************1

BDC23s-08 dated jun 6, 2023

THE FOLLOWING SUBSECTION IS ADDED:

1008.07  Concrete Vertical Curb Saw

Provide a power-driven vertical curb saw with horizontally-oriented blade capable of sawing to the required dimensions without causing uncontrolled cracking.  Equip the vertical curb saw with water-cooled, circular, diamond-edge blades or abrasive wheels, and alignment guides.  Ensure that the vertical curb saw is capable of immediately collecting the slurry produced from the operations.  The Contractor may use a vertical curb saw that does not collect slurry if the RE approves an alternate slurry collection method.

When sawcutting grooves, use a multi-bladed saw with an adequate number of blades and alignment wheels.

Provide within the Project Limits spare saw blades and at least one standby saw that meets the above requirements.

1**************************************************************************************************************************1

Section 1009 – HMA Plant Equipment

1009.01  HMA Plant

A.	Requirements for HMA Mixing Plants.

1**************************************************************************************************************************1

BDC24S-06 dated May 28, 2024

4.	Equipment for Preparation of Asphalt Binder.

The 4th paragraph in Part 4 is changed to:

Provide valves according to ASTMD 140, except ensure that a sampling valve is also located in the lowest third of each storage tank.  If any additive is added at the HMA plant, provide a sampling valve according to AASHTO M 156, Section 4.3.4.

1**************************************************************************************************************************1

1**************************************************************************************************************************1

BDC20S-09 dated Jul 6, 2020

8.	Safety.

THE 3rd paragraph in part 8 is changed to:

When plant production occurs during night operations, provide permanently fixed lighting throughout the plant operations, plant laboratory, and truck scale areas to ensure a clear view of the operations.  Also provide permanently mounted lighting at the sampling platforms to sufficiently illuminate the bed of the truck for inspection and sampling operations.

1**************************************************************************************************************************1

1009.03  Asphalt-Rubber Binder Blending Equipment

1**************************************************************************************************************************1

BDC24S-06 dated May 28, 2024

The 5th paragraph is changed to:

Provide valves according to ASTMD 140, except ensure that a sampling valve is also located in the lowest third of each storage tank.

1**************************************************************************************************************************1

NJDOT Test Methods

NJDOT R-1 – Determining Ride Quality of Pavement Surfaces

B.	Apparatus.

1**************************************************************************************************************************1

BDC21S-06 dated Jun 11, 2021

Part b is changed to:

Use the following apparatus:

1.	Class 1 IPS that meets the requirements of ASTM E 950, Sections 4.0, 5.0 and 6.0 of AASHTO M 328, and the following:

a.	Valid certification.

b	Recertification after any major component repairs or replacements.

c.	The data system provides the raw profile data in format readable in ProVal.

d.	Current version of  pavement profile analysis software installed on the IPS computer to compute the IRI.

2.	Base plate and gauge blocks, of 1 inch and 2 inch thickness, provided by the manufacturer to verify daily vertical calibration.

3.	Retro-reflective traffic marking tape or other approved mechanism to automatically trigger the start and stop of profile measurements.

1**************************************************************************************************************************1

C.	Procedure.

1**************************************************************************************************************************1

BDC21S-06 dated Jun 11, 2021

Part C is changed to:

Perform the following steps:

1.	Turn on the inertial profiler and warm up all electronic equipment in accordance with the manufacturer recommendations before testing.

2.	Perform Block and Bounce tests each day before collecting data. Record the results in the calibration log. Ensure tolerances are within the certified limits.

3.	Ensure retro-reflective traffic marking tape or other approved mechanism is placed at the beginning and end of each direction of travel lane.

4.	Enter project information in the test equipment system.

5.	Make provisions to start and stop recording profile at the beginning and end of testing.  If an automatic trigger mechanism is not installed, make provision to initiate start and end of data recording manually by pressing an appropriate key(s) on the computer.

6.	Ensure that the required speed, as recommended by the manufacturer, is achieved and that the system is collecting profile data before recording profile.

2**************************************************************************************2

BDC26S-02 dated MAY 29, 2026

part 7 is changed to:

7.	For each test section, perform 3 test runs to collect data from two test areas of each lane in the longitudinal direction of travel.  The test areas are defined as longitudinal strips with the inside edge of each test area located approximately 15 inches from the centerline of the lane on both the left and right sides and the outside edge extending to the lane striping for the full length of the paving limits.  Lanes are defined by striping.

2**************************************************************************************2

8.	Save data from each run separately before the next run or lane testing, clearly identifying each test run, lane identification, and run number.

1**************************************************************************************************************************1

Attachments

INclude fhwa attachments for FHWA funded projects

1*********************************************************************************************************************1

upon receiving the information, fill in the blank (GOALS FOR THIS PROJECT) on the appropriate FHWA attachment.

(FHWA ATTAchment NO. 1 for ESBE or FHWA attachment NO. 1 for DBE)

delete the inappropriate fhwa ATTACHMENT

FEDERAL AID PROJECT ATTACHMENT 1

DISADVANTAGED BUSINESS ENTERPRISE UTILIZATION

This page left intentionally blank. There is no DBE or ESBE goal for the project.

FEDERAL AID PROJECT ATTACHMENT 2

SPECIFIC EQUAL EMPLOYMENT OPPORTUNITY RESPONSIBILITIES ON NJDOT FEDERAL AID PROJECTS

(23 CFR, PART 230, SUBPART A, APPENDIX A TO SUBPART A - SPECIAL PROVISIONS)

The Contractor is obligated to comply with the policies, procedures and guidelines relative to the implementation of an Equal Employment Opportunity Program on Federal and Federal Aid Highway construction contracts, except for those contracts awarded under 23 U.S.C. 117, and to the preparation and submission of reports pursuant thereto as per 23 CFR, Part 230, Subpart A, Appendix A to Subpart A - Special Provisions.

A.	General

1.	Equal Employment Opportunity Requirements.  Equal employment opportunity requirements not to discriminate and to take affirmative action to assure equal employment opportunity as required by Executive Order 11246 and Executive Order 11375 are set forth in Required Contract, Provisions (Form FHWA-1273 or 1316, as appropriate) and these Special Provisions which are imposed pursuant to 23 USC 140, as established by Section 22 of the Federal Aid Highway Act of 1968.  The requirements set forth in the Contract constitute the specific affirmative action requirements for project activities under this Contract and supplement the equal employment opportunity requirements set forth in the Required Contract Provisions.

2.	The Contractor will work with the State agencies and the Federal Government in carrying out Equal Employment Opportunity obligations and in their review of activities under the contract.

3.	The Contractor, and all subcontractors holding subcontracts, not including material suppliers, of $10,000 or more, will comply with the following minimum specific requirement activities of Equal Employment Opportunity:  (The equal employment opportunity requirements of Executive Order 11246, as set forth in Volume 6, Chapter 4, Section 1, Subsection 1 of the Federal-Aid Highway Program Manual, are applicable to material suppliers, as well as contractors and subcontractors.)  The Contractor will include these requirements in every subcontract of $10,000 or more with such modification of language as is necessary to make them binding on the subcontractor.

B.	Equal Employment Opportunity Policy.  The Contractor will accept as its operating policy the following statement which is designed to further the provisions of equal employment opportunity to all persons without regard to their race, color, religion, sex, or national origin, and to promote the full realization of equal employment opportunity through a positive continuing program:

“It is the policy of this Company to assure that applicants are employed, and that employees are treated during employment, without regard to their race, religion, sex, sexual orientation, gender identity, color, national origin, age or disability.  Such action shall include:  employment, upgrading, demotion, or transfer; recruitment or recruitment advertising; layoff or termination; rates of pay or other forms of compensation; and selection for training, including apprenticeship, pre-apprenticeship, and/or on-the-job training.”

C.	Equal Employment Opportunity Officer.  The Contractor will designate and make known to the Department contracting officers an Equal Employment Opportunity Officer (hereinafter referred to as the EEO Officer) who will have the responsibility for and must be capable of effectively administering and promoting an active EEO contractor program and who must be assigned adequate authority and responsibility to do so.

D.	Dissemination of Policy

1.	Implementation.  All members of the Contractor’s staff who are authorized to hire, supervise, promote, and discharge employees, or who recommended such action, or who are substantially involved in such action, will be made fully cognizant of, and will implement, the Contractor’s equal employment opportunity policy and contractual responsibilities to provide equal employment opportunity in each grade and classification of employment.  To ensure compliance with the above agreement, the following actions will be taken as a minimum:

a.	Initial Project Site Meeting.  Conduct an initial project site meeting with key supervisory and office personnel before or at the start of work, and then not less than once every 6 months, at which time the Contractor’s Equal Employment Opportunity policy and its implementation will be reviewed and explained.  The meetings will be conducted by the EEO Officer.

b.	EEO Obligations.  All new supervisory and office personnel will be given a thorough indoctrination by the EEO Officer or other knowledgeable company official covering all major aspects of the Contractor’s equal employment opportunity obligations within 30 days following their reporting for duty with the Contractor.

c.	All personnel engaged in direct recruitment for the project will be instructed by the EEO Officer or appropriate company official in the Contractor’s procedures for locating and hiring minority and female employees.

2.	Take the following actions to  make the Contractor’s equal employment opportunity policy known to all employees, prospective employees and potential sources of employees, i.e., schools, employment agencies, labor unions (where appropriate), college placement officers, etc.:

a.	Place notices and posters setting forth the Contractor’s equal employment opportunity policy in areas readily accessible to employees, applicants for employment and potential employees.

b.	Bring the Contractor’s equal employment opportunity policy and the procedures to implement such policy to the attention of employees by means of meetings, employee handbooks, and/or other appropriate means.

E.	Recruitment

1.	When advertising for employees, include in all advertisements for employees the notation: “An Equal Opportunity Employer”.  Publish all such advertisements in newspapers or other publications having a large circulation among minorities and women in the area from which the project work force would normally be derived.

2.	Unless precluded by a valid bargaining agreement, conduct systematic and direct recruitment through public and private employee referral sources likely to yield qualified minority and female applicants, including, but not limited to, State employment agencies, schools, colleges and minority-group organizations.  To meet this requirement, the Contractor will, through their EEO Officer, identify sources of potential minority and female group employees, and establish procedures with such identified sources whereby minority and female group applicants may be referred to the Contractor for employment consideration.

3.	In the event the Contractor has a valid bargaining agreement providing for exclusive hiring hall referrals, he/she is expected to observe the provisions of that agreement to the extent that the system permits the Contractor’s compliance with the equal employment opportunity contract provisions.  (The US Department of Labor has held that where implementation of such agreements have the effect of discriminating against minorities or women, or obligates the Contractor to do the same, such implementation violates Executive Order 11246, as amended).

4.	Encourage present employees to refer minority and female applicants for employment by posting appropriate notices or bulletins in areas accessible to all such employees.  In addition, information and procedures pertaining to the referral of applicants will be discussed with employees.

F.	Personnel Actions.  Wages, working conditions and employee benefits shall be established and administered, and personnel actions of every type, including hiring, upgrading, promotion, transfer, demotion, layoff, and termination, shall be taken without regard to race, color, religion, sex, sexual orientation, gender identity, national origin, age or disability.  The following procedures shall be followed:

1.	Conduct a project site inspection at the start of work, and periodically thereafter, to ensure that working conditions and employee facilities do not indicate discriminatory treatment of project site personnel.

2.	Periodically evaluate the spread of wages paid within each classification to determine any evidence of discriminatory wage practices.

3.	Periodically review selected personnel actions in depth to determine whether there is evidence of discrimination.  Where evidence is found, the Contractor will promptly take corrective action.  If the review indicates that the discrimination may extend beyond the actions reviewed, such corrective action shall include all affected persons.

4.	Promptly investigate all complaints of alleged discrimination made to the Contractor in connection with its obligations under this Contract, and will resolve or attempt to resolve such complaints, within a reasonable time.  If the investigation indicates that the discrimination may affect persons other than the complainant, corrective action shall include such other persons.  Upon completion of each investigation, inform complainants of all their avenues of appeal.

G.	Training and Promotion

1.	Assist in locating, qualifying, and increasing the skills of minority and women who are applicants for employment or current employees.

2.	Advise employees and applicants for employment of available training programs and entrance requirements for each.

3.	Periodically review the training and promotion potential of minority and female employees and encourage eligible employees to apply for such training and promotion.

H.	On-the-Job Training.  The Contractor, as part of their equal employment opportunity affirmative action program, shall provide on-the-job training aimed at developing full journey people in the type of craft or job classification involved on the project.

1.	Apprenticeship and Training Programs

The minimum length and type of training for each position will be established in the training program selected by the Contractor and approved by the Department and the Federal Highway Administration.  The Department will approve a program if it is reasonably calculated to meet the equal employment opportunity obligations of the Contractor and to qualify the average apprentice or trainee for journeyperson status in the craft concerned by the end of the training period.

Apprenticeship programs registered with the US Department of Labor, Bureau of Apprenticeship and Training, (BAT) or with a State apprenticeship agency recognized by USDOL BAT and training programs approved but not necessarily sponsored by the US Department of Labor, Employment and Training Administration, Bureau of Apprenticeship and Training shall also be considered acceptable provided such programs are being administered in a manner consistent with the equal employment obligations of Federal-aid highway contracts.  Approval or acceptance of a training program shall be obtained from the DCR/AA prior to commencing work in the classifications covered by the Contractor’s training program.  The Division will review guidelines developed by the Contractor for approval or disapproval in accordance with the Training Guideline Approval Process described in the “Revised Standard Training Guidelines”.  The Division will also review existing guidelines for revision based on the same process.

It is the intention of these provisions that training be provided in construction crafts rather than clerk-typist or secretarial-type positions.  Training is permitted in lower level management positions (e.g., timekeepers), where the training is oriented toward project site applications.  Training in semi-skilled laborer positions is permitted provided that significant and meaningful training is available on the project site and approved by DCR/AA.  Some offsite, classroom training (e.g., safety, first aid instruction) may be permitted as long as such training is an integral part of an approved training program and does not comprise a significant part of the overall training.

2.	Contractor Submission and Department Approval of the Initial Training Program

At or after the preconstruction conference, and prior to the start of Work, submit a Training Program to the RE for review and comments prior to DCR/AA review and approval.  The Contractor’s training program shall include:

a.	Number of trainees or apprentices to be trained in all selected Training Positions,

b.	Standard Program Hours for all positions,

c.	Estimate of the Minimum Available Hours actually feasible on the project toward completion of the Standard Program Hours per position,

d.	Training schedule of Estimated Start Dates for the apprentices or trainees, developed and coordinated with the project’s work progress schedule,

e.	Training Guidelines for all positions, and

f.	Training that will be provided by the Contractor and provided by Subcontractors.

The number of apprentices and trainees shall be distributed among the work classifications on the basis of the Contractor’s needs and the availability of journey people in the various crafts within a reasonable area of recruitment.  Submit timely, revised Training Programs, as required throughout the project to ensure that feasible and Maximum Available Training is provided.  Maximum Available Training is defined as bringing each apprentice or trainee onto the project when work first becomes available in his/her craft and providing all available training until hours are no longer available.

3.	Assignment of Training to Subcontractors

In the event that portions of the Contract work are subcontracted, determine how many, if any, of the apprentices or trainees are to be trained by subcontractors, provided, however, that the Contractor shall retain the primary responsibility for meeting the training requirements imposed by these Training Special Provisions.  Ensure that these Training Special Provisions are made applicable to such subcontracts.

4.	Reimbursement of the Contractor for Providing Training

The Contractor will be credited for each apprentice or trainee employed on the construction site who is currently enrolled or becomes enrolled in an approved program and will be reimbursed for such apprentices or trainees as provided hereinafter.  Payment will be made under the pay item Trainees at the bid price in the Proposal per person-hour of training given an employee on this contract in accordance with an approved training program. If approved, payment will be made for training persons in excess of the number specified herein.  This reimbursement will be made even though the Contractor receives additional training program funds from other sources, provided such other sources do not specifically prohibit the Contractor from receiving other reimbursement.  Offsite, classroom training reimbursement may only be made to the Contractor when the company does one or more of the following and the apprentices or trainees are concurrently employed on a Federal-aid project:  contributes to the cost of the training and/or provides instruction to apprentices or trainees or pays their wages during the offsite, classroom training (e.g., safety, first aid instruction) period.

Pay apprentices and trainees according to the project-specific New Jersey Department of Labor Prevailing Wage Rate Determination for the project.  Trainees will be paid at least 60 percent of the appropriate minimum journeyman's rate specified in the Contract for the first half of the training period, 75 percent for the third quarter of the training period, and 90 percent for the last quarter of the training period, unless apprentices or trainees in an approved existing program are enrolled as trainees on this project.  In that case, the appropriate rates approved by the Departments of Labor or Transportation in connection with the existing program shall apply to all trainees being trained for the same classification who are covered by this Training Special Provision.

I.	Apprentice/Trainee Requirements of the Contract

The number of training positions will be____0____, where feasible, consisting of at least___0___ APPRENTICES and ___0___TRAINEES.  TRAINEE HOURS=_____0_____.

Apprentices are defined as registered members of an approved apprenticeship program recognized by the United States Department of Labor (USDOL) Bureau of Apprenticeship and Training (BAT) or a New Jersey State apprenticeship agency recognized by USDOL BAT (e.g., New Jersey Department of Education).  Graduates of the Pre-Apprenticeship Training Cooperative Program shall be classified as apprentices.  Trainees are defined as skilled, semi-skilled or lower level management individuals receiving training per one of the approved NJDOT “Revised Standard Training Guidelines” (available from the DCR/AA).

Where feasible, assign at least 50% of the training positions to Skilled Crafts which include but are not limited to Carpenters, Dockbuilders, Electricians, Ironworkers and Operating Engineers.

1.	Requirements for Recruitment, Selection and Approval of Apprentices and Trainees

Apprentices or trainees should be in their first year of apprenticeship or training.  Interview and screen trainee candidates to determine if their actual work experience is equivalent to or exceeds that offered by the training program prior to submitting candidates on the Apprentice/Trainee Approval Memorandum (Form CR-1), via the RE, to the Division for review and approval or disapproval.

Training and upgrading of minorities (e.g., Blacks, Asians or Pacific Islanders, Native Americans or Alaskan Natives, Hispanics) and women toward journeyperson status is a primary objective of these Training Special Provisions.  Accordingly, the Contractor shall make every effort to enroll minorities and women, by conducting systematic and direct recruitment through public and private sources likely to yield minority and female apprentices or trainees, to the extent that such persons are available within a reasonable area of recruitment.  This training commitment is not intended, and shall not be used, to discriminate against any applicant for training, whether a member of a minority group or not.

No employee shall be employed as an apprentice or trainee in any position in which he or she has successfully completed a training course leading to journeyperson status or in which he or she has been employed as a journeyperson.  The Contractor shall satisfy this requirement by including appropriate questions in the employment application or by other suitable means and by submitting an accurate and complete “Apprentice/Trainee Approval Memorandum” (Form CR-1) prior to the apprentice or trainee starting work on the project.  Regardless of the methods used, the Contractor’s records should document the findings in each case.

Skilled craft trainees may complete up to 3,000 total training hours on Department projects, with an extension of an additional 1,000 hours permitted on a case-by-case basis.  Semi-skilled and lower-level management trainees attain journeyperson status upon completion of a training guideline and may complete up to three (3) different positions.

2.	Documentation Required to be Signed by Apprentices or Trainees, and provided to the Department

Prior to the apprentice/trainee starting work on the project, submit an accurate, complete and signed Apprentice/Trainee Approval Memorandum for each apprentice/trainee to the RE for review, and final approval by DCR/AA.  Once the notice that said apprentice/trainee has been approved to work on the Contract, said apprentice/trainee may start work on the Contract.  No credit will be given for apprentices/trainees prior to said apprentice/trainee being approved by DCR/AA.

At the start of training, provide the RE and each apprentice or trainee with an applicable “Training Guideline” and, at the conclusion of training, an accurate and complete “Training Certificate for Reporting Hours to NJDOT” (Form CR-3), showing hours of training satisfactorily completed.

Maintain and submit an accurate and complete “NJDOT Contractor’s 1409 Quarterly Training Report” (Form-CR-1409) to the RE within ten (10) days of the end of each training quarter (e.g., January 10, April 10, July 10, October 10); also provide a copy to each apprentice or trainee.

Maintain and submit accurate and complete “Biweekly Training Reports” (Form CR-2) to the RE, and each apprentice or trainee, as periodic reports documenting their performance under the Contract.

3.	Determining Good Faith Compliance of Contract Apprentice/Trainee Program

Per the approved program or guideline, provide Maximum Available Training to apprentices and trainees by beginning their training as soon as feasible with the start of craft work utilizing the skill involved on the project construction site and by retaining them as long as training opportunities exist in their crafts or until their training program positions are completed.

Recall apprentices or trainees released due to reductions in force when the work scope permits and they are available to return.  When they are unavailable to resume training on the project site, submit written proof of recall efforts and replacement candidates and/or positions in a timely manner.  Do not terminate apprentices or trainees prior to completion of their training program positions without Department consultation and authorization.  Apprentices or trainees are not required to be on board for the entire length of the Contract.

The Contractor shall have fulfilled the contractual responsibilities under these Training Special Provisions as specified in 23 CFR 230, Appendix B to Subpart A of Part 230 if the company has provided Acceptable Training to the number of apprentices or trainees specified in this contract and/or by providing the remaining hours required to complete training positions begun by apprentices or trainees on other projects.  The number trained shall be determined on the basis of the total number enrolled on the Contract for a significant period.

Demonstrate all steps that have been taken in pursuance of enrolling minorities and women in the training program positions, prior to a determination as to whether the Contractor is in compliance with the Training Special Provisions of the Contract.

Submit to the RE written training program summaries at the 50% time and/or cost stage of the contract and also prior to project completion, describing all good faith efforts and particularly addressing Maximum Available Training for incomplete training positions, per the procedure found in the revised “Instructions for Implementing the Training Special Provisions”.

4.	Enforcement Measures and Contractor’s Rating

Payment will not be made if either the failure to provide the required training or the failure to hire the apprentice or trainee as a journeyperson is caused by the Contractor and evidences a lack of good faith on the part of the Contractor in meeting the requirements of the Contract.

Per established procedures and scheduled Contract Compliance Reviews, the Contractor’s performance will be rated and reviewed periodically by the Department.

Failure of a Contractor to comply with the Training Special Provisions of the Contract, and as specified in 23 CFR Part 230, Appendix B to Subpart A of Part 230, may result in the actions as set forth as specified in Section 105.

J.	Unions.  If the Contractor relies in whole or in part upon unions as a source of employees, use good faith efforts to obtain the cooperation of such unions to increase opportunities for minorities and women within the unions, and to effect such union referrals to the construction project.  Actions by the Contractor, either directly or through a contractor’s association acting as agent, will include the procedures set forth below:

1.	Use maximum effort to develop, in cooperation with the unions, joint training programs aimed at qualifying more minorities and women for union membership and increasing their skills in order for them to qualify for higher paying employment.

2.	Use maximum effort to incorporate an equal employment opportunity clause into each union agreement to the end that such union will be contractually bound to refer applicants without regard to race, color, religion, sex, sexual orientation, gender identity, national origin, age, or disability.

3.	Obtain information concerning the referral practices and policies of the labor unions except that to the extent such information is within the exclusive possession of the labor unions and such labor unions refuse to furnish this information to the Contractor, certify to the Department and set forth what efforts have been made to obtain this information.

4.	In the event the unions are unable to provide the Contractor with a reasonable flow of minority and female referrals within the time limit set forth in the collective bargaining agreement, through independent recruitment efforts, fill the employment vacancies without regard to race, color, religion, sex, sexual orientation, gender identity, national origin, age or disability, making full efforts to obtain qualified and/or qualifiable minorities and women.  (The US Department of Labor has held that it shall be no excuse that the union with which the Contractor has a collective bargaining agreement providing for exclusive referral failed to refer minority employees).  In the event the union referral practice prevents the Contractor from meeting the obligations pursuant to Executive Order 11246, as amended, and these special provisions, such Contractor shall immediately notify the Department.

K.	Subcontracting

1.	Use maximum effort to solicit bids from and to utilize minority subcontractors or subcontractors with meaningful minority and female representation among their employees.  The Contractor may use lists of minority-owned construction firms as issued by the Department.

2.	Ensure subcontractor compliance with the Contract Equal Employment Opportunity obligations.

L.	Records and Reports

1.	Maintain records necessary to determine compliance with the Contractor’s equal employment opportunity requirements.  Documents will include the following:

a.	Number of minorities, non-minorities, and women employed in each work classification on the Contract.

b.	Progress and efforts being made in cooperation with unions to increase employment opportunities for minorities and women (applicable only to Contractors who rely in whole or in part on unions as a source of their work force).

c.	Progress and efforts being made in locating, hiring, training, qualifying, and upgrading minority and female employees, and

d.	Progress and efforts being made in securing the services of minority and female subcontractors or subcontractors with meaningful minority and female representation among their employees.

2.	All such documents must be retained for a period of 3 years following completion of the Contract work and shall be available at reasonable times and places for inspection by authorized representatives of the Department and the Federal funding agencies.

FEDERAL AID PROJECT ATTACHMENT 3

REQUIREMENTS FOR AFFIRMATIVE ACTION TO ENSURE EQUAL EMPLOYMENT OPPORTUNITY ON NJDOT FEDERAL AID PROJECTS

(MINORITY AND WOMEN WORK EMPLOYMENT GOAL OBLIGATIONS)

A.	Employment Goals.

The goals for minority and female participation, in the covered area, expressed in percentage terms for the Contractor’s aggregate work force in each trade, on all construction work are:

| Minority And Women Employment Goal Obligations For Construction Contractors and Subcontractors on Federal Aid Projects |  |  |
| --- | --- | --- |
| County | Minority Participation | Women Participation |
|  | Percent | Percent |
| Atlantic | 18.2 | 6.9 |
| Bergen | 22.6 | 6.9 |
| Burlington | 17.3 | 6.9 |
| Camden | 17.3 | 6.9 |
| Cape May | 14.5 | 6.9 |
| Cumberland | 16.0 | 6.9 |
| Essex | 17.3 | 6.9 |
| Gloucester | 17.3 | 6.9 |
| Hudson | 12.8 | 6.9 |
| Hunterdon | 17 | 6.9 |
| Mercer | 16.4 | 6.9 |
| Middlesex | 5.8 | 6.9 |
| Monmouth | 9.5 | 6.9 |
| Morris | 17.3 | 6.9 |
| Ocean | 17 | 6.9 |
| Passaic | 12.9 | 6.9 |
| Salem | 12.3 | 6.9 |
| Somerset | 17.3 | 6.9 |
| Sussex | 17 | 6.9 |
| Union | 17.3 | 6.9 |
| Warren | 1.6 | 6.9 |

These goals are applicable to all the Contractor’s construction work (whether or not it is Federal or Federally assisted) performed in the covered area.  If the Contractor performs construction work in a geographical area located outside of the covered area, it shall apply the goals established for such geographical area where the work is actually performed.  With regard to this second area, the Contractor also is subject to the goals for both its Federally involved and non-Federally involved construction.

The Contractor’s compliance with the Executive Order 11246 and the regulations in 41 CFR Part 60-4 shall be based on its implementation of the Equal Opportunity Clause, specific affirmative action obligations required by the specifications set forth in 41 CFR 60-4(3)a, and its efforts to meet the goals.  The hours of minority and female employment and training must be substantially uniform throughout the length of the Contract, and in each trade.  Make a good faith effort to employ minorities and women evenly on each project.  The transfer of minority or female employees or trainees from Contractor to Contractor or from project to project for sole purpose of meeting the Contractor’s goals is a violation of the Contract, the Executive Order and the regulations in 41 CFR Part 60-4.  Compliance with the goals will be measured against the total work hours performed.

If a project is located in more than one county, the minority work hour goal, only, will be determined by the county which serves as the primary source of hiring or, if workers are obtained almost equally from one or more counties, the single minority goal will be the average of the affected county goals.

B.	Reporting Requirements.

1.	Provide the Department with written notification in triplicate within 10 working days of award of any construction subcontract in excess of $10,000 at any tier for construction work under the contract resulting from this solicitation.  The notification will list the name, address and telephone number of the subcontractor; employer identification number of the subcontractor; estimated dollar amount of the subcontract; estimated starting and completion dates of the subcontract; and the geographical area in which the contract is to be performed.

2.	Directly provide the Department with employment workforce data of the number and work hours of minority and non-minority group members and women employed in each work classification for the Contract.  The Contractor, subcontractors, professional service firms and others working on the project must submit this information via a web-based application through the New Jersey portal, Vendor Workforce Reporting Manager.  Instructions on how to complete Form CC-257R are provided within the web application.  Instructions for registering and receiving the authentication code to access the web based application can be found at the Contractor Manpower Project Reporting CC-257R website at: http://www.state.nj.us/transportation/business/civilrights/pdf/cc257.pdf.

a.	On a monthly basis, submit Form CC-257R through the web based application within 10 days following the end of each reporting month.

b.	In addition to the above, submit a hard copy of the electronic Form CC-257R to the RE within 10 days following the end of each reporting month

c.	Submit a copy of the confirmation e-mail of the successful submission of Monthly Employment Utilization Report to the RE within 10 days following the end of each reporting month.

3.	All employment data must be accurate and consistent with the certified payroll records.  The Contractor is responsible for ensuring compliance with these reporting requirements.  Failure of the Contractor, subcontractors, professional service firms and others working on the Contract, to report monthly employment data may result in payments being delayed or withheld as per 105.01, or impact the Contractor’s prequalification rating with the Department.

FEDERAL AID PROJECT ATTACHMENT 4

FEDERAL EQUAL EMPLOYMENT OPPORTUNITY CONTRACT SPECIFICATIONS FOR NJDOT FEDERAL AID PROJECTS

(AS REQUIRED PER EXECUTIVE ORDER 11246 AS AMENDED BY EXECUTIVE ORDER 11375 AND IMPLEMENTING REGULATIONS AT 41 C.F.R. PART 60)

A.	As used in these Specifications:

1.	Covered area means the County or Counties in which the Project is located.

2.	Director means Director, Office of Federal Contract Compliance Programs, United States Department of Labor or any person to whom the Director delegates authority.

3.	Employer identification number means the Federal Social Security number used on the Employer’s Quarterly Federal Tax Return, US Treasury Department Form 941.

4.	Minority includes:

a.	Black (a person having origins in any of the black African racial groups not of Hispanic origin);

b.	Hispanic (a person of Mexican, Puerto Rican, Cuban, Central or South American or other Spanish culture or origin, regardless of race);

c.	Asian and Pacific Islander (a person having originals in any of the original peoples of the Far East, Southeast Asia, the Indian Subcontinent, or the Pacific Islands); and

d.	American Indian or Alaskan Native (a person having origins in any of the original peoples of North America and maintaining identifiable tribal affiliations through membership and participating or community identification).

B.	Whenever the Contractor, or any subcontractor at any tier, subcontracts a portion of the work involving any construction trade, it shall physically include in each subcontract in excess of $10,000 the provisions of these specifications and the Notice which contains the applicable goals for minority and female participation and which is set forth in the solicitations from which this contract resulted.

C.	Implement the specific affirmative action standards provided in paragraphs F1 through 16 of these specifications.  The goals set forth in the solicitation from which this contract resulted are expressed as percentages of the total hours of employment and training of minority and female utilization the Contractor should reasonably be able to achieve in each construction trade in which it has employees in the covered area.  Covered Construction Contractors performing construction work in geographical areas where they do not have a Federal or Federally assisted construction contract shall apply the minority and female goals established for the geographical area where the work is being performed.  The Contractor is expected to make substantially uniform progress in meeting its goals in each craft during the period specified.

D.	Neither the provisions of any collective bargaining agreement nor the failure by a union with whom the Contractor has a collective bargaining agreement to refer either minorities or women excuses the Contractor’s obligations under these Specifications, Executive Order 11246, or the regulations promulgated pursuant thereto.

E.	In order for the nonworking training hours of apprentices and trainees to be counted in meeting the goals, such apprentices and trainees must be employed by the Contractor during the training period, and the Contractor must have made a commitment to employ the apprentices and trainees at the completion of their training, subject to the availability of employment opportunities.  Trainees must be trained pursuant to training programs approved by the US Department of Labor.

F.	Take specific affirmative action to ensure equal employment opportunity.  The evaluation of the Contractor’s compliance with these specifications shall be based upon its effort to achieve maximum results from its actions.  Document these efforts fully, and shall implement affirmative action steps at least as extensive as the following:

1.	Ensure and maintain a working environment free of harassment, intimidation, and coercion at all sites, and in all facilities at which the Contractor’s employees are assigned to work.  Where possible, assign two or more women to each construction project.  Specifically ensure that all foreman, superintendents, and other on-site supervisory personnel are aware of and carry out the Contractor’s obligation to maintain such a working environment with specific attention to minority or female individuals working at such sites or in such facilities.

2.	Establish and maintain a current list of minority and female recruitment sources, provide written notification to minority and female recruitment sources and to community organizations when the Contractor or its unions have employment opportunities available, and maintain a record of the organizations’ responses.

3.	Maintain a current file of the names, addresses and telephone numbers of each minority and female off-the-street applicant and minority or female referral from a union, a recruitment source or community and of what action was taken with respect to each such individual.  If such individual was sent to the union hiring hall for referral and was not referred back to the Contractor by the union or, if referred back to the Contractor, this shall be documented in the file with the reason therefore, along with whatever additional actions the Contractor may have taken.

4.	Provide immediate written notification to the Director when the union or unions with which the Contractor has a collective bargaining agreement has not referred to the Contractor a minority person or woman sent by the Contractor, or when the Contractor has other information that the union referral process has impeded the Contractor’s efforts to meet its obligations.

5.	Develop on-the-job training opportunities and/or participate in training programs for the area which expressly include minorities and women including upgrading programs and apprenticeship and trainee programs relevant to the Contractor’s employment needs, especially those programs funded or approved by the Department of Labor.  The Contractor shall provide notice of these programs to the source compiled under F2 above.

6.	Disseminate the Contractor’s EEO policy by providing notice of the policy to unions and training programs and requesting their cooperation in assisting the Contractor in meeting its EEO obligations by including it in any policy manual and collective bargaining agreement; by publicizing it in the company newspaper, annual report, etc. by specific review of the policy with all management personnel and with all minority and female employees at least once a year; and by posting the company EEO policy on bulletin boards accessible to all employees at each location where construction work is performed.

7.	Review, at least annually, the company’s EEO policy and affirmative action obligations under these specifications with all employees having any responsibility for hiring, assignment, layoff, termination or other employment decisions including specific review of these items with on-site supervisory personnel such as Superintendents, General Foremen, etc., prior to the initiation of construction work at any job site.  A written record shall be made and maintained identifying the time and place of these meetings, persons attending, subject matter discussed, and disposition of the subject matter.

8.	Disseminate the Contractor’s EEO policy externally by including it in any advertising in the news media, specifically including minority and female news media, and providing written notification to and discussing the Contractor’s EEO policy with other Contractors and Subcontractors with whom the Contractor does or anticipates doing business.

9.	Direct its recruitment efforts, both oral and written, to minority, female and community organizations, to schools with minority and female students and to minority and female recruitment and training organizations serving the Contractor’s recruitment area and employment needs.  Not later than one month prior to the date for the acceptance of applications for apprenticeship or other training by any recruitment source, the Contractor shall send written notification to organizations such as the above, describing the openings, screening procedures, and tests to be used in the selection process.

10.	Encourage present minority and female employees to recruit other minority persons and women and, where reasonable, provide after school, summer and vacation employment to minority and female youth both on the site and in other areas of a Contractor’s work force.

11.	Validate all tests and other selection requirements where there is an obligation to do so under 41 CFR Part 60-3.

12.	Conduct, at least annually, an inventory and evaluation at least of all minority and female personnel for promotional opportunities and encourage these employees to seek or to prepare for, through appropriate training, etc., such opportunities.

13.	Ensure that seniority practices, job classifications, work assignments and other personnel practices do not have a discriminatory effect by continually monitoring all personnel and employment related activities to ensure that the EEO policy and the Contractor’s obligations under these specifications are being carried out.

14.	Ensure that all facilities and company activities are provided in a manner such that segregation on the basis of race, color, religion, sex, sexual orientation, gender identity, national origin cannot result.  Do not require such segregated use by written or oral policies, nor tolerate such use by employee custom.  Provide separate or single-user restrooms and necessary dressing or sleeping areas to assure privacy between sexes.

15.	Document and maintain a record of all solicitations of offers for subcontracts from minority and female construction Contraction and suppliers, including circulation of solicitations to minority and female contractor associations and other business associations.

16.	Conduct a review, at least annually, of all supervisors’ adherence to and performance under the Contractor’s EEO policies and affirmative action obligations.

G.	Contractors are encouraged to participate in voluntary associations which assist in fulfilling one or more of their affirmative action obligations (F1 through 16).  The efforts of a Contractor association, joint contractor union, Contractor-Community, or other similar group of which the Contractor is a member and participant may be asserted as fulfilling any one or more of its obligations under F1 through 16 of these Specifications provided that the Contractor actively participates in the group, makes every effort to assure that the group has a positive impact on the employment of minorities and women in the industry, ensures that the concrete benefits of the program are reflected in the Contractor’s minority and female work force participation, makes a good faith effort to meet its individual goals and timetables, and can provide access to documentation which demonstrates the effectiveness of actions taken on behalf of the Contractor. The obligation to comply, however, is the Contractor’s and failure of such a group to fulfill an obligation shall not be a defense for the Contractor’s noncompliance.

H.	A single goal for minorities and a separate single goal for women have been established.  The Contractor, however, is required to provide equal employment opportunity and to take affirmative action for all minority groups, both male and female, and all women both minority and nonminority.  Consequently, the Contractor may be in violation of the Executive Order if a particular group is employed in a substantially disparate manner (for example, even though the Contractor has achieved its goals for women generally, the Contractor may be in violation of the Executive Order if a specific minority group of women is underutilized).

I.	Do not use the goals and timetables or affirmative action standards to discriminate against any person because of race, creed, color, national origin, age, ancestry, nationality, gender, disability, sex, affectional or sexual orientation, gender identity or expression, religion, and liability for military service.

J.	Do not enter into any Subcontract with any person or firm debarred from Government contracts pursuant to Executive Order 11246.

K.	Carry out such sanctions and penalties for violation of these specifications and of the Equal Opportunity Clause, including suspensions, termination and cancellation of existing subcontracts as may be imposed or ordered pursuant to Executive Order 11246, as amended, and its implementing regulations, by the Office of Federal Contract Compliance Programs.  Any Contractor who fails to carry out such sanctions and penalties shall be in violation of these specifications and Executive Order 11246 as amended.

L.	Implement specific affirmative action steps, at least as extensive as those standards prescribed in paragraph F of these specifications, so as to achieve maximum results from its efforts to ensure equal employment opportunity.  If the Contractor fails to comply with the requirements of the Executive Order, the implementing regulations or these specifications, the Director shall proceed in accordance with 41 CFR 60-4.8.

M.	Designate a responsible official to monitor all employment related activity to ensure that the company EEO policy is being carried out, to submit reports relating to the provisions hereof as may be required by the Government and to keep records.  Records shall at least include for each employee the name, address, telephone number, construction trade, union affiliation if any, employee identification number when assigned, social security number, race, sex, status (such as mechanic, apprentice, trainee, helper, or laborer), dates of changes in status, hours worked per week in the indicated trade, rate of pay, and locations at which the work was performed.  Records shall be maintained in an easily understandable and retrievable form; however, to the degree that existing records satisfy this requirement, contractors shall not be required to maintain separate records.

N.	Nothing herein provided shall be construed as a limitation upon the application of other laws which establish different standards of compliance or upon the application of requirements for the hiring of local or other area residents (such as those under the Public Works Employment Act of 1977 and the Community Development Block Grant Program).

O.	Failure of the Contractor or subcontractors to comply with the nondiscrimination provisions of the Contract may result in payment being delayed or withheld as specified in 105.01; default as specified in 108.14, liquidated damages as specified in 108.20, or termination of the Contract as specified in 108.15.02 pending corrective and appropriate measures taken by the Contractor to the satisfaction of the Department.

FEDERAL AID PROJECT ATTACHMENT 5

STATE OF NEW JERSEY MANDATORY EQUAL EMPLOYMENT OPPORTUNITY LANGUAGE ON NJDOT FEDERAL AID PROJECTS

(N.J.S.A. 10:5-31 et seq. (P.L.1975, c.127) N.J.A.C. 17:27-1.1 et seq.)

During the performance of this contract, the contractor agrees as follows:

The Contractor or subcontractor, where applicable, will not discriminate against any employee or applicant for employment because of age, race, creed, color, national origin, ancestry, marital status, affectional or sexual orientation, gender identity or expression, disability, nationality or sex.  Except with respect to affectional or sexual orientation and gender identity or expression, the Contractor will ensure that equal employment opportunity is afforded to such applicants in recruitment and employment, and that employees are treated during employment, without regard to their age, race, creed, color, national origin, ancestry, marital status, affectional or sexual orientation, gender identity or expression, disability, nationality or sex.  Such equal employment opportunity shall include, but not be limited to the following: employment, upgrading, demotion, or transfer; recruitment or recruitment advertising; layoff or termination; rates of pay or other forms of compensation; and selection for training, including apprenticeship.  The Contractor agrees to post in conspicuous places, available to employees and applicants for employment, notices to be provided by the Department setting forth provisions of this nondiscrimination clause.

The Contractor or subcontractor, where applicable will, in all solicitations or advertisements for employees placed by or on behalf of the Contractor, state that all qualified applicants will receive consideration for employment without regard to age, race, creed, color, national origin, ancestry, marital status, affectional or sexual orientation, gender identity or expression, disability, nationality or sex.

The Contractor or subcontractor will send to each labor union, with which it has a collective bargaining agreement, a notice, to be provided by the agency contracting officer, advising the labor union or workers' representative of the contractor's commitments under this act and shall post copies of the notice in conspicuous places available to employees and applicants for employment.

The Contractor or subcontractor, where applicable, agrees to comply with any regulations promulgated by the Treasurer, pursuant to N.J.S.A. 10:5-31 et seq., as amended and supplemented from time to time and the Americans with Disabilities Act.

When hiring or scheduling workers in each construction trade, the Contractor or subcontractor agrees to make good faith efforts to employ minority and women workers in each construction trade consistent with the targeted employment goal prescribed by N.J.A.C. l7:27-7.2; provided, however, that the Dept. of LWD, Construction EEO Monitoring Program, may, in its discretion, exempt a contractor or subcontractor from compliance with the good faith procedures prescribed by the following provisions, A, B, and C, as long as the Dept. of LWD, Construction EEO Monitoring Program is satisfied that the Contractor or subcontractor is employing workers provided by a union which provides evidence, in accordance with standards prescribed by the Dept. of LWD, Construction EEO Monitoring Program, that its percentage of active “card carrying” members who are minority and women workers is equal to or greater than the targeted employment goal established in accordance with N.J.A.C. 17:27-7.2. The Contractor or subcontractor agrees that a good faith effort shall include compliance with the following procedures:

A.	If the Contractor or subcontractor has a referral agreement or arrangement with a union for a construction trade, the Contractor or subcontractor shall, within three business days of the contract award, seek assurances from the union that it will cooperate with the Contractor or sub-contractor as it fulfills its affirmative action obligations under this contract and in accordance with the rules promulgated by the Treasurer pursuant to N.J.S.A. 10:5-31 et. seq., as supplemented and amended from time to time and the Americans with Disabilities Act.  If the Contractor or subcontractor is unable to obtain said assurances from the construction trade union at least five business days prior to the commencement of construction work, the Contractor or sub-contractor agrees to afford equal employment opportunities minority and women workers directly, consistent with this chapter.  If the Contractor's or subcontractor's prior experience with a construction trade union, regardless of whether the union has provided said assurances, indicates a significant possibility that the trade union will not refer sufficient minority and women workers consistent with affording equal employment opportunities as specified in this chapter, the Contractor or subcontractor agrees to be prepared to provide such opportunities to minority and women workers directly, consistent with this chapter, by complying with the hiring or scheduling procedures prescribed under “B” below; and the Contractor or subcontractor further agrees to take said action immediately if it determines that the union is not referring minority and women workers consistent with the equal employment opportunity goals set forth in this chapter.

B.	If good faith efforts to meet targeted employment goals have not or cannot be met for each construction trade by adhering to the procedures of “A” above, or if the contractor does not have a referral agreement or arrangement with a union for a construction trade, the Contractor or subcontractor agrees to take the following actions:

1.	To notify the public agency compliance officer, the Dept. of LWD, Construction EEO Monitoring Program, and minority and women referral organizations listed by the Division pursuant to N.J.A.C. 17:27-5.3, of its workforce needs, and request referral of minority and women workers;

2.	To notify any minority and women workers who have been listed with it as awaiting available vacancies;

3.	Prior to commencement of work, to request that the local construction trade union refer minority and women workers to fill job openings, provided the Contractor or subcontractor has a referral agreement or arrangement with a union for the construction trade;

4.	To leave standing requests for additional referral to minority and women workers with the local construction trade union, provided the Contractor or subcontractor has a referral agreement or arrangement with a union for the construction trade, the State Training and Employment Service and other approved referral sources in the area;

5.	If it is necessary to lay off some of the workers in a given trade on the construction site, layoffs shall be conducted in compliance with the equal employment opportunity and non-discrimination standards set forth in this regulation, as well as with applicable Federal and State court decisions;

6.	To adhere to the following procedure when minority and women workers apply or are referred to the contractor or subcontractor:

a.	The Contactor or subcontractor shall interview the referred minority or women worker.

b.	If said individuals have never previously received any document or certification signifying a level of qualification lower than that required in order to perform the work of the construction trade, the contractor or subcontractor shall in good faith determine the qualifications of such individuals.  The Contractor or subcontractor shall hire or schedule those individuals who satisfy appropriate qualification standards in conformity with the equal employment opportunity and non-discrimination principles set forth in this chapter.  However, a Contractor or subcontractor shall determine that the individual at least possesses the requisite skills, and experience recognized by a union, apprentice program or a referral agency, provided the referral agency is acceptable to the Dept. of LWD, Construction EEO Monitoring Program.  If necessary, the Contractor or subcontractor shall hire or schedule minority and women workers who qualify as trainees pursuant to these rules.  All of the requirements, however, are limited by the provisions of “C” below.

c.	The name of any interested women or minority individual shall be maintained on a waiting list, and shall be considered for employment as described in (i) above, whenever vacancies occur.  At the request of the Dept. of LWD, Construction EEO Monitoring Program, the Contractor or subcontractor shall provide evidence of its good faith efforts to employ women and minorities from the list to fill vacancies.

d.	If, for any reason, said Contractor or subcontractor determines that a minority individual or a woman is not qualified or if the individual qualifies as an advanced trainee or apprentice, the Contractor or subcontractor shall inform the individual in writing of the reasons for the determination, maintain a copy of the determination in its files, and send a copy to the public agency compliance officer and to the Dept. of LWD, Construction EEO Monitoring Program.

7.	To keep a complete and accurate record of all requests made for the referral of workers in any trade covered by the contract, on forms made available by the Dept. of LWD, Construction EEO Monitoring Program and submitted promptly to the Dept. of LWD, Construction EEO Monitoring Program upon request.

C.	The Contractor or subcontractor agrees that nothing contained in “B” above shall preclude the Contractor or subcontractor from complying with the union hiring hall or apprentice-ship policies in any applicable collective bargaining agreement or union hiring hall arrangement, and, where required by custom or agreement, it shall send journeymen and trainees to the union for referral, or to the apprenticeship program for admission, pursuant to such agreement or arrangement.  However, where the practices of a union or apprenticeship program will result in the exclusion of minorities and women or the failure to refer minorities and women consistent with the targeted county employment goal, the Contractor or subcontractor shall consider for employment persons referred pursuant to “B” above without regard to such agreement or arrangement; provided further, however, that the Contractor or subcontractor shall not be required to employ women and minority advanced trainees and trainees in numbers which result in the employment of advanced trainees and trainees as a percentage of the total workforce for the construction trade, which percentage significantly exceeds the apprentice to journey worker ratio specified in the applicable collective bargaining agreement, or in the absence of a collective bargaining agreement, exceeds the ratio established by practice in the area for said construction trade. Also, the Contractor or subcontractor agrees that, in implementing the procedures of “B” above, it shall, where applicable, employ minority and women workers residing within the geographical jurisdiction of the union.

After notification of award, but prior to signing a construction contract, the Contractor shall submit to the Department and the Department of LWD, Construction EEO Monitoring Program an initial project workforce report (Form AA-201) electronically provided to the Department by the Department of LWD, Construction EEO Monitoring Program, through its web-site, for distribution to and completion by the Contractor, in accordance with N.J.A.C. 17:27-7.  The Contractor also agrees to submit a copy of the Monthly Project Workforce Report once a month thereafter for the duration of this contract to the Dept. of LWD, Construction EEO Monitoring Program, and to the Department.

The Contractor agrees to cooperate with the public agency in the payment of budgeted funds, as is necessary, for on-the-job and/or off-the-job programs for outreach and training of minorities and women.

D.	The Contractor and its subcontractors shall furnish such reports or other documents to the Dept. of LWD, Construction EEO Monitoring Program as may be requested by the Dept. of LWD, Construction EEO Monitoring Program from time to time in order to carry out the purposes of these regulations, and public agencies shall furnish such information as may be re-quested by the Dept. of LWD, Construction EEO Monitoring Program for conducting a compliance investigation pursuant to N.J.A.C. 17:27-1.1 et seq.

It is the policy of the NJDOT that its contracts should create a work-force that reflects the diversity of the State of New Jersey.  Therefore, contractors engaged by the NJDOT to perform under a construction contract shall put forth a good faith effort to engage in recruitment and employment practices that further the goal of fostering equal opportunities to minorities and women.

The Contractor must demonstrate to the NJDOT satisfaction that a good faith effort was made to ensure that minorities and women have been afforded equal opportunity to gain employment under the NJDOT contract with the Contractor.  Payment may be withheld from a Contractor’s contract for failure to comply with these provisions.

Evidence of a “good faith effort” includes, but is not limited to:

1.	The Contractor shall recruit prospective employees through the New Jersey Career Connections website, managed by the Department of Labor and Workforce Development, available online at:  Error! Hyperlink reference not valid. http://careerconnections.nj.gov/careerconnections/for_businesses.shtml

2.	The Contractor shall keep specific records of its efforts, including records of all individuals interviewed and hired, including the specific numbers of minorities and women;

3.	The Contractor shall actively solicit and shall provide the NJDOT with proof of solicitations for employment, including but not limited to advertisements in general circulation media, professional service publications and electronic media; and

4.	The Contractor shall provide evidence of efforts described at 2 above to the NJDOT no less frequently than once every 12 months.

5.	The Contractor shall comply with the requirements set forth at N.J.A.C. 17:27-1.1 et seq.

FEDERAL AID PROJECT ATTACHMENT 6

INVESTIGATING, REPORTING AND RESOLVING EMPLOYMENT DISCRIMINATION AND SEXUAL HARASSMENT COMPLAINTS ON NJDOT FEDERAL AID PROJECTS

The Contractor and subcontractors agrees to the following requirements in order to implement fully the nondiscrimination provisions of the Contract.

The Contractor agrees that in instances when it receives from any person working on the project site a verbal or written complaint of employment discrimination, prohibited under N.J.S.A. 10:5-1 et seq., 10:2-1 et seq., 42 U.S.C. 2000(d) et seq., 42 U.S.C. 2000 (e) et seq. and Executive Order 11246, it shall take the following actions:

A.	Within one (1) working day commence an investigation of the complaint which shall include but not be limited to interviewing the complainant, the respondent, and all possible witnesses to the alleged act or acts of discrimination or sexual harassment.

B.	Prepare and keep for its use and file a detailed written investigative report which includes the following information:

1.	Investigatory activities and findings.

2.	Dates and parties involved and activities involved in resolving the complaint.

3.	Resolution and corrective action taken if discrimination or sexual harassment is found to have taken place.

4.	A signed copy of resolution of complaint by complainant and Contractor.

In addition to keeping in its files the above-noted detailed written investigative report, the Contractor shall keep for possible future review by the Department all other records, including but not limited to, interview memos and statements.

C.	Upon the request of the Department, provides to the Department within ten (10) calendar days a copy of its detailed written investigative report and all other records on the complaint investigation and resolution.

D.	Take appropriate disciplinary action against any Contractor employee, official or agent who has committed acts of discrimination or sexual harassment against any contractor employee or person working on the project.  If the person committing the discrimination is a subcontractor employee, then the Contractor is required to attempt to effectuate corrective and/or disciplinary action by the subcontractor in order to establish compliance with the Contract.

E.	Take appropriate disciplinary action against any Contractor employee, official or agent who retaliates, coerces or intimidates any complaint and/or person who provides information or assistance to any investigation of complaints of discrimination or sexual harassment.  If the person retaliating, coercing or intimidating a complainant or other person assisting an investigation is a subcontractor’s employee, then the Contractor is required to attempt to effectuate corrective and/or disciplinary action by the subcontractor in order to establish compliance with the Contract.

F.	Ensure to the maximum extent possible that the privacy interests of all persons who give confidential information in aid of the Contractor’s employment discrimination investigation are protected.

In conjunction with the above requirements, the Contractor shall develop and post a written sexual harassment policy for its work force.

Failure by the Contractor and subcontractors to comply with the above requirements may be cause for the Department to institute against the Contractor any and all enforcement proceedings and/or sanctions authorized by the Contract or by State and/or Federal law.

FEDERAL AID PROJECT ATTACHMENT 7

PAYROLL REQUIREMENTS FOR NJDOT FEDERAL AID PROJECTS

A.	Certified Payroll Reports.  Each Contractor and subcontractor shall furnish the RE with certified payroll reports for each week of contract work.  Such reports shall be submitted within 7 days of the date of payment covered thereby and shall contain the following information:

1.	Each employee’s full name and an individually identifying number, (e.g. the last four digits of the employee’s social security number) of each such employee.

2.	The ethnicity and gender of each employee.

3.	Each employee’s correct classification (s) of work actually performed.

4.	Entries indicating each employee’s basis hourly wage rate(s) and, where applicable, the overtime hourly wage rate(s).  Any rates of contributions for fringe benefits paid to approved plans, funds or programs on behalf of the employee must be indicated.  Any fringe benefits paid to the employee in cash must be indicated.

5.	Each employee’s daily and weekly hours worked in each classification, including actual overtime hours worked (not adjusted).

6.	Each employee’s gross wage.

7.	The itemized deductions made.

8.	The net wages paid.

B.	Statement of Wages.  Each Contractor or subcontractor shall furnish a statement each week to the RE with respect to the wages paid each of its employees engaged in contract work covered by the Copeland Act, as amended during the preceding weekly payroll period.  The statement shall be executed by the Contractor or subcontractor or by an authorized officer or employee of the Contractor or subcontractors who supervises the payment of wages.  Contractors and subcontractors must use the certification set forth on the Department’s CR-347, or any form with identical wording.  Each payroll submitted must be accompanied by a signed “Statement of Compliance”.

C.	Maintaining Records.  Contractor and subcontractors shall maintain payroll records for a period of three (3) years after all work on the Contract is completed. Contractor and subcontractor shall maintain complete payroll records to include: name, social security numbers, last known address, telephone number and e-mail address for each employee.  Government agencies are entitled to request or review all relevant payroll information, including social security numbers and addresses of employees.  Contractors and subcontractors are required to provide such information upon request.

D.	Lack of Compliance.  Failure of the Contractor or subcontractor to comply with the payroll requirements may result in payment being delayed or withheld as specified in Section 105, default as specified in Section 108 or termination of the Contract as specified in Section 108.

E.	Diane B. Allen Equal Pay Act, N.J.S.A. 34:11-56.14b

For Local Aid projects.

This requirement DOES NOT apply to employers who are contracting with local governments (for example: municipalities and counties).

## FEDERAL AID PROJECT ATTACHMENT 8

### FHWA-1273

REQUIRED CONTRACT PROVISIONS

FEDERAL-AID CONSTRUCTION CONTRACTS

- General

- Nondiscrimination

- Non-segregated Facilities

- Davis-Bacon and Related Act Provisions

- Contract Work Hours and Safety Standards Act Provisions

- Subletting or Assigning the Contract

- Safety: Accident Prevention

- False Statements Concerning Highway Projects

- Implementation of Clean Air Act and Federal Water Pollution Control Act

- Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion

- Certification Regarding Use of Contract Funds for Lobbying

- Use of United States-Flag Vessels: ATTACHMENTS

A. Employment and Materials Preference for Appalachian Development Highway System or Appalachian Local Access Road Contracts (included in Appalachian contracts only)

  - GENERAL

    - Form FHWA-1273 must be physically incorporated in each construction contract funded under title 23, United States Code, as required in 23 CFR 633.102(b) (excluding emergency contracts solely intended for debris removal). The contractor (or subcontractor) must insert this form in each subcontract and further require its inclusion in all lower tier subcontracts (excluding purchase orders, rental agreements and other agreements for supplies or services). 23 CFR 633.102(e).

The applicable requirements of Form FHWA-1273 are incorporated by reference for work done under any purchase order, rental agreement or agreement for other services. The prime contractor shall be responsible for compliance by any subcontractor, lower-tier subcontractor or service provider. 23 CFR 633.102(e).

Form FHWA-1273 must be included in all Federal-aid design- build contracts, in all subcontracts and in lower tier subcontracts (excluding subcontracts for design services, purchase orders, rental agreements and other agreements for supplies or services) in accordance with 23 CFR 633.102. The design-builder shall be responsible for compliance by any subcontractor, lower-tier subcontractor or service provider.

Contracting agencies may reference Form FHWA-1273 in solicitation-for-bids or request-for-proposals documents, however, the Form FHWA-1273 must be physically incorporated (not referenced) in all contracts, subcontracts and lower-tier subcontracts (excluding purchase orders, rental agreements and other agreements for supplies or services related to a construction contract). 23 CFR 633.102(b).

    - Subject to the applicability criteria noted in the following sections, these contract provisions shall apply to all work

performed on the contract by the contractor's own organization and with the assistance of workers under the contractor's immediate superintendence and to all work performed on the contract by piecework, station work, or by subcontract. 23 CFR 633.102(d).

    - A breach of any of the stipulations contained in these Required Contract Provisions may be sufficient grounds for withholding of progress payments, withholding of final payment, termination of the contract, suspension / debarment or any other action determined to be appropriate by the contracting agency and FHWA.

    - Selection of Labor: During the performance of this contract, the contractor shall not use convict labor for any purpose within the limits of a construction project on a Federal-aid highway unless it is labor performed by convicts who are on parole, supervised release, or probation. 23 U.S.C. 114(b). The term Federal-aid highway does not include roadways functionally classified as local roads or rural minor collectors. 23 U.S.C. 101(a).

  - NONDISCRIMINATION (23 CFR 230.107(a); 23 CFR Part

230, Subpart A, Appendix A; EO 11246)

The provisions of this section related to 23 CFR Part 230, Subpart A, Appendix A are applicable to all Federal-aid construction contracts and to all related construction subcontracts of $10,000 or more. The provisions of 23 CFR Part 230 are not applicable to material supply, engineering, or architectural service contracts.

In addition, the contractor and all subcontractors must comply with the following policies: Executive Order 11246, 41 CFR Part 60, 29 CFR Parts 1625-1627, 23 U.S.C. 140, Section 504

of the Rehabilitation Act of 1973, as amended (29 U.S.C. 794), Title VI of the Civil Rights Act of 1964, as amended (42 U.S.C. 2000d et seq.), and related regulations including 49 CFR Parts 21, 26, and 27; and 23 CFR Parts 200, 230, and 633.

The contractor and all subcontractors must comply with: the requirements of the Equal Opportunity Clause in 41 CFR 60- 1.4(b) and, for all construction contracts exceeding $10,000, the Standard Federal Equal Employment Opportunity Construction Contract Specifications in 41 CFR 60-4.3.

Note: The U.S. Department of Labor has exclusive authority to determine compliance with Executive Order 11246 and the policies of the Secretary of Labor including 41 CFR Part 60, and 29 CFR Parts 1625-1627. The contracting agency and the FHWA have the authority and the responsibility to ensure compliance with 23 U.S.C. 140, Section 504 of the Rehabilitation Act of 1973, as amended (29 U.S.C. 794), and Title VI of the Civil Rights Act of 1964, as amended (42 U.S.C. 2000d et seq.), and related regulations including 49 CFR Parts 21, 26, and 27; and 23 CFR Parts 200, 230, and 633.

The following provision is adopted from 23 CFR Part 230, Subpart A, Appendix A, with appropriate revisions to conform to the U.S. Department of Labor (US DOL) and FHWA requirements.

    - Equal Employment Opportunity: Equal Employment Opportunity (EEO) requirements not to discriminate and to take affirmative action to assure equal opportunity as set forth under laws, executive orders, rules, regulations (see 28 CFR Part 35, 29 CFR Part 1630, 29 CFR Parts 1625-1627, 41 CFR

Part 60 and 49 CFR Part 27) and orders of the Secretary of Labor as modified by the provisions prescribed herein, and imposed pursuant to 23 U.S.C. 140, shall constitute the EEO and specific affirmative action standards for the contractor's project activities under this contract. The provisions of the Americans with Disabilities Act of 1990 (42 U.S.C. 12101 et seq.) set forth under 28 CFR Part 35 and 29 CFR Part 1630 are incorporated by reference in this contract. In the execution of this contract, the contractor agrees to comply with the following minimum specific requirement activities of EEO:

- The contractor will work with the contracting agency and the Federal Government to ensure that it has made every good faith effort to provide equal opportunity with respect to all of its terms and conditions of employment and in their review of activities under the contract. 23 CFR 230.409 (g)(4) & (5).

- The contractor will accept as its operating policy the following statement:

"It is the policy of this Company to assure that applicants are employed, and that employees are treated during employment, without regard to their race, religion, sex, sexual orientation, gender identity, color, national origin, age or disability. Such action shall include: employment, upgrading, demotion, or transfer; recruitment or recruitment advertising; layoff or termination; rates of pay or other forms of compensation; and selection for training, including apprenticeship, pre-apprenticeship, and/or on-the-job training."

    - EEO Officer: The contractor will designate and make known to the contracting officers an EEO Officer who will have the responsibility for and must be capable of effectively administering and promoting an active EEO program and who must be assigned adequate authority and responsibility to do so.

    - Dissemination of Policy: All members of the contractor's staff who are authorized to hire, supervise, promote, and discharge employees, or who recommend such action or are substantially involved in such action, will be made fully cognizant of and will implement the contractor's EEO policy and contractual responsibilities to provide EEO in each grade and classification of employment. To ensure that the above agreement will be met, the following actions will be taken as a minimum:

- Periodic meetings of supervisory and personnel office employees will be conducted before the start of work and then not less often than once every six months, at which time the contractor's EEO policy and its implementation will be reviewed and explained. The meetings will be conducted by the EEO Officer or other knowledgeable company official.

- All new supervisory or personnel office employees will be given a thorough indoctrination by the EEO Officer, covering all major aspects of the contractor's EEO obligations within thirty days following their reporting for duty with the contractor.

- All personnel who are engaged in direct recruitment for the project will be instructed by the EEO Officer in the contractor's procedures for locating and hiring minorities and women.

- Notices and posters setting forth the contractor's EEO policy will be placed in areas readily accessible to employees, applicants for employment and potential employees.

- The contractor's EEO policy and the procedures to implement such policy will be brought to the attention of employees by means of meetings, employee handbooks, or other appropriate means.

    - Recruitment: When advertising for employees, the contractor will include in all advertisements for employees the notation: "An Equal Opportunity Employer." All such advertisements will be placed in publications having a large circulation among minorities and women in the area from which the project work force would normally be derived.

- The contractor will, unless precluded by a valid bargaining agreement, conduct systematic and direct recruitment through public and private employee referral sources likely to yield qualified minorities and women. To meet this requirement, the contractor will identify sources of potential minority group employees and establish with such identified sources procedures whereby minority and women applicants may be referred to the contractor for employment consideration.

- In the event the contractor has a valid bargaining agreement providing for exclusive hiring hall referrals, the contractor is expected to observe the provisions of that agreement to the extent that the system meets the contractor's compliance with EEO contract provisions. Where implementation of such an agreement has the effect of discriminating against minorities or women, or obligates the contractor to do the same, such implementation violates Federal nondiscrimination provisions.

- The contractor will encourage its present employees to refer minorities and women as applicants for employment. Information and procedures with regard to referring such applicants will be discussed with employees.

    - Personnel Actions: Wages, working conditions, and employee benefits shall be established and administered, and personnel actions of every type, including hiring, upgrading, promotion, transfer, demotion, layoff, and termination, shall be taken without regard to race, color, religion, sex, sexual orientation, gender identity, national origin, age or disability. The following procedures shall be followed:

- The contractor will conduct periodic inspections of project sites to ensure that working conditions and employee facilities do not indicate discriminatory treatment of project site personnel.

- The contractor will periodically evaluate the spread of wages paid within each classification to determine any evidence of discriminatory wage practices.

- The contractor will periodically review selected personnel actions in depth to determine whether there is evidence of discrimination. Where evidence is found, the contractor will promptly take corrective action. If the review indicates that the discrimination may extend beyond the actions reviewed, such corrective action shall include all affected persons.

- The contractor will promptly investigate all complaints of alleged discrimination made to the contractor in connection with its obligations under this contract, will attempt to resolve such complaints, and will take appropriate corrective action

within a reasonable time. If the investigation indicates that the discrimination may affect persons other than the complainant, such corrective action shall include such other persons. Upon completion of each investigation, the contractor will inform every complainant of all of their avenues of appeal.

    - Training and Promotion:

- The contractor will assist in locating, qualifying, and increasing the skills of minorities and women who are applicants for employment or current employees. Such efforts should be aimed at developing full journey level status employees in the type of trade or job classification involved.

- Consistent with the contractor's work force requirements and as permissible under Federal and State regulations, the contractor shall make full use of training programs (i.e., apprenticeship and on-the-job training programs for the geographical area of contract performance). In the event a special provision for training is provided under this contract, this subparagraph will be superseded as indicated in the special provision. The contracting agency may reserve training positions for persons who receive welfare assistance in accordance with 23 U.S.C. 140(a).

- The contractor will advise employees and applicants for employment of available training programs and entrance requirements for each.

- The contractor will periodically review the training and promotion potential of employees who are minorities and women and will encourage eligible employees to apply for such training and promotion.

    - Unions: If the contractor relies in whole or in part upon unions as a source of employees, the contractor will use good faith efforts to obtain the cooperation of such unions to increase opportunities for minorities and women. 23 CFR

230.409. Actions by the contractor, either directly or through a contractor's association acting as agent, will include the procedures set forth below:

- The contractor will use good faith efforts to develop, in cooperation with the unions, joint training programs aimed toward qualifying more minorities and women for membership in the unions and increasing the skills of minorities and women so that they may qualify for higher paying employment.

- The contractor will use good faith efforts to incorporate an EEO clause into each union agreement to the end that such union will be contractually bound to refer applicants without regard to their race, color, religion, sex, sexual orientation, gender identity, national origin, age, or disability.

- The contractor is to obtain information as to the referral practices and policies of the labor union except that to the extent such information is within the exclusive possession of the labor union and such labor union refuses to furnish such information to the contractor, the contractor shall so certify to the contracting agency and shall set forth what efforts have been made to obtain such information.

- In the event the union is unable to provide the contractor with a reasonable flow of referrals within the time limit set forth in the collective bargaining agreement, the contractor will, through independent recruitment efforts, fill the employment vacancies without regard to race, color, religion, sex, sexual orientation, gender identity, national origin, age, or disability; making full efforts to obtain qualified and/or qualifiable minorities and women. The failure of a union to provide

sufficient referrals (even though it is obligated to provide exclusive referrals under the terms of a collective bargaining agreement) does not relieve the contractor from the requirements of this paragraph. In the event the union referral practice prevents the contractor from meeting the obligations pursuant to Executive Order 11246, as amended, and these special provisions, such contractor shall immediately notify the contracting agency.

    - Reasonable Accommodation for Applicants / Employees with Disabilities: The contractor must be familiar with the requirements for and comply with the Americans with Disabilities Act and all rules and regulations established thereunder. Employers must provide reasonable accommodation in all employment activities unless to do so would cause an undue hardship.

    - Selection of Subcontractors, Procurement of Materials and Leasing of Equipment: The contractor shall not discriminate on the grounds of race, color, religion, sex, sexual orientation, gender identity, national origin, age, or disability in the selection and retention of subcontractors, including procurement of materials and leases of equipment. The contractor shall take all necessary and reasonable steps to ensure nondiscrimination in the administration of this contract.

- The contractor shall notify all potential subcontractors, suppliers, and lessors of their EEO obligations under this contract.

- The contractor will use good faith efforts to ensure subcontractor compliance with their EEO obligations.

    - Assurances Required:

- The requirements of 49 CFR Part 26 and the State DOT’s FHWA-approved Disadvantaged Business Enterprise (DBE) program are incorporated by reference.

- The contractor, subrecipient or subcontractor shall not discriminate on the basis of race, color, national origin, or sex in the performance of this contract. The contractor shall carry out applicable requirements of 49 CFR part 26 in the award and administration of DOT-assisted contracts. Failure by the contractor to carry out these requirements is a material breach of this contract, which may result in the termination of this contract or such other remedy as the recipient deems appropriate, which may include, but is not limited to:

  - Withholding monthly progress payments;

  - Assessing sanctions;

  - Liquidated damages; and/or

  - Disqualifying the contractor from future bidding as non- responsible.

- The Title VI and nondiscrimination provisions of U.S. DOT Order 1050.2A at Appendixes A and E are incorporated by reference. 49 CFR Part 21.

    - Records and Reports: The contractor shall keep such records as necessary to document compliance with the EEO requirements. Such records shall be retained for a period of three years following the date of the final payment to the contractor for all contract work and shall be available at reasonable times and places for inspection by authorized representatives of the contracting agency and the FHWA.

- The records kept by the contractor shall document the following:

  - The number and work hours of minority and non- minority group members and women employed in each work classification on the project;

  - The progress and efforts being made in cooperation with unions, when applicable, to increase employment opportunities for minorities and women; and

  - The progress and efforts being made in locating, hiring, training, qualifying, and upgrading minorities and women.

- The contractors and subcontractors will submit an annual report to the contracting agency each July for the duration of the project indicating the number of minority, women, and non- minority group employees currently engaged in each work classification required by the contract work. This information is to be reported on Form FHWA-1391. The staffing data should represent the project work force on board in all or any part of the last payroll period preceding the end of July. If on-the-job training is being required by special provision, the contractor will be required to collect and report training data. The employment data should reflect the work force on board during all or any part of the last payroll period preceding the end of July.

  - NONSEGREGATED FACILITIES

This provision is applicable to all Federal-aid construction contracts and to all related construction subcontracts of more than $10,000. 41 CFR 60-1.5.

As prescribed by 41 CFR 60-1.8, the contractor must ensure that facilities provided for employees are provided in such a manner that segregation on the basis of race, color, religion, sex, sexual orientation, gender identity, or national origin cannot result. The contractor may neither require such segregated use by written or oral policies nor tolerate such use by employee custom. The contractor's obligation extends further to ensure that its employees are not assigned to perform their services at any location under the contractor's control where the facilities are segregated. The term "facilities" includes waiting rooms, work areas, restaurants and other eating areas, time clocks, restrooms, washrooms, locker rooms and other storage or dressing areas, parking lots, drinking fountains, recreation or entertainment areas, transportation, and housing provided for employees. The contractor shall provide separate or single-user restrooms and necessary dressing or sleeping areas to assure privacy between sexes.

  - DAVIS-BACON AND RELATED ACT PROVISIONS

This section is applicable to all Federal-aid construction projects exceeding $2,000 and to all related subcontracts and lower-tier subcontracts (regardless of subcontract size), in accordance with 29 CFR 5.5. The requirements apply to all projects located within the right-of-way of a roadway that is functionally classified as Federal-aid highway. 23 U.S.C. 113. This excludes roadways functionally classified as local roads or rural minor collectors, which are exempt. 23 U.S.C. 101.

Where applicable law requires that projects be treated as a project on a Federal-aid highway, the provisions of this subpart will apply regardless of the location of the project. Examples include: Surface Transportation Block Grant Program projects funded under 23 U.S.C. 133 [excluding recreational trails projects], the Nationally Significant Freight and Highway

Projects funded under 23 U.S.C. 117, and National Highway Freight Program projects funded under 23 U.S.C. 167.

The following provisions are from the U.S. Department of Labor regulations in 29 CFR 5.5 “Contract provisions and related matters” with minor revisions to conform to the FHWA- 1273 format and FHWA program requirements.

    - Minimum wages (29 CFR 5.5)

- Wage rates and fringe benefits. All laborers and mechanics employed or working upon the site of the work (or otherwise working in construction or development of the project under a development statute), will be paid unconditionally and not less often than once a week, and without subsequent deduction or rebate on any account (except such payroll deductions as are permitted by regulations issued by the Secretary of Labor under the Copeland Act (29 CFR part 3)), the full amount of basic hourly wages and bona fide fringe benefits (or cash equivalents thereof) due at time of payment computed at rates not less than those contained in the wage determination of the Secretary of Labor which is attached hereto and made a part hereof, regardless of any contractual relationship which may be alleged to exist between the contractor and such laborers and mechanics. As provided in paragraphs (d) and (e) of 29 CFR 5.5, the appropriate wage determinations are effective by operation of law even if they have not been attached to the contract. Contributions made or costs reasonably anticipated for bona fide fringe benefits under the Davis-Bacon Act (40

U.S.C. 3141(2)(B)) on behalf of laborers or mechanics are considered wages paid to such laborers or mechanics, subject to the provisions of paragraph 1.e. of this section; also, regular contributions made or costs incurred for more than a weekly period (but not less often than quarterly) under plans, funds, or programs which cover the particular weekly period, are deemed to be constructively made or incurred during such weekly period. Such laborers and mechanics must be paid the appropriate wage rate and fringe benefits on the wage determination for the classification(s) of work actually performed, without regard to skill, except as provided in paragraph 4. of this section. Laborers or mechanics performing work in more than one classification may be compensated at the rate specified for each classification for the time actually worked therein: Provided, That the employer's payroll records accurately set forth the time spent in each classification in which work is performed. The wage determination (including any additional classifications and wage rates conformed under paragraph 1.c. of this section) and the Davis-Bacon poster (WH–1321) must be posted at all times by the contractor and its subcontractors at the site of the work in a prominent and accessible place where it can be easily seen by the workers.

- Frequently recurring classifications. (1) In addition to wage and fringe benefit rates that have been determined to be prevailing under the procedures set forth in 29 CFR part 1, a wage determination may contain, pursuant to § 1.3(f), wage and fringe benefit rates for classifications of laborers and mechanics for which conformance requests are regularly submitted pursuant to paragraph 1.c. of this section, provided that:

- The work performed by the classification is not performed by a classification in the wage determination for which a prevailing wage rate has been determined;

- The classification is used in the area by the construction industry; and

- The wage rate for the classification bears a reasonable relationship to the prevailing wage rates contained in the wage determination.

(2) The Administrator will establish wage rates for such classifications in accordance with paragraph 1.c.(1)(iii) of this section. Work performed in such a classification must be paid at no less than the wage and fringe benefit rate listed on the wage determination for such classification.

- Conformance. (1) The contracting officer must require that any class of laborers or mechanics, including helpers, which is not listed in the wage determination and which is to be employed under the contract be classified in conformance with the wage determination. Conformance of an additional classification and wage rate and fringe benefits is appropriate only when the following criteria have been met:

- The work to be performed by the classification requested is not performed by a classification in the wage determination; and

- The classification is used in the area by the construction industry; and

- The proposed wage rate, including any bona fide fringe benefits, bears a reasonable relationship to the wage rates contained in the wage determination.

- The conformance process may not be used to split, subdivide, or otherwise avoid application of classifications listed in the wage determination.

- If the contractor and the laborers and mechanics to be employed in the classification (if known), or their representatives, and the contracting officer agree on the classification and wage rate (including the amount designated for fringe benefits where appropriate), a report of the action taken will be sent by the contracting officer by email to DBAconformance@dol.gov. The Administrator, or an authorized representative, will approve, modify, or disapprove every additional classification action within 30 days of receipt and so advise the contracting officer or will notify the contracting officer within the 30–day period that additional time is necessary.

- In the event the contractor, the laborers or mechanics to be employed in the classification or their representatives, and the contracting officer do not agree on the proposed classification and wage rate (including the amount designated for fringe benefits, where appropriate), the contracting officer will, by email to DBAconformance@dol.gov, refer the questions, including the views of all interested parties and the recommendation of the contracting officer, to the Administrator for determination. The Administrator, or an authorized representative, will issue a determination within 30 days of receipt and so advise the contracting officer or will notify the contracting officer within the 30–day period that additional time is necessary.

- The contracting officer must promptly notify the contractor of the action taken by the Wage and Hour Division

under paragraphs 1.c.(3) and (4) of this section. The contractor must furnish a written copy of such determination to each affected worker or it must be posted as a part of the wage determination. The wage rate (including fringe benefits where appropriate) determined pursuant to paragraph 1.c.(3) or (4) of this section must be paid to all workers performing work in the classification under this contract from the first day on which work is performed in the classification.

- Fringe benefits not expressed as an hourly rate. Whenever the minimum wage rate prescribed in the contract for a class of laborers or mechanics includes a fringe benefit which is not expressed as an hourly rate, the contractor may either pay the benefit as stated in the wage determination or may pay another bona fide fringe benefit or an hourly cash equivalent thereof.

- Unfunded plans. If the contractor does not make payments to a trustee or other third person, the contractor may consider as part of the wages of any laborer or mechanic the amount of any costs reasonably anticipated in providing bona fide fringe benefits under a plan or program, Provided, That the Secretary of Labor has found, upon the written request of the contractor, in accordance with the criteria set forth in

§ 5.28, that the applicable standards of the Davis-Bacon Act have been met. The Secretary of Labor may require the contractor to set aside in a separate account assets for the meeting of obligations under the plan or program.

- Interest. In the event of a failure to pay all or part of the wages required by the contract, the contractor will be required to pay interest on any underpayment of wages.

    - Withholding (29 CFR 5.5)

- Withholding requirements. The contracting agency may, upon its own action, or must, upon written request of an authorized representative of the Department of Labor, withhold or cause to be withheld from the contractor so much of the accrued payments or advances as may be considered necessary to satisfy the liabilities of the prime contractor or any subcontractor for the full amount of wages and monetary relief, including interest, required by the clauses set forth in this section for violations of this contract, or to satisfy any such liabilities required by any other Federal contract, or federally assisted contract subject to Davis-Bacon labor standards, that is held by the same prime contractor (as defined in § 5.2). The necessary funds may be withheld from the contractor under this contract, any other Federal contract with the same prime contractor, or any other federally assisted contract that is subject to Davis-Bacon labor standards requirements and is held by the same prime contractor, regardless of whether the other contract was awarded or assisted by the same agency, and such funds may be used to satisfy the contractor liability for which the funds were withheld. In the event of a contractor's failure to pay any laborer or mechanic, including any apprentice or helper working on the site of the work all or part of the wages required by the contract, or upon the contractor's failure to submit the required records as discussed in paragraph 3.d. of this section, the contracting agency may on its own initiative and after written notice to the contractor, take such action as may be necessary to cause the suspension of any further payment, advance, or guarantee of funds until such violations have ceased.

- Priority to withheld funds. The Department has priority to funds withheld or to be withheld in accordance with paragraph

      - of this section or Section V, paragraph 3.a., or both, over claims to those funds by:

        - A contractor's surety(ies), including without limitation performance bond sureties and payment bond sureties;

        - A contracting agency for its reprocurement costs;

        - A trustee(s) (either a court-appointed trustee or a U.S. trustee, or both) in bankruptcy of a contractor, or a contractor's bankruptcy estate;

        - A contractor's assignee(s);

        - A contractor's successor(s); or

        - A claim asserted under the Prompt Payment Act, 31

U.S.C. 3901–3907.

    - Records and certified payrolls (29 CFR 5.5)

- Basic record requirements (1) Length of record retention. All regular payrolls and other basic records must be maintained by the contractor and any subcontractor during the course of the work and preserved for all laborers and mechanics working at the site of the work (or otherwise working in construction or development of the project under a development statute) for a period of at least 3 years after all the work on the prime contract is completed.

- Information required. Such records must contain the name; Social Security number; last known address, telephone number, and email address of each such worker; each worker's correct classification(s) of work actually performed; hourly rates of wages paid (including rates of contributions or costs anticipated for bona fide fringe benefits or cash equivalents thereof of the types described in 40 U.S.C. 3141(2)(B) of the Davis-Bacon Act); daily and weekly number of hours actually worked in total and on each covered contract; deductions made; and actual wages paid.

- Additional records relating to fringe benefits. Whenever the Secretary of Labor has found under paragraph 1.e. of this section that the wages of any laborer or mechanic include the amount of any costs reasonably anticipated in providing benefits under a plan or program described in 40 U.S.C. 3141(2)(B) of the Davis-Bacon Act, the contractor must maintain records which show that the commitment to provide such benefits is enforceable, that the plan or program is financially responsible, and that the plan or program has been communicated in writing to the laborers or mechanics affected, and records which show the costs anticipated or the actual cost incurred in providing such benefits.

- Additional records relating to apprenticeship. Contractors with apprentices working under approved programs must maintain written evidence of the registration of apprenticeship programs, the registration of the apprentices, and the ratios and wage rates prescribed in the applicable programs.

- Certified payroll requirements (1) Frequency and method of submission. The contractor or subcontractor must submit weekly, for each week in which any DBA- or Related Acts- covered work is performed, certified payrolls to the contracting

agency. The prime contractor is responsible for the submission of all certified payrolls by all subcontractors. A contracting agency or prime contractor may permit or require contractors to submit certified payrolls through an electronic system, as long as the electronic system requires a legally valid electronic signature; the system allows the contractor, the contracting agency, and the Department of Labor to access the certified payrolls upon request for at least 3 years after the work on the prime contract has been completed; and the contracting agency or prime contractor permits other methods of submission in situations where the contractor is unable or limited in its ability to use or access the electronic system.

- Information required. The certified payrolls submitted must set out accurately and completely all of the information required to be maintained under paragraph 3.a.(2) of this section, except that full Social Security numbers and last known addresses, telephone numbers, and email addresses must not be included on weekly transmittals. Instead, the certified payrolls need only include an individually identifying number for each worker ( e.g., the last four digits of the worker's Social Security number). The required weekly certified payroll information may be submitted using Optional Form WH–347 or in any other format desired. Optional Form WH–347 is available for this purpose from the Wage and Hour Division website at https://www.dol.gov/sites/dolgov/files/WHD/ legacy/files/wh347/.pdf or its successor website. It is not a violation of this section for a prime contractor to require a subcontractor to provide full Social Security numbers and last known addresses, telephone numbers, and email addresses to the prime contractor for its own records, without weekly submission by the subcontractor to the contracting agency.

- Statement of Compliance. Each certified payroll submitted must be accompanied by a “Statement of Compliance,” signed by the contractor or subcontractor, or the contractor's or subcontractor's agent who pays or supervises the payment of the persons working on the contract, and must certify the following:

  - That the certified payroll for the payroll period contains the information required to be provided under paragraph 3.b. of this section, the appropriate information and basic records are being maintained under paragraph 3.a. of this section, and such information and records are correct and complete;

  - That each laborer or mechanic (including each helper and apprentice) working on the contract during the payroll period has been paid the full weekly wages earned, without rebate, either directly or indirectly, and that no deductions have been made either directly or indirectly from the full wages earned, other than permissible deductions as set forth in 29 CFR part 3; and

  - That each laborer or mechanic has been paid not less than the applicable wage rates and fringe benefits or cash equivalents for the classification(s) of work actually performed, as specified in the applicable wage determination incorporated into the contract.

- Use of Optional Form WH–347. The weekly submission of a properly executed certification set forth on the reverse side of Optional Form WH–347 will satisfy the requirement for submission of the “Statement of Compliance” required by paragraph 3.b.(3) of this section.

- Signature. The signature by the contractor, subcontractor, or the contractor's or subcontractor's agent must be an original handwritten signature or a legally valid electronic signature.

- Falsification. The falsification of any of the above certifications may subject the contractor or subcontractor to civil or criminal prosecution under 18 U.S.C. 1001 and 31

U.S.C. 3729.

- Length of certified payroll retention. The contractor or subcontractor must preserve all certified payrolls during the course of the work and for a period of 3 years after all the work on the prime contract is completed.

- Contracts, subcontracts, and related documents. The contractor or subcontractor must maintain this contract or subcontract and related documents including, without limitation, bids, proposals, amendments, modifications, and extensions. The contractor or subcontractor must preserve these contracts, subcontracts, and related documents during the course of the work and for a period of 3 years after all the work on the prime contract is completed.

- Required disclosures and access (1) Required record disclosures and access to workers. The contractor or subcontractor must make the records required under paragraphs 3.a. through 3.c. of this section, and any other documents that the contracting agency, the State DOT, the FHWA, or the Department of Labor deems necessary to determine compliance with the labor standards provisions of any of the applicable statutes referenced by § 5.1, available for inspection, copying, or transcription by authorized representatives of the contracting agency, the State DOT, the FHWA, or the Department of Labor, and must permit such representatives to interview workers during working hours on the job.

- Sanctions for non-compliance with records and worker access requirements. If the contractor or subcontractor fails to submit the required records or to make them available, or refuses to permit worker interviews during working hours on the job, the Federal agency may, after written notice to the contractor, sponsor, applicant, owner, or other entity, as the case may be, that maintains such records or that employs such workers, take such action as may be necessary to cause the suspension of any further payment, advance, or guarantee of funds. Furthermore, failure to submit the required records upon request or to make such records available, or to permit worker interviews during working hours on the job, may be grounds for debarment action pursuant to § 5.12. In addition, any contractor or other person that fails to submit the required records or make those records available to WHD within the time WHD requests that the records be produced will be precluded from introducing as evidence in an administrative proceeding under 29 CFR part 6 any of the required records that were not provided or made available to WHD. WHD will take into consideration a reasonable request from the contractor or person for an extension of the time for submission of records. WHD will determine the reasonableness of the request and may consider, among other things, the location of the records and the volume of production.

- Required information disclosures. Contractors and subcontractors must maintain the full Social Security number and last known address, telephone number, and email address

of each covered worker, and must provide them upon request to the contracting agency, the State DOT, the FHWA, the contractor, or the Wage and Hour Division of the Department of Labor for purposes of an investigation or other compliance action.

    - Apprentices and equal employment opportunity (29 CFR 5.5)

- Apprentices (1) Rate of pay. Apprentices will be permitted to work at less than the predetermined rate for the work they perform when they are employed pursuant to and individually registered in a bona fide apprenticeship program registered with the U.S. Department of Labor, Employment and Training Administration, Office of Apprenticeship (OA), or with a State Apprenticeship Agency recognized by the OA. A person who is not individually registered in the program, but who has been certified by the OA or a State Apprenticeship Agency (where appropriate) to be eligible for probationary employment as an apprentice, will be permitted to work at less than the predetermined rate for the work they perform in the first 90 days of probationary employment as an apprentice in such a program. In the event the OA or a State Apprenticeship Agency recognized by the OA withdraws approval of an apprenticeship program, the contractor will no longer be permitted to use apprentices at less than the applicable predetermined rate for the work performed until an acceptable program is approved.

- Fringe benefits. Apprentices must be paid fringe benefits in accordance with the provisions of the apprenticeship program. If the apprenticeship program does not specify fringe benefits, apprentices must be paid the full amount of fringe benefits listed on the wage determination for the applicable classification. If the Administrator determines that a different practice prevails for the applicable apprentice classification, fringe benefits must be paid in accordance with that determination.

- Apprenticeship ratio. The allowable ratio of apprentices to journeyworkers on the job site in any craft classification must not be greater than the ratio permitted to the contractor as to the entire work force under the registered program or the ratio applicable to the locality of the project pursuant to paragraph 4.a.(4) of this section. Any worker listed on a payroll at an apprentice wage rate, who is not registered or otherwise employed as stated in paragraph 4.a.(1) of this section, must be paid not less than the applicable wage rate on the wage determination for the classification of work actually performed. In addition, any apprentice performing work on the job site in excess of the ratio permitted under this section must be paid not less than the applicable wage rate on the wage determination for the work actually performed.

- Reciprocity of ratios and wage rates. Where a contractor is performing construction on a project in a locality other than the locality in which its program is registered, the ratios and wage rates (expressed in percentages of the journeyworker's hourly rate) applicable within the locality in which the construction is being performed must be observed. If there is no applicable ratio or wage rate for the locality of the project, the ratio and wage rate specified in the contractor's registered program must be observed.

- Equal employment opportunity. The use of apprentices and journeyworkers under this part must be in conformity with

the equal employment opportunity requirements of Executive Order 11246, as amended, and 29 CFR part 30.

- Apprentices and Trainees (programs of the U.S. DOT).

Apprentices and trainees working under apprenticeship and skill training programs which have been certified by the Secretary of Transportation as promoting EEO in connection with Federal-aid highway construction programs are not subject to the requirements of paragraph 4 of this Section IV. 23 CFR 230.111(e)(2). The straight time hourly wage rates for apprentices and trainees under such programs will be established by the particular programs. The ratio of apprentices and trainees to journeyworkers shall not be greater than permitted by the terms of the particular program.

    - Compliance with Copeland Act requirements. The contractor shall comply with the requirements of 29 CFR part 3, which are incorporated by reference in this contract as provided in 29 CFR 5.5.

    - Subcontracts. The contractor or subcontractor must insert FHWA-1273 in any subcontracts, along with the applicable wage determination(s) and such other clauses or contract modifications as the contracting agency may by appropriate instructions require, and a clause requiring the subcontractors to include these clauses and wage determination(s) in any lower tier subcontracts. The prime contractor is responsible for the compliance by any subcontractor or lower tier subcontractor with all the contract clauses in this section. In the event of any violations of these clauses, the prime contractor and any subcontractor(s) responsible will be liable for any unpaid wages and monetary relief, including interest from the date of the underpayment or loss, due to any workers of lower-tier subcontractors, and may be subject to debarment, as appropriate. 29 CFR 5.5.

    - Contract termination: debarment. A breach of the contract clauses in 29 CFR 5.5 may be grounds for termination of the contract, and for debarment as a contractor and a subcontractor as provided in 29 CFR 5.12.

    - Compliance with Davis-Bacon and Related Act requirements. All rulings and interpretations of the Davis- Bacon and Related Acts contained in 29 CFR parts 1, 3, and 5 are herein incorporated by reference in this contract as provided in 29 CFR 5.5.

    - Disputes concerning labor standards. As provided in 29 CFR 5.5, disputes arising out of the labor standards provisions of this contract shall not be subject to the general disputes clause of this contract. Such disputes shall be resolved in accordance with the procedures of the Department of Labor set forth in 29 CFR parts 5, 6, and 7. Disputes within the meaning of this clause include disputes between the contractor (or any of its subcontractors) and the contracting agency, the

U.S. Department of Labor, or the employees or their representatives.

    - Certification of eligibility. a. By entering into this contract, the contractor certifies that neither it nor any person or firm who has an interest in the contractor's firm is a person or firm ineligible to be awarded Government contracts by virtue of 40

U.S.C. 3144(b) or § 5.12(a).

b. No part of this contract shall be subcontracted to any person or firm ineligible for award of a Government contract by virtue of 40 U.S.C. 3144(b) or § 5.12(a).

c. The penalty for making false statements is prescribed in the U.S. Code, Title 18 Crimes and Criminal Procedure, 18

U.S.C. 1001.

    - Anti-retaliation. It is unlawful for any person to discharge, demote, intimidate, threaten, restrain, coerce, blacklist, harass, or in any other manner discriminate against, or to cause any person to discharge, demote, intimidate, threaten, restrain, coerce, blacklist, harass, or in any other manner discriminate against, any worker or job applicant for:

- Notifying any contractor of any conduct which the worker reasonably believes constitutes a violation of the DBA, Related Acts, this part, or 29 CFR part 1 or 3;

- Filing any complaint, initiating or causing to be initiated any proceeding, or otherwise asserting or seeking to assert on behalf of themselves or others any right or protection under the DBA, Related Acts, this part, or 29 CFR part 1 or 3;

- Cooperating in any investigation or other compliance action, or testifying in any proceeding under the DBA, Related Acts, this part, or 29 CFR part 1 or 3; or

- Informing any other person about their rights under the DBA, Related Acts, this part, or 29 CFR part 1 or 3.

  - CONTRACT WORK HOURS AND SAFETY STANDARDS ACT

Pursuant to 29 CFR 5.5(b), the following clauses apply to any Federal-aid construction contract in an amount in excess of

$100,000 and subject to the overtime provisions of the Contract Work Hours and Safety Standards Act. These clauses shall be inserted in addition to the clauses required by 29 CFR 5.5(a) or 29 CFR 4.6. As used in this paragraph, the terms laborers and mechanics include watchpersons and guards.

    - Overtime requirements. No contractor or subcontractor contracting for any part of the contract work which may require or involve the employment of laborers or mechanics shall require or permit any such laborer or mechanic in any workweek in which he or she is employed on such work to work in excess of forty hours in such workweek unless such laborer or mechanic receives compensation at a rate not less than one and one-half times the basic rate of pay for all hours worked in excess of forty hours in such workweek. 29 CFR 5.5.

    - Violation; liability for unpaid wages; liquidated damages. In the event of any violation of the clause set forth in paragraph 1. of this section the contractor and any subcontractor responsible therefor shall be liable for the unpaid wages and interest from the date of the underpayment. In addition, such contractor and subcontractor shall be liable to the United States (in the case of work done under contract for the District of Columbia or a territory, to such District or to such territory), for liquidated damages. Such liquidated damages shall be computed with respect to each individual laborer or

mechanic, including watchpersons and guards, employed in violation of the clause set forth in paragraph 1. of this section, in the sum currently provided in 29 CFR 5.5(b)(2)* for each calendar day on which such individual was required or permitted to work in excess of the standard workweek of forty hours without payment of the overtime wages required by the clause set forth in paragraph 1. of this section.

* $31 as of January 15, 2023 (See 88 FR 88 FR 2210) as may be adjusted annually by the Department of Labor, pursuant to the Federal Civil Penalties Inflation Adjustment Act of 1990.

    - Withholding for unpaid wages and liquidated damages

- Withholding process. The FHWA or the contracting agency may, upon its own action, or must, upon written request of an authorized representative of the Department of Labor, withhold or cause to be withheld from the contractor so much of the accrued payments or advances as may be considered necessary to satisfy the liabilities of the prime contractor or any subcontractor for any unpaid wages; monetary relief, including interest; and liquidated damages required by the clauses set forth in this section on this contract, any other Federal contract with the same prime contractor, or any other federally assisted contract subject to the Contract Work Hours and Safety Standards Act that is held by the same prime contractor (as defined in § 5.2). The necessary funds may be withheld from the contractor under this contract, any other Federal contract with the same prime contractor, or any other federally assisted contract that is subject to the Contract Work Hours and Safety Standards Act and is held by the same prime contractor, regardless of whether the other contract was awarded or assisted by the same agency, and such funds may be used to satisfy the contractor liability for which the funds were withheld.

- Priority to withheld funds. The Department has priority to funds withheld or to be withheld in accordance with Section IV paragraph 2.a. or paragraph 3.a. of this section, or both, over claims to those funds by:

  - A contractor's surety(ies), including without limitation performance bond sureties and payment bond sureties;

  - A contracting agency for its reprocurement costs;

  - A trustee(s) (either a court-appointed trustee or a U.S. trustee, or both) in bankruptcy of a contractor, or a contractor's bankruptcy estate;

  - A contractor's assignee(s);

  - A contractor's successor(s); or

  - A claim asserted under the Prompt Payment Act, 31

U.S.C. 3901–3907.

    - Subcontracts. The contractor or subcontractor must insert in any subcontracts the clauses set forth in paragraphs 1. through 5. of this section and a clause requiring the subcontractors to include these clauses in any lower tier subcontracts. The prime contractor is responsible for compliance by any subcontractor or lower tier subcontractor with the clauses set forth in paragraphs 1. through 5. In the

event of any violations of these clauses, the prime contractor and any subcontractor(s) responsible will be liable for any unpaid wages and monetary relief, including interest from the date of the underpayment or loss, due to any workers of lower- tier subcontractors, and associated liquidated damages and may be subject to debarment, as appropriate.

    - Anti-retaliation. It is unlawful for any person to discharge, demote, intimidate, threaten, restrain, coerce, blacklist, harass, or in any other manner discriminate against, or to cause any person to discharge, demote, intimidate, threaten, restrain, coerce, blacklist, harass, or in any other manner discriminate against, any worker or job applicant for:

- Notifying any contractor of any conduct which the worker reasonably believes constitutes a violation of the Contract Work Hours and Safety Standards Act (CWHSSA) or its implementing regulations in this part;

- Filing any complaint, initiating or causing to be initiated any proceeding, or otherwise asserting or seeking to assert on behalf of themselves or others any right or protection under CWHSSA or this part;

- Cooperating in any investigation or other compliance action, or testifying in any proceeding under CWHSSA or this part; or

- Informing any other person about their rights under CWHSSA or this part.

  - SUBLETTING OR ASSIGNING THE CONTRACT

This provision is applicable to all Federal-aid construction contracts on the National Highway System pursuant to 23 CFR 635.116.

    - The contractor shall perform with its own organization contract work amounting to not less than 30 percent (or a greater percentage if specified elsewhere in the contract) of the total original contract price, excluding any specialty items designated by the contracting agency. Specialty items may be performed by subcontract and the amount of any such specialty items performed may be deducted from the total original contract price before computing the amount of work required to be performed by the contractor's own organization (23 CFR 635.116).

- The term “perform work with its own organization” in paragraph 1 of Section VI refers to workers employed or leased by the prime contractor, and equipment owned or rented by the prime contractor, with or without operators. Such term does not include employees or equipment of a subcontractor or lower tier subcontractor, agents of the prime contractor, or any other assignees. The term may include payments for the costs of hiring leased employees from an employee leasing firm meeting all relevant Federal and State regulatory requirements. Leased employees may only be included in this term if the prime contractor meets all of the following conditions: (based on longstanding interpretation)

  - the prime contractor maintains control over the supervision of the day-to-day activities of the leased employees;

  - the prime contractor remains responsible for the quality of the work of the leased employees;

  - the prime contractor retains all power to accept or exclude individual employees from work on the project; and

  - the prime contractor remains ultimately responsible for the payment of predetermined minimum wages, the submission of payrolls, statements of compliance and all other Federal regulatory requirements.

- "Specialty Items" shall be construed to be limited to work that requires highly specialized knowledge, abilities, or equipment not ordinarily available in the type of contracting organizations qualified and expected to bid or propose on the contract as a whole and in general are to be limited to minor components of the overall contract. 23 CFR 635.102.

    - Pursuant to 23 CFR 635.116(a), the contract amount upon which the requirements set forth in paragraph (1) of Section VI is computed includes the cost of material and manufactured products which are to be purchased or produced by the contractor under the contract provisions.

    - Pursuant to 23 CFR 635.116(c), the contractor shall furnish

(a) a competent superintendent or supervisor who is employed by the firm, has full authority to direct performance of the work in accordance with the contract requirements, and is in charge of all construction operations (regardless of who performs the work) and (b) such other of its own organizational resources (supervision, management, and engineering services) as the contracting officer determines is necessary to assure the performance of the contract.

    - No portion of the contract shall be sublet, assigned or otherwise disposed of except with the written consent of the contracting officer, or authorized representative, and such consent when given shall not be construed to relieve the contractor of any responsibility for the fulfillment of the contract. Written consent will be given only after the contracting agency has assured that each subcontract is evidenced in writing and that it contains all pertinent provisions and requirements of the prime contract. (based on long- standing interpretation of 23 CFR 635.116).

    - The 30-percent self-performance requirement of paragraph

(1) is not applicable to design-build contracts; however, contracting agencies may establish their own self-performance requirements. 23 CFR 635.116(d).

  - SAFETY: ACCIDENT PREVENTION

This provision is applicable to all Federal-aid construction contracts and to all related subcontracts.

    - In the performance of this contract the contractor shall comply with all applicable Federal, State, and local laws governing safety, health, and sanitation (23 CFR Part 635). The contractor shall provide all safeguards, safety devices and protective equipment and take any other needed actions as it determines, or as the contracting officer may determine, to be reasonably necessary to protect the life and health of employees on the job and the safety of the public and to protect property in connection with the performance of the work covered by the contract. 23 CFR 635.108.

    - It is a condition of this contract, and shall be made a condition of each subcontract, which the contractor enters into pursuant to this contract, that the contractor and any subcontractor shall not permit any employee, in performance of the contract, to work in surroundings or under conditions which are unsanitary, hazardous or dangerous to his/her health or safety, as determined under construction safety and

health standards (29 CFR Part 1926) promulgated by the Secretary of Labor, in accordance with Section 107 of the Contract Work Hours and Safety Standards Act (40 U.S.C. 3704). 29 CFR 1926.10.

    - Pursuant to 29 CFR 1926.3, it is a condition of this contract that the Secretary of Labor or authorized representative thereof, shall have right of entry to any site of contract performance to inspect or investigate the matter of compliance with the construction safety and health standards and to carry out the duties of the Secretary under Section 107 of the Contract Work Hours and Safety Standards Act (40 U.S.C. 3704).

  - FALSE STATEMENTS CONCERNING HIGHWAY PROJECTS

This provision is applicable to all Federal-aid construction contracts and to all related subcontracts.

In order to assure high quality and durable construction in conformity with approved plans and specifications and a high degree of reliability on statements and representations made by engineers, contractors, suppliers, and workers on Federal- aid highway projects, it is essential that all persons concerned with the project perform their functions as carefully, thoroughly, and honestly as possible. Willful falsification, distortion, or misrepresentation with respect to any facts related to the project is a violation of Federal law. To prevent any misunderstanding regarding the seriousness of these and similar acts, Form FHWA-1022 shall be posted on each Federal-aid highway project (23 CFR Part 635) in one or more places where it is readily available to all persons concerned with the project:

18 U.S.C. 1020 reads as follows:

"Whoever, being an officer, agent, or employee of the United States, or of any State or Territory, or whoever, whether a person, association, firm, or corporation, knowingly makes any false statement, false representation, or false report as to the character, quality, quantity, or cost of the material used or to be used, or the quantity or quality of the work performed or to be performed, or the cost thereof in connection with the submission of plans, maps, specifications, contracts, or costs of construction on any highway or related project submitted for approval to the Secretary of Transportation; or

Whoever knowingly makes any false statement, false representation, false report or false claim with respect to the character, quality, quantity, or cost of any work performed or to be performed, or materials furnished or to be furnished, in connection with the construction of any highway or related project approved by the Secretary of Transportation; or

Whoever knowingly makes any false statement or false representation as to material fact in any statement, certificate, or report submitted pursuant to provisions of the Federal-aid Roads Act approved July 11, 1916, (39 Stat. 355), as amended and supplemented;

Shall be fined under this title or imprisoned not more than 5 years or both."

  - IMPLEMENTATION OF CLEAN AIR ACT AND FEDERAL WATER POLLUTION CONTROL ACT (42 U.S.C. 7606; 2

CFR 200.88; EO 11738)

This provision is applicable to all Federal-aid construction contracts in excess of $150,000 and to all related subcontracts. 48 CFR 2.101; 2 CFR 200.327.

By submission of this bid/proposal or the execution of this contract or subcontract, as appropriate, the bidder, proposer, Federal-aid construction contractor, subcontractor, supplier, or vendor agrees to comply with all applicable standards, orders or regulations issued pursuant to the Clean Air Act (42 U.S.C. 7401-7671q) and the Federal Water Pollution Control Act, as amended (33 U.S.C. 1251-1387). Violations must be reported to the Federal Highway Administration and the Regional Office of the Environmental Protection Agency. 2 CFR Part 200, Appendix II.

The contractor agrees to include or cause to be included the requirements of this Section in every subcontract, and further agrees to take such action as the contracting agency may direct as a means of enforcing such requirements. 2 CFR 200.327.

  - CERTIFICATION REGARDING DEBARMENT, SUSPENSION, INELIGIBILITY AND VOLUNTARY EXCLUSION

This provision is applicable to all Federal-aid construction contracts, design-build contracts, subcontracts, lower-tier subcontracts, purchase orders, lease agreements, consultant contracts or any other covered transaction requiring FHWA approval or that is estimated to cost $25,000 or more – as defined in 2 CFR Parts 180 and 1200. 2 CFR 180.220 and

1200.220.

    - Instructions for Certification – First Tier Participants:

- By signing and submitting this proposal, the prospective first tier participant is providing the certification set out below.

- The inability of a person to provide the certification set out below will not necessarily result in denial of participation in this covered transaction. The prospective first tier participant shall submit an explanation of why it cannot provide the certification set out below. The certification or explanation will be considered in connection with the department or agency's determination whether to enter into this transaction. However, failure of the prospective first tier participant to furnish a certification or an explanation shall disqualify such a person from participation in this transaction. 2 CFR 180.320.

- The certification in this clause is a material representation of fact upon which reliance was placed when the contracting agency determined to enter into this transaction. If it is later determined that the prospective participant knowingly rendered an erroneous certification, in addition to other remedies available to the Federal Government, the contracting agency may terminate this transaction for cause of default. 2 CFR 180.325.

- The prospective first tier participant shall provide immediate written notice to the contracting agency to whom this proposal is submitted if any time the prospective first tier participant learns that its certification was erroneous when submitted or has become erroneous by reason of changed circumstances. 2 CFR 180.345 and 180.350.

- The terms "covered transaction," "debarred," "suspended," "ineligible," "participant," "person," "principal," and "voluntarily excluded," as used in this clause, are defined in 2 CFR Parts 180, Subpart I, 180.900-180.1020, and 1200. “First Tier Covered Transactions” refers to any covered transaction between a recipient or subrecipient of Federal funds and a participant (such as the prime or general contract). “Lower Tier Covered Transactions” refers to any covered transaction under a First Tier Covered Transaction (such as subcontracts). “First Tier Participant” refers to the participant who has entered into a covered transaction with a recipient or subrecipient of Federal funds (such as the prime or general contractor). “Lower Tier Participant” refers any participant who has entered into a covered transaction with a First Tier Participant or other Lower Tier Participants (such as subcontractors and suppliers).

- The prospective first tier participant agrees by submitting this proposal that, should the proposed covered transaction be entered into, it shall not knowingly enter into any lower tier covered transaction with a person who is debarred, suspended, declared ineligible, or voluntarily excluded from participation in this covered transaction, unless authorized by the department or agency entering into this transaction. 2 CFR 180.330.

- The prospective first tier participant further agrees by submitting this proposal that it will include the clause titled "Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion-Lower Tier Covered Transactions," provided by the department or contracting agency, entering into this covered transaction, without modification, in all lower tier covered transactions and in all solicitations for lower tier covered transactions exceeding the $25,000 threshold. 2 CFR

180.220 and 180.300.

- A participant in a covered transaction may rely upon a certification of a prospective participant in a lower tier covered transaction that is not debarred, suspended, ineligible, or voluntarily excluded from the covered transaction, unless it knows that the certification is erroneous. 2 CFR 180.300; 180.320, and 180.325. A participant is responsible for ensuring that its principals are not suspended, debarred, or otherwise ineligible to participate in covered transactions. 2 CFR 180.335. To verify the eligibility of its principals, as well as the eligibility of any lower tier prospective participants, each participant may, but is not required to, check the System for Award Management website (https://www.sam.gov/). 2 CFR 180.300, 180.320, and 180.325.

- Nothing contained in the foregoing shall be construed to require the establishment of a system of records in order to render in good faith the certification required by this clause. The knowledge and information of the prospective participant is not required to exceed that which is normally possessed by a prudent person in the ordinary course of business dealings.

- Except for transactions authorized under paragraph (f) of these instructions, if a participant in a covered transaction knowingly enters into a lower tier covered transaction with a person who is suspended, debarred, ineligible, or voluntarily excluded from participation in this transaction, in addition to other remedies available to the Federal Government, the department or agency may terminate this transaction for cause or default. 2 CFR 180.325.

* * * * *

    - Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion – First Tier Participants:

- The prospective first tier participant certifies to the best of its knowledge and belief, that it and its principals:

  - Are not presently debarred, suspended, proposed for debarment, declared ineligible, or voluntarily excluded from participating in covered transactions by any Federal department or agency, 2 CFR 180.335;.

  - Have not within a three-year period preceding this proposal been convicted of or had a civil judgment rendered against them for commission of fraud or a criminal offense in connection with obtaining, attempting to obtain, or performing a public (Federal, State, or local) transaction or contract under a public transaction; violation of Federal or State antitrust statutes or commission of embezzlement, theft, forgery, bribery, falsification or destruction of records, making false statements, or receiving stolen property, 2 CFR 180.800;

  - Are not presently indicted for or otherwise criminally or civilly charged by a governmental entity (Federal, State or local) with commission of any of the offenses enumerated in paragraph (a)(2) of this certification, 2 CFR 180.700 and 180.800; and

  - Have not within a three-year period preceding this application/proposal had one or more public transactions (Federal, State or local) terminated for cause or default. 2 CFR 180.335(d).

  - Are not a corporation that has been convicted of a felony violation under any Federal law within the two-year period preceding this proposal (USDOT Order 4200.6 implementing appropriations act requirements); and

  - Are not a corporation with any unpaid Federal tax liability that has been assessed, for which all judicial and administrative remedies have been exhausted, or have lapsed, and that is not being paid in a timely manner pursuant to an agreement with the authority responsible for collecting the tax liability (USDOT Order 4200.6 implementing appropriations act requirements).

- Where the prospective participant is unable to certify to any of the statements in this certification, such prospective participant should attach an explanation to this proposal. 2 CFR 180.335 and 180.340.

* * * * *

    - Instructions for Certification - Lower Tier Participants:

(Applicable to all subcontracts, purchase orders, and other lower tier transactions requiring prior FHWA approval or estimated to cost $25,000 or more - 2 CFR Parts 180 and 1200). 2 CFR 180.220 and 1200.220.

- By signing and submitting this proposal, the prospective lower tier participant is providing the certification set out below.

- The certification in this clause is a material representation of fact upon which reliance was placed when this transaction was entered into. If it is later determined that the prospective lower tier participant knowingly rendered an erroneous certification, in addition to other remedies available to the Federal Government, the department, or agency with which

this transaction originated may pursue available remedies, including suspension and/or debarment.

- The prospective lower tier participant shall provide immediate written notice to the person to which this proposal is submitted if at any time the prospective lower tier participant learns that its certification was erroneous by reason of changed circumstances. 2 CFR 180.365.

- The terms "covered transaction," "debarred," "suspended," "ineligible," "participant," "person," "principal," and "voluntarily excluded," as used in this clause, are defined in 2 CFR Parts 180, Subpart I, 180.900 – 180.1020, and 1200. You may contact the person to which this proposal is submitted for assistance in obtaining a copy of those regulations. “First Tier Covered Transactions” refers to any covered transaction between a recipient or subrecipient of Federal funds and a participant (such as the prime or general contract). “Lower Tier Covered Transactions” refers to any covered transaction under a First Tier Covered Transaction (such as subcontracts). “First Tier Participant” refers to the participant who has entered into a covered transaction with a recipient or subrecipient of Federal funds (such as the prime or general contractor). “Lower Tier Participant” refers any participant who has entered into a covered transaction with a First Tier Participant or other Lower Tier Participants (such as subcontractors and suppliers).

- The prospective lower tier participant agrees by submitting this proposal that, should the proposed covered transaction be entered into, it shall not knowingly enter into any lower tier covered transaction with a person who is debarred, suspended, declared ineligible, or voluntarily excluded from participation in this covered transaction, unless authorized by the department or agency with which this transaction originated. 2 CFR 1200.220 and 1200.332.

- The prospective lower tier participant further agrees by submitting this proposal that it will include this clause titled "Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion-Lower Tier Covered Transaction," without modification, in all lower tier covered transactions and in all solicitations for lower tier covered transactions exceeding the $25,000 threshold. 2 CFR 180.220 and 1200.220.

- A participant in a covered transaction may rely upon a certification of a prospective participant in a lower tier covered transaction that is not debarred, suspended, ineligible, or voluntarily excluded from the covered transaction, unless it knows that the certification is erroneous. A participant is responsible for ensuring that its principals are not suspended, debarred, or otherwise ineligible to participate in covered transactions. To verify the eligibility of its principals, as well as the eligibility of any lower tier prospective participants, each participant may, but is not required to, check the System for Award Management website (https://www.sam.gov/), which is compiled by the General Services Administration. 2 CFR 180.300, 180.320, 180.330, and 180.335.

- Nothing contained in the foregoing shall be construed to require establishment of a system of records in order to render in good faith the certification required by this clause. The knowledge and information of participant is not required to exceed that which is normally possessed by a prudent person in the ordinary course of business dealings.

- Except for transactions authorized under paragraph e of these instructions, if a participant in a covered transaction knowingly enters into a lower tier covered transaction with a person who is suspended, debarred, ineligible, or voluntarily

excluded from participation in this transaction, in addition to other remedies available to the Federal Government, the department or agency with which this transaction originated may pursue available remedies, including suspension and/or debarment. 2 CFR 180.325.

* * * * *

    - Certification Regarding Debarment, Suspension, Ineligibility and Voluntary Exclusion--Lower Tier Participants:

- The prospective lower tier participant certifies, by submission of this proposal, that neither it nor its principals:

  - is presently debarred, suspended, proposed for debarment, declared ineligible, or voluntarily excluded from participating in covered transactions by any Federal department or agency, 2 CFR 180.355;

  - is a corporation that has been convicted of a felony violation under any Federal law within the two-year period preceding this proposal (USDOT Order 4200.6 implementing appropriations act requirements); and

  - is a corporation with any unpaid Federal tax liability that has been assessed, for which all judicial and administrative remedies have been exhausted, or have lapsed, and that is not being paid in a timely manner pursuant to an agreement with the authority responsible for collecting the tax liability. (USDOT Order 4200.6 implementing appropriations act requirements)

- Where the prospective lower tier participant is unable to certify to any of the statements in this certification, such prospective participant should attach an explanation to this proposal.

* * * * *

  - CERTIFICATION REGARDING USE OF CONTRACT FUNDS FOR LOBBYING

This provision is applicable to all Federal-aid construction contracts and to all related subcontracts which exceed

$100,000. 49 CFR Part 20, App. A.

    - The prospective participant certifies, by signing and submitting this bid or proposal, to the best of his or her knowledge and belief, that:

- No Federal appropriated funds have been paid or will be paid, by or on behalf of the undersigned, to any person for influencing or attempting to influence an officer or employee of any Federal agency, a Member of Congress, an officer or employee of Congress, or an employee of a Member of Congress in connection with the awarding of any Federal contract, the making of any Federal grant, the making of any Federal loan, the entering into of any cooperative agreement, and the extension, continuation, renewal, amendment, or modification of any Federal contract, grant, loan, or cooperative agreement.

- If any funds other than Federal appropriated funds have been paid or will be paid to any person for influencing or attempting to influence an officer or employee of any Federal agency, a Member of Congress, an officer or employee of Congress, or an employee of a Member of Congress in connection with this Federal contract, grant, loan, or

cooperative agreement, the undersigned shall complete and submit Standard Form-LLL, "Disclosure Form to Report Lobbying," in accordance with its instructions.

    - This certification is a material representation of fact upon which reliance was placed when this transaction was made or entered into. Submission of this certification is a prerequisite for making or entering into this transaction imposed by 31

U.S.C. 1352. Any person who fails to file the required certification shall be subject to a civil penalty of not less than

$10,000 and not more than $100,000 for each such failure.

    - The prospective participant also agrees by submitting its bid or proposal that the participant shall require that the language of this certification be included in all lower tier subcontracts, which exceed $100,000 and that all such recipients shall certify and disclose accordingly.

  - USE OF UNITED STATES-FLAG VESSELS:

This provision is applicable to all Federal-aid construction contracts, design-build contracts, subcontracts, lower-tier subcontracts, purchase orders, lease agreements, or any other covered transaction. 46 CFR Part 381.

This requirement applies to material or equipment that is acquired for a specific Federal-aid highway project. 46 CFR

381.7. It is not applicable to goods or materials that come into inventories independent of an FHWA funded-contract.

When oceanic shipments (or shipments across the Great Lakes) are necessary for materials or equipment acquired for a specific Federal-aid construction project, the bidder, proposer, contractor, subcontractor, or vendor agrees:

- To utilize privately owned United States-flag commercial vessels to ship at least 50 percent of the gross tonnage (computed separately for dry bulk carriers, dry cargo liners, and tankers) involved, whenever shipping any equipment, material, or commodities pursuant to this contract, to the extent such vessels are available at fair and reasonable rates for United States-flag commercial vessels. 46 CFR 381.7.

- To furnish within 20 days following the date of loading for shipments originating within the United States or within 30 working days following the date of loading for shipments originating outside the United States, a legible copy of a rated, ‘on-board’ commercial ocean bill-of-lading in English for each shipment of cargo described in paragraph (b)(1) of this section to both the Contracting Officer (through the prime contractor in the case of subcontractor bills-of-lading) and to the Office of Cargo and Commercial Sealift (MAR-620), Maritime Administration, Washington, DC 20590. (MARAD requires copies of the ocean carrier's (master) bills of lading, certified onboard, dated, with rates and charges. These bills of lading may contain business sensitive information and therefore may be submitted directly to MARAD by the Ocean Transportation Intermediary on behalf of the contractor). 46 CFR 381.7.

# ATTACHMENT A - EMPLOYMENT AND MATERIALS PREFERENCE FOR APPALACHIAN DEVELOPMENT HIGHWAY SYSTEM OR APPALACHIAN LOCAL ACCESS

ROAD CONTRACTS (23 CFR 633, Subpart B, Appendix B) This provision is applicable to all Federal-aid projects funded under the Appalachian Regional Development Act of 1965.

  - During the performance of this contract, the contractor undertaking to do work which is, or reasonably may be, done as on-site work, shall give preference to qualified persons who regularly reside in the labor area as designated by the DOL wherein the contract work is situated, or the subregion, or the Appalachian counties of the State wherein the contract work is situated, except:

    - To the extent that qualified persons regularly residing in the area are not available.

    - For the reasonable needs of the contractor to employ supervisory or specially experienced personnel necessary to assure an efficient execution of the contract work.

    - For the obligation of the contractor to offer employment to present or former employees as the result of a lawful collective bargaining contract, provided that the number of nonresident persons employed under this subparagraph (1c) shall not exceed 20 percent of the total number of employees employed by the contractor on the contract work, except as provided in subparagraph (4) below.

  - The contractor shall place a job order with the State Employment Service indicating (a) the classifications of the laborers, mechanics and other employees required to perform the contract work, (b) the number of employees required in each classification, (c) the date on which the participant estimates such employees will be required, and (d) any other pertinent information required by the State Employment Service to complete the job order form. The job order may be placed with the State Employment Service in writing or by telephone. If during the course of the contract work, the information submitted by the contractor in the original job order is substantially modified, the participant shall promptly notify the State Employment Service.

  - The contractor shall give full consideration to all qualified job applicants referred to him by the State Employment Service. The contractor is not required to grant employment to any job applicants who, in his opinion, are not qualified to perform the classification of work required.

  - If, within one week following the placing of a job order by the contractor with the State Employment Service, the State Employment Service is unable to refer any qualified job applicants to the contractor, or less than the number requested, the State Employment Service will forward a certificate to the contractor indicating the unavailability of applicants. Such certificate shall be made a part of the contractor's permanent project records. Upon receipt of this certificate, the contractor may employ persons who do not normally reside in the labor area to fill positions covered by the certificate, notwithstanding the provisions of subparagraph (1c) above.

  - The provisions of 23 CFR 633.207(e) allow the contracting agency to provide a contractual preference for the use of mineral resource materials native to the Appalachian region.

  - The contractor shall include the provisions of Sections 1 through 4 of this Attachment A in every subcontract for work which is, or reasonably may be, done as on-site work.

FEDERAL AID ATTACHMENT 9

This page left intentionally blank. See Federal Aid Project Attachment 8 – FHWA-1273

FEDERAL AID ATTACHMENT 10

FEDERAL MANDATORY EQUAL OPPORTUNITY LANGUAGE ON FEDERAL AID PROJECTS

(AUTHORITY SUBJECT TO 41 CFR 60-1.4 IN COMPLIANCE WITH 2 CFR PART 200 AND 2 CFR PART 200 APPENDIX II)

All Contractors regardless of the value of the contract shall have this mandatory clause with their subcontractors:

The Contractor/Subcontractor hereby agrees that it will incorporate or cause to be incorporated into any contract for construction work, or modification thereof, as defined in the regulations of the Secretary of Labor at 41 CFR Chapter 60, which is paid for in whole or in part with funds obtained from the Federal Government or borrowed on the credit of the Federal Government pursuant to a grant, contract, loan, insurance, or guarantee, or undertaken pursuant to any Federal program involving such grant, contract, loan, insurance, or guarantee, the following equal opportunity clause:

During the performance of this contract, the contractor agrees as follows:

(1)	The contractor will not discriminate against any employee or applicant for employment because of race, color, religion, sex, sexual orientation, gender identity, or national origin. The contractor will take affirmative action to ensure that applicants are employed, and that employees are treated during employment without regard to their race, color, religion, sex, sexual orientation, gender identity, or national origin. Such action shall include, but not be limited to the following:

Employment, upgrading, demotion, or transfer; recruitment or recruitment advertising; layoff or termination; rates of pay or other forms of compensation; and selection for training, including apprenticeship. The contractor agrees to post in conspicuous places, available to employees and applicants for employment, notices to be provided setting forth the provisions of this nondiscrimination clause.

(2)	The contractor will, in all solicitations or advertisements for employees placed by or on behalf of the contractor, state that all qualified applicants will receive consideration for employment without regard to race, color, religion, sex, sexual orientation, gender identity, or national origin.

(3)	The contractor will not discharge or in any other manner discriminate against any employee or applicant for employment because such employee or applicant has inquired about, discussed, or disclosed the compensation of the employee or applicant or another employee or applicant. This provision shall not apply to instances in which an employee who has access to the compensation information of other employees or applicants as a part of such employee's essential job functions discloses the compensation of such other employees or applicants to individuals who do not otherwise have access to such information, unless such disclosure is in response to a formal complaint or charge, in furtherance of an investigation, proceeding, hearing, or action, including an investigation conducted by the employer, or is consistent with the contractor's legal duty to furnish information.

(4) The contractor will send to each labor union or representative of workers with which he has a collective bargaining agreement or other contract or understanding, a notice to be provided advising the said labor union or workers' representatives of the contractor's commitments under this section, and shall post copies of the notice in conspicuous places available to employees and applicants for employment.

(5) The contractor will comply with all provisions of Executive Order 11246 of September 24, 1965, and of the rules, regulations, and relevant orders of the Secretary of Labor.

(6) The contractor will furnish all information and reports required by Executive Order 11246 of September 24, 1965, and by rules, regulations, and orders of the Secretary of Labor, or pursuant thereto, and will permit access to his books, records, and accounts by the administering agency and the Secretary of Labor for purposes of investigation to ascertain compliance with such rules, regulations, and orders.

(7) In the event of the contractor's noncompliance with the nondiscrimination clauses of this contract or with any of the said rules, regulations, or orders, this contract may be canceled, terminated, or suspended in whole or in part and the contractor may be declared ineligible for further Government contracts or federally assisted construction contracts in accordance with procedures authorized in Executive Order 11246 of September 24, 1965, and such other sanctions may be imposed and remedies invoked as provided in Executive Order 11246 of September 24, 1965, or by rule, regulation, or order of the Secretary of Labor, or as otherwise provided by law.

(8) The contractor will include the portion of the sentence immediately preceding paragraph (1) and the provisions of paragraphs (1) through (8) in every subcontract or purchase order unless exempted by rules, regulations, or orders of the Secretary of Labor issued pursuant to section 204 of Executive Order 11246 of September 24, 1965, so that such provisions will be binding upon each subcontractor or vendor. The contractor will take such action with respect to any subcontract or purchase order as the administering agency may direct as a means of enforcing such provisions, including sanctions for noncompliance:

Provided, however, that in the event a contractor becomes involved in, or is threatened with, litigation with a subcontractor or vendor as a result of such direction by the administering agency, the contractor may request the United States to enter into such litigation to protect the interests of the United States.

The applicant further agrees that it will be bound by the above equal opportunity clause with respect to its own employment practices when it participates in federally assisted construction work: Provided, That if the applicant so participating is a State or local government, the above equal opportunity clause is not applicable to any agency, instrumentality or subdivision of such government which does not participate in work on or under the contract.

The applicant agrees that it will assist and cooperate actively with the administering agency and the Secretary of Labor in obtaining the compliance of contractors and subcontractors with the equal opportunity clause and the rules, regulations, and relevant orders of the Secretary of Labor, that it will furnish the administering agency and the Secretary of Labor such information as they may require for the supervision of such compliance, and that it will otherwise assist the administering agency in the discharge of the agency's primary responsibility for securing compliance.

The applicant further agrees that it will refrain from entering into any contract or contract modification subject to Executive Order 11246 of September 24, 1965, with a contractor debarred from, or who has not demonstrated eligibility for, Government contracts and federally assisted construction contracts pursuant to the Executive Order and will carry out such sanctions and penalties for violation of the equal opportunity clause as may be imposed upon contractors and subcontractors by the administering agency or the Secretary of Labor pursuant to Part II, Subpart D of the Executive Order. In addition, the applicant agrees that if it fails or refuses to comply with these undertakings, the administering agency may take any or all of the following actions: Cancel, terminate, or suspend in whole or in part this grant (contract, loan, insurance, guarantee); refrain from extending any further assistance to the applicant under the program with respect to which the failure or refund occurred until satisfactory assurance of future compliance has been received from such applicant; and refer the case to the Department of Justice for appropriate legal proceedings.

FEDERAL AID ATTACHMENT 11

BYRD ANTI-LOBBYING CERTIFICATION

Pursuant to 31 USC 1352 and 49 CFR part 21, Contractor and all subcontractors are required to comply with this Attachment.  Contractor and all subcontractors shall be responsible to fill out Disclosure of Lobbying Activities Standard Form – LLL (as contained in this Attachment) and report it to the NJDOT Contract Compliance Unit for appropriate disclosure to the Federal Government.

All Contracts and subcontracts over $100,000 shall require the following mandatory language in every contract:

The undersigned certifies, to the best of his or her knowledge and belief, that:

1.	No federal appropriate funds have been paid or will be paid, by or on behalf of the undersigned, to any person for influencing or attempting to influence an officer or employee of an agency, a Member of Congress, an officer or employee of Congress, or an employee of a Member of Congress in connection with the awarding of any Federal contract, the making of any Federal grant, the making of any Federal loan, the entering into of any cooperative agreement, and the extension, continuation, renewal, amendment or modification of any Federal contract, grant, loan, or cooperative agreement.

2.	If any funds other than Federal appropriated funds have been paid or will be paid to any person for influencing or attempting to influence an officer or employee of any agency, a Member of Congress, or any employee of a Member of Congress in connection with this Federal contract, grant, loan or cooperative agreement, the undersigned shall complete and submit Disclosure of Lobbying Activities Standard Form – LLL (Federal Aid Attachment Form 11) in accordance with its instructions.

3.	The undersigned shall require that the language of this certification be included in the award documents for all subawards at all tiers (including subcontracts, subgrants, and contracts under grants, loans, and cooperative agreements) and that all subrecipients shall certify and disclose accordingly.

This certification is a material representation of fact upon which reliance was placed when the transaction was made or entered into.  Submission of this certification is a prerequisite for making or entering into this transaction imposed by section 1352, title 31, U.S. Code. Any person who fails to file the required certification shall be subject to a civil penalty of not that $10,000 and not more than $100,000 for each such failure.

Pt. 21, App. B	31 CFR Subtitle A (7–1–10 Edition)

APPENDIX B TO PART 21—DISCLOSURE FORM TO REPORT LOBBYING

Office of the Secretary of the Treasury	                                           Pt. 21, App. B

Pt. 21, App. B	31 CFR Subtitle A (7–1–10 Edition)